3GPP TSG-RAN WG4 Meeting#107                              R4-2309192

Incheon , KR, May 22 – May 26, 2023

Agenda item:	x.x.x

Source: 	ZTE Corporation

Title: 	Further discussion on RRM requirements for NCR-MT

Document for:	Approval

Introduction

In the latest RAN#97e meeting, the work item [RP-222673] on NR network-controlled repeater was approved after the completion of study item phase led by RAN1. In the last RAN4 meeting, we made good progress on NCR-MT RRM, however there are still some remaining issues left for further discussion. In this contribution, we want to share some further inputs on these remaining issues.

Issue 4-2: adaptive beamforming for NCR access link - Agreements - FFS whether to define RRM requirement for NCR-Fwd access link for beam configuration and switching latency. Issue 4-3: adaptive beamforming for NCR backhaul link - Agreement: - Not to define the RRM requirement for NCR-Fwd backhual link. Issue 5-1: BFD/BFR/RLM for NCR-MT - Agreements - For LA NCR-MT - Define RLM/BFD/BFR requirements - For WA NCR-MT - Do not define BFD/BFR requirements - FFS for RLM requirements.

Discussion

## 2.1. Adaptive beamforming fro NCR-Fwd access link

At RAN1#112 meeting, for the beam indication on NCR-Fwd access link, it was agreed to have following two approaches, one is DCI based aperiodic beam indication and another one is semi-persistent beam indication. The detailed procedures for the two approaches are shown in the following Figure 1/2 for the ease of further discussions.

Agreements: For the aperiodic beam indication, the reference of slot offset for each time resource is defined as the slot n+k where n refers to the slot that NCR-MT receive the DCI carrying the indication and: Option-2: k refers to the offset value [defined by NCR-MT capability and/or declared by vendor]. Note: This k is different from the parameter used to define the Slot offset for the time resource. For the semi-persistent beam indication, the indicated information should be applied starting from the first slot that is after slot n+3N_slot^(subframe u), where n refers to the slot that NCR-MT would transmit HARQ-ACK information corresponding to the PDSCH carrying the activation command and u is the SCS configuration for the channel carrying the HARQ-ACK information.

As shown in the following figure 1/2, the latency for the execution of beam indication at the access link has already been clearly defined by RAN1 regardless of DCI based indication or MAC-CE based indication, from the procedure wise, it should be quite clear from RAN1 perspective without any further clarification or specification work in RAN4. The concerns might be coming from the verification of execution of NCR-Fwd access link. This might be similar as DCI based TCI switching if target TCI is known, from the procedure wise, it should be clearly defined in RAN1/RAN2, however there are still RRM requirement defined in RAN for the verification of UE behavior by RRM test case design.

Figure 1. DCI based beam indication for NCR-Fwd access link

Figure 2. MAC CE based beam indication for NCR-Fwd access link

In the following figure 3, we try to figure out the measurement setup for NCR-Fwd access link beam switching based on beam indication from NCR-MT link if necessary. As we discussed in last RAN4 meeting, since the beam switching is occurring on the access link instead of control link, therefore measurement equipment is supposed to detect or measure the beam at the NCR-Fwd access link. Since the OTA signal forwarded by NCR-Fwd part is transparent without any further regeneration as relay or IAB, then how the measurement equipment is supposed to detect the beams from access link should be one essential issue to be further analyzed. At least from our understanding, if switched beams consist of both SSB and PDCCH/PDSCH at the access link, then SSS-based RSRP measurement or EIRP measurement could one measurement metric to verify whether beam switch has already been executed, or switched beam consist of only PDCCH/PDSCH signals, then EIRP measurement could one measurement metric to verify whether beam switch has already been executed.

However as we know that, weighting factor for each beams at the access link is implementation based without any specific beam directions specified in 3GPP and the mapping between weighting factor for each beam or each beam direction and beam indication index is also up to the implementation. Therefore it should be quite difficult to define the EIRP measurement difference between light orange @t0 and gray beams@t1 in the RRM test case. In addition, if two switched beams are quite close to each other and EIRP measurement uncertainty defined in TS38.106 is around 2.2dB for FR1 and 3.4dB for FR2 in normal conditions, then it would be also quite difficult to verify the beam switching executed.

Figure 3. measurement setup for NCR-Fwd access link beam indication measurement

In short, based on the above analysis especially from the verification of execution of beam switching at NCR-Fwd access link, it seems quite difficult to figure out the universally applicable test parameter configurations for it since both mapping between beam index and each beam direction and weighting factor for each beams are implementation based.

Proposal 1: not to define the RRM requirement for NCR-Fwd access link.

## 2.2. BFD/BFR/RLM for NCR-MT

Issue 5-1: BFD/BFR/RLM for NCR-MT - Agreements - For LA NCR-MT - Define RLM/BFD/BFR requirements - For WA NCR-MT - Do not define BFD/BFR requirements - FFS for RLM requirements.

In last RAN4 meeting, we reached the consensus on the necessity of defining RLM/BFD/BFR requirement for LA NCR-MT, however there were no discussion yet on RLM/BFD/BFR requirement itself for LA NCR-MT. Compared with Rel-16 IAB-MT, RLM/BFD/BFR requirement was defined corresponding in clause 12.3 of TS38.174 without considering DRX configuration and it’s also reasonable to reuse the RLM/BFD/RLM requirement of IAB-MT for NCR-MT. In addition, from our understanding, DRX configuration for NCR-MT could be still allowed in practice which is slightly different from IAB-MT for backhaul link transmission.

Proposal 2: to reuse the existing IAB-MT RLM/BFD/BFR requirement in TS38.174 clause 12.3 as baseline and further consider the DRX configuration for NCR-MT.

In addition, in last RAN4 meeting, it is still FFS for RLM requirement of WA NCR-MT. We could understand that RLM is one important functionality or feature of NCR-MT to maintain the link connection between NCR-MT and donor gNB which is also quite important for connection between NCR-Fwd link and donor gNB and make sure that it could return back to normal connection after the RLF happened, however it has already been agreed that not to define RRC re-establishment for WA NCR-MT. In other words, from the procedure wise or form latency requirement perspective, it would not make much sense to define the partial requirements for whole procedure (e.g. after RLF, to trigger the RRC re-establishment).  Based on above analysis, we propose not to define the RLM requirement for WA NCR-MT.

Proposal 3: not to define the RLM requirement for WA NCR-MT,

Conclusions

In this contribution, we want to share further views on NCR-MT from the RRM perspective and proposals are made as following:

Proposal 1: not to define the RRM requirement for NCR-Fwd access link.

Proposal 2: to reuse the existing IAB-MT RLM/BFD/BFR requirement in TS38.174 clause 12.3 as baseline and further consider the DRX configuration for NCR-MT.

Proposal 3: not to define the RLM requirement for WA NCR-MT,

References

- RP-222673, New WID on NR network-controlled repeater, ZTE, Sanechips, Approved

- RP-222323, TR 38.867 for NR Network-Controlled repeater, RAN#97-e

- R4-2217263,WF on RRM requirements for NCR, ZTE, approved.

- R4-2217471,WF on NCR RF requirements, ZTE, approved.

- R4-2220708,WF on NR Network-controlled Repeaters RRM requirements, ZTE

- R4-2220076,Topic summary for [105][230] NR_netcon_repeater_RRM

- R4-2219845,Discussion on RRM impacts for NR network-controlled repeaters, Huawei.

- R4-2303257, WF on NCR-MT RRM requirements, ZTE, approved.

- R4-2306360, WF on NR Network-controlled Repeaters RRM requirements, ZTE, approved.