3GPP TSG-RAN WG4 Meeting #107	R4-2309220

Incheon, Korea, 22 – 26 May 2023

Agenda Item:	9.8.4

Source:	Ericsson

Title:	Discussions on RRM requirements for IoT NTN enhancements

Document for:	Discussions

# 1	Introduction

A new Rel-18 work item (WI) on IoT NTN enhancements was approved [1]. RAN4 continued to discuss the RRM impact of this WI which were summarized in [2].  In this contribution, we provide discuss and provide our view on those.

# 2	Discussions

Neighbour cell measurements

Following agreement was reached in [1]:

#### Issue 2: Requirements which not applicable due to lack of neighbor cell ephemeris The requirements which not applicable due to lack of neighbour cell ephemeris in R17 could be applied in R18, the requirements defined during R17 can be the starting point for further discussion.

RAN2 has agreed to introduce the neighbor cell/satellite information in a new SIBx. The RAN4 agreement from last meteing states that the earlier agreement can be used as baseline. Our view is that the requirements which were first agreed but later excluded due to lack of neighbour cell ephemeris in R17 could be reused directly.

RAN4 to confirm reusing of neighbour cell measurement requirements which were agreed in the first IoT NTN WI.

At last meeting it as agreed to include a scaling factor for multiple NGSO satellites for RRC IDLE measurements (Tdetect, Tevaluate, Tmeasure) [1]. Whether to introduce similar scaling factor for CONNECTED mode neighbour cell measurements remains open. Firstly it is noted that the CONNECTED mode intra-frequency neighbour cell measurements are defined assuming a fixed measurement time period. But the inter-frequency neighbour cell measurement requirements are defined assuming best-effort method, i.e. that the UE measures during vacant time occasions. In both cases, it is noted that the total cell identification delay is scaled with the number of frequency layers. Following the same principle as in IDLE mode, the intra-frequency measurement requirements can be scaled for the multiple NGSO satellites. However, for inter-frequency measurements can be discussed after a conclusion on inter-frequency measurements is reached.

Introduce scaling factor for multiple NGSO satellites for NB intra-freuquency neighbour cell measurement in connected mode. FFS for inter-frequency measurements.

RAN2 has made following agreement [RAN2#120]:

- For NB-IoT we support a trigger for neighbour cell measurements based on T-service (in the quasi-Earth fixed case) (this does not preclude anything for eMTC discussion) - At least for NB-IoT NTN, for quasi-earth fixed cells, UE shall start intra/inter frequency measurement in connected mode before the t-Service if present. The exact time to start measurements in connected mode before t-Service can be left to UE implementation” (can revisit if we agree other proposal based on neighbour cell coverage) - RAN2 will not specify the condition of stopping UE measurement before t-Service

From RAN4 measurement perspective, there is benefit for the UE to start the measurements at least certain time before start of t-Service in order to have a complete measurement before entering the discontinuous coverage. Otherwise, if the measurements are started too late (e.g. very close in time to t-Start) then the UE may not have sufficient time to collect the number of samples needed to perform the averaging. In this case, the UE may need to discard the already started measurements and collected samples since it could not take all samples before the t-Service was reached. This may have considerable impact on the UE power consumption and may lead to unnecessary power consumption which should be avoided especially given that these are IoT devices with long battery life. Hence, it is essential to start the neighbour cell measurements at least time T1 before start of t-Service, where T1 corresponds to number of samples (i.e. measurement period from legacy requirements) needed to perform the intended measurements.

The NB-IoT UE shall start the intra/inter-frequency measurements at least before time T1 before start of t-Service, where T1 is the time required to perform one measurement.

Improved GNSS operations

Regarding the objective on improved GNSS operations, RAN4 agreed to postpone the discussions on GNSS re-acquisition until more progress has been reached in other WGs. In order to reacquire GNSS, the gNB can configure the UE with aperiodic measurement gap, where the gap length is based on the reported GNSS position fix time duration. Different gap lengths are being discussed in RNA1, for example 1,2,3,7,13,19,25, X] seconds. The RAN4 impact is not yet clear since related discussions affecting RAN4 are still ongoing in RAN1, e.g. the starting time of the gap, gap length, etc. Therefore, we propose to postpone the RAN4 discussions on the GNSS improvement objective until more progress is reached in other WGs.

RAN4 to postpone the discussions on GNSS re-acquisition until more progress has been reached on this objective in other WGs.

# 3	Summary

In this contribution we have discussed and provided our on the open issues on RRM impact identified at last meeting. Based on the discussions, we have made following observations and proposals:

- RAN4 to confirm reusing of neighbour cell measurement requirements which were agreed in the first IoT NTN WI.

Introduce scaling factor for multiple NGSO satellites for NB intra-freuquency neighbour cell measurement in connected mode. FFS for inter-frequency measurements.

The NB-IoT UE shall start the intra/inter-frequency measurements at least before time T1 before start of t-Service, where T1 is the time required to perform one measurement.

RAN4 to postpone the discussions on GNSS re-acquisition until more progress has been reached on this objective in other WGs.

# References

RP-223519, “Revised WID on IoT NTN enhancements”, MediaTek Inc.

R4-2306387, “WF on IoT_NTN_enh”, Moderator (MediaTek inc.)