3GPP TSG-RAN WG4 Meeting #107	R4-2309222

Incheon, Korea, 22 – 26 May, 2023

Agenda Item:	9.7.6

Source:	Ericsson

Title:	Discussions on NTN IoT RRM performance requirements

Document for:	Discussion

# 1	Introduction

Good progress was reached at previous RAN4 meeting on the performance part of the IoT NTN WI as most of the test cases were completed. However, some new test cases were agreed and work split was agreed. There are still few open issues remaining to be resolved which are discussed in this contribution.

# 2	Discussion

PHR reporting

New power headroom reporting requirements were introduced in section 9.1.23 in TS 36.133 for NB-IoT in LTE. The motivations for introducing the new requirements were the multiple coverage levels and new power class that the NB-IoT UE could operate in, and mainly considering the limited number of reportable values compared to number of reportable values of a legacy LTE UE. More specifically, the NB-IoT UE has only 2 bits that can be used for reporting the power headroom, and this is considerably fewer values compared to the 6 bits used for PHR reporting by the legacy LTE UEs or cat-M UEs. This means that the NB-IoT UE can report only 4 different values while the cat-M or LTE UE can report up to 64 values for power headroom which gives more flexibility to adjust the reporting values to better reflect the actual power usage in the UE.

Table 1 shows the PHR reporting values in legacy requirements for normal coverage [2]:

Table 1: Power headroom report mapping for UE category NB1 UEs not supporting enhanced PHR [31] when the enhanced coverage level 0 is selected during random access procedure [17]

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | -54 PH 5 |
| POWER_HEADROOM_1 | 5 PH 8 |
| POWER_HEADROOM_2 | 8 PH 11 |
| POWER_HEADROOM_3 | PH ≥ 11 |

One important difference between GEO and LEO is in the altitude they are operating at. The geostationary satellites operate around 36 000 km above the earth while the LEO satellites operate at altitude less than 1 200 km above the earth. This affects the path loss as studied and shown in the technical report in TR 38.821, see Table 2 below.

Table 2: Link budgets results

| Case | Transmission mode | Frequency [GHz] | TX: EIRP [dBm] | RX: G/T [dB/T] | Bandwidth [MHz] | Free space path loss [dB] | Atmospheric loss [dB] | Shadow fading margin [dB] | Scintillation Loss [dB] | Polarization loss [dB] | Additional losses [dB] | CNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SC1 | DL | 20.0 | 96.0 | 15.9 | 400.0 | 210.6 | 1.2 | 0.0 | 1.1 | 0.0 | 0.0 | 11.6 |
|  | UL | 30.0 | 76.2 | 28.0 | 400.0 | 214.1 | 1.1 | 0.0 | 1.1 | 0.0 | 0.0 | 0.5 |
| SC2 | DL | 20.0 | 91.2 | 15.9 | 133.3 | 210.6 | 1.2 | 0.0 | 1.1 | 0.0 | 0.0 | 11.6 |
|  | UL | 30.0 | 76.2 | 28.0 | 133.3 | 214.1 | 1.1 | 0.0 | 1.1 | 0.0 | 0.0 | 5.2 |
| SC3 | DL | 20.0 | 93.0 | 15.9 | 200.0 | 210.6 | 1.2 | 0.0 | 1.1 | 0.0 | 0.0 | 11.6 |
|  | UL | 30.0 | 76.2 | 28.0 | 200.0 | 214.1 | 1.1 | 0.0 | 1.1 | 0.0 | 0.0 | 3.5 |
| SC4 | DL | 2.0 | 103.8 | -31.6 | 30.0 | 190.6 | 0.2 | 3.0 | 2.2 | 0.0 | 0.0 | 0.0 |
|  | UL | 2.0 | 23.0 | 19.0 | 0.4 | 190.6 | 0.2 | 3.0 | 2.2 | 0.0 | 0.0 | -10.9 |
| SC5 | DL | 2.0 | 99.0 | -31.6 | 10.0 | 190.6 | 0.2 | 3.0 | 2.2 | 0.0 | 0.0 | 0.0 |
|  | UL | 2.0 | 23.0 | 19.0 | 0.4 | 190.6 | 0.2 | 3.0 | 2.2 | 0.0 | 0.0 | -10.9 |
| SC6 | DL | 20.0 | 60.0 | 15.9 | 400.0 | 179.1 | 0.5 | 0.0 | 0.3 | 0.0 | 0.0 | 8.5 |
|  | UL | 30.0 | 76.2 | 13.0 | 400.0 | 182.6 | 0.5 | 0.0 | 0.3 | 0.0 | 0.0 | 18.4 |
| SC7 | DL | 20.0 | 55.2 | 15.9 | 133.3 | 179.1 | 0.5 | 0.0 | 0.3 | 0.0 | 0.0 | 8.5 |
|  | UL | 30.0 | 76.2 | 13.0 | 133.3 | 182.6 | 0.5 | 0.0 | 0.3 | 0.0 | 0.0 | 23.1 |
| SC8 | DL | 20.0 | 57.0 | 15.9 | 200.0 | 179.1 | 0.5 | 0.0 | 0.3 | 0.0 | 0.0 | 8.5 |
|  | UL | 30.0 | 76.2 | 13.0 | 200.0 | 182.6 | 0.5 | 0.0 | 0.3 | 0.0 | 0.0 | 21.4 |
| SC9 | DL | 2.0 | 78.8 | -31.6 | 30.0 | 159.1 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | 6.6 |
|  | UL | 2.0 | 23.0 | 1.1 | 0.4 | 159.1 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | 2.8 |
| SC10 | DL | 2.0 | 74.0 | -31.6 | 10.0 | 159.1 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | 6.6 |
|  | UL | 2.0 | 23.0 | 1.1 | 0.4 | 159.1 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | 2.8 |
| SC11 | DL | 20.0 | 66.0 | 15.9 | 400.0 | 184.5 | 0.5 | 0.0 | 0.3 | 0.0 | 0.0 | 9.1 |
|  | UL | 30.0 | 76.2 | 13.0 | 400.0 | 188.0 | 0.5 | 0.0 | 0.3 | 0.0 | 0.0 | 13.0 |
| SC12 | DL | 20.0 | 61.2 | 15.9 | 133.3 | 184.5 | 0.5 | 0.0 | 0.3 | 0.0 | 0.0 | 9.1 |
|  | UL | 30.0 | 76.2 | 13.0 | 133.3 | 188.0 | 0.5 | 0.0 | 0.3 | 0.0 | 0.0 | 17.8 |
| SC13 | DL | 20.0 | 63.0 | 15.9 | 200.0 | 184.5 | 0.5 | 0.0 | 0.3 | 0.0 | 0.0 | 9.1 |
|  | UL | 30.0 | 76.2 | 13.0 | 200.0 | 188.0 | 0.5 | 0.0 | 0.3 | 0.0 | 0.0 | 16.0 |
| SC14 | DL | 2.0 | 84.8 | -31.6 | 30.0 | 164.5 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | 7.2 |
|  | UL | 2.0 | 23.0 | 1.1 | 0.4 | 164.5 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | -2.6 |
| SC15 | DL | 2.0 | 80.0 | -31.6 | 10.0 | 164.5 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | 7.2 |
|  | UL | 2.0 | 23.0 | 1.1 | 0.4 | 164.5 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | -2.6 |
| SC16 | DL | 20.0 | 88.0 | 15.9 | 400.0 | 210.4 | 0.8 | 0.0 | 0.5 | 0.0 | 0.0 | 4.8 |
|  | UL | 30.0 | 76.2 | 20.0 | 400.0 | 213.9 | 0.7 | 0.0 | 0.5 | 0.0 | 0.0 | -6.3 |
| SC17 | DL | 20.0 | 83.2 | 15.9 | 133.3 | 210.4 | 0.8 | 0.0 | 0.5 | 0.0 | 0.0 | 4.8 |
|  | UL | 30.0 | 76.2 | 20.0 | 133.3 | 213.9 | 0.7 | 0.0 | 0.5 | 0.0 | 0.0 | -1.6 |
| SC18 | DL | 20.0 | 85.0 | 15.9 | 200.0 | 210.4 | 0.8 | 0.0 | 0.5 | 0.0 | 0.0 | 4.8 |
|  | UL | 30.0 | 76.2 | 20.0 | 200.0 | 213.9 | 0.7 | 0.0 | 0.5 | 0.0 | 0.0 | -3.3 |
| SC19 | DL | 2.0 | 98.3 | -31.6 | 30.0 | 190.4 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | -5.2 |
|  | UL | 2.0 | 23.0 | 14.0 | 0.4 | 190.4 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | -15.7 |
| SC20 | DL | 2.0 | 93.5 | -31.6 | 10.0 | 190.4 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | -5.2 |
|  | UL | 2.0 | 23.0 | 14.0 | 0.4 | 190.4 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | -15.7 |
| SC21 | DL | 20.0 | 52.0 | 15.9 | 400.0 | 179.1 | 0.5 | 0.0 | 0.3 | 0.0 | 0.0 | 0.5 |
|  | UL | 30.0 | 76.2 | 5.0 | 400.0 | 182.6 | 0.5 | 0.0 | 0.3 | 0.0 | 0.0 | 10.4 |
| SC22 | DL | 20.0 | 47.2 | 15.9 | 133.3 | 179.1 | 0.5 | 0.0 | 0.3 | 0.0 | 0.0 | 0.5 |
|  | UL | 30.0 | 76.2 | 5.0 | 133.3 | 182.6 | 0.5 | 0.0 | 0.3 | 0.0 | 0.0 | 15.1 |
| SC23 | DL | 20.0 | 49.0 | 15.9 | 200.0 | 179.1 | 0.5 | 0.0 | 0.3 | 0.0 | 0.0 | 0.5 |
|  | UL | 30.0 | 76.2 | 5.0 | 200.0 | 182.6 | 0.5 | 0.0 | 0.3 | 0.0 | 0.0 | 13.4 |
| SC24 | DL | 2.0 | 72.8 | -31.6 | 30.0 | 159.1 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | 0.6 |
|  | UL | 2.0 | 23.0 | -4.9 | 0.4 | 159.1 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | -3.2 |
| SC25 | DL | 2.0 | 68.0 | -31.6 | 10.0 | 159.1 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | 0.6 |
|  | UL | 2.0 | 23.0 | -4.9 | 0.4 | 159.1 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | -3.2 |
| SC26 | DL | 20.0 | 58.0 | 15.9 | 400.0 | 184.5 | 0.5 | 0.0 | 0.3 | 0.0 | 0.0 | 1.1 |
|  | UL | 30.0 | 76.2 | 5.0 | 400.0 | 188.0 | 0.5 | 0.0 | 0.3 | 0.0 | 0.0 | 5.0 |
| SC27 | DL | 20.0 | 53.2 | 15.9 | 133.3 | 184.5 | 0.5 | 0.0 | 0.3 | 0.0 | 0.0 | 1.1 |
|  | UL | 30.0 | 76.2 | 5.0 | 133.3 | 188.0 | 0.5 | 0.0 | 0.3 | 0.0 | 0.0 | 9.8 |
| SC28 | DL | 20.0 | 55.0 | 15.9 | 200.0 | 184.5 | 0.5 | 0.0 | 0.3 | 0.0 | 0.0 | 1.1 |
|  | UL | 30.0 | 76.2 | 5.0 | 200.0 | 188.0 | 0.5 | 0.0 | 0.3 | 0.0 | 0.0 | 8.0 |
| SC29 | DL | 2.0 | 78.8 | -31.6 | 30.0 | 164.5 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | 1.2 |
|  | UL | 2.0 | 23.0 | -4.9 | 0.4 | 164.5 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | -8.6 |
| SC30 | DL | 2.0 | 74.0 | -31.6 | 10.0 | 164.5 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | 1.2 |
|  | UL | 2.0 | 23.0 | -4.9 | 0.4 | 164.5 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | -8.6 |
| NOTE: The link budget calculations including CIR and CINR results contributed by the companies are available in [24]. |  |  |  |  |  |  |  |  |  |  |  |  |

The table 2 shows the free space pathloss for GEO and LEO satellites for the scenarios defined in table Table 6.1.1.1-9 in [3]. These scenarios are also included in the Appendix of this contribution for convenient reading. SC1-SC5, SC 16- shows the results GEO with different assumptions. SC6-SC15 and SDC21-SC30 shows the results for LEO with different assumptions.  Higher pathloss for GEO compared to LEO can be observed from these results. The UE may therefore use more uplink transmit power for reaching a satellite at 36 00 km above the earth compared to reaching a satellite above 16 000 km above the earth. This means, for a UE with only 4 reportable values, it is reasonable to assume that more negative values are needed to report for GEO compared to LEO since the UE is expected to operate at higher power for GEO compared to LEO.

Observation 1: GEO satellites operate around 36 000 km above the earth while LEO satellites operates around 1 200 km above the earth.

- For UE in normal coverage managed by GEO with only 4 reportable values, the PHR reporting values are defined as shown in Table 3.

Table 3: Power headroom report mapping for UE category NB1 UEs for a cell (e.g. cell1) served by GEO satellite during random access procedure

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | -54 PH -10 |
| POWER_HEADROOM_1 | -10 PH -2 |
| POWER_HEADROOM_2 | -2 PH 6 |
| POWER_HEADROOM_3 | PH ≥ 6 |

- For UE in enhanced coverage managed by GEO with only 4 reportable values, the PHR reporting values are defined as shown in Table 4.

Table 4 Power headroom report mapping for UE category NB1 UEs not supporting enhanced PHR when the enhanced coverage level other than 0 is selected during random access procedure and UE is served by GEO satellite

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | -54 PH -30 |
| POWER_HEADROOM_1 | -30 PH -20 |
| POWER_HEADROOM_2 | -20 PH 6 |
| POWER_HEADROOM_3 | PH ≥ 6 |

Test for segmented UL transmissions

Need for introducing test for segmented UL transmissions was discussed at last meeting with following outcome [1]:

#### Issue 4: Test for Segmented UL Transmissions - Proposal 1: Do not define dedicated UE initial transmission timing test cases for UL segmented transmission. - Proposal 2: Define UE initial transmission timing test for UL segmented transmission, the test parameter in A.7.1.18 Test 1 can be the starting point. - In test case, set Nsegmentprecompensation (npusch-TxDuration) as 32ms, and Ngapprecompensation (ntn-SegmentedPrecompensationGaps) as 2 slots. - For the test requirement of UL segmented transmission, - The test system shall verify that the adjustment step size and the adjustment rate shall be according to requirements specified in core requirements until the UE transmit timing offset is withinNTA_Ref+NTA-offset+NTA,adjcommon+NTA,adjUE×Ts ± Te_NTN respective to the first detected path (in time) of DL SSB. The adjustment will be only performed at the start of a transmission segment boundary. - For the first transmission in each segment, the test system shall verify that the UE transmit timing offset stays within NTA_Ref+NTA-offset+NTA,adjcommon+NTA,adjUE×Ts ± Te_NTN of the first detected path of DL SSB.

RAN4 did not introduce any new requirements for the segmented UL transmission, instead new conditions were captured in the specification. Therefore, it is not necessary to verify the requirements, however at the same time it can be noted that the legacy test cases will not verify the UE behavior with segmented UL transmissions. We are therefore fine to compromise to proposal 2, i.e. to define new test case for UE initial transmission timing with UL segmented transmission.

- Define UE initial transmission timing test for UL segmented transmission.

# Conclusion

In this contribution we have provided discussed the performance requirements for NB-IoT and eMTC NTN. Based on the discussions, we have made following agreements and observations:

Observation 1: GEO satellites operate around 36 000 km above the earth while LEO satellites operates around 1 200 km above the earth.

- For UE in normal coverage managed by GEO with only 4 reportable values, the PHR reporting values are defined as shown in Table 3.

Table 3: Power headroom report mapping for UE category NB1 UEs for a cell (e.g. cell1) served by GEO satellite during random access procedure

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | -54 PH -10 |
| POWER_HEADROOM_1 | -10 PH -2 |
| POWER_HEADROOM_2 | -2 PH 6 |
| POWER_HEADROOM_3 | PH ≥ 6 |

- For UE in enhanced coverage managed by GEO with only 4 reportable values, the PHR reporting values are defined as shown in Table 4.

Table 4 Power headroom report mapping for UE category NB1 UEs not supporting enhanced PHR when the enhanced coverage level other than 0 is selected during random access procedure and UE is served by GEO satellite

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | -54 PH -30 |
| POWER_HEADROOM_1 | -30 PH -20 |
| POWER_HEADROOM_2 | -20 PH 6 |
| POWER_HEADROOM_3 | PH ≥ 6 |

- Define UE initial transmission timing test for UL segmented transmission.

# References

R4-2303311, "WF on NB-IoT/eMTC RRM requirements for NTN ", Moderator (MediaTek inc.)

TS 36.133 V18.1.0

TR 38.821 V16.2.0

# Appendix

Table 6.1.1.1-9: List of calibration study cases

| Case | Satellite orbit | Satellite parameter set | Central beam elevation | Terminal | Frequency Band | Frequency/ Polarization Reuse |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | GEO | Set 1 | 45 deg | VSAT | Ka-band | Option 1 |
| 2 | GEO | Set 1 | 45 deg | VSAT | Ka-band | Option 2 |
| 3* | GEO | Set 1 | 45 deg | VSAT | Ka-band | Option 3 |
| 4* | GEO | Set 1 | 45 deg | Handheld | S-band | Option 1 |
| 5* | GEO | Set 1 | 45 deg | Handheld | S-band | Option 2 |
| 6 | LEO-600 | Set 1 | 90 deg | VSAT | Ka-band | Option 1 |
| 7 | LEO-600 | Set 1 | 90 deg | VSAT | Ka-band | Option 2 |
| 8* | LEO-600 | Set 1 | 90 deg | VSAT | Ka-band | Option 3 |
| 9 | LEO-600 | Set 1 | 90 deg | Handheld | S-band | Option 1 |
| 10 | LEO-600 | Set 1 | 90 deg | Handheld | S-band | Option 2 |
| 11* | LEO-1200 | Set 1 | 90 deg | VSAT | Ka-band | Option 1 |
| 12* | LEO-1200 | Set 1 | 90 deg | VSAT | Ka-band | Option 2 |
| 13* | LEO-1200 | Set 1 | 90 deg | VSAT | Ka-band | Option 3 |
| 14 | LEO-1200 | Set 1 | 90 deg | Handheld | S-band | Option 1 |
| 15 | LEO-1200 | Set 1 | 90 deg | Handheld | S-band | Option 2 |
| 16** | GEO | Set 2 | 45 deg | VSAT | Ka-band | Option 1 |
| 17** | GEO | Set 2 | 45 deg | VSAT | Ka-band | Option 2 |
| 18** | GEO | Set 2 | 45 deg | VSAT | Ka-band | Option 3 |
| 19** | GEO | Set 2 | 45 deg | Handheld | S-band | Option 1 |
| 20** | GEO | Set 2 | 45 deg | Handheld | S-band | Option 2 |
| 21** | LEO-600 | Set 2 | 90 deg | VSAT | Ka-band | Option 1 |
| 22** | LEO-600 | Set 2 | 90 deg | VSAT | Ka-band | Option 2 |
| 23** | LEO-600 | Set 2 | 90 deg | VSAT | Ka-band | Option 3 |
| 24** | LEO-600 | Set 2 | 90 deg | Handheld | S-band | Option 1 |
| 25** | LEO-600 | Set 2 | 90 deg | Handheld | S-band | Option 2 |
| 26** | LEO-1200 | Set 2 | 90 deg | VSAT | Ka-band | Option 1 |
| 27** | LEO-1200 | Set 2 | 90 deg | VSAT | Ka-band | Option 2 |
| 28** | LEO-1200 | Set 2 | 90 deg | VSAT | Ka-band | Option 3 |
| 29** | LEO-1200 | Set 2 | 90 deg | Handheld | S-band | Option 1 |
| 30** | LEO-1200 | Set 2 | 90 deg | Handheld | S-band | Option 2 |
| NOTE 1: no star = 1st priority, * = second priority scenario, ** = third priority scenario NOTE 2: Only 1st priority cases will be considered for calibration phase 1 |  |  |  |  |  |  |