3GPP TSG-RAN4 Meeting #107	        R4-2309268 Incheon, South Korea, 22 – 26 May 2023

Source: 	Qualcomm Inc.

Title: 	eRedCap UE RF impacts

Agenda Item:	8.32.2

Document for:	Approval

# Introduction

Work item for Enhanced support of reduced capability NR devices have been agreed in [1]. In this contribution we discuss the UE RF specification impacts based on the work item objectives as well as agreements reached in RAN1.

# Discussion

Power saving/energy efficiency enhancements

- Enhanced eDRX in RRC_INACTIVE (>10.24s) [RAN2, RAN3, RAN4]

  - Note that this objective requires SA2, CT1 and CT4 involvement

Complexity/cost reduction

- Further reduced UE complexity in FR1 [RAN1, RAN2, RAN4]

  - UE BB bandwidth reduction

    - 5 MHz BB bandwidth only for PDSCH (for both unicast and broadcast) and PUSCH, with 20 MHz RF bandwidth for UL and DL

    - The other physical channels and signals are still allowed to use a BWP up to the 20 MHz maximum UE RF+BB bandwidth.

    - Support additional separate early indication(s) [RAN1, RAN2]

  - UE peak data rate reduction

    - Relaxation of the constraint (vLayers·Qm·f ≥ 4) for peak data rate reduction

    - The relaxed constraint is, e.g., 1 (instead of 4).

    - The parameters (vLayers, Qm, f) can be as in Rel-17 RedCap.

  - Both 15 kHz SCS and 30 kHz SCS are supported.

  - Aim to define at most one Rel-18 RedCap UE type for further UE complexity reduction.

  - The existing UE capability framework is used, and changes to capability signalling are specified only if necessary. By default, all UE capabilities applicable to a Rel-17 RedCap UE are applicable unless otherwise specified.

Notes:

- The work defined as part of this WI is not to overlap with LPWA use cases.

- Coexistence with non-RedCap UEs and Rel-17 RedCap UEs should be ensured.

- This WI considers all applicable duplex modes unless otherwise specified.

Check in RAN#99 regarding:

- Whether UE peak data rate reduction for UE is limited only with UE BB bandwidth reduction or standalone

Work item objectives from [1] are reproduced below.

In RAN#99, further proposal was endorsed in [2] as captured below

Rel-18 eRedCap UE capable of 20MHz + PR1 and Rel-18 eRedCap UE capable of BW3/PR3 + PR1 are designed/targeted to same peak data rate, i.e., 10Mbps

Note 1: Peak data rate of "Rel-18 eRedCap: UE capable of 20MHz + PR1" and "Rel-18 eRedCap: UE capable of BW3/PR3 + PR1" is same including unicast and broadcast respectively.

Note 2: PRB processing capability of "Rel-18 eRedCap: UE capable of 20MHz + PR1" is not limited to "25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS" and it corresponds to PRB size corresponding to 20 MHz.

Note 3: The only difference between "Rel-18 eRedCap: UE capable of 20MHz + PR1" and "Rel-18 eRedCap: UE capable of BW3/PR3 + PR1" is Note 2 and vLayers·Qm·f   in order to have the same peak rate.

Note 4: The initial access procedure of Rel-18 eRedCap UE capable of 20MHz + PR1 is realized by following:

- Same as Rel-18 eRedCap UE capable of BW3/PR3 + PR1

It can be seen from the WID objectives previous agreements reached so far, that there is limited amount specification impact for RF. RF channel bandwidth can extend to 20 MHz similar to Rel-17 RedCap and the main aspect is to capture the baseband bandwidth reduction impacts.

Compared to the discussion in previous RAN4 meeting on the UEs with bandwidth reduction, based on RAN agreement [2] second UE type is now considered without any bandwidth reduction. That is, both Tx and Rx can cover 20 MHz channel bandwidth. However, both UE types are restricted to peak data rate of 10 Mbps.

For UE without bandwidth reduction, Rel-17 requirements can be applied as long as peak data rate does not exceed 10 Mbps. For Tx requirements we consider this to mean that core RF requirements can cover all RB allocations similar to Rel-17, but in test case selection in RAN5 only cases which do not exceed the Tput rate are used. For Rx requirements reference channels cannot exceed 10 Mbps peak rate.

Proposal 1: Apply Rel-17 RedCap RF requirements for UE without bandwidth reduction with the restriction that peak data rate does not exceed 10 Mbps.

For DL requirements, Rel-17 RedCap and also regular NR face situations in the field where DL signal is not covering the full RF channel. However, the reference channels used for specifying minimum requirements assume fully allocated channel. Some work is needed to select appropriate DL RB allocation for eRedCap requirements for the bandwidth restricted UE. The need for new refsens requirements is captured also in agreed WF from RAN4#106 [3]. It should be noted that this work is needed only for the UE type with bandwidth reduction.

Observation 1: Further work for UL and DL PRB placement in reference sensitivity and blocking tests is required only for the UE type with bandwidth reduction.

In Figure 1 we have drawn examples of possible RB allocations.

Figure 1: Example possibilities for RB allocations for DL requirements

Assuming contiguous PRB allocation is used, the requirements can become more stringent than Rel-17 RedCap requirements in case PRBs at channel edge are used. When Rx RBs would be allocated as close as possible to Tx, Tx non-linearities and noise will have stronger impact. The magnitude of the impact is operating band specific.

If the channel edge furthest away from Tx is used, similar effect is present due to the non-linearities and noise caused by the Rx interferer. Therefore neither case 1 nor 2 in Figure 1 can be used without detailed analysis on receiver impacts.

A third option would be to place the RBs in the middle of RF channel. This has the following benefits:

- Tx-Rx frequency separation stays the same as in Rel-17

- No excessive leakage from neither jammer signals nor own Tx is present

Therefore, we suggest to adopt case 3. Despite this, some further reference sensitivity degradation may need to be allowed for bands with narrow duplex spacing. For example, band n71 uses only 20 RB UL allocation in refsens test.

Proposal 2: Consider placing both Tx and Rx RB allocations in the middle of the RF channel in RF requirements for bandwidth limited UE type

Proposal 3: Rx requirements of eRedCap UE shall not be more stringent than RedCap or NR UE requirements. This shall be taken into account in Tx and Rx RB frequency location in receiver tests.

# Conclusions

In this contribution eRedCap RF specification impact was discussed. Following observations and proposals were made.

Proposal 1: Apply Rel-17 RedCap RF requirements for UE without bandwidth reduction with the restriction that peak data rate does not exceed 10 Mbps.

Observation 1: Further work for UL and DL PRB placement in reference sensitivity and blocking tests is required only for the UE type with bandwidth reduction.

Proposal 2: Consider placing both Tx and Rx RB allocations in the middle of the RF channel in RF requirements for bandwidth limited UE type

Proposal 3: Rx requirements of eRedCap UE shall not be more stringent than RedCap or NR UE requirements. This shall be taken into account in Tx and Rx RB frequency location in receiver tests.

# References

[1]	RP-223544, Revised WID on Enhanced support of reduced capability NR devices, Ericsson

[2]	RP-230778, Proposal for PR1 in eRedCap, Moderator (CMCC)

[3]	R4-2303565. WF on eRedCap UE RF requirements