3GPP TSG-RAN WG4 Meeting # 106-Bis-e 	R4-2309284

# e-Meeting, Apr. 2023

Source: Qualcomm Incorporated

Title: 	 On UE RF requirements for 2AoA FR2 DL MIMO

Agenda item:	8.8.2.2

Document for:	Discussion

# Introduction

A previous agreement [1] identified a baseline requirement metric, and an alternative in addition to the baseline. We compare the two metrics to investigate if the baseline assumption can benefit from improvement. We also propose to establish scope of applicability of UE RF requirements as they apply to UE orientation, DL polarization.

# Discussion

## Metric for UE RF requirement

Agreements in a previous WF [1] identified the baseline RF metric (option1), but further study on both options was encouraged. For convenient reference, the options are reproduced below as:

- For a specific angular separation between 2 TRPs and a specific UE orientation under standardized DL power level which is equal between 2 TRPs, the result at each test point is constructed based on two AoA pairs containing that test point, i.e., AoA+ pair and AoA- pair. Overall result (probability to support 2TRP DL) is by averaging regional results.

- The result is averaged over all tested AoA pairs. Weighting is products of the area weights for both AoAs in the pair (AW = sinθ1.sinθ2) and the average is further normalized by the sum of the weights (used in the integration.

These schemes are fundamentally similar in establishing a probability metric but differ in how the outcomes over all tested AoAs are combined into the final quantification. We evaluated both options for two UE types (modules on opposite faces and modules on adjacent faces), and two representative orientations (module coverage covering polar region of grid or not). The ‘grid’ referenced here is the UE-centred lat-long grid that is defined by the positioner axes. ‘Front’ refers to the north polar region, ‘top, left, right’ refer to different equatorial regions of the grid.

Since neither option enjoys evaluation over the whole N2 space, neither can correctly capture the overall probability to support 2TRP DL for the general case (N2 case). Both are modified from equivalent general formulations in some way to account for the significant pruning of the evaluation space due to TE constraints. The key to evaluating the options is to understand what decisions have been taken to compensate for the loss of AoA pairs from the ideal N2 formulation and how it manifests in results. Some early observations are below in table 2.1-1:

|  | Metric option 1 | Metric option 2 | Notes |
| --- | --- | --- | --- |
| Weighting applied in case of full N2 grid | Sin2 weighting | Sin2 weighting | Both methods are equivalent when a full suite of test AoAs are available (no TE constraint) |
| Compensation when not all AoA pairs are available to test (for example due to TE constraints) | Weighting is adjusted to reflect much reduced test AoA pair set compared to N2 case | No compensation, same weights as N2 case reused regardless of reduction in number of AoA pairs | Option 2 determines weights for one sample space of AoA pairs but uses them for a different sample space for the metric. Could this incorrect use of weights lead to a problem? |
| sum of weights AKA ‘normalizing factor’ | Sum of weights =1 for each stage. | Varies | no physical reason for variation as a function of AoA separation |

To evaluate these options, it is necessary to establish a ‘gold standard’ or reference that is insulated from sensitivity to UE orientation. For this purpose, we turned to the technique described previously [2,3] (‘three-axis positioner’). This technique was not pursued as an enhancement to the positioner for pragmatic reasons for the industry, but it suffers no such challenges in the simulation domain. To summarize, this technique establishes a test AoA set that enables evaluation agnostic to UE orientation (‘bias-free set’). These reference performance trends are represented by dashed lines in the figures below. The RF conditions comply with agreements in [7] and [1] including the sensitivity calibration condition for ~11 dB gain drop for low bands (n257,n58, n261). The results are tabulated below in table 2.1-2:

| Module location in the positioner centric grid (*) | Module coverage relative to positioner centric grid (*) | Probability to support 2 TRP DL |
| --- | --- | --- |
| Front and Back Legacy ant. gain: |  |  |
| Left and Right Legacy ant. gain: |  |  |
| Front and Right Legacy ant. gain: |  |  |
| Top and Right Legacy ant. gain: |  |  |

The configurations in the table above require some notion of how the module coverages are oriented in the system. Descriptions like ‘sources in the XZ plane’ and ‘P0/P60/Pxx’ were useful for discussion, but it remains strongly reliant on tribal knowledge, and is perhaps exact only for some TE implementations. There is also  strong interdependence between agreed TE constraints and the requirement derivation process. Any requirement derived with the agreed TE constraints would not be valid for a UE tested with a suitable 3-axis positioner and scan strategy [2,3], for example.

For a complete RF requirement specification therefore, the geometry and the TE constraints require a formal and general description. The necessary definitions must be defined in the standard itself or alternatively, in an external TR. Regardless of where this is captured, it is necessary to establish a coordinate system that compactly captures the constraints imposed by the TE system. We also note that any future TR for the WI would only be an internal TR, like TR38.810 and TR38.871 (for the test SI associated with this WI) and unsuitable to reference in the standard.

Proposal 1: The final requirement includes a description of the TE constraints assumed during the requirement derivation process to reflect their interdependence.

The positioner centric grid (*) referenced in the table above are the grid points of a spherical coordinate system that is fixed to the DUT-holder in the positioner, centered at the DUT location, and is defined by the axes of the positioner motion in relation to the stationary sources. Since the polar axis of this system remains aligned to the corresponding axis in the positioner that causes the azimuth motion, it is not attached directly to the DUT (i.e., the polar axis can emerge from any face of the UE). The grid points of this positioner centric spherical coordinate system are the locations where a stationary source in the agreed TE system would appear to a UE placed in the DUT positioner and the agreed constant step size scan is implemented. Since these grid points (source locations during 3D spherical scan) remain invariant of UE orientation, this reference system is well suited for analysis operations in context of the agreed TE constraints. The mathematical formulations of the requirement metric captured in [1] are only meaningful in this reference system because the AoAs are defined without consideration for the UE orientation and only depend on positioner motion. The grid along with an example UE orientation relative to this grid is captured in figure 2.1-1 (excerpted from TR38.871).

Figure 2.1-1: Reference coordinate system for calculating probability metric.

Observation 1: The agreed baseline requirement metric in WF R4-2306604 [1] was derived using a positioner-centric spherical coordinate system grid that is fixed to the DUT-holder in the positioner, centered at the DUT location, and is defined by the positioner axes in relation to the stationary sources. It is invariant of UE orientation in the DUT holder.

Analysis of projected performance in the table above shows that for some UEs (those with modules on opposite faces), option 2 shows odd behavior that must be first investigated and explained before it can be considered for this feature:

- Option 2 may show un-natural peaks in performance trends, see the solid green trend curve for a UE with ‘front and back’ modules. See green dotted highlight in first row in table 2.1-2. ‘Un-natural’ is judged relative to the data from the bias-free test AoA set. The option 1 metric does not suffer from this problem.

- The overall probability result can exceed 50% by a significant amount for option 2, see green dotted highlight in second row in table 2.1-2. This is unnatural for a UE that is calibrated to have only 50% probability to be ‘in-coverage’ per legacy requirements. The option 1 metric does not suffer from this problem.

- The most problematic observation for option 2 and perhaps the root cause of the problems above is that it introduces yet another source of sensitivity to UE orientation. In the figures below (2⁰ step size), option 1 and option 2 are compared on the bias-free reference test AoA set for both example UE types. Option 1 curves show good alignment regardless of UE orientation as expected. Option 2 however  shows significant deviation even with the benefit of the bias-free reference AoA pair set, see dotted green highlight in figure 2.1-2.

Figure 2.1-2: sin2 weighting introduces a new mechanism of sensitivity to UE orientation.

Observation 2: sin2 weighting (option 2 metric in WF R4-2306604) introduces a new mechanism of sensitivity to UE orientation and therefore not preferred.

Option 2’s problems are likely due to a weighting scheme that does not correctly compensate for the loss due to TE constraints of test AoA pairs from the general N2 case, as identified as a note in table 2.1-1.

Proposal 2: The baseline metric (option 1 from WF R4-2306604) is retained as the UE requirement metric.

## Optimal orientation for UEs

For this section we again reference the positioner-centric spherical coordinate system (*) defined in section 2.1.

We previously established that it was possible to institute a no-block strategy even with multiple DL sources [4] if the sources were arranged such that they were always contained in planes of constant  in the positioner centric grid. i.e. absolute probe locations are not necessary, and perhaps counterproductive. ‘No-block’ in this context means preserving line of sight between the sources and DUT. There is a second type of blockage to consider, which is module blockage. Module blockage is a condition when the module of the UE in the positioner is such that its coverage areas are partially or fully blocked by the positioner mechanism. For example, for a TE positioner implementation that uses roll-on-azimuth (see figure 2.3-1), the ‘south pole’ of the positioner centric spherical coordinate system is blocked by the roll motor. It would not be useful for the UE to be oriented such that one of its modules is shrouded by the roll motor, because this proximity between the module and the TE mechanism can impact results in an unpredictable way.

Turn-table axis

Legacy source location

Second source candidate locations

Figure 2.3-1: Example 2-axis UE positioner and source locations for both TRPs to fall on the same grid from the positioner (and UE)’s perspective. Example positioner representation excerpted from R4-2219852.

Roll axis

Separately, we have shown that measured overall probability of the UE has strong dependence on its orientation, a characteristic that is true for multiple different UE implementations [3]. In that study, the following conclusions were drawn:

- UE with identical modules on adjacent faces: the measured probability is maximized when one of the module coverage regions coincides with the polar region of the positioner centric grid. To avoid module blockage, the polar region for this purpose would be the north pole (module covering ‘front’).

- UEs had identical modules on opposite faces: While this type of UE also benefits in the simulation domain when one of the modules covers a polar region, this configuration causes module blockage in a realistic TE: when one module faces the north pole (‘front’), the second module faces the south (‘back’) and is blocked by the roll motor. This type of UE is therefore forced to only cover equatorial regions, where lower measured probability is expected.

Observation 3: The UE orientation that maximizes the probability metric for a UE with identical modules on adjacent faces is one in which one of the modules covers the polar region of the positioner centric grid.

Examples of the UE orientations referenced in observation 5 are those that place the individual module coverage regions in one of 4 principal configurations: front+(top/right/left/bottom).

Observation 4: The only UE orientation that dodges module blockage for a UE with identical modules on opposite faces is one in which neither of the modules covers the polar region of the positioner centric grid.

Examples of the UE orientations referenced in observation 6 are those that place the individual module coverage regions in one of 2 principal configurations: top+bottom or left+right.

If the ‘declared UE orientation’ method is agreed, then for consistency the requirement derivation process must also ensure the proposals position the UE optimally in the simulation phase. Some additional information surrounding the module location relative to the positioner centric grid  would be useful to ensure this. Alternatively, the UE can be evaluated in multiple different orientations, so the optimal orientation is self-evident.

Proposal 3: If the ‘declared UE orientation’ method is agreed, the simulation phase must include multiple different UE orientations, so the optimal orientation is self-evident and data pertaining to that orientation can be chosen for the requirement specification process.

## Requirement applicability to AoA separations

Projected performance trends in section 2.1 demonstrated that they depend strongly on the module coverage areas in relation to the polar regions of the reference coordinate system defined by the positioner axes. If one fixed test orientation is forced on all UEs, it follows that some UE implementations may register as inferior even though their real-world performance may be equivalent or even superior to other UEs.

Observation 5: From a UE implementation perspective, the limitations of the agreed TE architecture are incompatible with not allowing a UE to dictate its preferred orientation during test.

Even if UEs were allowed a preferred orientation, data from section 2.1 show that some UE implementations have modest performance for all AoA separations while others have strong performance for wider separations and poor performance for narrower separations.  From a network perspective, the first characteristic may be useful for co-located mTRP arrays at the gNB, while the latter may be useful for non-co-located TRPs. During the Rel-16 work phase, it was specifically agreed to not discriminate between those two deployment cases.

Observation 6: Wide and narrow AoA separations correspond to non-co-located and co-located TRPs, scenarios that were equally considered for the Rel-16 FR2 inter-band CA WI.

Moreover, forcing a requirement on all implementations at multiple AoA separations would mean requirements at each AoA separation, the requirement derivation process would have to consider all reasonable UE implementations, including those that do not work well for that AoA separation. The candidate values would be spread, and the MU would be defined by the worst configuration. In contrast, the declaration method allows limiting the candidate values to the best value for each UE.

Observation 7: If a UE must be specified over multiple AoA separations, the requirement would be defined by the worst-performing implementation at each AoA separation.

Finally, there is the test time argument against testing at multiple AoA separations. In conclusions, to minimize MU and test time, and for consistency with the decision to stay with the legacy positioner:

Proposal 4: The UE RF requirement is specified at the UE’s declared pair of preferred AoA separation and preferred orientation in the positioner. The UE RF requirement is derived assuming the UE orientation that yields the best metric value.

## DUT legacy spherical coverage

Sometimes it is preferable to use actual module coverage data, but in conjunction with the spherical coverage calibration condition, we may end up with an over-specified set of boundary conditions. (i.e cannot meet both REFSENS and spherical coverage gain drop at the same time.). In these cases, the baseline assumption in a previous WF [3] suggests to ‘scale antenna gain’, see excerpt below (figure 2.4-1).

There are several ways to achieve this scaling: the first method would apply to both modules equally (symmetric scaling of gains in way to ‘tune’ the spherical coverage gain drop), and the second method would reduce the gain of one of the modules relative to the other (asymmetric scaling). Asymmetric scaling makes the sensitivity worse for one module over the other, so it can impact the probability metric significantly, see tables 2.4-2 and -3.

| Left and Right modules, legacy antenna gain | Module coverage relative to positioner centric grid (*) | Probability to support 2 TRP DL (option 1 metric [1]+ OR combining) |
| --- | --- | --- |
| Equal modules |  |  |
| Asymm. scaled modules (9.0 dB reduction in one) |  |  |

| Left and Front modules, legacy antenna gain | Module coverage relative to positioner centric grid (*) | Probability to support 2 TRP DL (option 1 metric [1]+ OR combining) |
| --- | --- | --- |
| Equal modules |  |  |
| Asymm. scaled modules (7.5 dB reduction in one) |  |  |

Note that all the UEs above are calibrated to legacy spherical coverage, but the probability metrics can be vastly different between UEs with equal modules and UEs with asymmetric modules. This difference may be tolerable for small differences ( 2 dB or less) across modules, but this type of implementation is not suitable for 2 TRP reception if the asymmetric gain split is larger across the modules. Larger gain splits become necessary if the individual module performance is good to begin with, and large compensation is required to degrade it to match the standards expectation.

Observation 8: Non-identical modules can result in significantly degraded 2TRP reception performance even when the UE is calibrated to just meet legacy sensitivity requirements (REFSENS + spherical coverage).

If actual module data is preferred to be used, option 2 or 3 in the same WF [1, 2.6] can be used instead. Option 2 and 3 in the same WF are aligned, although option 3 is a more general treatment.

Proposal 5: RAN4 to clarify that ‘scale the antenna gain’ method to make UE align with both peak EIS and spherical coverage applies only to schemes that degrade all modules equally. If this is not possible option 3 of 2.6 in WF R4-2306604 is used instead (‘Meet one calibration condition as long as the other is met or exceeded’).

The asymmetric adjustments considered above demonstrate that it may be useful to retain the ‘network benefit’ sanity check option in the future to ensure that the UE variant being considered is meaningful for the 2TRP reception feature.

Proposal 6: There is no need to preclude a network benefit concept as a possible sanity check option to use for various UE proposals during the requirement specification stage.

## DL test polarizations

It is recognized that some UEs may have asymmetric performance across polarization angles perhaps due to differing H- and V- antenna gains. Legacy practice is to use an averaging method across multiple DL polarizations. Extending this argument, some form of averaging results obtained from different orthogonal DL polarizations seems reasonable.

Recall previous agreement that the UE RF requirement would assume a worst-case polarization combination incident on the UE. The analysis to support this agreement is accomplished by accruing the power at each module separately for each TRP and calculating the SINR for the TRP assigned to the module by the UE algorithm. Any chosen DL combination during verification would produce equal or better results because it is not practical to determine the worst-case polarizations during verification. Consequently, the UE can be verified with either  and  or  and  DL pol. configurations (shown as TRP1 DL pol., TRP2 DL pol.) without any additional consideration. Unless more debate is justified, we propose to identify one of them for the verification step,  say  and  :

Proposal 7: The UE RF requirement is defined as the average (arithmetic mean) probability of supporting 2TRP DL for two DL polarization test conditions: the first condition is when the DL polarizations at both TRPs are ‘’ and the second condition is when they are ‘’.

For the UE RF requirement condition, note that neither proposal for DL polarizations (- or - DL pols.) has any direct bearing on how the DL power from each TRP is distributed among the H- and V- pol receivers in each module. The H- and V- power split in each module depends on the vector projections of the incoming rank 1 DL signal along the H and V axes of the module.  This determination must be done at each grid point not only because of the changing incident angle from the UE’s perspective, but also to mimic the changing unit vector directions (transmitted pol. directions) as a function of grid position.

Observation 9: HH-only or VV-only reception cannot be used as a proxy for either DL transmitted polarization pairing (- or -).

## Requirement applicability to various channel bandwidths

Unlike a sensitivity measurement  procedure, this requirement presents ‘sufficient DL power’ to the UE and measures if the feature is supported. Ability to support 2TRP reception in the UE RF context in turn depends on the perceived SINR associated with the two TRPs. The SINR behavior in any dual-pol receiver with differing DL bandwidths is below:

- The DL power from the desired TRP is linked to the spherical coverage requirement which scales with channel bandwidth.

- The noise in any dual-pol receiver too scales with the channel bandwidth.

- The interference power from the other TRP scales with the bandwidth of the interferer.

For the case where both TRPs transmit their DL over identical channel bandwidths, a UE’s SINR statistics would remain invariant of different channel bandwidths of test.

Proposal 8: The UE RF requirement for 2AoA Rx feature is specified to be the same for all channel bandwidths.

# Conclusions

Unless otherwise noted, observations and proposals below apply to UEs that support either mDCI or sDCI schemes.

Proposal 1: The final requirement includes a description of the TE constraints assumed during the requirement derivation process to reflect their interdependence.

Observation 1: The agreed baseline requirement metric in WF R4-2306604 [1] was derived using a positioner-centric spherical coordinate system grid that is fixed to the DUT-holder in the positioner, centered at the DUT location, and is defined by the positioner axes in relation to the stationary sources. It is invariant of UE orientation in the DUT holder.

Observation 2: sin2 weighting (option 2 metric in WF R4-2306604) introduces a new mechanism of sensitivity to UE orientation and therefore not preferred.

Proposal 2: The baseline metric (option 1 from WF R4-2306604) is retained as the UE requirement metric.

Observation 3: The UE orientation that maximizes the probability metric for a UE with identical modules on adjacent faces is one in which one of the modules covers the polar region of the positioner centric grid.

Observation 4: The only UE orientation that dodges module blockage for a UE with identical modules on opposite faces is one in which neither of the modules covers the polar region of the positioner centric grid.

Proposal 3: If the ‘declared UE orientation’ method is agreed, the simulation phase must include multiple different UE orientations, so the optimal orientation is self-evident and data pertaining to that orientation can be chosen for the requirement specification process.

Observation 5: From a UE implementation perspective, the limitations of the agreed TE architecture are incompatible with not allowing a UE to dictate its preferred orientation during test.

Observation 6: Wide and narrow AoA separations correspond to non-co-located and co-located TRPs, scenarios that were equally considered for the Rel-16 FR2 inter-band CA WI.

Observation 7: If a UE must be specified over multiple AoA separations, the requirement would be defined by the worst-performing implementation at each AoA separation.

Proposal 4: The UE RF requirement is specified at the UE’s declared pair of preferred AoA separation and preferred orientation in the positioner. The UE RF requirement is derived assuming the UE orientation that yields the best metric value.

Observation 8: Non-identical modules can result in significantly degraded 2TRP reception performance even when the UE is calibrated to just meet legacy sensitivity requirements (REFSENS  + spherical coverage).

Proposal 5: RAN4 to clarify that ‘scale the antenna gain’ method to make UE align with both peak EIS and spherical coverage applies only to schemes that degrade all modules equally. If this is not possible option 3 of 2.6 in WF R4-2306604 is used instead (‘Meet one calibration condition as long as the other is met or exceeded’).

Proposal 6: There is no need to preclude a network benefit concept as a possible sanity check option to use for various UE proposals during the requirement specification stage.

Proposal 7: The UE RF requirement is defined as the average (arithmetic mean) probability of supporting 2TRP DL for two DL polarization test conditions: the first condition is when the DL polarizations at both TRPs are ‘’ and the second condition is when they are ‘’.

Observation 9: HH-only or VV-only reception cannot be used as a proxy for either DL transmitted polarization pairing (- or -).

Proposal 8: The UE RF requirement for 2AoA Rx feature is specified to be the same for all channel bandwidths.

# References

[1]	R4-2306604, ‘WF on FR2-1 UERF requirements for FR2 multi-Rx’, vivo et al., RAN4#106-Bis-e, Apr. 2023

[2]		R4-2304679, ‘Discussion on test method for FR2 multi-Rx UE’, Qualcomm, RAN4#106-Bis-e, Apr. 2023

[3]	R4-2304603, ‘On UE RF requirements for 2AoA FR2 DL MIMO’, Qualcomm, RAN4#106-Bis-e, Apr. 2023

[4]	R4-2300087, ‘On test method for FR2 multi-Rx UE’, Qualcomm, RAN4#106, Athens, GR, Feb-Mar. 2022