3GPP TSG-RAN WG4 Meeting # 107	R4-2309295

# Incheon, KR, May 22 – May 26, 2023

Source: 	Skyworks Solutions, Inc.

Title: 	Input on CA_n5-n8 options and MSD

Agenda Item:	7.30.2.1	CA configuration of CA_n5-n8	[NR_700800900_combo_enh-Core]

Document for:	Approval

# Introduction

In RAN4#106b-e, the different options and related MSDs were discussed in [2] for CA_n5-n8 and further agreements were captured in [1]. In this contribution, we provide our inputs regarding architecture and options to be specified together with the related MSD evaluations.

# Discussion

A number of agreements were captured in way forward [1] notably on:

- Delta T and Delta R

- 1UL cross-band MSD for dedicated restricted range n8 UL filter.

Still, there are pending issues that were captured in following agreements:

Way forward/Agreement on band combinations to be specified

- In single band operation, the entire band n5 and n8 frequency ranges must be supported

- In CA_n5-n8 operation the following options are discussed for sepcification:

  - pending RAN2 input on option 2:

    - CA_n5-n8 1UL n5, the entire band n5 and n8 frequency ranges must be supported. This configuration is needed as fallback of the next two 2UL cases.

    - CA_n5-n8 2UL non-simultaneous n5Rx/n8Tx as optional, the entire band n5 and n8 frequency ranges must be supported

    - CA_n5-n8 2UL CA_n5-n8R as optional, the entire band n5 and n8 frequency ranges must be supported except for n8ULthat is restricted to 904-915MHz

  - Further down selection is discussed at next meeting

Way forward/Agreement on capturing band combinations to be specified and potential signaling needs: following options are discussed as potential signaling solutions:

- Option 1: use different BCS with notes clarifying the UL restrictions based on agreed cases to be specified

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Bandwidth combination set |
| --- | --- | --- | --- |
| CA_n5A-n8A | n5X | n5 | 0 |
|  |  | n8 |  |
|  | CA_n5A-n8AY | n5 | 1 |
|  |  | n8 |  |
|  | CA_n5A-n8AZ | n5 | 2 |
|  |  | n8 |  |
| NOTE X: In BCS0, n8 UL is not supported NOTE Y: In BCS1, the concurrent operation of band n5 DL and band n8 UL is not supported NOTE Z: In BCS2 the band n8 support is restricted to the 904-915MHz frequency range in UL |  |  |  |

  - Channel bandwidth support per UL configuration case is FFS (some case can be above 10MHz CBW)

- Option 2: Optional capability is needed for option 2: CA_n5-n8 2UL non-simultaneous n5Rx/n8Tx

- Option 3: Frequency restriction or New frequency band (for n8) is needed for option 3: CA_n5-n8 2UL with n8 restricted to 904-915MHz

- Option 4: Other are not precluded

- Options are not necessarily exclusive

Way forward/Agreement

-	Following cross-band MSD test point is evaluated for future meetings based on existing band n5/n26 duplexer band n8 rejection performance and n5 transmitter noise floor and specified if justified

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n8 | 844 | 10 | 15 | 25 (RBstart=27) | 951.5 | 5 | TBD | >ACLR2 |

<Way forward>:

-	The Following near miss IMD3 MSD test point is evaluated for future meetings and specified if justified

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode | of IMD |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n5-n8 | n5 | 846.5 | 5 | 25 | 891.5 | N/A | FDD | N/A |
|  | n8 | 906.5 | 5 | 25 | 951.5 | TBD | FDD | Near miss IMD3 |

- A note is added to clarify applicability to the case where n8 UL only supports 904-915MHz frequency range

In this contribution, we provide our input on some of the above aspects.

## Options for CA_n5-n8 combination support

If RAN4 still needs to wait for the RAN2 answer on whether an n8 DL configuration associated with a CA_n5-n8 UL configuration is feasible and can be signaled, it is still useful to discuss how to distinguish the different CA_n5-n8 supported in the specification and any down selection.

Out of the 3 cases, the CA_n5-n8 DL with n5 UL only is a subset of the two options discussed for 2UL support and can be supported with minimal RF front end impact from legacy UEs. Also, the 2UL cases should be optional making the CA_n5-n8 DL with n5 UL only the mandatory support and, as such, should be BCS0.

For the non-simultaneous n5Rx/n8Tx case, the introduction is pending the RAN2 discussion on feasibility and the need for specific signaling or not. Even in the later case, we believe this is an interesting optional operation that can also cover other operators that have spectrum in both Band n5 and Band n8 and does not rely on a 24MHz gap between n5DL and n8UL.

Finally, the restricted Band n8 UL, although very specific to the band n8 deployment for one operator in China, is really the only way to fulfill the 20MHz aggregated throughput in both DL and UL and, as such, may be supported as an optional operation.

Thus, we support specifying all three BCS and, although only in BCS2, n8 is limited to 10MHz CBW, we believe that all options can be limited to 10MHz CBW as in any case there cannot be simultaneous allocation of n5DL and n8UL in the 880MHz-894MHz frequency range (note that it would still allow up to 20MHz CBW in n8 for BCS0 and BCS1).

Proposal on supported BCS for CA_n5-n8:

- The following BCS in Table 1 are introduced in 38.101-1

- BCS 1 is introduced pending the feasibility from RAN2 with/without dedicated signaling

- BCS0 is the baseline support while BCS1 and BCS2 are optional (via related BCS being signaled or not)

- UEs supporting BCS1 are not mandated to support BCS2

- UEs supporting BCS2 are not mandated to support BCS1.

Table 1: BCS for CA_n5-n8 with different UL configurations

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5A-n8A | n5X | n5 | 5, 10 | 0 |
|  |  | n8 | 5, 10 |  |
|  | CA_n5A-n8AY | n5 | 5, 10 | 1 |
|  |  | n8 | 5, 10 |  |
|  | CA_n5A-n8AZ | n5 | 5, 10 | 2 |
|  |  | n8 | 5, 10 |  |
| NOTE X: In BCS0, n8 UL is not supported. NOTE Y: In BCS1, the concurrent operation of band n5 DL and band n8 UL is not supported and BCS2 cannot be supported. NOTE Z: In BCS2 the band n8 support is restricted to the 904-915MHz frequency range in UL and BCS1 cannot be supported. |  |  |  |  |

## 1UL cross-band MSD for n5 UL in n8DL

For all the different options to support CA_n5-n8, the 1UL case of Band n5 with DL CA_n5-n8 exists. If the requested 10MHz CBW for Band n5 is used, the overlap is >ACLR4. However, if the UL BW considered is up to 20MHz, there is an overlap with ACLR4. In both cases, the MSD is not related to the IMDs of the allocated UL RBs and their image, and we should mostly evaluate if there is an issue with the transmitter noise floor.

To evaluate cross-band isolation and rejection, one should account for the fact that Band n5 is typically implemented with a Band n26 filter and that there are multiple LBLB combinations that will need to be supported within the same UE that include Band n5 and or Band n26, including cases where the same operator requests the support of multiple LBLB combinations:

- CA_n5-n105

- CA_n5-n71

- CA_n5-n28

- CA_n26-n28

- Note that LBLBLB cases are also requested.

It is thus impossible to optimize the band-to-band Tx/Rx isolations and Tx rejections for all bands and to enable multiple LBLB support in a UE, conservative numbers should be used.

Still, given the distance between the two bands it should be feasible, with the proper Delta T and Delta R, to target a two-antenna architecture with two triplexers supporting n5 with an n26 implementation and should be used for the study:

- n26 DL+ n8 UL + n8 DL triplexer

- n8 DL+ n26 UL + n26 DL triplexer.

With the architecture aspects discussed above and the need to support multiple LBLB cases, conservative values should be considered:

- 45dB cross band Tx-Rx isolation for the triplexers

- 40dB Tx rejection in the other DL band

- 10dB antenna coupling

- 4dB post PA losses

- Transmitter noise floor of -125dBm/Hz.

Based on this, the MRC combining MSD can be calculated as shown in Table 2.

Table 2: MRC combining calculations

| CBW/NRB | [MHz]/# | 5 | 25 |
| --- | --- | --- | --- |
| transmitter noise floor | [dBm/CBW] | -58.5 |  |
| 5MHz CBW REFSENS | [dBm/CBW] | -98.0 |  |
| RX noise floor wo Tx noise: Main/Div | [dBm/CBW] | -94.0 | -94.0 |
| Tx-Rx / Tx-Ant duplexer isolation/rejection | [dB] | 45 | 40 |
| LNA to antenna insertion loss | [dB] | 4 |  |
| Antenna isolation | [dB] | 0 | 10 |
| PC3 1Tx interference levels: Main/Div | [dBm/CBW] | -99.5 | -104.5 |
| noise degradation due to Tx noise: Main/Div | [dB] | 1.1 | 0.4 |
| # noise degradation due to Tx noise: Main/Div | # [lin] | # 1.3 | # 1.1 |
| REFSENS degradation after MRC | uncor [dB] | 0.7 |  |
| REFSENS degradation after MRC | cor [dB] | 1.3 |  |

Proposal on 1UL cross-band MSD for n5UL and n8DL: see Table 3 below.

Table 3: REFSENS exceptions due to cross-band isolation for n5 UL and n8 DL

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n8 | 844 | 10 | 15 | 25 (RBstart=27) | 927.5 | 5 | 1.3 | >ACLR2 |

## 1 2UL MSD for cases without spectrum restriction on n8 UL

Without frequency restrictions on Band n5 and Band n8, assuming there is no concurrent n5 DL and n8 UL, there is a direct hit 2UL IMD3 on Band n8 DL for the 2UL CA_n5-n8 configuration. Based on previous LBLB cases, the MSD is large at 25dB.

Proposal on 2UL MSD without n8 UL restriction: Pending RAN2 input on non-concurrent n5DL/n8UL case, Table 4 below provides the required 2UL MSD with the specific note on UL bandwidth restriction.

Table 4: REFSENS exceptions due to IMD without frequency restriction on n8 UL

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode | of IMD |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n5-n8 | n5 | 846.5 | 5 | 25 (RBstart=0) | N/A | N/A | FDD | N/A |
|  | n8 | 892.5 | 5 | 25 (RBstart=0) | 937.5 | 25 X | FDD | IMD3 |
| Note X: This MSD applies for UEs supporting CA_n5-n8 without restrictions on band n8 UL frequency range. |  |  |  |  |  |  |  |  |

## 1UL and 2UL MSD for cases with spectrum restriction on n8 UL

When CA_n5-n8 is implemented with a Band n8 UL filter with 904-915MHz UL, there is concurrent operation of n8UL and n5DL creating a ACLR2 cross-band MSD as agreed in [1].

Proposal on 1UL cross band MSD with restricted n8 UL: see Table 5 below with the specific note on UL bandwidth restriction.

Table 5: REFSENS exceptions due to cross-band isolation with frequency restriction on n8 UL

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n8 | n5 | 909 | 10 | 15 | 25 (RBstart=0) | 877.5 | 5 | 12.3 X | ACLR2 |
| Note X: This MSD applies for UEs supporting CA_n5-n8 with the band n8 UL restricted to 904-915MHz frequency range. |  |  |  |  |  |  |  |  |  |

In the case where there is a frequency restriction on n8 UL frequency range, the 2UL IMD3 of n5UL at the top of n5 and n8UL at the bottom of n8R UL falls just above n8 DL at 966.5MHz. With 25 RB, the IMD3 spectrum is 13.5MHz wide and thus stops at 959.75MHz which is 5.75MHz above the upper edge of the corresponding n8 DL channel. Thus, there is no direct hit of the IMD3. However, the UL being at the maximum power, both Band n5 and Band n8 ACLR may be close to 30dBc and their IMD3 spans over 40.5MHz overlapping with the band n8 DL. Since the direct it IMD3 results in 25dB MSD, there may still be residual MSD from the near miss IMD3.

With the short time between the meetings, we could not evaluate this precisely with measurement, but one can assume that the IMD3 of the ACLR regions is 30dB lower than the direct hit case. Which, after MRC combining calculations, results in 1.6dB MSD.

Proposal on restricted n8 UL 2UL MSD: see Table 6 below with the specific note on UL bandwidth restriction.

Table 6: REFSENS exceptions due to IMD with frequency restriction on n8 UL

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode | of IMD |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n5-n8 | n5 | 846.5 | 5 | 25 (RBstart=0) | 891.5 | N/A | FDD | N/A |
|  | n8 | 906.5 | 5 | 25 (RBstart=0) | 951.5 | [1.6]X | FDD | Near miss IMD3 |
| Note X: This MSD applies for UEs supporting CA_n5-n8 with the band n8 UL restricted to 904-915MHz frequency range. |  |  |  |  |  |  |  |  |

# Conclusions

In this contribution, we have discussed the requirements for CA_n5-n8 based on way forward [1], and make the following proposals.

Proposal on supported BCS for CA_n5-n8:

- The following BCS in Table 1 are introduced in 38.101-1

- BCS 1 is introduced pending the feasibility from RAN2 with/without dedicated signaling

- BCS0 is the baseline support while BCS1 and BCS2 are optional (via related BCS being signaled or not)

- UEs supporting BCS1 are not mandated to support BCS2

- UEs supporting BCS2 are not mandated to support BCS1.

Table 1: BCS for CA_n5-n8 with different UL configurations

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5A-n8A | n5X | n5 | 5, 10 | 0 |
|  |  | n8 | 5, 10 |  |
|  | CA_n5A-n8AY | n5 | 5, 10 | 1 |
|  |  | n8 | 5, 10 |  |
|  | CA_n5A-n8AZ | n5 | 5, 10 | 2 |
|  |  | n8 | 5, 10 |  |
| NOTE X: In BCS0, n8 UL is not supported. NOTE Y: In BCS1, the concurrent operation of band n5 DL and band n8 UL is not supported and BCS2 cannot be supported. NOTE Z: In BCS2 the band n8 support is restricted to the 904-915MHz frequency range in UL and BCS1 cannot be supported. |  |  |  |  |

Proposal on 1UL cross-band MSD for n5UL and n8DL: see Table 3 below.

Table 3: REFSENS exceptions due to cross-band isolation for n5 UL and n8 DL

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n8 | 844 | 10 | 15 | 25 (RBstart=27) | 927.5 | 5 | 1.3 | >ACLR2 |

Proposal on 2UL MSD without n8 UL restriction: Pending RAN2 input on non-concurrent n5DL/n8UL case, Table 4 below provides the required 2UL MSD with the specific note on UL bandwidth restriction.

Table 4: REFSENS exceptions due to IMD without frequency restriction on n8 UL

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode | of IMD |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n5-n8 | n5 | 846.5 | 5 | 25 (RBstart=0) | N/A | N/A | FDD | N/A |
|  | n8 | 892.5 | 5 | 25 (RBstart=0) | 937.5 | 25 X | FDD | IMD3 |
| Note X: This MSD applies for UEs supporting CA_n5-n8 without restrictions on band n8 UL frequency range. |  |  |  |  |  |  |  |  |

Proposal on 1UL cross band MSD with restricted n8 UL: see Table 5 below with the specific note on UL bandwidth restriction.

Table 5: REFSENS exceptions due to cross-band isolation with frequency restriction on n8 UL

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n8 | n5 | 909 | 10 | 15 | 25 (RBstart=0) | 877.5 | 5 | 12.3 X | ACLR2 |
| Note X: This MSD applies for UEs supporting CA_n5-n8 with the band n8 UL restricted to 904-915MHz frequency range. |  |  |  |  |  |  |  |  |  |

Proposal on restricted n8 UL 2UL MSD: see Table 6 below with the specific note on UL bandwidth restriction.

Table 6: REFSENS exceptions due to IMD with frequency restriction on n8 UL

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode | of IMD |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n5-n8 | n5 | 846.5 | 5 | 25 (RBstart=0) | 891.5 | N/A | FDD | N/A |
|  | n8 | 906.5 | 5 | 25 (RBstart=0) | 951.5 | [1.6]X | FDD | Near miss IMD3 |
| Note X: This MSD applies for UEs supporting CA_n5-n8 with the band n8 UL restricted to 904-915MHz frequency range. |  |  |  |  |  |  |  |  |

# References

- R4-2306467 WF on CA_n5-n8, Skyworks Solutions, Inc., RAN4#106b-e

- R4-2304563 Input to CA_n5-n8 for LBLB and LBLBLB band combinations WI, Skyworks Solutions, Inc., RAN4#106b-e