3GPP TSG-RAN WG4 Meeting #107	R4-2309314

# Incheon, 22nd - 26th May, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0383 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for HST SFN scheme A and B test setup from Rel-17 (TS 38.101-4, Rel-17) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_feMIMO-Perf |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1. Current configuration of CSI-RS for tracking leads to CSI-RS resources being mapped to the same slot and same symbol. 2. CSI-RS periodicity of TDD test cases are not aligned between CSI-RS resource sets. 3. Some brackets are still left in the specification 4. TDD UL-DL pattern is missing in minimum performance tables of TDD test cases |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update clause 5.2.2.1.20, 5.2.2.1.21, 5.2.2.2.21, 5.2.2.2.22, 5.2.3.1.19, 5.2.3.1.20, 5.2.3.2.20, 5.2.3.2.21. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be inconsist between specification and RAN4 agreements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.1.20, 5.2.2.1.21, 5.2.2.2.21, 5.2.2.2.22, 5.2.3.1.19, 5.2.3.1.20, 5.2.3.2.20, 5.2.3.2.21. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<START OF THE CHANGE 1>

##### 5.2.2.1.20	Minimum requirements for HST-SFN Scheme A

The performance requirements are specified in Table 5.2.2.1.20-3, with the addition of test parameters in Table 5.2.2.1.20-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.2.1.20-1.

Table 5.2.2.1.20-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify UE performance in the HST-SFN Scheme A scenario defined in B.3. 5 | 1-1 |

Table 5.2.2.1.20-2: Test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  |  | FDD |
| Active DL BWP index |  |  |  | 1 |
| PDCCH configuration | TCI state |  |  | Note 1 |
| PDSCH configuration | Mapping type |  |  | Type A |
|  | k0 |  |  | 0 |
|  | Starting symbol (S) |  |  | 2 |
|  | Length (L) |  |  | 12 |
|  | PDSCH aggregation factor |  |  | 1 |
|  | PRB bundling type |  |  | Static |
|  | PRB bundling size |  |  | 2 |
|  | Resource allocation type |  |  | Type 0 |
|  | RBG size |  |  | Config2 |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |
|  | TCI state |  |  | Note 1 |
| PDSCH DMRS configuration | DMRS Type |  |  | Type 1 |
|  | Number of additional DMRS |  |  | 2 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |
| CSI-RS for tracking | Resource set #1 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
|  |  | CSI-RS periodicity | Slots | 10 for CSI-RS resource 1,2,3,4. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 1 and 2 2 for CSI-RS resource 3 and 4 |
|  |  | QCL info |  | TCI state #3 |
|  | Resource set #2 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 6 for CSI-RS resource 5 and 7 l0 = 10 for CSI-RS resource 6 and 8 |
|  |  | CSI-RS periodicity | Slots | 10 for CSI-RS resource 5,6,7,8. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 5 and 6 2 for CSI-RS resource 7 and 8 |
|  |  | QCL info |  | TCI state #4 |
|  | Resource set #3 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 4 for CSI-RS resource 9 and 11 l0 = 8 for CSI-RS resource 10 and 12 |
|  |  | CSI-RS periodicity | Slots | 10 for CSI-RS resource 9,10,11,12. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 9 and 10 2 for CSI-RS resource 11 and 12 |
|  |  | QCL info |  | TCI state #5 |
| NZP CSI-RS for CSI acquisition | Resource set #4 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  | CSI-RS periodicity | Slots | 20 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #0 |
|  | Resource set #5 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  | CSI-RS periodicity | Slots | 20 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #1 |
|  | Resource set #6 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 7 |
|  |  | CSI-RS periodicity | Slots | 20 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #2 |
| TCI state #0 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking Resource set #1' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 5 from 'CSI-RS for tracking Resource set #2' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #2 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 9 from 'CSI-RS for tracking Resource set #3' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #3 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #4 | Type 1 QCL information | SSB index |  | SSB #1 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #5 | Type 1 QCL information | SSB index |  | SSB #2 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| Number of HARQ Processes |  |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  | 2 |
| Note 1: SSB # (k mod 3) , CSI-RS (for tracking) resource set # ((k mod 3) + 1) and CSI-RS (for CSI acquisition) resource set # ((k mod 3) + 4) are transmitted by kth RRH. Codepoint #0 is activated when UE receives PDCCH/PDSCH from RRH#3k and RRH#3k+1 with TCI States TCI state #0, TCI State #1. Codepoint #1 is activated when UE receives PDCCH/PDSCH from RRH#3k+1 and RRH#3k+2 with TCI States TCI state #1, TCI State #2. Codepoint #2 is activated when UE receives PDCCH/PDSCH from RRH#3k+2 and RRH#3k+3 with TCI States TCI state #2, TCI State #0. |  |  |  |  |

Table 5.2.2.1.20-3: Minimum performance for HST-SFN Scheme A

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-8.5 FDD | 10 / 15 | 16QAM, 0.48 | HST-SFN Scheme A | 2x2 | 70 | 11.8 |

<END OF THE CHANGE 1>

<START OF THE CHANGE 2>

##### 5.2.2.1.21	Minimum requirements for HST-SFN Scheme B

The performance requirements are specified in Table 5.2.2.1.21-3, with the addition of test parameters in Table 5.2.2.1.21-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.2.1.21-1.

Table 5.2.2.1.21-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify UE performance in the HST-SFN Scheme B scenario defined in B.3.6 | 1-1 |

Table 5.2.2.1.21-2: Test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  |  | FDD |
| Active DL BWP index |  |  |  | 1 |
| PDCCH configuration | TCI state |  |  | Note 1 |
| PDSCH configuration | Mapping type |  |  | Type A |
|  | k0 |  |  | 0 |
|  | Starting symbol (S) |  |  | 2 |
|  | Length (L) |  |  | 12 |
|  | PDSCH aggregation factor |  |  | 1 |
|  | PRB bundling type |  |  | Static |
|  | PRB bundling size |  |  | 2 |
|  | Resource allocation type |  |  | Type 0 |
|  | RBG size |  |  | Config2 |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |
|  | TCI state |  |  | Note 1 |
| PDSCH DMRS configuration | DMRS Type |  |  | Type 1 |
|  | Number of additional DMRS |  |  | 2 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |
| CSI-RS for tracking | Resource set #1 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
|  |  | CSI-RS periodicity | Slots | 10 for CSI-RS resource 1,2,3,4. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 1 and 2 2 for CSI-RS resource 3 and 4 |
|  |  | QCL info |  | TCI state #3 |
|  | Resource set #2 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 6 for CSI-RS resource 5 and 7 l0 = 10 for CSI-RS resource 6 and 8 |
|  |  | CSI-RS periodicity | Slots | 10 for CSI-RS resource 5,6,7,8. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 5 and 6 2 for CSI-RS resource 7 and 8 |
|  |  | QCL info |  | TCI state #4 |
|  | Resource set #3 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 4 for CSI-RS resource 9 and 11 l0 = 8 for CSI-RS resource 10 and 12 |
|  |  | CSI-RS periodicity | Slots | 10 for CSI-RS resource 9,10,11,12. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 9 and 10 2 for CSI-RS resource 11 and 12 |
|  |  | QCL info |  | TCI state #5 |
| NZP CSI-RS for CSI acquisition | Resource set #4 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  | CSI-RS periodicity | Slots | 20 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #0 |
|  | Resource set #5 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  | CSI-RS periodicity | Slots | 20 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #1 |
|  | Resource set #6 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 7 |
|  |  | CSI-RS periodicity | Slots | 20 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #2 |
| TCI state #0 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking Resource set #1' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 5 from 'CSI-RS for tracking Resource set #2' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #2 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 9 from 'CSI-RS for tracking Resource set #3' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #3 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #4 | Type 1 QCL information | SSB index |  | SSB #1 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #5 | Type 1 QCL information | SSB index |  | SSB #2 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| Number of HARQ Processes |  |  | 4 |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |  |
| Note 1: SSB # (k mod 3), CSI-RS (for tracking) resource set # ((k mod 3) + 1) and CSI-RS (for CSI acquisition) resource set # ((k mod 3) + 4) are transmitted by kth RRH. Codepoint#0 {TCI state #0, TCI State #1} is activated when UE receives PDCCH/PDSCH from RRH#3k and RRH#3k+1. Codepoint#1 {TCI state #1, TCI State #2} is activated when UE receives PDCCH/PDSCH from RRH#3k+1 and RRH#3k+2. Codepoint#2 {TCI state #2, TCI State #0} is activated when UE receives PDCCH/PDSCH from RRH#3k+2 and RRH#3k+3. The second indicated TCI state in each codepoint is not used for quasi co-location parameters {Doppler shift, Doppler spread}. |  |  |  |  |

Table 5.2.2.1.21-3: Minimum performance for HST-SFN Scheme B

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-8.5 FDD | 10/15 | 16QAM, 0.48 | HST-SFN-Scheme B | 2x2 | 70 | 11.3 |

<END OF THE CHANGE 2>

<START OF THE CHANGE 3>

##### 5.2.2.2.21	Minimum requirements for HST-SFN Scheme A

The performance requirements are specified in Table 5.2.2.2.21-3, with the addition of test parameters in Table 5.2.2.2.21-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.2.2.21-1.

Table 5.2.2.2.21-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify UE performance in the HST-SFN Scheme A scenario defined in B.3.5 | 1-1 |

Table 5.2.2.2.21-2: Test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  |  | TDD |
| Active DL BWP index |  |  |  | 1 |
| PDCCH configuration | TCI state |  |  | Note 1 |
| PDSCH configuration | Mapping type |  |  | Type A |
|  | k0 |  |  | 0 |
|  | Starting symbol (S) |  |  | 2 |
|  | Length (L) |  |  | 12 |
|  | PDSCH aggregation factor |  |  | 1 |
|  | PRB bundling type |  |  | Static |
|  | PRB bundling size |  |  | 2 |
|  | Resource allocation type |  |  | Type 0 |
|  | RBG size |  |  | Config2 |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |
|  | TCI state |  |  | Note 1 |
| PDSCH DMRS configuration | DMRS Type |  |  | Type 1 |
|  | Number of additional DMRS |  |  | 2 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |
| CSI-RS for tracking | Resource set #1 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
|  |  | CSI-RS periodicity | Slots | 20 for CSI-RS resource 1,2,3,4. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 1 and 2 2 for CSI-RS resource 3 and 4 |
|  |  | QCL info |  | TCI state #3 |
|  | Resource set #2 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 6 for CSI-RS resource 5 and 7 l0 = 10 for CSI-RS resource 6 and 8 |
|  |  | CSI-RS periodicity | Slots | 20 for CSI-RS resource 5,6,7,8. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 5 and 6 2 for CSI-RS resource 7 and 8 |
|  |  | QCL info |  | TCI state #4 |
|  | Resource set #3 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 4 for CSI-RS resource 9 and 11 l0 = 8 for CSI-RS resource 10 and 12 |
|  |  | CSI-RS periodicity | Slots | 20 for CSI-RS resource 9,10,11,12. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 9 and 10 2 for CSI-RS resource 11 and 12 |
|  |  | QCL info |  | TCI state #5 |
| NZP CSI-RS for CSI acquisition | Resource set #4 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  | CSI-RS periodicity | Slots | 40 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #0 |
|  | Resource set #5 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  | CSI-RS periodicity | Slots | 40 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #1 |
|  | Resource set #6 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 7 |
|  |  | CSI-RS periodicity | Slots | 40 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #2 |
| TCI state #0 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking Resource set #1' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 5 from 'CSI-RS for tracking Resource set #2' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #2 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 9 from 'CSI-RS for tracking Resource set #3' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #3 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #4 | Type 1 QCL information | SSB index |  | SSB #1 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #5 | Type 1 QCL information | SSB index |  | SSB #2 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| Number of HARQ Processes |  |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |
| Note 1: SSB # (k mod 3) , CSI-RS (for tracking) resource set # ((k mod 3) + 1) and CSI-RS (for CSI acquisition) resource set # ((k mod 3) + 4) are transmitted by kth RRH. Codepoint #0 is activated when UE receives PDCCH/PDSCH from RRH#3k and RRH#3k+1 with TCI States TCI state #0, TCI State #1. Codepoint #1 is activated when UE receives PDCCH/PDSCH from RRH#3k+1 and RRH#3k+2 with TCI States TCI state #1, TCI State #2. Codepoint #2 is activated when UE receives PDCCH/PDSCH from RRH#3k+2 and RRH#3k+3 with TCI States TCI state #2, TCI State #0. |  |  |  |  |

Table 5.2.2.2.21-3: Minimum performance for HST-SFN Scheme A

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.2-30.1 TDD | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | HST-SFN Scheme A | 2x2 | 70 | 12.9 |

<END OF THE CHANGE 3>

<START OF THE CHANGE 4>

##### 5.2.2.2.22	Minimum requirements for HST-SFN Scheme B

The performance requirements are specified in Table 5.2.2.2.22-3, with the addition of test parameters in Table 5.2.2.2.22-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.2.2.22-1.

Table 5.2.2.2.22-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify UE performance in the HST-SFN Scheme B scenario defined in B.3.6 | 1-1 |

Table 5.2.2.2.22-2: Test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  |  | TDD |
| Active DL BWP index |  |  |  | 1 |
| PDCCH configuration | TCI state |  |  | Note 1 |
| PDSCH configuration | Mapping type |  |  | Type A |
|  | k0 |  |  | 0 |
|  | Starting symbol (S) |  |  | 2 |
|  | Length (L) |  |  | Specific to each Reference channel |
|  | PDSCH aggregation factor |  |  | 1 |
|  | PRB bundling type |  |  | Static |
|  | PRB bundling size |  |  | 2 |
|  | Resource allocation type |  |  | Type 0 |
|  | RBG size |  |  | Config2 |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |
|  | TCI state |  |  | Note 1 |
| PDSCH DMRS configuration | DMRS Type |  |  | Type 1 |
|  | Number of additional DMRS |  |  | 2 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |
| CSI-RS for tracking | Resource set #1 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
|  |  | CSI-RS periodicity | Slots | 20 for CSI-RS resource 1,2,3,4. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 1 and 2 2 for CSI-RS resource 3 and 4 |
|  |  | QCL info |  | TCI state #3 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB = 52 |
|  | Resource set #2 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 6 for CSI-RS resource 5 and 7 l0 = 10 for CSI-RS resource 6 and 8 |
|  |  | CSI-RS periodicity | Slots | 20 for CSI-RS resource 5,6,7,8. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 5 and 6 2 for CSI-RS resource 7 and 8 |
|  |  | QCL info |  | TCI state #4 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB = 52 |
|  | Resource set #3 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 4 for CSI-RS resource 9 and 11 l0 = 8 for CSI-RS resource 10 and 12 |
|  |  | CSI-RS periodicity | Slots | 20 for CSI-RS resource 9,10,11,12. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 9 and 10 2 for CSI-RS resource 11 and 12 |
|  |  | QCL info |  | TCI state #5 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB = 52 |
| NZP CSI-RS for CSI acquisition | Resource set #4 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  | CSI-RS periodicity | Slots | 40 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #0 |
|  | Resource set #5 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  | CSI-RS periodicity | Slots | 40 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #1 |
|  | Resource set #6 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 7 |
|  |  | CSI-RS periodicity | Slots | 40 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #2 |
| TCI state #0 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking Resource set #1' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 5 from 'CSI-RS for tracking Resource set #2' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #2 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 9 from 'CSI-RS for tracking Resource set #3' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #3 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #4 | Type 1 QCL information | SSB index |  | SSB #1 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #5 | Type 1 QCL information | SSB index |  | SSB #2 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| Number of HARQ Processes |  |  | 8 |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |  |
| Note 1: SSB # (k mod 3), CSI-RS (for tracking) resource set # ((k mod 3) + 1) and CSI-RS (for CSI acquisition) resource set # ((k mod 3) + 4) are transmitted by kth RRH. Codepoint#0 {TCI state #0, TCI State #1} is activated when UE receives PDCCH/PDSCH from RRH#3k and RRH#3k+1. Codepoint#1 {TCI state #1, TCI State #2} is activated when UE receives PDCCH/PDSCH from RRH#3k+1 and RRH#3k+2. Codepoint#2 {TCI state #2, TCI State #0} is activated when UE receives PDCCH/PDSCH from RRH#3k+2 and RRH#3k+3. The second indicated TCI state in each codepoint is not used for quasi co-location parameters {Doppler shift, Doppler spread}. |  |  |  |  |

Table 5.2.2.2.22-3: Minimum performance for HST-SFN Scheme B

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.2-30.1 TDD | 40/30 | 16QAM, 0.48 | FR1.30-1 | HST-SFN-Scheme B | 2x2 | 70 | 11.8 |

# <END OF THE CHANGE 4>

# <START OF THE CHANGE 5>

##### 5.2.3.1.19	Minimum requirements for HST-SFN Scheme A

The performance requirements are specified in Table 5.2.3.1.19-3, with the addition of test parameters in Table 5.2.3.1.19-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.3.1.19-1.

Table 5.2.3.1.19-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify UE performance in the HST-SFN Scheme A scenario defined in B.3.5 | 1-1 |

Table 5.2.2.1.19-2: Test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  |  | FDD |
| Active DL BWP index |  |  |  | 1 |
| PDCCH configuration | TCI state |  |  | Note 1 |
| PDSCH configuration | Mapping type |  |  | Type A |
|  | k0 |  |  | 0 |
|  | Starting symbol (S) |  |  | 2 |
|  | Length (L) |  |  | 12 |
|  | PDSCH aggregation factor |  |  | 1 |
|  | PRB bundling type |  |  | Static |
|  | PRB bundling size |  |  | 2 |
|  | Resource allocation type |  |  | Type 0 |
|  | RBG size |  |  | Config2 |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |
|  | TCI state |  |  | Note 1 |
| PDSCH DMRS configuration | DMRS Type |  |  | Type 1 |
|  | Number of additional DMRS |  |  | 2 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |
| CSI-RS for tracking | Resource set #1 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
|  |  | CSI-RS periodicity | Slots | 10 for CSI-RS resource 1,2,3,4. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 1 and 2 2 for CSI-RS resource 3 and 4 |
|  |  | QCL info |  | TCI state #3 |
|  | Resource set #2 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 6 for CSI-RS resource 5 and 7 l0 = 10 for CSI-RS resource 6 and 8 |
|  |  | CSI-RS periodicity | Slots | 10 for CSI-RS resource 5,6,7,8. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 5 and 6 2 for CSI-RS resource 7 and 8 |
|  |  | QCL info |  | TCI state #4 |
|  | Resource set #3 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 4 for CSI-RS resource 9 and 11 l0 = 8 for CSI-RS resource 10 and 12 |
|  |  | CSI-RS periodicity | Slots | 10 for CSI-RS resource 9,10,11,12. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 9 and 10 2 for CSI-RS resource 11 and 12 |
|  |  | QCL info |  | TCI state #5 |
| NZP CSI-RS for CSI acquisition | Resource set #4 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  | CSI-RS periodicity | Slots | 20 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #0 |
|  | Resource set #5 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  | CSI-RS periodicity | Slots | 20 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #1 |
|  | Resource set #6 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 7 |
|  |  | CSI-RS periodicity | Slots | 20 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #2 |
| TCI state #0 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking Resource set #1' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 5 from 'CSI-RS for tracking Resource set #2' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #2 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 9 from 'CSI-RS for tracking Resource set #3' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #3 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #4 | Type 1 QCL information | SSB index |  | SSB #1 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #5 | Type 1 QCL information | SSB index |  | SSB #2 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| Number of HARQ Processes |  |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  | 2 |
| Note 1: SSB # (k mod 3) , CSI-RS (for tracking) resource set # ((k mod 3) + 1) and CSI-RS (for CSI acquisition) resource set # ((k mod 3) + 4) are transmitted by kth RRH. Codepoint #0 is activated when UE receives PDCCH/PDSCH from RRH#3k and RRH#3k+1 with TCI States TCI state #0, TCI State #1. Codepoint #1 is activated when UE receives PDCCH/PDSCH from RRH#3k+1 and RRH#3k+2 with TCI States TCI state #1, TCI State #2. Codepoint #2 is activated when UE receives PDCCH/PDSCH from RRH#3k+2 and RRH#3k+3 with TCI States TCI state #2, TCI State #0. |  |  |  |  |

Table 5.2.3.1.19-3: Minimum performance for HST-SFN Scheme A

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-8.5 FDD | 10 / 15 | 16QAM, 0.48 | HST-SFN Scheme A | 2x4 | 70 | 9.1 |

# <END OF THE CHANGE 5>

# <START OF THE CHANGE 6>

##### 5.2.3.1.20	Minimum requirements for HST-SFN Scheme B

The performance requirements are specified in Table 5.2.3.1.20-3, with the addition of test parameters in Table 5.2.3.1.20-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.3.1.20-1.

Table 5.2.3.1.20-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify UE performance in the HST-SFN Scheme B scenario defined in B.3.6 | 1-1 |

Table 5.2.3.1.20-2: Test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  |  | FDD |
| Active DL BWP index |  |  |  | 1 |
| PDCCH configuration | TCI state |  |  | Note 1 |
| PDSCH configuration | Mapping type |  |  | Type A |
|  | k0 |  |  | 0 |
|  | Starting symbol (S) |  |  | 2 |
|  | Length (L) |  |  | 12 |
|  | PDSCH aggregation factor |  |  | 1 |
|  | PRB bundling type |  |  | Static |
|  | PRB bundling size |  |  | 2 |
|  | Resource allocation type |  |  | Type 0 |
|  | RBG size |  |  | Config2 |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |
|  | TCI state |  |  | Note 1 |
| PDSCH DMRS configuration | DMRS Type |  |  | Type 1 |
|  | Number of additional DMRS |  |  | 2 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |
| CSI-RS for tracking | Resource set #1 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
|  |  | CSI-RS periodicity | Slots | 10 for CSI-RS resource 1,2,3,4. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 1 and 2 2 for CSI-RS resource 3 and 4 |
|  |  | QCL info |  | TCI state #3 |
|  | Resource set #2 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 6 for CSI-RS resource 5 and 7 l0 = 10 for CSI-RS resource 6 and 8 |
|  |  | CSI-RS periodicity | Slots | 10 for CSI-RS resource 5,6,7,8. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 5 and 6 2 for CSI-RS resource 7 and 8 |
|  |  | QCL info |  | TCI state #4 |
|  | Resource set #3 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 4 for CSI-RS resource 9 and 11 l0 = 8 for CSI-RS resource 10 and 12 |
|  |  | CSI-RS periodicity | Slots | 10 for CSI-RS resource 9,10,11,12. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 9 and 10 2 for CSI-RS resource 11 and 12 |
|  |  | QCL info |  | TCI state #5 |
| NZP CSI-RS for CSI acquisition | Resource set #4 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  | CSI-RS periodicity | Slots | 20 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #0 |
|  | Resource set #5 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  | CSI-RS periodicity | Slots | 20 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #1 |
|  | Resource set #6 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 7 |
|  |  | CSI-RS periodicity | Slots | 20 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #2 |
| TCI state #0 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking Resource set #1' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 5 from 'CSI-RS for tracking Resource set #2' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #2 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 9 from 'CSI-RS for tracking Resource set #3' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #3 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #4 | Type 1 QCL information | SSB index |  | SSB #1 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #5 | Type 1 QCL information | SSB index |  | SSB #2 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| Number of HARQ Processes |  |  | 4 |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |  |
| Note 1: SSB # (k mod 3), CSI-RS (for tracking) resource set # ((k mod 3) + 1) and CSI-RS (for CSI acquisition) resource set # ((k mod 3) + 4) are transmitted by kth RRH. Codepoint#0 {TCI state #0, TCI State #1} is activated when UE receives PDCCH/PDSCH from RRH#3k and RRH#3k+1. Codepoint#1 {TCI state #1, TCI State #2} is activated when UE receives PDCCH/PDSCH from RRH#3k+1 and RRH#3k+2. Codepoint#2 {TCI state #2, TCI State #0} is activated when UE receives PDCCH/PDSCH from RRH#3k+2 and RRH#3k+3. The second indicated TCI state in each codepoint is not used for quasi co-location parameters {Doppler shift, Doppler spread}. |  |  |  |  |

Table 5.2.3.1.20-3: Minimum performance for HST-SFN Scheme B

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-8.5 FDD | 10/15 | 16QAM, 0.48 | HST-SFN-Scheme B | 2x4 | 70 | 8.4 |

# <END OF THE CHANGE 6>

# <START OF THE CHANGE 7>

##### 5.2.3.2.20	Minimum requirements for HST-SFN Scheme A

The performance requirements are specified in Table 5.2.3.2.20-3, with the addition of test parameters in Table 5.2.3.2.20-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.3.2.20-1.

Table 5.2.3.2.20-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify UE performance in the HST-SFN Scheme A scenario defined in B.3.5 | 1-1 |

Table 5.2.3.2.20-2: Test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  |  | TDD |
| Active DL BWP index |  |  |  | 1 |
| PDCCH configuration | TCI state |  |  | Note 1 |
| PDSCH configuration | Mapping type |  |  | Type A |
|  | k0 |  |  | 0 |
|  | Starting symbol (S) |  |  | 2 |
|  | Length (L) |  |  | 12 |
|  | PDSCH aggregation factor |  |  | 1 |
|  | PRB bundling type |  |  | Static |
|  | PRB bundling size |  |  | 2 |
|  | Resource allocation type |  |  | Type 0 |
|  | RBG size |  |  | Config2 |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |
|  | TCI state |  |  | Note 1 |
| PDSCH DMRS configuration | DMRS Type |  |  | Type 1 |
|  | Number of additional DMRS |  |  | 2 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |
| CSI-RS for tracking | Resource set #1 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
|  |  | CSI-RS periodicity | Slots | 20 for CSI-RS resource 1,2,3,4. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 1 and 2 2 for CSI-RS resource 3 and 4 |
|  |  | QCL info |  | TCI state #3 |
|  | Resource set #2 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 6 for CSI-RS resource 5 and 7 l0 = 10 for CSI-RS resource 6 and 8 |
|  |  | CSI-RS periodicity | Slots | 20 for CSI-RS resource 5,6,7,8. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 5 and 6 2 for CSI-RS resource 7 and 8 |
|  |  | QCL info |  | TCI state #4 |
|  | Resource set #3 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 4 for CSI-RS resource 9 and 11 l0 = 8 for CSI-RS resource 10 and 12 |
|  |  | CSI-RS periodicity | Slots | 20 for CSI-RS resource 9,10,11,12. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 9 and 10 2 for CSI-RS resource 11 and 12 |
|  |  | QCL info |  | TCI state #5 |
| NZP CSI-RS for CSI acquisition | Resource set #4 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  | CSI-RS periodicity | Slots | 40 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #0 |
|  | Resource set #5 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  | CSI-RS periodicity | Slots | 40 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #1 |
|  | Resource set #6 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 7 |
|  |  | CSI-RS periodicity | Slots | 40 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #2 |
| TCI state #0 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking Resource set #1' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 5 from 'CSI-RS for tracking Resource set #2' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #2 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 9 from 'CSI-RS for tracking Resource set #3' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #3 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #4 | Type 1 QCL information | SSB index |  | SSB #1 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #5 | Type 1 QCL information | SSB index |  | SSB #2 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| Number of HARQ Processes |  |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |
| Note 1: SSB # (k mod 3) , CSI-RS (for tracking) resource set # ((k mod 3) + 1) and CSI-RS (for CSI acquisition) resource set # ((k mod 3) + 4) are transmitted by kth RRH. Codepoint #0 is activated when UE receives PDCCH/PDSCH from RRH#3k and RRH#3k+1 with TCI States TCI state #0, TCI State #1. Codepoint #1 is activated when UE receives PDCCH/PDSCH from RRH#3k+1 and RRH#3k+2 with TCI States TCI state #1, TCI State #2. Codepoint #2 is activated when UE receives PDCCH/PDSCH from RRH#3k+2 and RRH#3k+3 with TCI States TCI state #2, TCI State #0. |  |  |  |  |

Table 5.2.3.2.20-3: Minimum performance for HST-SFN Scheme A

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.2-30.1 TDD | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | HST-SFN Scheme A | 2x4 | 70 | 10.3 |

# <END OF THE CHANGE 7>

# <START OF THE CHANGE 8>

##### 5.2.3.2.21	Minimum requirements for HST-SFN Scheme B

The performance requirements are specified in Table 5.2.3.2.21-3, with the addition of test parameters in Table 5.2.3.2.21-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.3.2.21-1.

Table 5.2.3.2.21-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify UE performance in the HST-SFN Scheme B scenario defined in B.3.6 | 1-1 |

Table 5.2.3.2.21-2: Test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  |  | TDD |
| Active DL BWP index |  |  |  | 1 |
| PDCCH configuration | TCI state |  |  | Note 1 |
| PDSCH configuration | Mapping type |  |  | Type A |
|  | k0 |  |  | 0 |
|  | Starting symbol (S) |  |  | 2 |
|  | Length (L) |  |  | Specific to each Reference channel |
|  | PDSCH aggregation factor |  |  | 1 |
|  | PRB bundling type |  |  | Static |
|  | PRB bundling size |  |  | 2 |
|  | Resource allocation type |  |  | Type 0 |
|  | RBG size |  |  | Config2 |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |
|  | TCI state |  |  | Note 1 |
| PDSCH DMRS configuration | DMRS Type |  |  | Type 1 |
|  | Number of additional DMRS |  |  | 2 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |
| CSI-RS for tracking | Resource set #1 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
|  |  | CSI-RS periodicity | Slots | 20 for CSI-RS resource 1,2,3,4. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 1 and 2 2 for CSI-RS resource 3 and 4 |
|  |  | QCL info |  | TCI state #3 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB = 52 |
|  | Resource set #2 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 6 for CSI-RS resource 5 and 7 l0 = 10 for CSI-RS resource 6 and 8 |
|  |  | CSI-RS periodicity | Slots | 20 for CSI-RS resource 5,6,7,8. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 5 and 6 2 for CSI-RS resource 7 and 8 |
|  |  | QCL info |  | TCI state #4 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB = 52 |
|  | Resource set #3 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 4 for CSI-RS resource 9 and 11 l0 = 8 for CSI-RS resource 10 and 12 |
|  |  | CSI-RS periodicity | Slots | 20 for CSI-RS resource 9,10,11,12. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 9 and 10 2 for CSI-RS resource 11 and 12 |
|  |  | QCL info |  | TCI state #5 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB = 52 |
| NZP CSI-RS for CSI acquisition | Resource set #4 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  | CSI-RS periodicity | Slots | 40 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #0 |
|  | Resource set #5 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  | CSI-RS periodicity | Slots | 40 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #1 |
|  | Resource set #6 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 7 |
|  |  | CSI-RS periodicity | Slots | 40 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #2 |
| TCI state #0 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking Resource set #1' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 5 from 'CSI-RS for tracking Resource set #2' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #2 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 9 from 'CSI-RS for tracking Resource set #3' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #3 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #4 | Type 1 QCL information | SSB index |  | SSB #1 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #5 | Type 1 QCL information | SSB index |  | SSB #2 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| Number of HARQ Processes |  |  | 8 |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |  |
| Note 1: SSB # (k mod 3), CSI-RS (for tracking) resource set # ((k mod 3) + 1) and CSI-RS (for CSI acquisition) resource set # ((k mod 3) + 4) are transmitted by kth RRH. Codepoint#0 {TCI state #0, TCI State #1} is activated when UE receives PDCCH/PDSCH from RRH#3k and RRH#3k+1. Codepoint#1 {TCI state #1, TCI State #2} is activated when UE receives PDCCH/PDSCH from RRH#3k+1 and RRH#3k+2. Codepoint#2 {TCI state #2, TCI State #0} is activated when UE receives PDCCH/PDSCH from RRH#3k+2 and RRH#3k+3. The second indicated TCI state in each codepoint is not used for quasi co-location parameters {Doppler shift, Doppler spread}. |  |  |  |  |

Table 5.2.3.2.21-3: Minimum performance for HST-SFN Scheme B

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.2-30.1 TDD | 40/30 | 16QAM, 0.48 | FR1.30-1 | HST-SFN-SchemeB | 2x4 | 70 | 9.3 |

# <END OF THE CHANGE 8>