3GPP TSG-RAN4 Meeting #107	R4-2309350

# Incheon, KR, 22 – 26 May 2023

Agenda item:	8.16.3 / 8.17.5

Source:	Telecom Italia S.p.A.

Title:   	Further discussion on device’s information disclosure for FR1 TRP-TRS and MIMO OTA

Document for:	Discussion

# 1	Introduction

During RAN#106-bis-e meeting WF for Rel-18 TRP/TRS requirements [1] was approved. Regarding “Topic #5: Requirements work”, the following agreements on UE information disclosure were reached:

Issue 5-1-1: Disclosed UE information (and thresholds) for Rel-18 TRP TRS requirement work

Agreements:

  - For UE information disclosure activity, encourage companies to share views next meeting on which UE information are Key aspects to ensure a representative data pool, and corresponding technical reasons.

  - For thresholds of each UE information, encourage companies to share the analysis of current market status of NR smartphones, and technical reasons on each threshold.

  - RAN4 should discuss and decide UE information disclosure is mandatory or optional to volunteered test labs.

This contribution recaps the proposals provided in document R4-2305186 [2] with some additional information for what concern the thresholds definition. It is requested to adopt the same proposals also in the context of the performance part framework of MIMO OTA enhancement WI (agenda item 8.17.5).

# 2	Discussion

## 2.1 Information to be disclosed

The FR1 TRP/TRS performance requirements definition is based on a statistical analysis of the data derived from a measurement campaign performed on a limited number of devices.

Observation 1: The statistical sample set must be sufficiently representative of the market status/devices present in the network. Therefore, a comprehensive set of information describing the statistical sample is necessary to evaluate its validity.

Based on the observation above, the following information are considered relevant to guarantee the transparency of the framework as well as the statistical confidence of the measurement campaign.

Proposal 1: It is proposed to consider the following information in the framework of the performance part:

- Total number of devices

- Total number of models

- Total number of devices’ vendors

- Percentage of devices per vendor

- Percentage of devices per Power Class

- Percentage of devices per each supported band

- Percentage of devices per year of production

- Percentage of the devices that are certified by PTCRB and GCF

- Percentage of the devices per market segment (i.e. low, mid or high)

- Percentage of devices that are commercially available

## 2.2 Collection and management of the data

To obtain the information listed in the Proposal 1, device specific data need to be provided together with the measurement results.

Proposal 2: It is proposed to include the following fields in the datasheet that will be provided to the laboratories for collecting the measurement results:

- Device model

- Device vendor

- Power Class

- Supported bands

- Year of production

- Device certification (PTCRB, GCF, N/A)

- Market segment (low, mid or high)

- Commercially available (YES or NO)

Some data reported in the Proposal 2 can be considered sensitive (e.g. device model and/or device vendor).

Observation 2: Sensitive information cannot be disclosed to the RAN4 group. An appropriate methodology and a trusted third party need to be defined to collect and manage the device data.

Following the same approach that was used in ETSI MSG-TFES for the definition of the TRP/TRS minimum performance requirements for LTE OTA reported in [3], it is proposed the following:

Proposal 3: It is proposed that RAN4 Secretary will cover the role of the trusted third party to collect the measurements results provided by the laboratories and forward them to the RAN4 group after anonymizing the sensitive data

Device models can be anonymized as follow:

Proposal 4: It is proposed that each device model will be identified by a generic label, e.g. “Model A”, “Model B”, etc. If different laboratories will measure the same device model (not necessarily the same physical device), this will be anyway identified under the same label. For example, with reference to Proposal 1, 4 devices of the same model will count as 4 measured devices and 1 measured model.

Proposal 5: The information reported in the Proposal 1 will be provided by the WI rapporteur together with the curves analysis of the measurements results.

## 2.3 Information validation

To validate the statistical relevance of the measurement campaign, the thresholds reported in Proposal 6 have to be fulfilled by the information defined in Proposal 1. The following considerations can be made:

- According to GSA [4], the number of announced 5G devices rose by 3% between February and March 2023, reaching a total of 1,896 devices. Of these, 1,513 are understood to be commercially available, representing 79.8% of all announced 5G devices. Therefore, at least 40 device models need to be considered in the measurement campaign.

- According to IDC [5], the devices market share is divided between 5 main manufacturers. Therefore, at least 5 device’s vendors need to be considered in the measurement campaign.

- The year of production should focus on the most recent devices, i.e., from second-half 2021 to 2023; the percentage threshold per year to be discussed.

- According to IDC [6], the worldwide smartphone presence per market segment is reported in the diagram below and can be used as a reference to define the threshold for the percentage of low, mid, high market segments:

Proposal 6: It is proposed to adopt the following thresholds to be satisfied for the statistical relevance validation of the measurement campaign:

- Total number of devices: >= 50

- Total number of models: >= 40

- Total number of devices’ vendors: >=5

- Percentage of devices per vendor: [>= 10%]

- Percentage of devices per Power Class: TBD

- Percentage of devices per each supported band: TBD

- Percentage of devices per year of production: from second-half 2021 to 2023, percentage TBD

- Percentage of the devices that are certified by PTCRB and GCF: 100%

- Percentage of the devices per market segment (i.e. low, mid or high): TBD

- Percentage of devices that are commercially available: 100%

## 2.4 Application to the MIMO OTA enhancement WI

Proposal 7: It is proposed to adopt sections 2.1, 2.2 and 2.3 also for the performance part framework of the MIMO OTA enhancement WI.

# 3	Conclusions

In this contribution, the following proposals are provided:

Observation 1: The statistical sample set must be sufficiently representative of the market status/devices present in the network. Therefore, a comprehensive set of information describing the statistical sample is necessary to evaluate its validity.

Proposal 1: It is proposed to consider the following information in the framework of the performance part:

- Total number of devices

- Total number of models

- Total number of devices’ vendors

- Percentage of devices per vendor

- Percentage of devices per Power Class

- Percentage of devices per each supported band

- Percentage of devices per year of production

- Percentage of the devices that are certified by PTCRB and GCF

- Percentage of the devices per market segment (i.e. low, mid or high)

- Percentage of devices that are commercially available

Proposal 2: It is proposed to include the following fields in the datasheet that will be provided to the laboratories for collecting the measurement results:

- Device model

- Device vendor

- Power Class

- Supported bands

- Year of production

- Device certification (PTCRB, GCF, N/A)

- Market segment (low, mid or high)

- Commercially available (YES or NO)

Observation 2: Sensitive information cannot be disclosed to the RAN4 group. An appropriate methodology and a trusted third party need to be defined to collect and manage the device data.

Proposal 3: It is proposed that RAN4 Secretary will cover the role of the trusted third party to collect the measurements results provided by the laboratories and forward them to the RAN4 group after anonymizing the sensitive data

Proposal 4: It is proposed that each device model will be identified by a generic label, e.g. “Model A”, “Model B”, etc. If different laboratories will measure the same device model (not necessarily the same physical device), this will be anyway identified under the same label. For example, with reference to Proposal 1, 4 devices of the same model will count as 4 measured devices and 1 measured model.

Proposal 5: The information reported in the Proposal 1 will be provided by the WI rapporteur together with the curves analysis of the measurements results.

Proposal 6: It is proposed to adopt the following thresholds to be satisfied for the statistical relevance validation of the measurement campaign:

- Total number of devices: >= 50

- Total number of models: >= 40

- Total number of devices’ vendors: >=5

- Percentage of devices per vendor: [>= 10%]

- Percentage of devices per Power Class: TBD

- Percentage of devices per each supported band: TBD

- Percentage of devices per year of production: from second-half 2021 to 2023, percentage TBD

- Percentage of the devices that are certified by PTCRB and GCF: 100%

- Percentage of the devices per market segment (i.e. low, mid or high): TBD

- Percentage of devices that are commercially available: 100%

Proposal 7: It is proposed to adopt sections 2.1, 2.2 and 2.3 also for the performance part framework of the MIMO OTA enhancement WI.

# 4	References

R4-2305904, "WF for Rel-18 TRP/TRS requirements", vivo

R4-2305186, “Device’s information disclosure for FR1 TRP-TRS and MIMO OTA”, Telecom Italia

ETSI TR 103 803 V1.1.1 “Technical report on LTE OTA requirement derivation from MSG TFES ad hoc working group on LTE OTA”, December 2020

https://gsacom.com/technology/5g/

https://www.idc.com/promo/smartphone-market-share

https://telecoms.com/506561/smartphone-market-headed-for-mid-range-mediocrity/