3GPP TSG-RAN WG4 #107	R4-2309356

Incheon, Korea, May 22nd – 26th, 2023

Agenda item:	8.4.2.1

Source: 	Qualcomm Incorporated

Title: 	Considerations on CA_n5-n105

Document for:	Approval

# Introduction

Considerations on CA_n5-n105 are provided in this contribution.

# Discussion

## The objectives of SI

The objectives of SI are as follows:

- Investigate the feasibility and solutions to enable simultaneous transmission on two UL bands with 2Tx and simultaneous reception on two or three bands for the following sub-1GHz band combinations for smart phone form factor

Table 1: Summary of band combinations considered in the SI

| Configuration | Uplink Configuration | fallback configurations (Status) | Supported operators |
| --- | --- | --- | --- |
| CA_n5A-n105A | CA_n5A-n105A | DL_n5A-n105A_UL_n5A_BCS0 (New) DL_n5A-n105A_UL_n105A_BCS0 (New) | Spark NZ |
| CA_n28A-n105A | CA_n28A-n105A | DL_n28A-n105A_UL_n28A_BCS0 (New) DL_n28A-n105A_UL_n105A_BCS0 (New) | Spark NZ |
| CA_n26A-n28A | CA_n26A-n28A | DL_n26A-n28A_UL_n28A_BCS0 (Completed in RAN4#106) DL_n26A-n28A_UL_n26A_BCS0 (New) | Telstra |
| CA_n5A-n28A-n105A1 | CA_n5A-n28A | DL_n5A-n28A_UL_n5A-n28A_BCS0 (Ongoing in a separate WI) DL_n5A-n28A-n105A_UL_n5A_BCS0 (New) DL_n5A-n28A-n105A_UL_n28A_BCS0 (New) | Spark NZ |
| CA_n5A-n28A-n105A1 | CA_n5A-n105A | DL_n5A-n105A_UL_n5A-n105A_BCS0 (New) DL_n5A-n28A-n105A_UL_n5A_BCS0 (New) DL_n5A-n28A-n105A_UL_n105A_BCS0 (New) | Spark NZ |
| CA_n5A-n28A-n105A1 | CA_n28A-n105A | DL_n28A-n105A_UL_n28A-n105A_BCS0 (New) DL_n5A-n28A-n105A_UL_n28A_BCS0 (New) DL_n5A-n28A-n105A_UL_n105A_BCS0 (New) | Spark NZ |

Table 2: Bandwidth combination set for each band combination

| - NR CA configuration | - Uplink CA configuration or single uplink carrier | - NR Band | - Channel bandwidth (MHz) | - Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5A-n105A | CA_n5A-n105A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n28A-n105A | CA_n28A-n105A | n28 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n26A-n28A | CA_n26A-n28A | n26 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
| CA_n5A-n28A-n105A1 | CA_n5A-n28A CA_n5A-n105A CA_n28A-n105A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |

  - The following aspects need be studied

    - UE RF architecture including n-plexing, PA

    - Study feasibility of low band wideband antenna and multiple low band narrowband antennas.

    - Evaluate the MSD requirements if needed.

    - Evaluate if UE complexity can be reduced based on the frequency range and BCS proposed by operators.

  - Power class 3 (PC3) is considered in this study

- Identify potential impacts to relevant RAN4 requirements.

NOTE 1: The study of 3 band combination can only start after completion of 2 bands fallbacks

NOTE 2: Check at RAN4#107 whether CA_n5A-n28A-n105A could be included in the SI based on study progress for the fall back combinations listed in Table 1.

## WF from RAN4#106bis-e

Issue 2-1-1: RF architecture discussion

Agreement:

All the RF architectures can be captured into the TR for further study.

- Two-antenna architecture

  - Architecture #1: n5UL+n5DL+n105UL+n105DL quadplexer on antenna 1 and n5DL+n105DL duplexer on antenna 2

  - Architecture #2: n5UL+n5DL+n105DL triplexer on antenna 1 and n105UL+n105DL+5DL triplexer on antenna 2

- Three-antenna architecture

  - Architecture #3: n5UL+n5DL duplexer on antenna 1, n105UL+n105DL duplexer on antenna 2 and n5DL+n105DL duplexer on antenna 3

Issue 2-1-2: Which RF architecture can be used as baseline to study the RF requirements

Agreements:

Two-antenna architecture with dual triplexer can be used as a baseline to study Tib and Rib. For other requirements except for Tib and Rib, both two-antenna architecture and three-antenna architecture are not precluded.

Issue 2-2-1: MSD due to harmonic and IMD

Agreements:

- There is no MSD issue due to harmonic or harmonic mixing interference

- There is no MSD issue due to IMD interference when both Band n5 and Band n105 transmit the UL signals for CA_n5-n105.

Issue 2-2-2: MSD due to cross band isolation

- Proposals

  - Option 1: Study of cross band MSD in both n105 and n5 DL due to transmitter noise floor and accounting for UL filters rejection is needed, but should not result in significant MSD.

  - Option 2: There is no MSD due to cross band isolation (n105 Tx => n5Rx). FFS the MSD due to cross band isolation (n5 Tx => n105Rx) based on the attenuation assumption.

Way forwards:

FFS on MSD due to cross band isolation based on the assumption of filter rejection, triplexer isolation, etc. Companies are encouraged to provide analysis based on certain assumption.

## Analysis

We did a brief analysis for the Two Triplexers RF architecture option. One aspect that needs to be pointed out is that n105 is a very new band, with very challenging frequency arrangements from RF filtering point of view. Hence, especially given the band is new and implementations are just under development, we don’t see it possible to optimize the CA characteristics at the expense of attenuation at CH36 and n105 TX/RX isolations. Please note that the numbers are not derived from dedicated triplexers, but instead from what can be achieved by matching DPX and RX filters together, without optimizations which may offer improved performance over time. We are targeting to provide more detailed simulations during the WI phase, so it is possible that initial results change quite a bit.

With this in mind, we are using the following assumptions in the analysis.

- n5 ΔTIB=0.5dB

- n105 ΔTIB=0.5dB

- n5 ΔRIB=0.2dB

- n105 ΔRIB=0.2dB

- Cross band TX isolation at n5 RX ≥50dB

- Cross band RX isolation at n5 RX ≥42dB

- Cross band TX rejection at n5 RX ≥38dB

- Cross band TX attenuation at n5 DRX≥40dB

- Cross band TX isolation at n105 RX ≥30dB

- Cross band RX isolation at n105 RX ≥37dB

- Cross band TX rejection at n105 RX ≥32dB

- Cross band TX attenuation at n105 DRX≥40dB

The frequency arrangements were as follows.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n105 | n5 | 693 | 20 | 15 | 20 (RBstart=86) | 871.5 | 5 | TBD | >ACLR2 |
| n5 | n105 | 834 | 20 | 15 | 20 (RBstart=0) | 649.5 | 5 | TBD | >ACLR2 |

MSD for n5 DL:

MSD for n105 DL:

# Conclusions

MSD analysis for CA_n5-n105 was provided with the following initial results. We are targeting to provide more detailed simulations during the WI phase, so it is possible that initial results change quite a bit.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n105 | n5 | 693 | 20 | 15 | 20 (RBstart=86) | 871.5 | 5 | 2.6 | >ACLR2 |
| n5 | n105 | 834 | 20 | 15 | 20 (RBstart=0) | 649.5 | 5 | 5.6 | >ACLR2 |

| Inter-band CA combination | NR Band | ΔRIB,c (dB) | ΔTIB,c (dB) |
| --- | --- | --- | --- |
| CA_n5-n105 | n5 | 0.2 | 0.5 |
|  | n105 | 0.2 | 0.5 |

# Reference

- RP-230789, “Study on enhancement for sub-1GHz NR band combinations”, Spark NZ, Huawei, HiSilicon