3GPP TSG-RAN WG4 #107	R4-2309357

Incheon, Korea, May 22nd – 26th, 2023

Agenda item:	8.4.2.2

Source: 	Qualcomm Incorporated

Title: 	Considerations on CA_n28-n105

Document for:	Approval

# Introduction

Considerations on CA_n28-n105 are provided in this contribution.

# Discussion

## The objectives of SI

The objectives of SI [1] are as follows:

- Investigate the feasibility and solutions to enable simultaneous transmission on two UL bands with 2Tx and simultaneous reception on two or three bands for the following sub-1GHz band combinations for smart phone form factor

Table 1: Summary of band combinations considered in the SI

| Configuration | Uplink Configuration | fallback configurations (Status) | Supported operators |
| --- | --- | --- | --- |
| CA_n5A-n105A | CA_n5A-n105A | DL_n5A-n105A_UL_n5A_BCS0 (New) DL_n5A-n105A_UL_n105A_BCS0 (New) | Spark NZ |
| CA_n28A-n105A | CA_n28A-n105A | DL_n28A-n105A_UL_n28A_BCS0 (New) DL_n28A-n105A_UL_n105A_BCS0 (New) | Spark NZ |
| CA_n26A-n28A | CA_n26A-n28A | DL_n26A-n28A_UL_n28A_BCS0 (Completed in RAN4#106) DL_n26A-n28A_UL_n26A_BCS0 (New) | Telstra |
| CA_n5A-n28A-n105A1 | CA_n5A-n28A | DL_n5A-n28A_UL_n5A-n28A_BCS0 (Ongoing in a separate WI) DL_n5A-n28A-n105A_UL_n5A_BCS0 (New) DL_n5A-n28A-n105A_UL_n28A_BCS0 (New) | Spark NZ |
| CA_n5A-n28A-n105A1 | CA_n5A-n105A | DL_n5A-n105A_UL_n5A-n105A_BCS0 (New) DL_n5A-n28A-n105A_UL_n5A_BCS0 (New) DL_n5A-n28A-n105A_UL_n105A_BCS0 (New) | Spark NZ |
| CA_n5A-n28A-n105A1 | CA_n28A-n105A | DL_n28A-n105A_UL_n28A-n105A_BCS0 (New) DL_n5A-n28A-n105A_UL_n28A_BCS0 (New) DL_n5A-n28A-n105A_UL_n105A_BCS0 (New) | Spark NZ |

Table 2: Bandwidth combination set for each band combination

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5A-n105A | CA_n5A-n105A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n28A-n105A | CA_n28A-n105A | n28 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n26A-n28A | CA_n26A-n28A | n26 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
| CA_n5A-n28A-n105A1 | CA_n5A-n28A CA_n5A-n105A CA_n28A-n105A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |

  - The following aspects need be studied

    - UE RF architecture including n-plexing, PA

    - Study feasibility of low band wideband antenna and multiple low band narrowband antennas.

    - Evaluate the MSD requirements if needed.

    - Evaluate if UE complexity can be reduced based on the frequency range and BCS proposed by operators.

  - Power class 3 (PC3) is considered in this study

- Identify potential impacts to relevant RAN4 requirements.

NOTE 1: The study of 3 band combination can only start after completion of 2 bands fallbacks

NOTE 2: Check at RAN4#107 whether CA_n5A-n28A-n105A could be included in the SI based on study progress for the fall back combinations listed in Table 1.

## WF from RAN4#106bis-e

# UE RF architecture consideration

Four potential UE RF architectures for CA_n28A-n105A have been considered as shown below:

- Option 1a: 2-antenna implementation with a triplexer in main path and a duplexer in diversity path

- Option 1b: 2-antenna implementation with dual quadplexers (or triplexer + quadplexer as there is no frequency gap between n105 UL and n28 block A UL) in main path and a duplexer in diversity path

- Option 2: 2-antenna implementation with dual triplexers in main path and a triplexer in diversity path

- Option 3: 3-antenna implementation

Observation: Options 1a and 1b were considered rather challenging and impracticable for implementation due to the relatively wide frequency range in the middle sub-band for the triplexer.

Agreements:

- Only Option 2 and Option 3 UE architectures will be captured in the study item.

- Baseline architecture for UE RF requirements development will be further discussed.

# 2UL IMD issue and UL configuration

2UL IMD3/IM5 may fall into either n105 DL or n28 DL or both DL bands simultaneously to cause REFSENS degradation (MSD), as conceptually illustrated below:

Observation: MSD caused by 2UL IMD3 could be relatively high (> 36 dB) based on preliminary analysis which would render the 2UL operation rather impracticable for the combination.

Agreement:

- Only single UL configuration is considered in the study item where UL can be in either n28 or n105.

# Cross-band interference

Apart from the 2UL IMD concern, the combination may also be subject to cross-band UL to DL interference issue due to frequency proximity even under single UL operation where the REFSENS impact needs to be further analyzed for different UE architectures.

Agreement:

- Further study on REFSENS impact due to cross-band UL to DL interference

## Analysis

We did a brief analysis for Option 2/ Triplexers RF architecture option. One aspect that needs to be pointed out is that n105 is a very new band, with very challenging frequency arrangements from RF filtering point of view. Hence, especially given the band is new and implementations are just under development, we don’t see it possible to optimize the CA characteristics at the expense of attenuation at CH36 and n105 TX/RX isolations. Please note that the numbers are not derived from dedicated triplexers, but instead from what can be achieved by matching DPX and RX filters together, without optimizations which may offer improved performance over time. We are targeting to provide simulation results based on triplexers during the WI phase, so it is possible that initial results change quite a bit.

With this in mind, we are using the following assumptions in the analysis.

- n28 ΔTIB=0.8dB

- n105 ΔTIB=0.8dB

- n28 ΔRIB=0.4dB

- n105 ΔRIB=0.4dB

- Cross band TX isolation at n28 RX ≥35dB

- Cross band RX isolation at n28 RX ≥35dB

- Cross band TX rejection at n28 RX ≥35dB

- Cross band TX attenuation at n28 DRX≥31dB

- Cross band TX isolation at n105 RX ≥32dB

- Cross band RX isolation at n105 RX ≥37dB

- Cross band TX rejection at n105 RX ≥37dB

- Cross band TX attenuation at n105 DRX≥38dB

The frequency arrangements were as follows.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n105 | n28 | 693 | 20 | 15 | 20 (RBstart=86) | 760.5 | 5 | TBD | ACLR2 |
| n28 | n105 | 718 | 30 | 15 | 25 (RBstart=0) | 649.5 | 5 | TBD | >ACLR2 |

n28 MSD:

n105 MSD:

Regarding the RF architecture, the one to be used in specifying the requirements, our view is that 3GPP specification should enable feasible implementations. In spirit of this, the requirements should be specified allowing both 2 and 3 antenna implementations. In practice this means that the dTib, dRib, MSD are specified according to worse case among options 2 and 3. In practice, it seems that Option 2 would result as worse case, unless someone justifies that the MSD using option 3 is larger than that using option 2.

Proposal 1: Use Option 2 as baseline to derive requirements unless the MSD using Option 3 is larger than the respective MSD in Option 2

# Conclusions

MSD analysis for CA_n28-n105 was provided with the following initial results. We are targeting to provide more detailed simulations during the WI phase, so it is possible that initial results change quite a bit.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n105 | n28 | 693 | 20 | 15 | 20 (RBstart=86) | 760.5 | 5 | 10.8 | >ACLR2 |
| n28 | n105 | 718 | 30 | 15 | 25 (RBstart=0) | 649.5 | 5 | 10.1 | ACLR2 |

| Inter-band CA combination | NR Band | ΔRIB,c (dB) | ΔTIB,c (dB) |
| --- | --- | --- | --- |
| CA_n28-n105 | n28 | 0.4 | 0.8 |
|  | n105 | 0.4 | 0.8 |

Proposal 1: Use Option 2 as baseline to derive requirements unless the MSD using Option 3 is larger than the respective MSD in Option 2

# Reference

- RP-230789, “Study on enhancement for sub-1GHz NR band combinations”, Spark NZ, Huawei, HiSilicon