3GPP TSG-RAN WG4 #107	R4-2309360

Incheon, Korea, May 22nd – 26th, 2023

Agenda item:	7.30.2.2

Source: 	Qualcomm Incorporated

Title: 	Considerations on CA_n5-n28

Document for:	Approval

# Introduction

Analysis and proposals on CA_n5-n28 are provided in this contribution.

# Discussion

## Objectives of the WI

Specify the following requirements based on down-selection of options of the outcome of SI

- Define Tib/Rib for the band combinations.

- Define REFRENS degradation for some specific band combinations if it’s needed.

- Define other necessary requirements for some specific band combinations if they are identified to be needed.

For CA_n5-n8, study the feasibility of non-simultaneous n5 DL + n8 UL with the existing specifications [RAN2]

- Note: RAN2 work will be triggered by LS from RAN4

An overview table of the CA combinations studied in this WI is provided here:

Table 1: Band combinations studied in this WI

| Configuration | Uplink configuration | Supported operators |
| --- | --- | --- |
| CA_n5-n8 | CA_n5-n8 | China Telecom, Spark NZ, China Unicom |
| CA_n5-n28 | CA_n5-n28 | Spark NZ |
| CA_n8-n20-n28 | CA_n8-n20, CA_n8-n28, CA_n20-n28 | Vodafone, Telecom Italia, Orange, Deutsche Telekom, Telia Company |

The channel bandwidths per operating band for each band combination are listed in the following table.

Table 2: Channel bandwidths per operating band for each band combination

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5A-n8A | CA_n5A-n8A | n5 | 5, 10 | 0 |
|  |  | n8 | 5, 10 |  |
| CA_n5A-n28A | CA_n5A-n28A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
| CA_n8A-n20A-n28A | CA_n8A-n20A CA_n8A-n28A CA_n20A-n28A | n8 | 5, 10, 15, 20 | 0 |
|  |  | n20 | 5, 10, 15, 20 |  |
|  |  | n28 | 5, 10, 15, 20 |  |

## WF from RAN4#106bis-e

Issue 3-1: Tib/Rib for CA_n5-n28

<Agreement>: Option 1: Table 5.2.2.4-0 in TR 38.872

Table 5.2.2.4-0: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n5A-n28A | n5 | 0.7 |
|  | n28 | 0.7 |

Table 5.2.2.4-0a: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n5A-n28A | n5 | 0.2 |
|  | n28 | 0.2 |

Issue 3-2: MSD for 1UL CA_n5-n28

<Agreement>: The following MSD test point for cross band isolation issue with one UL band is agreeable.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n28 | 834 | 20 | 15 | 20 (RBstart=0) | 800.5 | 5 | 17.5 | ACLR2 |

Issue 3-3: MSD for 2UL CA_n5-n28

<Agreement>: The following MSD test point for cross band isolation issue with one UL band is agreeable by averaging the values in TR 38.872. RAN4 agrees to include a generic sentence into 2UL non-IMD table, which describes the generic principle for MSD in case of no IMD but still nonzero 2UL cross-band CA (i.e. the widest BW’s are used for both bands).

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n28 | 834 | 20 | 15 | 20 (RBstart=0) | 788 | 30 | 3.1 | ACLR2 from n5 UL band ACLR1+ACLR2 from n28 UL band |

# Discussion

As agreed in WF, Issue 3-3, 2UL MSD for CA_n5A-n28A is specified. This would in principle be valid case for any Inter-band combination which meets all three criteria listed below:

- Inter-band UL CA specified

- 1UL Cross-band CA MSD specified

- No 2UL IMD’s

The reason why 2UL “Non-IMD” MSD is relevant is because in case when there are neither IMD MSD or 2UL Non-IMD MSD specified, the 2UL REFSENS is considered exception-free.

In our view RAN4 should be very mindful when adding of new cases into specifications. We suggest limiting the scope of this new requirement to the extent possible including the alternative to revert the earlier agreement.

Observation 1: The 2UL Cross-band MSD is relevant to cases which have 1UL cross-band MSD but no REFSENS exception due to IMD MSD.

To limit the scope of 2UL Non-IMD MSD, we suggest the following options:

Proposal 1: Limit the scope of this new 2UL non-IMD MSD into combinations by choosing one of the options below

Option 1: Revert the earlier agreement and do not specify 2UL Non-IMD MSD

Option 2: Limit the scope of 2UL Non-IMSD MSD to combinations specified from Rel-18 onwards

Option 3: Other, please specify

If RAN4 decides to move forward with specifying 2UL Non-IMD MSD, we propose to create a dedicated table for those combinations in chapter 7.3.6A

Proposal 2: If RAN4 decides to move forward with specifying 2UL Non-IMD MSD, a dedicated table for those combinations is created in chapter 7.3.6A

# Conclusions

Considerations on 2UL Cross-band MSD and how to capture that in the specifications are provided with the following observation and proposal.

Observation 1: The 2UL Cross-band MSD is relevant to cases which have 1UL cross-band MSD but no REFSENS exception due to IMD MSD.

Proposal 1: Limit the scope of this new 2UL non-IMD MSD into combinations

Option 1: Revert the earlier agreement and do not specify 2UL Non-IMD MSD

Option 2: Limit the scope of 2UL Non-IMSD MSD to combinations specified from Rel-18 onwards

Option 3: Other, please specify

Proposal 2: If RAN4 decides to move forward with specifying 2UL Non-IMD MSD, a dedicated table for those combinations is created in chapter 7.3.6A

# Reference

- RP-230764, “New WID on enhancement for 700/800/900MHz band combinations”, CATT, China Telecom