3GPP TSG-RAN4 Meeting #107	R4-2309362

# Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | 0967 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Big CR for Rel-18 Dual Connectivity (DC) of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CHTTL |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | DC_R18_1BLTE_1BNR_2DL2UL-Core |  |  |  |  |  | Date: |  |  | 2023-05-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Completed DC configurations after RAN4#107 are specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Completed EN-DC configurations from the TR (R4-2309361): DC_8B_n78A with UL DC_8B_n78A DC_1A_n105A DC_3A_n105A DC_7A_n105A DC_71A_n12A Some configurations are completed via draft CR approach. This CR covers the following endorsed draft CR. Endorsed draft CR in RAN4#106bis: R4-2306488 Draft CR for TS 38.101-3 to add DC_7A-7A_n40A R4-2304959 Draft CR 38.101-3 to add 2CA combinations of 2 4 66 n258 R4-2305598 draft CR for missing Tx requirement for intra-band contiguous NE-DC with single switched UL only Endorsed draft CR in RAN4#107: R4-2307048 draft CR to add new EN-DC FR2 configurations R4-2310367 draft CR 38.101-3 for corrections in harmonic mixing table |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Completed DC configurations are not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5B.4.1, 5.5B.5.1, 6.2B.2.1a (new), 6.2B.4.1.1a (new), 6.2B.4.2.3.1, 6.3B.2a (new), 6.3B.3a (new), 6.4B.1.1a (new), 7.3B.2.3.1, 7.3B.2.3.2, 7.3B.2.3.4, 7.3B.2.3.5.1, 7.3B.3.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.521-3 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

5.5B.4	Inter-band EN-DC within FR1

5.5B.4.1	Inter-band EN-DC configurations within FR1 (two bands)

Table 5.5B.4.1-1: Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed | DL interruption allowed (Note 14) |
| --- | --- | --- | --- |
| DC_1A_n3A DC_1C_n3A | DC_1A_n3A DC_1C_n3A | DC_1_n3 |  |
| DC_1A_n5A | DC_1A_n5A | No |  |
| DC_1A_n7A DC_1A_n7B | DC_1A_n7A DC_1A_n7B | No |  |
| DC_1A-1A_n7A DC_1A-1A_n7B | DC_1A_n7A | No |  |
| DC_1A_n8A | DC_1A_n8A | No |  |
| DC_1A_n20A | DC_1A_n20A | No |  |
| DC_1A_n28A | DC_1A_n28A | No |  |
| DC_1A_n26A | DC_1A_n26A | No |  |
| DC_1A-1A_n28A | DC_1A_n28A | No |  |
| DC_1A_n38A DC_1C_n38A | DC_1A_n38A | No |  |
| DC_1A_n40A DC_1A_n40B | DC_1A_n40A | No |  |
| DC_1A_n41A7 | DC_1A_n41A | No |  |
| DC_1A_n50A | DC_1A_n50A | No |  |
| DC_1A_n51A | DC_1A_n51A | No |  |
| DC_1A_n71A DC_1A_n71B | DC_1A_n71A | No |  |
| DC_1A_n77A7 DC_1A_n77C7 | DC_1A_n77A | DC_1_n77 | No |
| DC_1A_n77(2A)7 DC_1A_n77(3A)7 | DC_1A_n77A | DC_1_n77 | No |
| DC_1A_n78A7 DC_1A_n78C7, 21 | DC_1A_n78A, 21 | No | No |
| DC_1A_n78(2A)7 DC_1A_n78(A-C)7 | DC_1A_n78A | No | No |
| DC_1A-1A_n78A | DC_1A_n78A | No | No |
| DC_1A_n79A7 DC_1A_n79C7 | DC_1A_n79A | No | No |
| DC_1A_n105A | DC_1A_n105A | No |  |
| DC_2A_n5A | DC_2A_n5A | No |  |
| DC_2A-2A_n5A | DC_2A_n5A | No |  |
| DC_2A_n7A | DC_2A_n7A | No |  |
| DC_2A_n7(2A) | DC_2A_n7A | No |  |
| DC_2A-2A_n7A | DC_2A_n7A | No |  |
| DC_2A_n12A | DC_2A_n12A | No |  |
| DC_2A_n25A11, 13, 20 | N/A | N/A |  |
| DC_2A_n28A | DC_2A_n28A | No |  |
| DC_2A_n30A | DC_2A_n30A | No |  |
| DC_2A-2A_n30A | DC_2A_n30A | No |  |
| DC_2A_n38A | DC_2A_n38A | No |  |
| DC_2A-2A_n38A | DC_2A_n38A | No |  |
| DC_2A_n41A DC_2A_n41C DC_2C_n41A | DC_2A_n41A DC_2C_n41A | No |  |
| DC_2A_n41(2A) | DC_2A_n41A | No |  |
| DC_2A-2A_n41A | DC_2A_n41A | No |  |
| DC_2A_n46A | DC_2A_n46A | No |  |
| DC_2A_n48A DC_2A_n48B | DC_2A_n48A | No |  |
| DC_2A_n66A | DC_2A_n66A | DC_2_n66 |  |
| DC_2A_n66(2A) | DC_2A_n66A | DC_2_n66 |  |
| DC_2A-2A_n66A | DC_2A_n66A | DC_2_n66 |  |
| DC_2A_n71A DC_2A_n71B DC_2C_n71A | DC_2A_n71A | No |  |
| DC_2A-2A_n71A | DC_2A_n71A | No |  |
| DC_2A_n77A DC_2A_n77C21 | DC_2A_n77A21 | DC_2_n77 |  |
| DC_2A_n77(2A) 21 | DC_2A_n77A21 | DC_2_n77 |  |
| DC_2A-2A_n77A21 DC_2A-2A_n77C21 | DC_2A_n77A21 | DC_2_n77 |  |
| DC_2A-2A_n77(2A) 21 | DC_2A_n77A21 | DC_2_n77 |  |
| DC_2A_n78A | DC_2A_n78A | DC_2_n78 |  |
| DC_2A-2A_n78(2A) | DC_2A_n78A | DC_2_n78 |  |
| DC_2A_n78(2A) | DC_2A_n78A | DC_2_n78 |  |
| DC_2A-2A_n78A | DC_2A_n78A | DC_2_n78 |  |
| DC_3A_n1A DC_3C_n1A | DC_3A_n1A DC_3C_n1A | DC_3_n1 |  |
| DC_3A-3A_n1A | DC_3A_n1A | DC_3_n1 |  |
| DC_3A_n5A DC_3C_n5A | DC_3A_n5A | DC_3_n5 |  |
| DC_3A_n7A DC_3A_n7B DC_3C_n7A DC_3C_n7B | DC_3A_n7A DC_3A_n7B DC_3C_n7A | No |  |
| DC_3A-3A_n7A DC_3A-3A_n7B | DC_3A_n7A | No |  |
| DC_3A_n8A | DC_3A_n8A | No |  |
| DC_3A-3A_n8A | DC_3A_n8A | No |  |
| DC_3A_n20A DC_3C_n20A | DC_3A_n20A | No |  |
| DC_3A_n26A DC_3C_n26A | DC_3A_n26A DC_3C_n26A | Yes |  |
| DC_3A_n28A DC_3C_n28A | DC_3A_n28A DC_3C_n28A | No |  |
| DC_3A_n34A | DC_3A_n34A | No |  |
| DC_3A_n38A DC_3C_n38A | DC_3A_n38A | No |  |
| DC_3A_n40A DC_3A_n40B | DC_3A_n40A | No |  |
| DC_3A_n41A7 DC_3A_n41C DC_3C_n41A7 | DC_3A_n41A DC_3C_n41A | DC_3_n41 | No |
| DC_3A_n50A | DC_3A_n50A | No |  |
| DC_3A_n51A | DC_3A_n51A | No |  |
| DC_3A_n71A DC_3A_n71B | DC_3A_n71A | No |  |
| DC_3A_n77A7 DC_3A_n77C7 DC_3C_n77A7 | DC_3A_n77A DC_3C_n77A | DC_3_n77 | No |
| DC_3A_n77(2A)7 DC_3A_n77(3A)7 DC_3C_n77(2A)7 | DC_3A_n77A DC_3C_n77A | DC_3_n77 | No |
| DC_3A-3A_n77A7 | DC_3A_n77A | DC_3_n77 | No |
| DC_3A_n78A7 DC_3A_n78C7 DC_3C_n78A7 | DC_3A_n78A DC_3C_n78A | DC_3_n78 | No |
| DC_3A_n78(2A)7 DC_3A_n78(A-C)7 DC_3C_n78(2A)7 | DC_3A_n78A DC_3C_n78A | DC_3_n78 | No |
| DC_3A-3A_n78A7, 21 | DC_3A_n78A21 | DC_3_n78 | No |
| DC_3A_n79A7 DC_3A_n79C7 DC_3C_n79A7 | DC_3A_n79A DC_3C_n79A | No | No |
| DC_3A_n105A | DC_3A_n105A | No |  |
| DC_4A_n2A | DC_4A_n2A | No |  |
| DC_4A_n5A | DC_4A_n5A | DC_4_n5 |  |
| DC_4A_n7A | DC_4A_n7A | No |  |
| DC_4A_n28A | DC_4A_n28A | No |  |
| DC_4A_n38A | DC_4A_n38A | No |  |
| DC_4A_n41A | DC_4A_n41A | No |  |
| DC_4A_n78A | DC_4A_n78A | No |  |
| DC_4A_n78(2A) | DC_4A_n78A | No |  |
| DC_5A_n1A | DC_5A_n1A | No |  |
| DC_5A_n2A DC_5B_n2A | DC_5A_n2A | No |  |
| DC_5A_n2(2A) | DC_5A_n2A | No |  |
| DC_5A-5A_n2A | DC_5A_n2A | No |  |
| DC_5A_n3A | DC_5A_n3A | DC_5_n3 |  |
| DC_5A_n7A | DC_5A_n7A | DC_5_n7 |  |
| DC_5A_n7(2A) | DC_5A_n7A | DC_5_n7 |  |
| DC_5A_n12A | DC_5A_n12A | No |  |
| DC_5A_n25A | DC_5A_n25A | No |  |
| DC_5A_n30A | DC_5A_n30A | No |  |
| DC_5A_n38A | DC_5A_n38A | DC_5_n38 |  |
| DC_5A_n40A | DC_5A_n40A | No |  |
| DC_5A_n41A | DC_5A_n41A | No |  |
| DC_5A_n48A DC_5A_n48B | DC_5A_n48A | No |  |
| DC_5A_n66A DC_5B_n66A | DC_5A_n66A | DC_5_n66 |  |
| DC_5A-5A_n66A | DC_5A_n66A | DC_5_n66 |  |
| DC_5A_n77A DC_5A_n77C21 | DC_5A_n77A21 | No |  |
| DC_5A_n77(2A) 21 DC_5A_n77(3A) | DC_5A_n77A21 | No |  |
| DC_5A_n71A | DC_5A_n71A | No |  |
| DC_5A_n78A7 DC_5A_n78C7 | DC_5A_n78A | No | No |
| DC_5A_n78(2A)7 DC_5A_n78(A-C)7 | DC_5A_n78A | No | No |
| DC_5A_n79A | DC_5A_n79A | No | No |
| DC_7A_n1A DC_7C_n1A | DC_7A_n1A DC_7C_n1A | No |  |
| DC_7A-7A_n1A | DC_7A_n1A | No |  |
| DC_7A_n2A DC_7C_n2A | DC_7A_n2A | No |  |
| DC_7A_n2(2A) | DC_7A_n2A | No |  |
| DC_7A_n3A DC_7C_n3A | DC_7A_n3A DC_7C_n3A | No |  |
| DC_7A_n5A DC_7C_n5A | DC_7A_n5A DC_7C_n5A | DC_7_n5 |  |
| DC_7A-7A_n5A | DC_7A_n5A | DC_7_n5 |  |
| DC_7A_n8A | DC_7A_n8A | No |  |
| DC_7A-7A_n8A | DC_7A_n8A | No |  |
| DC_7A_n12A | DC_7A_n12A | No |  |
| DC_7A-7A_n78(2A)7 | DC_7A_n78A | No |  |
| DC_7A_n20A | DC_7A_n20A | No |  |
| DC_7A_n25A DC_7C_n25A | DC_7A_n25A | No |  |
| DC_7A_n26A DC_7C_n26A | DC_7A_n26A DC_7C_n26A | Yes |  |
| DC_7A-7A_n25A | DC_7A_n25A | No |  |
| DC_7A_n28A DC_7C_n28A | DC_7A_n28A DC_7C_n28A | No |  |
| DC_7A_n40A | DC_7A_n40A | Yes |  |
| DC_7A-7A_n40A | DC_7A_n40A | Yes |  |
| DC_7A-7A_n28A | DC_7A_n28A | No |  |
| DC_7A_n51A | DC_7A_n51A | No |  |
| DC_7A_n66A DC_7C_n66A | DC_7A_n66A | No |  |
| DC_7A-7A_n66A | DC_7A_n66A | No |  |
| DC_7A_n71A | DC_7A_n71A | No |  |
| DC_7A_n77A7 DC_7C_n77A | DC_7A_n77A | No |  |
| DC_7A_n77(2A) DC_7A_n77(3A) DC_7C_n77(2A) | DC_7A_n77A | No |  |
| DC_7A-7A_n77A7 | DC_7A_n77A | No |  |
| DC_7A-7A_n77(2A) DC_7A-7A_n77(3A) | DC_7A_n77A | No |  |
| DC_7A_n78A7 DC_7C_n78A7 DC_7A_n78C7 | DC_7A_n78A DC_7C_n78A | No |  |
| DC_7A_n78(2A)7 DC_7A_n78(A-C)7 DC_7C_n78(2A)7 | DC_7A_n78A DC_7C_n78A | No |  |
| DC_7A-7A_n78A7, 21 DC_7A-7A_n78C7 | DC_7A_n78A21 | No |  |
| DC_7A-7A_n78(A-C)7 | DC_7A_n78A | No |  |
| DC_7A_n79A DC_7A_n79C | DC_7A_n79A | No |  |
| DC_7A_n105A | DC_7A_n105A | No |  |
| DC_8A_n1A | DC_8A_n1A | No |  |
| DC_8A_n2A | DC_8A_n2A | DC_8_n2 |  |
| DC_8A_n3A | DC_8A_n3A | No |  |
| DC_8A_n7A | DC_8A_n7A | No |  |
| DC_8A_n20A | DC_8A_n20A | Yes |  |
| DC_8A_n28A | DC_8A_n28A | No |  |
| DC_8A_n34A | DC_8A_n34A | No |  |
| DC_8A_n38A | DC_8A_n38A | No |  |
| DC_8A_n39A | DC_8A_n39A | No |  |
| DC_8A_n40A7 | DC_8A_n40A | No |  |
| DC_8A_n41A7 DC_8A_n41C | DC_8A_n41A | No | No |
| DC_8A_n41(2A) | DC_8A_n41A | No | No |
| DC_8A_n77A7 | DC_8A_n77A | No | No |
| DC_8A_n77(2A)7 DC_8A_n77(3A)7 | DC_8A_n77A | No | No |
| DC_8A_n78A7 DC_8B_n78A7 | DC_8A_n78A DC_8B_n78A | No | No |
| DC_8A_n78(2A)7 | DC_8A_n78A | No | No |
| DC_8A_n79A7 DC_8A_n79C | DC_8A_n79A DC_8A_n79C | No | No |
| DC_8A_n93A | DC_8A_n93A_ULSUP-TDM | N/A |  |
| DC_8A_n94A | DC_8A_n94A_ULSUP-TDM | N/A |  |
| DC_11A_n1A | DC_11A_n1A | No |  |
| DC_11A_n3A | DC_11A_n3A | No |  |
| DC_11A_n28A | DC_11A_n28A | No |  |
| DC_11A_n41A7 | DC_11A_n41A | No |  |
| DC_11A_n77A7 | DC_11A_n77A | No | No |
| DC_11A_n77(2A)7 DC_11A_n77(3A)7 | DC_11A_n77A | No | No |
| DC_11A_n78A7 | DC_11A_n78A | No | No |
| DC_11A_n78(2A) | DC_11A_n78A | No | No |
| DC_11A_n79A7 | DC_11A_n79A | No |  |
| DC_12A_n2A | DC_12A_n2A | No |  |
| DC_12A_n2(2A) | DC_12A_n2A | No |  |
| DC_12A_n5A | DC_12A_n5A | No |  |
| DC_12A_n7A | DC_12A_n7A | No |  |
| DC_12A_n7(2A) | DC_12A_n7A | No |  |
| DC_12A_n25A | DC_12A_n25A | No |  |
| DC_12A_n30A | DC_12A_n30A | No |  |
| DC_12A_n38A | DC_12A_n38A | No |  |
| DC_12A_n41A | DC_12A_n41A | No |  |
| DC_12A_n66A | DC_12A_n66A | No |  |
| DC_12A_n66(2A) | DC_12A_n66A | No |  |
| DC_12A_n71A | DC_12A_n71A18,19 | DC_12_n71 |  |
| DC_12A_n77A | DC_12A_n77A | DC_12_n77 |  |
| DC_12A_n77(2A) 21 | DC_12A_n77A21 | DC_12_n77 |  |
| DC_12A_n78A | DC_12A_n78A | DC_12_n78 |  |
| DC_12A_n78(2A) | DC_12A_n78A | DC_12_n78 |  |
| DC_13A_n2A | DC_13A_n2A | No |  |
| DC_13A_n5A | DC_13A_n5A | DC_13_n5 |  |
| DC_13A_n7A | DC_13A_n7A | No |  |
| DC_13A_n7(2A) | DC_13A_n7A | No |  |
| DC_13A_n25A | DC_13A_n25A | No |  |
| DC_13A_n48A DC_13A_n48B | DC_13A_n48A | No |  |
| DC_13A_n66A | DC_13A_n66A | No |  |
| DC_13A_n71A | DC_13A_n71A | No |  |
| DC_13A_n77A DC_13A_n77C21 | DC_13A_n77A21 | No |  |
| DC_13A_n78A | DC_13A_n78A | No |  |
| DC_13A_n78(2A) | DC_13A_n78A | No |  |
| DC_14A_n2A | DC_14A_n2A | No |  |
| DC_14A_n5A | DC_14A_n5A | DC_14_n5 |  |
| DC_14A_n30A | DC_14A_n30A | No |  |
| DC_14A_n66A | DC_14A_n66A | No |  |
| DC_14A_n77A | DC_14A_n77A | No |  |
| DC_14A_n77(2A) 21 | DC_14A_n77A21 | No |  |
| DC_18A_n3A | DC_18A_n3A | No |  |
| DC_18A_n28A | DC_18A_n28A | No |  |
| DC_18A_n41A16 | DC_18A_n41A | No |  |
| DC_18A_n77A7 DC_18A_n77(2A)7 | DC_18A_n77A | No | No |
| DC_18A_n78A7 | DC_18A_n78A | No | No |
| DC_18A_n78(2A)7 | DC_18A_n78A | No | No |
| DC_20A_n91A | DC_20A_n91A_ULSUP-TDM | N/A |  |
| DC_20A_n92A | DC_20A_n92A_ULSUP-TDM | N/A |  |
| DC_18A_n79A7 | DC_18A_n79A | No |  |
| DC_19A_n1A | DC_19A_n1A | No |  |
| DC_19A_n77A7 DC_19A_n77C7 | DC_19A_n77A | No |  |
| DC_19A_n77(2A)7 | DC_19A_n77A | No |  |
| DC_19A_n78A7 DC_19A_n78C7 | DC_19A_n78A | No | No |
| DC_19A_n78(2A)7 | DC_19A_n78A | No | No |
| DC_19A_n79A7 DC_19A_n79C7 | DC_19A_n79A | No | No |
| DC_20A_n1A | DC_20A_n1A | No |  |
| DC_20A_n3A | DC_20A_n3A | No |  |
| DC_20A_n7A | DC_20A_n7A | DC_20_n7 |  |
| DC_20A_n8A | DC_20A_n8A | DC_20_n8 |  |
| DC_20A_n28A8,11,13 | DC_20A_n28A | No |  |
| DC_20A_n38A | DC_20A_n38A | No |  |
| DC_20A_n41A | DC_20A_n41A | DC_20_n41 |  |
| DC_20A_n50A | DC_20A_n50A | No |  |
| DC_20A_n51A | DC_20A_n51A | No |  |
| DC_20A_n77A7 | DC_20A_n77A | No |  |
| DC_20A_n78A7 DC_20A_n78C7 | DC_20A_n78A | No |  |
| DC_20A_n78(2A)7 | DC_20A_n78A | No |  |
| DC_21A_n1A | DC_21A_n1A | No |  |
| DC_21A_n28A17 | DC_21A_n28A | DC_21_n28 |  |
| DC_21A_n77A7 DC_21A_n77C7 | DC_21A_n77A | No |  |
| DC_21A_n77(2A)7 | DC_21A_n77A | No |  |
| DC_21A_n78A7 DC_21A_n78C7 | DC_21A_n78A | No | No |
| DC_21A_n78(2A)7 | DC_21A_n78A | No | No |
| DC_21A_n79A7 DC_21A_n79C7 | DC_21A_n79A | No | No |
| DC_25A_n41A | DC_25A_n41A | No |  |
| DC_25A-25A_n41A | DC_25A_n41A | No |  |
| DC_25A_n77A | DC_25A_n77A | DC_25_n77 |  |
| DC_25A-25A_n77A | DC_25A_n77A | DC_25_n77 |  |
| DC_25A_n78A | DC_25A_n78A | DC_25_n78 |  |
| DC_25A-25A_n78A | DC_25A_n78A | DC_25_n78 |  |
| DC_26A_n25A | DC_26A_n25A | No |  |
| DC_26A_n41A | DC_26A_n41A | No |  |
| DC_26A_n77A7 | DC_26A_n77A | No |  |
| DC_26A_n78A7 DC_26A_n78A7 | DC_26A_n78A | No |  |
| DC_26A_n78(2A) | DC_26A_n78A | No |  |
| DC_26A_n79A7 | DC_26A_n79A | No |  |
| DC_28A_n1A | DC_28A_n1A | No |  |
| DC_28A_n2A | DC_28A_n2A | No |  |
| DC_28A_n3A | DC_28A_n3A | No |  |
| DC_28A_n5A | DC_28A_n5A | No |  |
| DC_28A_n7A DC_28A_n7B | DC_28A_n7A DC_28A_n7B | No |  |
| DC_28A_n51A | DC_28A_n51A | No |  |
| DC_28A_n8A | DC_28A_n8A | No |  |
| DC_28A_n20A8,11,13 | DC_28A_n20A | No |  |
| DC_28A_n38A | DC_28A_n38A | No |  |
| DC_28A_n40A | DC_28A_n40A | No |  |
| DC_28A_n41A7 | DC_28A_n41A | No |  |
| DC_28A_n50A | DC_28A_n50A | No |  |
| DC_28A_n66A | DC_28A_n66A | No |  |
| DC_28A_n77A7 DC_28A_n77C7 | DC_28A_n77A | No | No |
| DC_28A_n77(2A)7 | DC_28A_n77A | No | No |
| DC_28A_n78A7 DC_28A_n78C7 | DC_28A_n78A | No | No |
| DC_28A_n78(2A)7 | DC_28A_n78A | No | No |
| DC_28A_n79A7 DC_28A_n79C7 | DC_28A_n79A | No |  |
| DC_30A_n2A | DC_30A_n2A | No |  |
| DC_30A_n5A | DC_30A_n5A | No |  |
| DC_30A_n66A | DC_30A_n66A | No |  |
| DC_30A_n77A | DC_30A_n77A | No |  |
| DC_30A_n77(2A) 21 | DC_30A_n77A21 | No |  |
| DC_38A_n1A | DC_38A_n1A | No |  |
| DC_38A_n3A | DC_38A_n3A | No |  |
| DC_38A_n8A | DC_38A_n8A | No |  |
| DC_38A_n28A | DC_38A_n28A | No |  |
| DC_38A_n78A7 | DC_38A_n78A | No |  |
| DC_38A_n79A DC_38A_n79C | DC_38A_n79A | No |  |
| DC_39A_n40A3 | DC_39A_n40A | No |  |
| DC_39A_n41A3 DC_39C_n41A3 DC_39A_n41C3 | DC_39A_n41A DC_39C_n41A | No | No |
| DC_39A_n78A5,7 | DC_39A_n78A | No |  |
| DC_39A_n79A7 DC_39A_n79C7 | DC_39A_n79A | No | No |
| DC_40A_n1A DC_40C_n1A | DC_40A_n1A | No |  |
| DC_40A_n41A3 DC_40A_n41C3 DC_40C_n41A3 | DC_40A_n41A | No |  |
| DC_40A_n41(2A)3 | DC_40A_n41A | No |  |
| DC_40A_n77A DC_40A_n77C | DC_40A_n77A | No |  |
| DC_40A_n78A DC_40C_n78A | DC_40A_n78A DC_40C_n78A | No |  |
| DC_40A_n78(2A) DC_40C_n78(2A) | DC_40A_n78A DC_40C_n78A | No |  |
| DC_40A_n79A7,12 DC_40A_n79C7,12 DC_40C_n79A7,12 | DC_40A_n79A | No | No |
| DC_41A_n1A DC_41C_n1A | DC_41A_n1A DC_41C_n1A | No | DC_41A_n1A DC_41C_n1A |
| DC_41A_n3A7 DC_41C_n3A7 | DC_41A_n3A DC_41C_n3A | No |  |
| DC_41A_n28A7 DC_41C_n28A7 | DC_41A_n28A DC_41C_n28A | No |  |
| DC_41A_n77A DC_41C_n77A | DC_41A_n77A DC_41C_n77A | No |  |
| DC_41A_n77(2A) DC_41C_n77(2A) | DC_41A_n77A DC_41C_n77A | No |  |
| DC_41A_n78A DC_41C_n78A DC_41D_n78A | DC_41A_n78A DC_41C_n78A | No |  |
| DC_41A_n78(2A) DC_41C_n78(2A) | DC_41A_n78A DC_41C_n78A | No |  |
| DC_41A_n79A6,7 DC_41A_n79C6,7 DC_41C_n79A6,7 | DC_41A_n79A DC_41C_n79A | No | No |
| DC_42A_n1A7 DC_42C_n1A7 | DC_42A_n1A DC_42C_n1A | No |  |
| DC_42A_n3A7 DC_42C_n3A7 | DC_42A_n3A DC_42C_n3A | DC_42_n3 |  |
| DC_42A_n28A7 DC_42C_n28A7 | DC_42A_n28A DC_42C_n28A | No |  |
| DC_42A_n51A | DC_42A_n51A | No |  |
| DC_42A_n77A3,4,9,11 DC_42A_n77C3,4,9,11 DC_42C_n77A3,4,9,11 DC_42C_n77C3,4,9,11 DC_42D_n77A3,4,9,11 DC_42D_n77C DC_42E_n77A3,4,9,11 DC_42E_n77C | N/A | N/A |  |
| DC_42A_n77(2A)3,4,9,11 DC_42C_n77(2A)3,4,9,11 | N/A | N/A |  |
| DC_42A_n78A3,4,9,11 DC_42A_n78C3,4,9,11 DC_42C_n78A3,4,9,11 DC_42C_n78C3,4,9,11 DC_42D_n78A3,4,9,11 DC_42D_n78C3,4,9,11 DC_42E_n78A3,4,9,11 DC_42E_n78C3,4,9,11 | N/A | N/A |  |
| DC_42A_n79A9,15 DC_42A_n79C9,15 DC_42C_n79A9,15 DC_42C_n79C9,15 DC_42D_n79A9,15 DC_42D_n79C9,15 DC_42E_n79A9,15 DC_42E_n79C9,15 | N/A | N/A |  |
| DC_46A_n77A2 | N/A | N/A |  |
| DC_46A_n78A2 DC_46C_n78A2 DC_46D_n78A2 DC_46E_n78A2 | N/A | N/A |  |
| DC_48A_n2A DC_48C_n2A DC_48D_n2A DC_48E_n2A | DC_48A_n2A | No |  |
| DC_48A_n5A DC_48C_n5A DC_48D_n5A DC_48E_n5A | DC_48A_n5A | No |  |
| DC_48A_n12A | DC_48A_n12A | No |  |
| DC_48A_n25A DC_48C_n25A DC_48D_n25A | DC_48A_n25A | No |  |
| DC_48A_n46A DC_48B_n46A DC_48C_n46A DC_48D_n46A DC_48E_n46A DC_48A_n46B DC_48B_n46B DC_48C_n46B DC_48D_n46B DC_48E_n46B DC_48A_n46C DC_48B_n46C DC_48C_n46C DC_48D_n46C DC_48E_n46C DC_48A_n46D DC_48B_n46D DC_48C_n46D DC_48D_n46D DC_48E_n46D DC_48A_n46E DC_48B_n46E DC_48C_n46E DC_48D_n46E DC_48E_n46E | DC_48A_n46A DC_48B_n46A | No |  |
| DC_48A_n66A DC_48C_n66A DC_48D_n66A DC_48E_n66A | DC_48A_n66A | No |  |
| DC_48A_n71A DC_48B_n71A DC_48C_n71A DC_48D_n71A | DC_48A_n71A | No |  |
| DC_48A-48A_n71A DC_48A-48A-48A_n71A | DC_48A_n71A | No |  |
| DC_48A_n77A3. 4. 9, 11 DC_48C_n77A3. 4. 9, 11 DC_48A_n77C3. 4. 9, 11 DC_48C_n77C3. 4. 9, 11 DC_48D_n77A3. 4. 9, 11 DC_48D_n77C3. 4. 9, 11 DC_48E_n77A3. 4. 9, 11 | N/A | N/A |  |
| DC_48A-48A_n77A | N/A | N/A |  |
| DC_48A-48A-48A_n77A | N/A | N/A |  |
| DC_66A_n2A DC_66B_n2A DC_66C_n2A | DC_66A_n2A | DC_66_n2 |  |
| DC_66A_n2(2A) | DC_66A_n2A | DC_66_n2 |  |
| DC_66A-66A_n2A | DC_66A_n2A | DC_66_n2 |  |
| DC_66A-66A-66A_n2A | DC_66A_n2A | DC_66_n2 |  |
| DC_66A_n5A DC_66B_n5A DC_66C_n5A | DC_66A_n5A | DC_66_n5 |  |
| DC_66A-66A_n5A | DC_66A_n5A | DC_66_n5 |  |
| DC_66A-66A-66A_n5A | DC_66A_n5A | DC_66_n5 |  |
| DC_66A_n7A | DC_66A_n7A | No |  |
| DC_66A_n7(2A) | DC_66A_n7A | No |  |
| DC_66A-66A_n7A | DC_66A_n7A | No |  |
| DC_66A-66A_n7(2A) | DC_66A_n7A | No |  |
| DC_66A_n12A | DC_66A_n12A | No |  |
| DC_66A_n25A | DC_66A_n25A | DC_66_n25 |  |
| DC_66A_n28A | DC_66A_n28A | No |  |
| DC_66A_n30A | DC_66A_n30A | No |  |
| DC_66A-66A_n30A | DC_66A_n30A | No |  |
| DC_66A_n38A | DC_66A_n38A | No |  |
| DC_66A-66A_n38A | DC_66A_n38A | No |  |
| DC_66A_n41A DC_66A_n41C | DC_66A_n41A | No |  |
| DC_66A_n41(2A) | DC_66A_n41A | No |  |
| DC_66A_n46A | DC_66A_n46A | No |  |
| DC_66A_n48A DC_66A_n48B | DC_66A_n48A | No |  |
| DC_66A-66A_n48A DC_66A-66A_n48B | DC_66A_n48A | No |  |
| DC_66A_n71A DC_66C_n71A DC_66A_n71B | DC_66A_n71A | No |  |
| DC_66A-66A_n71A | DC_66A_n71A | No |  |
| DC_66A_n77A DC_66A_n77C21 | DC_66A_n77A21 | DC_66_n77 |  |
| DC_66A_n77(2A) 21 | DC_66A_n77A21 | DC_66_n77 |  |
| DC_66A-66A_n77A21 DC_66A-66A_n77C21 | DC_66A_n77A21 | DC_66_n77 |  |
| DC_66A-66A_n77(2A) 21 | DC_66A_n77A21 | DC_66_n77 |  |
| DC_66A-66A-66A_n77A21 DC_66A-66A-66A_n77C21 | DC_66A_n77A21 | DC_66_n77 |  |
| DC_66A-66A-66A_n77(2A)21 | DC_66A_ n77A21 | DC_66_n77 |  |
| DC_66A_n78A | DC_66A_n78A | No |  |
| DC_66A_n78(2A) | DC_66A_n78A | No |  |
| DC_66A-66A_n78A | DC_66A_n78A | No |  |
| DC_66A-66A_n78(2A) | DC_66A_n78A | No |  |
| DC_71A_n2A | DC_71A_n2A | No |  |
| DC_71A_n2(2A) | DC_71A_n2A | No |  |
| DC_71A_n5A | DC_71A_n5A | No |  |
| DC_71A_n12A | DC_71A_n12A18,19 | Yes |  |
| DC_71A_n38A | DC_71A_n38A | No |  |
| DC_71A_n7A | DC_71A_n7A | No |  |
| DC_71A_n25A | DC_71A_n7A | No |  |
| DC_71A_n41A | DC_71A_n41A | No |  |
| DC_71A_n48A | DC_71A_n48A | No |  |
| DC_71A_n66A | DC_71A_n66A | No |  |
| DC_71A_n77A DC_71A_n77C | DC_71A_n77A | No |  |
| DC_71A_n77(2A) | DC_71A_n77A | No |  |
| DC_71A_n78A | DC_71A_n78A | No |  |
| DC_71A_n78(2A) | DC_71A_n78A | No |  |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band for Band 46 is paired with the uplink operating band (external E-UTRA band) of the carrier aggregation configuration that is supporting the configured Pcell. NOTE 3: The minimum requirements apply only when there is non-simultaneous Tx/Rx operation between E-UTRA and NR carriers. This restriction applies also for these carriers when applicable EN-DC configuration is part of a higher order EN-DC configuration. NOTE 4: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band non-contiguous EN-DC apply for the Band 42/48 and Band n77/n78 combination. For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, when UE capability interBandContiguousMRDC is indicated, the minimum requirements for intra-band-contiguous EN-DC also should be met in addtion to intra-band non-contiguous EN-DC. The intra-band requirements also apply for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. NOTE 5: The frequency range above 3600 MHz for Band n78 is not used in this combination. NOTE 6: The frequency range below 2506 MHz for Band 41 is not used in this combination. NOTE 7: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability. NOTE 8: The frequency range in band n28 / 28 is restricted for this band combination to 703 - 733 MHz for the UL and 758-788 MHz for the DL. This restriction also apply for any band combinations when DC_20_n28/ DC_28_n20/ CA_20-28/ CA_n20-n28 is a subset of a higher order band combination. NOTE 9: The combination is not used alone as fall back mode of other band combinations in which UL in Band 42 or Band 48 is not used. NOTE 10: Void. NOTE 11: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for apply when the maximum power spectral density imbalance between downlink carriers is within 6 dB. For UEs indicating interBandMRDC-WithOverlapDL-Bands-r16, the power imbalance requirement defined in clause 7.6B.2.6 apply. For these UEs, the power spectral density imbalance condition also applies for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. NOTE 12: Applicable for frequency range above 4800 MHz for Band n79 in this combination. NOTE 13: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements apply for synchronized DL carriers with a maximum receive time difference ≤ 3 usec. The requirements also apply for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. NOTE 14: Applicable when dynamic switching between two uplink carriers is conducted. The DL interruption requirements for NR DL carrier(s) and E-UTRA DL carrier(s) are specified in clause 8.2.1.2.14 of 38.133 [15] and clause 7.32.2.12 of 36.133 [16] respectively. NOTE 15: Simultaneous Rx/Tx capability does not apply for UEs supporting band 42 with a n77 implementation only. Same restrictions are applied to related higher order configurations. NOTE 16: The frequency range in band n41 is restricted for this band combination to 2595 – 2645 MHz. NOTE 17: The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL and 783 - 793 MHz for the DL. This restriction applies also for these band combinations when applicable EN-DC configuration is part of a higher order EN-DC configuration. NOTE 18: Only single switched UL is supported. NOTE 19: The implementation with 4 antennas is targeted for FWA form factor for this band combination. NOTE 20: The combination is not used alone as fallback mode of other band combinations in which UL in Band 2 is not used. NOTE 21: For this DC configuration, reference sensitivity exceptions for Power Class 2, if allowed, are specified in Clause 7.3B.2.3. If the uplink EN-DC configuration supported in Table 6.2B.1.3-1 is applicable to the same EN-DC configuration, the note is not shown as the reference sensitivity exceptions, if any, have been confirmed. NOTE 22: The PC2 Uplink EN-DC configuration supported in Table 6.2B.1.3-1 is applicable to the same EN-DC configuration without additional indication of NOTE 21. |  |  |  |

<< Next Changes >>

5.5B.5	Inter-band EN-DC including FR2

5.5B.5.1	Inter-band EN-DC configurations including FR2 (two bands)

Table 5.5B.5.1-1: Inter-band EN-DC configurations including FR2 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A_n257A DC_1A_n257D DC_1A_n257E DC_1A_n257F DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_1A_n257J DC_1A_n257K DC_1A_n257L DC_1A_n257M | DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_1A_n257J DC_1A_n257K DC_1A_n257L DC_1A_n257M |
| DC_1A_n257(2A) DC_1A_n257(A-G) DC_1A_n257(2G) | DC_1A_n257A DC_1A_n257G DC_1A_n257(2A) DC_1A_n257(2G) |
| DC_1A_n258A DC_1A_n258B DC_1A_n258C DC_1A_n258D DC_1A_n258E DC_1A_n258F DC_1A_n258G DC_1A_n258H DC_1A_n258I DC_1A_n258J DC_1A_n258K DC_1A_n258L DC_1A_n258M | DC_1A_n258A DC_1A_n258D DC_1A_n258G DC_1A_n258H DC_1A_n258I |
| DC_1A_n258(2A) | DC_1A_n258A DC_1A_n258(2A) |
| DC_2A_n257A DC_2C_n257A DC_2A_n257G DC_2A_n257H DC_2A_n257I DC_2A_n257J DC_2A_n257K DC_2A_n257L DC_2A_n257M | DC_2A_n257A DC_2A_n257G DC_2A_n257H DC_2A_n257I DC_2A_n257J DC_2A_n257K DC_2A_n257L DC_2A_n257M |
| DC_2A_n257(2A) | DC_2A_n257A |
| DC_2A-2A_n257A | DC_2A_n257A |
| DC_2A_n258A DC_2A_n258D DC_2A_n258G DC_2A_n258H DC_2A_n258I DC_2A_n258J DC_2A_n258K DC_2A_n258L DC_2A_n258M DC_2A_n258O DC_2A_n258P DC_2A_n258Q | DC_2A_n258A DC_2A_n258D DC_2A_n258G DC_2A_n258H DC_2A_n258I DC_2A_n258J DC_2A_n258K DC_2A_n258L DC_2A_n258M DC_2A_n258O DC_2A_n258P DC_2A_n258Q |
| DC_2A_n258(2A) DC_2A_n258(3A) DC_2A_n258(4A) DC_2A_n258(5A) | DC_2A_n258A |
| DC_2A_n260A DC_2A_n260D DC_2A_n260E DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M DC_2A_n260O DC_2A_n260P DC_2A_n260Q DC_2C_n260A | DC_2A_n260A DC_2A_n260D DC_2A_n260E DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M DC_2A_n260O DC_2A_n260P DC_2A_n260Q |
| DC_2A_n260(2A) DC_2A_n260(3A) DC_2A_n260(4A) DC_2A_n260(5A) DC_2A_n260(6A) DC_2A_n260(7A) DC_2A_n260(8A) DC_2A_n260(2D) DC_2A_n260(2G) DC_2A_n260(3G) DC_2A_n260(4G) DC_2A_n260(2H) DC_2A_n260(2O) DC_2A_n260(3O) DC_2A_n260(4O) DC_2A_n260(2P) DC_2A_n260(A-D) DC_2A_n260(A-G) DC_2A_n260(A-H) DC_2A_n260(A-O) DC_2A_n260(A-P) DC_2A_n260(A-Q) DC_2A_n260(A-2G) DC_2A_n260(A-2H) DC_2A_n260(A-2O) DC_2A_n260(A-2P) DC_2A_n260(A-3G) DC_2A_n260(A-3O) DC_2A_n260(3A-G) DC_2A_n260(3A-H) DC_2A_n260(3A-O) DC_2A_n260(2A-G) DC_2A_n260(2A-H) DC_2A_n260(2A-O) DC_2A_n260(2A-P) DC_2A_n260(2A-Q) DC_2A_n260(2A-2G) DC_2A_n260(2A-2H) DC_2A_n260(2A-2O) DC_2A_n260(3A-2O) DC_2A_n260(3A-P) DC_2A_n260(4A-O) DC_2A_n260(4A-P) DC_2A_n260(4A-2O) DC_2A_n260(5A-O) DC_2A_n260(G-H) DC_2A_n260(O-P) DC_2A_n260(O-Q) DC_2A_n260(P-Q) DC_2A_n260(A-G-H) DC_2A_n260(A-O-P) DC_2A_n260(A-O-Q) DC_2A_n260(A-P-Q) DC_2A_n260(2A-O-P) DC_2A_n260(2A-G-H) DC_2A_n260(3A-O-P) | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260O DC_2A_n260P DC_2A_n260Q |
| DC_2A-2A_n260A DC_2A-2A_n260G DC_2A-2A_n260H DC_2A-2A_n260I DC_2A-2A_n260J DC_2A-2A_n260K DC_2A-2A_n260L DC_2A-2A_n260M | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M |
| DC_2A_n261A DC_2A_n261B DC_2A_n261C DC_2A_n261D DC_2A_n261E DC_2A_n261F DC_2A_n261G DC_2A_n261H DC_2A_n261I DC_2A_n261J DC_2A_n261K DC_2A_n261L DC_2A_n261M DC_2A_n261O DC_2A_n261P DC_2A_n261Q | DC_2A_n261A DC_2A_n261B DC_2A_n261C DC_2A_n261D DC_2A_n261E DC_2A_n261F DC_2A_n261G DC_2A_n261H DC_2A_n261I DC_2A_n261J DC_2A_n261K DC_2A_n261L DC_2A_n261M DC_2A_n261O DC_2A_n261P DC_2A_n261Q |
| DC_2A_n261(2A) DC_2A_n261(3A) DC_2A_n261(4A) DC_2A_n261(2I) DC_2A_n261(2H) DC_2A_n261(A-G) DC_2A_n261(A-J) DC_2A_n261(A-K) DC_2A_n261(A-L) DC_2A_n261(A-2G) DC_2A_n261(A-H) DC_2A_n261(A-I) DC_2A_n261(2A-G) DC_2A_n261(2A-I) DC_2A_n261(2A-H) DC_2A_n261(3A-G) DC_2A_n261(G-H) DC_2A_n261(G-I) DC_2A_n261(G-J) DC_2A_n261(2G) DC_2A_n261(H-I) DC_2A_n261(A-G-H) DC_2A_n261(A-G-I) | DC_2A_n261A DC_2A_n261G DC_2A_n261H DC_2A_n261I |
| DC_2A-2A_n261A DC_2A-2A_n261G DC_2A-2A_n261H DC_2A-2A_n261I DC_2A-2A_n261J DC_2A-2A_n261K DC_2A-2A_n261L DC_2A-2A_n261M | DC_2A_n261A DC_2A_n261G DC_2A_n261H DC_2A_n261I DC_2A_n261J DC_2A_n261K DC_2A_n261L DC_2A_n261M |
| DC_3A_n257A DC_3A_n257B DC_3A_n257C DC_3A_n257D DC_3A_n257E DC_3A_n257F DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_3A_n257L DC_3A_n257M DC_3C_n257A DC_3C_n257D DC_3C_n257E DC_3C_n257F DC_3C_n257G DC_3C_n257H DC_3C_n257I DC_3C_n257J DC_3C_n257K DC_3C_n257L DC_3C_n257M | DC_3A_n257A DC_3A_n257B DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_3A_n257L DC_3A_n257M DC_3C_n257A |
| DC_3A-3A_n257A DC_3A-3A_n257D DC_3A-3A_n257E DC_3A-3A_n257F DC_3A-3A_n257G DC_3A-3A_n257H DC_3A-3A_n257I DC_3A-3A_n257J DC_3A-3A_n257K DC_3A-3A_n257L DC_3A-3A_n257M | DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K |
| DC_3A_n258A DC_3A_n258B DC_3A_n258C DC_3A_n258D DC_3A_n258E DC_3A_n258F DC_3A_n258G DC_3A_n258H DC_3A_n258I DC_3A_n258J DC_3A_n258K DC_3A_n258L DC_3A_n258M DC_3C_n258A DC_3C_n258B DC_3C_n258C DC_3C_n258D DC_3C_n258E DC_3C_n258F DC_3C_n258G DC_3C_n258H DC_3C_n258I DC_3C_n258J DC_3C_n258K DC_3C_n258L DC_3C_n258M | DC_3A_n258A DC_3A_n258G DC_3A_n258H DC_3A_n258I DC_3C_n258A DC_3C_n258G DC_3C_n258H DC_3C_n258I |
| DC_4A_n258A DC_4A_n258G DC_4A_n258H DC_4A_n258I | DC_4A_n258A DC_4A_n258G DC_4A_n258H DC_4A_n258I |
| DC_4A_n260A DC_4A_n260D DC_4A_n260G DC_4A_n260H DC_4A_n260O DC_4A_n260P DC_4A_n260Q | DC_4A_n260A DC_4A_n260G DC_4A_n260H DC_4A_n260O DC_4A_n260P DC_4A_n260Q |
| DC_4A_n260(2A) DC_4A_n260(3A) DC_4A_n260(4A) DC_4A_n260(5A) DC_4A_n260(6A) DC_4A_n260(7A) DC_4A_n260(8A) DC_4A_n260(2D) DC_4A_n260(2G) DC_4A_n260(3G) DC_4A_n260(4G) DC_4A_n260(2H) DC_4A_n260(2O) DC_4A_n260(2P) DC_4A_n260(3O) DC_4A_n260(4O) DC_4A_n260(A-D) DC_4A_n260(2A-D) DC_4A_n260(3A-D) DC_4A_n260(4A-D) DC_4A_n260(A-O) DC_4A_n260(2A-O) DC_4A_n260(A-D-O) DC_4A_n260(2A-D-O) DC_4A_n260(A-2O) DC_4A_n260(D-2O) DC_4A_n260(A-D-2O) DC_4A_n260(2A-D-2O) DC_4A_n260(A-2D) DC_4A_n260(2A-2D) DC_4A_n260(A-P) DC_4A_n260(2A-P) DC_4A_n260(A-2P) DC_4A_n260(2A-2P) DC_4A_n260(A-G) DC_4A_n260(2A-G) DC_4A_n260(A-2G) DC_4A_n260(2A-2G) DC_4A_n260(G-O) DC_4A_n260(2G-O) DC_4A_n260(A-G-O) DC_4A_n260(2A-G-O) DC_4A_n260(2A-G-2O) DC_4A_n260(2A-G-3O) DC_4A_n260(A-2G-O) DC_4A_n260(2A-2G-O) DC_4A_n260(2A-2G-2O) DC_4A_n260(A-H) DC_4A_n260(A-2H) DC_4A_n260(2A-H) DC_4A_n260(2A-2H) DC_4A_n260(2A-2O) DC_4A_n260(A-3O) DC_4A_n260(2A-3O) DC_4A_n260(A-4O) DC_4A_n260(2A-4O) DC_4A_n260(3A-O) DC_4A_n260(3A-2O) DC_4A_n260(3A-3O) DC_4A_n260(3A-G) DC_4A_n260(3A-2G) DC_4A_n260(4A-G) DC_4A_n260(4A-2G) DC_4A_n260(4A-O) DC_4A_n260(4A-2O) DC_4A_n260(D-2G) DC_4A_n260(2D-O) DC_4A_n260(G-2O) DC_4A_n260(2G-2O) DC_4A_n260(G-3O) DC_4A_n260(2G-3O) DC_4A_n260(G-4O) DC_4A_n260(2G-4O) DC_4A_n260(3G-O) DC_4A_n260(4G-O) DC_4A_n260(H-O) DC_4A_n260(2H-O) DC_4A_n260(A-P-Q) DC_4A_n260(3A-O-P) DC_4A_n260(A-Q) DC_4A_n260(P-Q) DC_4A_n260(2A-O-P) DC_4A_n260(3A-P) DC_4A_n260(A-O-P) DC_4A_n260(2A-3G) DC_4A_n260(2A-G-H) DC_4A_n260(2A-Q) DC_4A_n260(3A-2G-O) DC_4A_n260(3A-D-O) DC_4A_n260(3A-G-O) DC_4A_n260(3A-G-2O) DC_4A_n260(3A-H) DC_4A_n260(4A-G-O) DC_4A_n260(4A-P) DC_4A_n260(5A-G) DC_4A_n260(5A-O) DC_4A_n260(A-G-2O) DC_4A_n260(A-G-3O) DC_4A_n260(A-G-4O) DC_4A_n260(A-G-H) DC_4A_n260(A-H-O) DC_4A_n260(A-2G-2O) DC_4A_n260(A-2G-3O) DC_4A_n260(A-3G-O) DC_4A_n260(A-3G) DC_4A_n260(A-4G) DC_4A_n260(A-D-G) DC_4A_n260(A-O-Q) DC_4A_n260(D-G) DC_4A_n260(D-O) DC_4A_n260(G-H) DC_4A_n260(G-H-O) DC_4A_n260(O-P) DC_4A_n260(O-Q) | DC_4A_n260A DC_4A_n260G DC_4A_n260H DC_4A_n260O DC_4A_n260P DC_4A_n260Q |
| DC_4A_n261A DC_4A_n261D DC_4A_n261G DC_4A_n261H DC_4A_n261I DC_4A_n261J DC_4A_n261K DC_4A_n261L DC_4A_n261M | DC_4A_n261A DC_4A_n261D DC_4A_n261G DC_4A_n261H DC_4A_n261I |
| DC_4A_n261(2A) DC_4A_n261(3A) DC_4A_n261(4A) DC_4A_n261(2G) DC_4A_n261(2H) DC_4A_n261(2I) DC_4A_n261(A-D) DC_4A_n261(A-H) DC_4A_n261(A-2H) DC_4A_n261(A-D-H) DC_4A_n261(A-G) DC_4A_n261(A-G-H) DC_4A_n261(A-I) DC_4A_n261(A-2I) DC_4A_n261(G-I) DC_4A_n261(A-G-I) DC_4A_n261(A-H-I) DC_4A_n261(G-H) DC_4A_n261(H-I) DC_4A_n261(D-H) DC_4A_n261(2A-D) DC_4A_n261(2A-G) DC_4A_n261(2A-H) DC_4A_n261(2A-I) DC_4A_n261(A-2G) DC_4A_n261(A-D-G) DC_4A_n261(D-G) | DC_4A_n261A DC_4A_n261H DC_4A_n261I DC_4A_n261G |
| DC_5A_n257A DC_5A_n257D DC_5A_n257E DC_5A_n257F DC_5A_n257G DC_5A_n257H DC_5A_n257I DC_5A_n257J DC_5A_n257K DC_5A_n257L DC_5A_n257M DC_5B_n257A | DC_5A_n257A DC_5A_n257D DC_5A_n257G DC_5A_n257H DC_5A_n257I DC_5B_n257A |
| DC_5A-5A_n257A | DC_5A_n257A |
| DC_5A_n258A DC_5A_n258D DC_5A_n258G DC_5A_n258H DC_5A_n258I DC_5A_n258J DC_5A_n258K DC_5A_n258L DC_5A_n258M DC_5A_n258O DC_5A_n258P DC_5A_n258Q | DC_5A_n258A DC_5A_n258D DC_5A_n258G DC_5A_n258H DC_5A_n258I DC_5A_n258J DC_5A_n258K DC_5A_n258L DC_5A_n258M DC_5A_n258O DC_5A_n258P DC_5A_n258Q |
| DC_5A_n260A DC_5A_n260B DC_5A_n260C DC_5A_n260D DC_5A_n260E DC_5A_n260F DC_5A_n260G DC_5A_n260H DC_5A_n260I DC_5A_n260J DC_5A_n260K DC_5A_n260L DC_5A_n260M DC_5A_n260O DC_5A_n260P DC_5A_n260Q DC_5B_n260A | DC_5A_n260A DC_5A_n260G DC_5A_n260H DC_5A_n260I DC_5A_n260J DC_5A_n260K DC_5A_n260L DC_5A_n260M DC_5A_n260O DC_5A_n260P DC_5A_n260Q DC_5B_n260A |
| DC_5A_n260(2A) DC_5A_n260(3A) DC_5A_n260(4A) DC_5A_n260(5A) DC_5A_n260(6A) DC_5A_n260(7A) DC_5A_n260(8A) DC_5A_n260(9A) DC_5A_n260(10A) DC_5A_n260(A-I) DC_5A_n260(A-P-Q) DC_5A_n260(3A-O-P) DC_5A_n260(D-G) DC_5A_n260(D-H) DC_5A_n260(D-I) DC_5A_n260(D-O) DC_5A_n260(D-P) DC_5A_n260(D-Q) DC_5A_n260(E-O) DC_5A_n260(E-P) DC_5A_n260(E-Q) DC_5A_n260(G-I) DC_5A_n260(2G) DC_5A_n260(2H) DC_5A_n260(2O) DC_5A_n260(3O) DC_5A_n260(4O) DC_5A_n260(2P) DC_5A_n260(3P) DC_5A_n260(4P) DC_5A_n260(2A-O) DC_5A_n260(A-2O) DC_5A_n260(2A-G) DC_5A_n260(A-2G) DC_5A_n260(2A-2G) DC_5A_n260(2G-O) DC_5A_n260(2A-2G-O) DC_5A_n260(A-2H) DC_5A_n260(2A-H) DC_5A_n260(2A-2H) DC_5A_n260(2A-2O) DC_5A_n260(2A-3O) DC_5A_n260(A-4O) DC_5A_n260(2A-4O) DC_5A_n260(3A-2O) DC_5A_n260(3A-2G) DC_5A_n260(4A-G) DC_5A_n260(4A-2G) DC_5A_n260(4A-O) DC_5A_n260(4A-2O) DC_5A_n260(A-O) DC_5A_n260(A-G) DC_5A_n260(G-O) DC_5A_n260(A-G-O) DC_5A_n260(2A-G-O) DC_5A_n260(A-2G-O) DC_5A_n260(A-H) DC_5A_n260(A-3O) DC_5A_n260(3A-O) DC_5A_n260(3A-G) DC_5A_n260(2D) DC_5A_n260(3G) DC_5A_n260(4G) DC_5A_n260(A-D) DC_5A_n260(2A-D) DC_5A_n260(A-D-O) DC_5A_n260(2A-D-O) DC_5A_n260(D-2O) DC_5A_n260(A-D-2O) DC_5A_n260(2A-D-2O) DC_5A_n260(A-2D) DC_5A_n260(2A-2D) DC_5A_n260(A-P) DC_5A_n260(2A-P) DC_5A_n260(A-2P) DC_5A_n260(2A-2P) DC_5A_n260(3A-3O) DC_5A_n260(D-2G) DC_5A_n260(2D-O) DC_5A_n260(G-2O) DC_5A_n260(2G-2O) DC_5A_n260(G-3O) DC_5A_n260(2G-3O) DC_5A_n260(G-4O) DC_5A_n260(2G-4O) DC_5A_n260(3G-O) DC_5A_n260(4G-O) DC_5A_n260(H-O) DC_5A_n260(2H-O) DC_5A_n260(A-Q) DC_5A_n260(P-Q) DC_5A_n260(2A-4P) DC_5A_n260(2O-2P) DC_5A_n260(3A-P) DC_5A_n260(4A-4O) DC_5A_n260(4A-2Q) DC_5A_n260(6A-2O) DC_5A_n260(6A-2P) DC_5A_n260(6A-3O) DC_5A_n260(8A-2O) DC_5A_n260(2A-O-P) DC_5A_n260(2A-2G-2O) DC_5A_n260(2A-2O-2P) DC_5A_n260(2A-2O-2Q) DC_5A_n260(O-P) DC_5A_n260(A-O-P) DC_5A_n260(2A-2O-P-Q) DC_5A_n260(2A-2O-P) DC_5A_n260(2A-2O-Q) DC_5A_n260(2A-2Q) DC_5A_n260(2A-3G) DC_5A_n260(2A-3O-P) DC_5A_n260(2A-3O-Q) DC_5A_n260(2A-3P) DC_5A_n260(2A-G-2O) DC_5A_n260(2A-G-3O) DC_5A_n260(2A-G-H) DC_5A_n260(2A-O-2P) DC_5A_n260(2A-O-2Q) DC_5A_n260(2A-O-3P) DC_5A_n260(2A-O-P-Q) DC_5A_n260(2A-O-Q) DC_5A_n260(2A-P-Q) DC_5A_n260(2A-Q) DC_5A_n260(2O-2Q) DC_5A_n260(2O-P-Q) DC_5A_n260(2O-P) DC_5A_n260(2O-Q) DC_5A_n260(2Q) DC_5A_n260(3A-2G-O) DC_5A_n260(3A-2O-P) DC_5A_n260(3A-2O-Q) DC_5A_n260(3A-2P) DC_5A_n260(3A-2Q) DC_5A_n260(3A-3P) DC_5A_n260(3A-4O) DC_5A_n260(3A-D-O) DC_5A_n260(3A-D) DC_5A_n260(3A-G-2O) DC_5A_n260(3A-G-O) DC_5A_n260(3A-H) DC_5A_n260(3A-O-2P) DC_5A_n260(3A-O-2Q) DC_5A_n260(3A-O-P-Q) DC_5A_n260(3A-O-Q) DC_5A_n260(3A-P-Q) DC_5A_n260(3A-Q) DC_5A_n260(3O-P) DC_5A_n260(3O-Q) DC_5A_n260(4A-2P) DC_5A_n260(4A-3O) DC_5A_n260(4A-D) DC_5A_n260(4A-G-O) DC_5A_n260(4A-O-P) DC_5A_n260(4A-O-Q) DC_5A_n260(4A-P-Q) DC_5A_n260(4A-P) DC_5A_n260(4A-Q) DC_5A_n260(5A-2O) DC_5A_n260(5A-2P) DC_5A_n260(5A-3O) DC_5A_n260(5A-G) DC_5A_n260(5A-O-P) DC_5A_n260(5A-O) DC_5A_n260(5A-P) DC_5A_n260(5A-Q) DC_5A_n260(6A-O-P) DC_5A_n260(6A-O) DC_5A_n260(6A-P) DC_5A_n260(7A-2O) DC_5A_n260(7A-O) DC_5A_n260(7A-P) DC_5A_n260(8A-O) DC_5A_n260(9A-O) DC_5A_n260(A-2G-2O) DC_5A_n260(A-2G-3O) DC_5A_n260(A-2O-2P) DC_5A_n260(A-2O-2Q) DC_5A_n260(A-2O-P-Q) DC_5A_n260(A-2O-P) DC_5A_n260(A-2O-Q) DC_5A_n260(A-2Q) DC_5A_n260(A-3G-O) DC_5A_n260(A-3G) DC_5A_n260(A-3O-P) DC_5A_n260(A-3O-Q) DC_5A_n260(A-3P) DC_5A_n260(A-4G) DC_5A_n260(A-4P) DC_5A_n260(A-D-G) DC_5A_n260(A-E) DC_5A_n260(A-G-2O) DC_5A_n260(A-G-3O) DC_5A_n260(A-G-4O) DC_5A_n260(A-G-H) DC_5A_n260(A-H-O) DC_5A_n260(A-O-2P) DC_5A_n260(A-O-2Q) DC_5A_n260(A-O-3P) DC_5A_n260(A-O-P-Q) DC_5A_n260(A-O-Q) DC_5A_n260(G-H-O) DC_5A_n260(G-H) DC_5A_n260(O-2P) DC_5A_n260(O-2Q) DC_5A_n260(O-3P) DC_5A_n260(O-P-Q) DC_5A_n260(O-Q) | DC_5A_n260A DC_5A_n260G DC_5A_n260H DC_5A_n260O DC_5A_n260P DC_5A_n260Q |
| DC_5A-5A_n260A | DC_5A_n260A |
| DC_5A_n261A DC_5A_n261B DC_5A_n261C DC_5A_n261D DC_5A_n261E DC_5A_n261F DC_5A_n261G DC_5A_n261H DC_5A_n261I DC_5A_n261J DC_5A_n261K DC_5A_n261L DC_5A_n261M DC_5A_n261O DC_5A_n261P DC_5A_n261Q | DC_5A_n261A DC_5A_n261G DC_5A_n261H DC_5A_n261I DC_5A_n261J DC_5A_n261K DC_5A_n261L DC_5A_n261M DC_5A_n261O |
| DC_5A_n261(2A) DC_5A_n261(2G) DC_5A_n261(3A) DC_5A_n261(4A) DC_5A_n261(5A) DC_5A_n261(6A) DC_5A_n261(7A) DC_5A_n261(8A) DC_5A_n261(D-G) DC_5A_n261(D-H) DC_5A_n261(D-I) DC_5A_n261(D-O) DC_5A_n261(D-P) DC_5A_n261(D-Q) DC_5A_n261(E-O) DC_5A_n261(E-P) DC_5A_n261(E-Q) DC_5A_n261(2H) DC_5A_n261(2I) DC_5A_n261(A-H) DC_5A_n261(A-I) DC_5A_n261(A-J) DC_5A_n261(A-K) DC_5A_n261(A-L) DC_5A_n261(A-D) DC_5A_n261(A-D-H) DC_5A_n261(A-D-2O) DC_5A_n261(A-G) DC_5A_n261(A-G-H) DC_5A_n261(G-I) DC_5A_n261(A-G-I) DC_5A_n261(A-H-I) DC_5A_n261(G-H) DC_5A_n261(G-J) DC_5A_n261(H-I) DC_5A_n261(A-2D) DC_5A_n261(A-2H) DC_5A_n261(A-2P) DC_5A_n261(A-2Q) DC_5A_n261(A-2I) DC_5A_n261(A-4G) DC_5A_n261(A-4O) DC_5A_n261(A-7O) DC_5A_n261(A-2G-2O) DC_5A_n261(A-3G-O) DC_5A_n261(2A-G) DC_5A_n261(2A-H) DC_5A_n261(2A-I) DC_5A_n261(3A-G) DC_5A_n261(2A-2G-O) DC_5A_n261(2A-2G) DC_5A_n261(2A-2O) DC_5A_n261(2A-3G) DC_5A_n261(2A-3O) DC_5A_n261(2A-4O) DC_5A_n261(2A-5O) DC_5A_n261(2A-6O) DC_5A_n261(2A-D-O) DC_5A_n261(2A-D) DC_5A_n261(2A-G-2O) DC_5A_n261(2A-G-O) DC_5A_n261(2A-O) DC_5A_n261(2A-P) DC_5A_n261(2A-Q) DC_5A_n261(2D) DC_5A_n261(2G-2O) DC_5A_n261(2G-O) DC_5A_n261(2O) DC_5A_n261(2P) DC_5A_n261(2Q) DC_5A_n261(3A-2G) DC_5A_n261(3A-2O) DC_5A_n261(3A-3O) DC_5A_n261(3A-4O) DC_5A_n261(3A-5O) DC_5A_n261(3A-D) DC_5A_n261(3A-G-O) DC_5A_n261(3A-O) DC_5A_n261(3G-O) DC_5A_n261(3G) DC_5A_n261(3O) DC_5A_n261(4A-2O) DC_5A_n261(4A-3O) DC_5A_n261(4A-4O) DC_5A_n261(4A-G) DC_5A_n261(4A-O) DC_5A_n261(4G) DC_5A_n261(4O) DC_5A_n261(5A-2O) DC_5A_n261(5A-3O) DC_5A_n261(5A-O) DC_5A_n261(5O) DC_5A_n261(6A-2O) DC_5A_n261(6A-O) DC_5A_n261(6O) DC_5A_n261(7A-O) DC_5A_n261(7O) DC_5A_n261(A-2G-O) DC_5A_n261(A-2G) DC_5A_n261(A-2O) DC_5A_n261(A-3G) DC_5A_n261(A-3O) DC_5A_n261(A-5O) DC_5A_n261(A-6O) DC_5A_n261(A-D-G) DC_5A_n261(A-D-O) DC_5A_n261(A-E) DC_5A_n261(A-G-2O) DC_5A_n261(A-G-O) DC_5A_n261(A-O-P) DC_5A_n261(A-O-Q) DC_5A_n261(A-O) DC_5A_n261(A-P-Q) DC_5A_n261(A-P) DC_5A_n261(A-Q) DC_5A_n261(D-2O) DC_5A_n261(G-2O) DC_5A_n261(G-O) DC_5A_n261(O-P) DC_5A_n261(O-Q) DC_5A_n261(P-Q) | DC_5A_n261A DC_5A_n261G DC_5A_n261H DC_5A_n261I |
| DC_7A_n257A DC_7A_n257D DC_7A_n257E DC_7A_n257F DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K DC_7A_n257L DC_7A_n257M | DC_7A_n257A DC_7A_n257D DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K |
| DC_7A_n257(2A) DC_7A_n257(A-G) DC_7A_n257(2G) | DC_7A_n257A DC_7A_n257G DC_7A_n257(2A) DC_7A_n257(2G) |
| DC_7A-7A_n257A DC_7A-7A_n257D DC_7A-7A_n257E DC_7A-7A_n257F DC_7A-7A_n257G DC_7A-7A_n257H DC_7A-7A_n257I DC_7A-7A_n257J DC_7A-7A_n257K DC_7A-7A_n257L DC_7A-7A_n257M | DC_7A_n257A DC_7A_n257D DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K |
| DC_7A_n258A DC_7A_n258B DC_7A_n258C DC_7A_n258D DC_7A_n258E DC_7A_n258F DC_7A_n258G DC_7A_n258H DC_7A_n258I DC_7A_n258J DC_7A_n258K DC_7A_n258L DC_7A_n258M DC_7C_n258A DC_7C_n258B DC_7C_n258C DC_7C_n258D DC_7C_n258E DC_7C_n258F DC_7C_n258G DC_7C_n258H DC_7C_n258I DC_7C_n258J DC_7C_n258K DC_7C_n258L DC_7C_n258M | DC_7A_n258A DC_7A_n258B DC_7A_n258C DC_7A_n258D DC_7A_n258E DC_7A_n258F DC_7A_n258G DC_7A_n258H DC_7A_n258I DC_7A_n258J DC_7A_n258K DC_7A_n258L DC_7A_n258M DC_7C_n258A DC_7C_n258B DC_7C_n258C DC_7C_n258D DC_7C_n258E DC_7C_n258F DC_7C_n258G DC_7C_n258H DC_7C_n258I DC_7C_n258J DC_7C_n258K DC_7C_n258L DC_7C_n258M |
| DC_7A_n258(2A) | DC_7A_n258A DC_7A_n258(2A) |
| DC_7A_n260A DC_7A_n260G DC_7A_n260H DC_7A_n260I DC_7A_n260J DC_7A_n260K DC_7A_n260L DC_7A_n260M DC_7A_n260O DC_7A_n260P DC_7A_n260Q | DC_7A_n260A DC_7A_n260G DC_7A_n260H DC_7A_n260I DC_7A_n260J DC_7A_n260K DC_7A_n260L DC_7A_n260M DC_7A_n260O DC_7A_n260P DC_7A_n260Q |
| DC_7A_n261A DC_7A_n261G DC_7A_n261H DC_7A_n261I DC_7A_n261J DC_7A_n261K DC_7A_n261L DC_7A_n261M DC_7A_n261O DC_7A_n261P DC_7A_n261Q | DC_7A_n261A DC_7A_n261G DC_7A_n261H DC_7A_n261I DC_7A_n261J DC_7A_n261K DC_7A_n261L DC_7A_n261M DC_7A_n261O DC_7A_n261P DC_7A_n261Q |
| DC_8A_n257A DC_8A_n257D DC_8A_n257E DC_8A_n257F DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_8A_n257J DC_8A_n257K DC_8A_n257L DC_8A_n257M | DC_8A_n257A DC_8A_n257D DC_8A_n257G DC_8A_n257H DC_8A_n257I |
| DC_8A_n258A DC_8A_n258B DC_8A_n258C DC_8A_n258D DC_8A_n258E DC_8A_n258F DC_8A_n258G DC_8A_n258H DC_8A_n258I DC_8A_n258J DC_8A_n258K DC_8A_n258L DC_8A_n258M | DC_8A_n258A |
| DC_11A_n257A DC_11A_n257D DC_11A_n257G DC_11A_n257H DC_11A_n257I | DC_11A_n257A DC_11A_n257D DC_11A_n257G DC_11A_n257H DC_11A_n257I |
| DC_12A_n257A DC_12A_n257G DC_12A_n257H DC_12A_n257I DC_12A_n257J DC_12A_n257K DC_12A_n257L DC_12A_n257M | DC_12A_n257A DC_12A_n257G DC_12A_n257H DC_12A_n257I DC_12A_n257J DC_12A_n257K DC_12A_n257L DC_12A_n257M |
| DC_12A_n258A DC_12A_n258D DC_12A_n258G DC_12A_n258H DC_12A_n258I DC_12A_n258J DC_12A_n258K DC_12A_n258L DC_12A_n258M DC_12A_n258O DC_12A_n258P DC_12A_n258Q | DC_12A_n258A DC_12A_n258D DC_12A_n258G DC_12A_n258H DC_12A_n258I DC_12A_n258J DC_12A_n258K DC_12A_n258L DC_12A_n258M DC_12A_n258O DC_12A_n258P DC_12A_n258Q |
| DC_12A_n260A DC_12A_n260D DC_12A_n260E DC_12A_n260G DC_12A_n260H DC_12A_n260I DC_12A_n260J DC_12A_n260K DC_12A_n260L DC_12A_n260M DC_12A_n260O DC_12A_n260P DC_12A_n260Q | DC_12A_n260A DC_12A_n260D DC_12A_n260E DC_12A_n260G DC_12A_n260H DC_12A_n260I DC_12A_n260J DC_12A_n260K DC_12A_n260L DC_12A_n260M DC_12A_n260O DC_12A_n260P DC_12A_n260Q |
| DC_12A_n260(2A) DC_12A_n260(2G) DC_12A_n260(A-G) DC_12A_n260(A-H) DC_12A_n260(G-H) DC_12A_n260(A-I) DC_12A_n260(G-I) | DC_12A_n260A |
| DC_12A_n261A DC_12A_n261B DC_12A_n261C DC_12A_n261D DC_12A_n261E DC_12A_n261F DC_12A_n261G DC_12A_n261H DC_12A_n261I DC_12A_n261J DC_12A_n261K DC_12A_n261L DC_12A_n261M DC_12A_n261O DC_12A_n261P DC_12A_n261Q | DC_12A_n261A DC_12A_n261B DC_12A_n261C DC_12A_n261D DC_12A_n261E DC_12A_n261F DC_12A_n261G DC_12A_n261H DC_12A_n261I DC_12A_n261J DC_12A_n261K DC_12A_n261L DC_12A_n261M DC_12A_n261O DC_12A_n261P DC_12A_n261Q |
| DC_13A_n257A | DC_13A_n257A |
| DC_13A_n260A DC_13A_n260D DC_13A_n260G DC_13A_n260H DC_13A_n260I DC_13A_n260J DC_13A_n260K DC_13A_n260L DC_13A_n260M DC_13A_n260O DC_13A_n260P DC_13A_n260Q | DC_13A_n260A DC_13A_n260G DC_13A_n260H DC_13A_n260I DC_13A_n260J DC_13A_n260K DC_13A_n260L DC_13A_n260M DC_13A_n260O DC_13A_n260P DC_13A_n260Q |
| DC_13A_n260(2A) DC_13A_n260(3A) DC_13A_n260(4A) DC_13A_n260(5A) DC_13A_n260(6A) DC_13A_n260(7A) DC_13A_n260(8A) DC_13A_n260(2D) DC_13A_n260(2G) DC_13A_n260(3G) DC_13A_n260(4G) DC_13A_n260(2H) DC_13A_n260(2O) DC_13A_n260(3O) DC_13A_n260(4O) DC_13A_n260(A-G) DC_13A_n260(A-2G) DC_13A_n260(A-P) DC_13A_n260(A-Q) DC_13A_n260(2A-G) DC_13A_n260(2A-H) DC_13A_n260(2A-2G) DC_13A_n260(2A-2H) DC_13A_n260(3A-G) DC_13A_n260(3A-O) DC_13A_n260(3A-P) DC_13A_n260(3A-2O) DC_13A_n260(4A-O) DC_13A_n260(4A-2O) DC_13A_n260(P-Q) DC_13A_n260(A-P-Q) DC_13A_n260(2A-O-P) DC_13A_n260(3A-O-P) DC_13A_n260(A-H) DC_13A_n260(A-2H) DC_13A_n260(2A-O) DC_13A_n260(A-O) DC_13A_n260(2A-P) DC_13A_n260(A-O-P) DC_13A_n260(O-P) DC_13A_n260(2A-2O) DC_13A_n260(A-2O) DC_13A_n260(G-H) DC_13A_n260(2A-G-H) DC_13A_n260(2A-Q) DC_13A_n260(2P) DC_13A_n260(3A-H) DC_13A_n260(4A-P) DC_13A_n260(5A-O) DC_13A_n260(A-2P) DC_13A_n260(A-3G) DC_13A_n260(A-3O) DC_13A_n260(A-D) DC_13A_n260(A-G-H) DC_13A_n260(A-O-Q) DC_13A_n260(O-Q) | DC_13A_n260A DC_13A_n260G DC_13A_n260H DC_13A_n260O DC_13A_n260P DC_13A_n260Q |
| DC_13A_n261A DC_13A_n261G DC_13A_n261H DC_13A_n261J DC_13A_n261K DC_13A_n261I DC_13A_n261L DC_13A_n261M | DC_13A_n261A DC_13A_n261G DC_13A_n261H DC_13A_n261I DC_13A_n261J DC_13A_n261K DC_13A_n261L DC_13A_n261M |
| DC_13A_n261(2A) DC_13A_n261(2G) DC_13A_n261(3A) DC_13A_n261(4A) DC_13A_n261(2H) DC_13A_n261(2I) DC_13A_n261(A-G) DC_13A_n261(A-K) DC_13A_n261(A-L) DC_13A_n261(A-2G) DC_13A_n261(A-H) DC_13A_n261(A-I) DC_13A_n261(A-J) DC_13A_n261(2A-G) DC_13A_n261(2A-H) DC_13A_n261(2A-I) DC_13A_n261(3A-G) DC_13A_n261(G-H) DC_13A_n261(G-I) DC_13A_n261(G-J) DC_13A_n261(H-I) DC_13A_n261(A-G-H) DC_13A_n261(A-G-I) | DC_13A_n261A DC_13A_n261G DC_13A_n261H DC_13A_n261I |
| DC_14A_n258A | DC_14A_n258A |
| DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_14A_n260J DC_14A_n260K DC_14A_n260L DC_14A_n260M | DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_14A_n260J DC_14A_n260K DC_14A_n260L DC_14A_n260M |
| DC_18A_n257A DC_18A_n257D DC_18A_n257E DC_18A_n257F DC_18A_n257G DC_18A_n257H DC_18A_n257I DC_18A_n257J DC_18A_n257K DC_18A_n257L DC_18A_n257M | DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I |
| DC_19A_n257A DC_19A_n257D DC_19A_n257E DC_19A_n257F DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_19A_n257J DC_19A_n257K DC_19A_n257L DC_19A_n257M | DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_19A_n257J DC_19A_n257K DC_19A_n257L DC_19A_n257M |
| DC_20A_n257A DC_20A_n257B DC_20A_n257C DC_20A_n257D DC_20A_n257E DC_20A_n257F DC_20A_n257G DC_20A_n257H DC_20A_n257I DC_20A_n257J DC_20A_n257K DC_20A_n257L DC_20A_n257M | DC_20A_n257A |
| DC_20A_n258A DC_20A_n258B DC_20A_n258C DC_20A_n258D DC_20A_n258E DC_20A_n258F DC_20A_n258G DC_20A_n258H DC_20A_n258I DC_20A_n258J DC_20A_n258K DC_20A_n258L DC_20A_n258M | DC_20A_n258A |
| DC_21A_n257A DC_21A_n257D DC_21A_n257E DC_21A_n257F DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_21A_n257J DC_21A_n257K DC_21A_n257L DC_21A_n257M | DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_21A_n257J DC_21A_n257K DC_21A_n257L DC_21A_n257M |
| DC_26A_n257A | DC_26A_n257A |
| DC_26A_n258A DC_26A_n258B DC_26A_n258C DC_26A_n258D DC_26A_n258E DC_26A_n258F DC_26A_n258G DC_26A_n258H DC_26A_n258I DC_26A_n258J DC_26A_n258K DC_26A_n258L DC_26A_n258M | DC_26A_n258A DC_26A_n258G DC_26A_n258H DC_26A_n258I |
| DC_28A_n257A DC_28A_n257D DC_28A_n257E DC_28A_n257F DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_28A_n257J DC_28A_n257K DC_28A_n257L DC_28A_n257M | DC_28A_n257A DC_28A_n257D DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_28A_n257J DC_28A_n257K DC_28A_n257L DC_28A_n257M |
| DC_28A_n258A DC_28A_n258B DC_28A_n258C DC_28A_n258D DC_28A_n258E DC_28A_n258F DC_28A_n258G DC_28A_n258H DC_28A_n258I DC_28A_n258J DC_28A_n258K DC_28A_n258L DC_28A_n258M | DC_28A_n258A DC_28A_n258G DC_28A_n258H DC_28A_n258I |
| DC_30A_n258A | DC_30A_n258A |
| DC_30A_n260A DC_30A_n260G DC_30A_n260H DC_30A_n260I DC_30A_n260J DC_30A_n260K DC_30A_n260L DC_30A_n260M | DC_30A_n260A DC_30A_n260G DC_30A_n260H DC_30A_n260I DC_30A_n260J DC_30A_n260K DC_30A_n260L DC_30A_n260M |
| DC_30A_n260(2A) DC_30A_n260(2G) DC_30A_n260(A-G) DC_30A_n260(A-H) DC_30A_n260(G-H) DC_30A_n260(A-I) DC_30A_n260(G-I) | DC_30A_n260A |
| DC_38A_n257A DC_38A_n257G DC_38A_n257H DC_38A_n257I DC_38A_n257J DC_38A_n257K DC_38A_n257L DC_38A_n257M | DC_38A_n257A DC_38A_n257G DC_38A_n257H DC_38A_n257I DC_38A_n257J DC_38A_n257K DC_38A_n257L DC_38A_n257M |
| DC_38A_n258A DC_38A_n258G DC_38A_n258H DC_38A_n258I DC_38A_n258J DC_38A_n258K DC_38A_n258L DC_38A_n258M | DC_38A_n258A DC_38A_n258G DC_38A_n258H DC_38A_n258I DC_38A_n258J DC_38A_n258K DC_38A_n258L DC_38A_n258M |
| DC_39A_n257A DC_39A_n257D DC_39A_n257E DC_39A_n257F DC_39A_n257G DC_39A_n257H DC_39A_n257I DC_39A_n257J DC_39A_n257K DC_39A_n257L DC_39A_n257M | DC_39A_n257A |
| DC_39A_n258A DC_39A_n258B DC_39A_n258C DC_39A_n258D DC_39A_n258E DC_39A_n258F DC_39A_n258G DC_39A_n258H DC_39A_n258I DC_39A_n258J DC_39A_n258K DC_39A_n258L DC_39A_n258M | DC_39A_n258A DC_39A_n258B DC_39A_n258C DC_39A_n258D DC_39A_n258E DC_39A_n258F DC_39A_n258G DC_39A_n258H DC_39A_n258I DC_39A_n258J DC_39A_n258K DC_39A_n258L DC_39A_n258M |
| DC_40A_n257A DC_40A_n257D DC_40A_n257E DC_40A_n257F DC_40A_n257G DC_40A_n257H DC_40A_n257I DC_40A_n257J DC_40A_n257K DC_40A_n257L DC_40A_n257M DC_40C_n257A DC_40C_n257D DC_40C_n257E DC_40C_n257F DC_40C_n257G DC_40C_n257H DC_40C_n257I DC_40C_n257J DC_40C_n257K DC_40C_n257L DC_40C_n257M | DC_40A_n257A DC_40A_n257G DC_40A_n257H DC_40A_n257I DC_40A_n257J DC_40A_n257K DC_40A_n257L DC_40A_n257M |
| DC_40A-40A_n257A DC_40A-40A_n257D DC_40A-40A_n257E DC_40A-40A_n257F DC_40A-40A_n257G DC_40A-40A_n257H DC_40A-40A_n257I DC_40A-40A_n257J DC_40A-40A_n257K DC_40A-40A_n257L DC_40A-40A_n257M | DC_40A_n257A DC_40A_n257G DC_40A_n257H DC_40A_n257I DC_40A_n257J DC_40A_n257K DC_40A_n257L DC_40A_n257M |
| DC_40A_n258A DC_40A_n258B DC_40A_n258C DC_40A_n258D DC_40A_n258E DC_40A_n258F DC_40A_n258G DC_40A_n258H DC_40A_n258I DC_40A_n258J DC_40A_n258K DC_40A_n258L DC_40A_n258M | DC_40A_n258A DC_40A_n258B DC_40A_n258C DC_40A_n258D DC_40A_n258E DC_40A_n258F DC_40A_n258G DC_40A_n258H DC_40A_n258I DC_40A_n258J DC_40A_n258K DC_40A_n258L DC_40A_n258M |
| DC_41A_n257A DC_41A_n257D DC_41A_n257E DC_41A_n257F DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41A_n257J DC_41A_n257K DC_41A_n257L DC_41A_n257M DC_41C_n257A DC_41C_n257D DC_41C_n257E DC_41C_n257F DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_41C_n257J DC_41C_n257K DC_41C_n257L DC_41C_n257M | DC_41A_n257A DC_41A_n257D DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n257A DC_41C_n257D DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_41A_n258A DC_41A_n258B DC_41A_n258C DC_41A_n258D DC_41A_n258E DC_41A_n258F DC_41A_n258G DC_41A_n258H DC_41A_n258I DC_41A_n258J DC_41A_n258K DC_41A_n258L DC_41A_n258M | DC_41A_n258A DC_41A_n258B DC_41A_n258C DC_41A_n258D DC_41A_n258E DC_41A_n258F DC_41A_n258G DC_41A_n258H DC_41A_n258I DC_41A_n258J DC_41A_n258K DC_41A_n258L DC_41A_n258M |
| DC_42A_n257A DC_42A_n257D DC_42A_n257E DC_42A_n257F DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42A_n257J DC_42A_n257K DC_42A_n257L DC_42A_n257M DC_42C_n257A DC_42C_n257D DC_42C_n257E DC_42C_n257F DC_42C_n257G DC_42C_n257H DC_42C_n257I DC_42C_n257J DC_42C_n257K DC_42C_n257L DC_42C_n257M DC_42D_n257A DC_42D_n257D DC_42D_n257E DC_42D_n257F DC_42D_n257G DC_42D_n257H DC_42D_n257I DC_42D_n257J DC_42D_n257K DC_42D_n257L DC_42D_n257M DC_42E_n257A DC_42E_n257D DC_42E_n257E DC_42E_n257F DC_42E_n257G DC_42E_n257H DC_42E_n257I DC_42E_n257J DC_42E_n257K DC_42E_n257L DC_42E_n257M | DC_42A_n257A DC_42A_n257D DC_42A_n257E DC_42A_n257F DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42A_n257J DC_42A_n257K DC_42A_n257L DC_42A_n257M DC_42C_n257A DC_42C_n257D DC_42C_n257E DC_42C_n257F DC_42D_n257A DC_42D_n257D DC_42D_n257E DC_42D_n257F DC_42E_n257A DC_42E_n257D DC_42E_n257E DC_42E_n257F |
| DC_42A-42A_n257A DC_42A-42A_n257D DC_42A-42A_n257E DC_42A-42A_n257F DC_42A-42A_n257G DC_42A-42A_n257H DC_42A-42A_n257I DC_42A-42A_n257J DC_42A-42A_n257K DC_42A-42A_n257L DC_42A-42A_n257M | DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42A_n257J DC_42A_n257K DC_42A_n257L DC_42A_n257M |
| DC_46A_n260A3 | N/A |
| DC_46A_n261A3 | N/A |
| DC_48A_n257A DC_48C_n257A | DC_48A_n257A DC_48C_n257A |
| DC_48A-48A_n257A | DC_48A_n257A |
| DC_48A_n260A DC_48A_n260G DC_48A_n260H DC_48A_n260I DC_48A_n260J DC_48A_n260K DC_48A_n260L DC_48A_n260M DC_48C_n260A DC_48D_n260A | DC_48A_n260A DC_48A_n260G DC_48A_n260H DC_48A_n260I DC_48C_n260A |
| DC_48A_n260(2A) DC_48C_n260(2A) DC_48D_n260(2A) DC_48A_n260(3A) DC_48C_n260(3A) DC_48D_n260(3A) DC_48A_n260(4A) DC_48C_n260(4A) DC_48D_n260(4A) | DC_48A_n260A DC_48A_n260G DC_48A_n260H DC_48A_n260I DC_48C_n260A |
| DC_48A-48A_n260A | DC_48A_n260A |
| DC_48A_n261A DC_48A_n261G DC_48A_n261H DC_48A_n261I DC_48A_n261J DC_48A_n261K DC_48A_n261L DC_48A_n261M DC_48C_n261A DC_48D_n261A | DC_48A_n261A DC_48A_n261G DC_48A_n261H DC_48A_n261I DC_48C_n261A |
| DC_48A_n261(A-G) DC_48A_n261(A-G-H) DC_48A_n261(A-G-I) DC_48A_n261(A-H) DC_48A_n261(A-I) DC_48A_n261(A-J) DC_48A_n261(A-K) DC_48A_n261(A-L) DC_48A_n261(A-2G) DC_48A_n261(G-H) DC_48A_n261(G-I) DC_48A_n261(G-J) DC_48A_n261(H-I) DC_48A_n261(2A) DC_48C_n261(2A) DC_48D_n261(2A) DC_48A_n261(3A) DC_48A_n261(2A-G) DC_48A_n261(2A-H) DC_48A_n261(2A-I) DC_48A_n261(2G) DC_48A_n261(2H) DC_48A_n261(4A) DC_48A_n261(3A-G) | DC_48A_n261A DC_48A_n261G DC_48A_n261H DC_48A_n261I DC_48C_n261A |
| DC_66A_n257A DC_66A_n257G DC_66A_n257H DC_66A_n257I DC_66A_n257J DC_66A_n257K DC_66A_n257L DC_66A_n257M DC_66C_n257A | DC_66A_n257A DC_66A_n257G |
| DC_66A_n257(2A) DC_66A-66A_n257A | DC_66A_n257A |
| DC_66A_n258A DC_66A_n258D DC_66A_n258G DC_66A_n258H DC_66A_n258I DC_66A_n258J DC_66A_n258K DC_66A_n258L DC_66A_n258M DC_66A_n258O DC_66A_n258P DC_66A_n258Q | DC_66A_n258A DC_66A_n258D DC_66A_n258G DC_66A_n258H DC_66A_n258I DC_66A_n258J DC_66A_n258K DC_66A_n258L DC_66A_n258M DC_66A_n258O DC_66A_n258P DC_66A_n258Q |
| DC_66A_n258(2A) DC_66A_n258(3A) DC_66A_n258(4A) DC_66A_n258(5A) | DC_66A_n258A |
| DC_66A_n260A DC_66A_n260D DC_66A_n260E DC_66A_n260F DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M DC_66A_n260O DC_66A_n260P DC_66A_n260Q | DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M DC_66A_n260O DC_66A_n260P DC_66A_n260Q |
| DC_66A_n260(2A) DC_66A_n260(3A) DC_66A_n260(4A) DC_66A_n260(5A) DC_66A_n260(6A) DC_66A_n260(7A) DC_66A_n260(8A) DC_66A_n260(9A) DC_66A_n260(10A) DC_66A_n260(A-I) DC_66A_n260(D-G) DC_66A_n260(D-H) DC_66A_n260(D-I) DC_66A_n260(D-O) DC_66A_n260(D-P) DC_66A_n260(D-Q) DC_66A_n260(E-O) DC_66A_n260(E-P) DC_66A_n260(E-Q) DC_66A_n260(G-I) DC_66A_n260(2G) DC_66A_n260(2H) DC_66A_n260(2O) DC_66A_n260(3O) DC_66A_n260(4O) DC_66A_n260(2P) DC_66A_n260(3P) DC_66A_n260(4P) DC_66A_n260(2A-O) DC_66A_n260(A-2O) DC_66A_n260(2A-G) DC_66A_n260(A-2G) DC_66A_n260(2A-2G) DC_66A_n260(2G-O) DC_66A_n260(2A-2G-O) DC_66A_n260(A-2H) DC_66A_n260(2A-H) DC_66A_n260(2A-2H) DC_66A_n260(2A-2O) DC_66A_n260(2A-3O) DC_66A_n260(A-4O) DC_66A_n260(2A-4O) DC_66A_n260(3A-2O) DC_66A_n260(3A-2G) DC_66A_n260(4A-G) DC_66A_n260(4A-2G) DC_66A_n260(4A-O) DC_66A_n260(4A-2O) DC_66A_n260(A-O) DC_66A_n260(A-G) DC_66A_n260(G-O) DC_66A_n260(A-G-O) DC_66A_n260(2A-G-O) DC_66A_n260(A-2G-O) DC_66A_n260(A-H) DC_66A_n260(A-3O) DC_66A_n260(3A-O) DC_66A_n260(3A-O-P) DC_66A_n260(3A-P) DC_66A_n260(3A-G) DC_66A_n260(2D) DC_66A_n260(3G) DC_66A_n260(4G) DC_66A_n260(A-D) DC_66A_n260(2A-D) DC_66A_n260(A-D-O) DC_66A_n260(2A-D-O) DC_66A_n260(D-2O) DC_66A_n260(A-D-2O) DC_66A_n260(2A-D-2O) DC_66A_n260(2A-O-P) DC_66A_n260(A-2D) DC_66A_n260(2A-2D) DC_66A_n260(A-P) DC_66A_n260(A-P-Q) DC_66A_n260(2A-P) DC_66A_n260(A-2P) DC_66A_n260(2A-2P) DC_66A_n260(3A-3O) DC_66A_n260(D-2G) DC_66A_n260(2D-O) DC_66A_n260(G-H) DC_66A_n260(G-2O) DC_66A_n260(2G-2O) DC_66A_n260(G-3O) DC_66A_n260(2G-3O) DC_66A_n260(G-4O) DC_66A_n260(2G-4O) DC_66A_n260(3G-O) DC_66A_n260(4G-O) DC_66A_n260(H-O) DC_66A_n260(2H-O) DC_66A_n260(2A-2G-2O) DC_66A_n260(6A-2O) DC_66A_n260(8A-2O) DC_66A_n260(2A-2O-2P) DC_66A_n260(6A-3O) DC_66A_n260(4A-4O) DC_66A_n260(6A-2P) DC_66A_n260(2O-2P) DC_66A_n260(2A-4P) DC_66A_n260(4A-2Q) DC_66A_n260(2A-2O-2Q) DC_66A_n260(A-Q) DC_66A_n260(P-Q) DC_66A_n260(A-O-P) DC_66A_n260(O-P) DC_66A_n260(2A-2O-P-Q) DC_66A_n260(2A-2O-P) DC_66A_n260(2A-2O-Q) DC_66A_n260(2A-2Q) DC_66A_n260(2A-3G) DC_66A_n260(2A-3O-P) DC_66A_n260(2A-3O-Q) DC_66A_n260(2A-3P) DC_66A_n260(2A-G-2O) DC_66A_n260(2A-G-3O) DC_66A_n260(2A-G-H) DC_66A_n260(2A-O-2P) DC_66A_n260(2A-O-2Q) DC_66A_n260(2A-O-3P) DC_66A_n260(2A-O-P-Q) DC_66A_n260(2A-O-Q) DC_66A_n260(2A-P-Q) DC_66A_n260(2A-Q) DC_66A_n260(2O-2Q) DC_66A_n260(2O-P-Q) DC_66A_n260(2O-P) DC_66A_n260(2O-Q) DC_66A_n260(2Q) DC_66A_n260(3A-2G-O) DC_66A_n260(3A-2O-P) DC_66A_n260(3A-2O-Q) DC_66A_n260(3A-2P) DC_66A_n260(3A-2Q) DC_66A_n260(3A-3P) DC_66A_n260(3A-4O) DC_66A_n260(3A-D-O) DC_66A_n260(3A-D) DC_66A_n260(3A-G-2O) DC_66A_n260(3A-G-O) DC_66A_n260(3A-H) DC_66A_n260(3A-O-2P) DC_66A_n260(3A-O-2Q) DC_66A_n260(3A-O-P-Q) DC_66A_n260(3A-O-Q) DC_66A_n260(3A-P-Q) DC_66A_n260(3A-Q) DC_66A_n260(3O-P) DC_66A_n260(3O-Q) DC_66A_n260(4A-2P) DC_66A_n260(4A-3O) DC_66A_n260(4A-D) DC_66A_n260(4A-G-O) DC_66A_n260(4A-O-P) DC_66A_n260(4A-O-Q) DC_66A_n260(4A-P-Q) DC_66A_n260(4A-P) DC_66A_n260(4A-Q) DC_66A_n260(5A-2O) DC_66A_n260(5A-2P) DC_66A_n260(5A-3O) DC_66A_n260(5A-G) DC_66A_n260(5A-O-P) DC_66A_n260(5A-O) DC_66A_n260(5A-P) DC_66A_n260(5A-Q) DC_66A_n260(6A-O-P) DC_66A_n260(6A-O) DC_66A_n260(6A-P) DC_66A_n260(7A-2O) DC_66A_n260(7A-O) DC_66A_n260(7A-P) DC_66A_n260(8A-O) DC_66A_n260(9A-O) DC_66A_n260(A-2G-2O) DC_66A_n260(A-2G-3O) DC_66A_n260(A-2O-2P) DC_66A_n260(A-2O-2Q) DC_66A_n260(A-2O-P-Q) DC_66A_n260(A-2O-P) DC_66A_n260(A-2O-Q) DC_66A_n260(A-2Q) DC_66A_n260(A-3G-O) DC_66A_n260(A-3G) DC_66A_n260(A-3O-P) DC_66A_n260(A-3O-Q) DC_66A_n260(A-3P) DC_66A_n260(A-4G) DC_66A_n260(A-4P) DC_66A_n260(A-D-G) DC_66A_n260(A-E) DC_66A_n260(A-G-2O) DC_66A_n260(A-G-3O) DC_66A_n260(A-G-4O) DC_66A_n260(A-G-H) DC_66A_n260(A-H-O) DC_66A_n260(A-O-2P) DC_66A_n260(A-O-2Q) DC_66A_n260(A-O-3P) DC_66A_n260(A-O-P-Q) DC_66A_n260(A-O-Q) DC_66A_n260(G-H-O) DC_66A_n260(O-2P) DC_66A_n260(O-2Q) DC_66A_n260(O-3P) DC_66A_n260(O-P-Q) DC_66A_n260(O-Q) | DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260O DC_66A_n260P DC_66A_n260Q |
| DC_66A-66A_n260A DC_66A-66A_n260G DC_66A-66A_n260H DC_66A-66A_n260I DC_66A-66A_n260J DC_66A-66A_n260K DC_66A-66A_n260L DC_66A-66A_n260M | DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M |
| DC_66A-66A_n260(2A) DC_66A-66A_n260(2G) DC_66A-66A_n260(2H) DC_66A-66A_n260(3A) DC_66A-66A_n260(4A) DC_66A-66A_n260(5A) DC_66A-66A_n260(6A) DC_66A-66A_n260(A-G) DC_66A-66A_n260(A-H) DC_66A-66A_n260(A-2G) DC_66A-66A_n260(G-H) DC_66A-66A_n260(2A-G) DC_66A-66A_n260(2A-2G) DC_66A-66A_n260(3A-G) | DC_66A_n260A DC_66A_n260G DC_66A_n260H |
| DC_66A_n261A DC_66A_n261B DC_66A_n261C DC_66A_n261D DC_66A_n261E DC_66A_n261F DC_66A_n261G DC_66A_n261H DC_66A_n261I DC_66A_n261J DC_66A_n261K DC_66A_n261L DC_66A_n261M DC_66A_n261O DC_66A_n261P DC_66A_n261Q | DC_66A_n261A DC_66A_n261B DC_66A_n261C DC_66A_n261G DC_66A_n261H DC_66A_n261I DC_66A_n261J DC_66A_n261K DC_66A_n261L DC_66A_n261M DC_66A_n261O |
| DC_66A_n261(2A) DC_66A_n261(3A) DC_66A_n261(4A) DC_66A_n261(2G) DC_66A_n261(D-G) DC_66A_n261(D-H) DC_66A_n261(D-I) DC_66A_n261(D-O) DC_66A_n261(D-P) DC_66A_n261(D-Q) DC_66A_n261(E-O) DC_66A_n261(E-P) DC_66A_n261(E-Q) DC_66A_n261(2H) DC_66A_n261(2I) DC_66A_n261(A-H) DC_66A_n261(A-I) DC_66A_n261(A-J) DC_66A_n261(A-K) DC_66A_n261(A-L) DC_66A_n261(A-D) DC_66A_n261(A-D-H) DC_66A_n261(A-G) DC_66A_n261(A-G-H) DC_66A_n261(G-I) DC_66A_n261(G-J) DC_66A_n261(A-G-I) DC_66A_n261(A-H-I) DC_66A_n261(G-H) DC_66A_n261(H-I) DC_66A_n261(A-D-2O) DC_66A_n261(A-2D) DC_66A_n261(A-2G) DC_66A_n261(A-2G-2O) DC_66A_n261(A-3G-O) DC_66A_n261(A-4G) DC_66A_n261(A-2H) DC_66A_n261(A-2I) DC_66A_n261(A-4O) DC_66A_n261(A-7O) DC_66A_n261(A-2P) DC_66A_n261(A-2Q) DC_66A_n261(2A-G) DC_66A_n261(2A-H) DC_66A_n261(2A-I) DC_66A_n261(3A-G) DC_66A_n261(2A-2G-O) DC_66A_n261(2A-2G) DC_66A_n261(2A-2O) DC_66A_n261(2A-3G) DC_66A_n261(2A-3O) DC_66A_n261(2A-4O) DC_66A_n261(2A-5O) DC_66A_n261(2A-6O) DC_66A_n261(2A-D-O) DC_66A_n261(2A-D) DC_66A_n261(2A-G-2O) DC_66A_n261(2A-G-O) DC_66A_n261(2A-O) DC_66A_n261(2A-P) DC_66A_n261(2A-Q) DC_66A_n261(2D) DC_66A_n261(2G-2O) DC_66A_n261(2G-O) DC_66A_n261(2O) DC_66A_n261(2P) DC_66A_n261(2Q) DC_66A_n261(3A-2G) DC_66A_n261(3A-2O) DC_66A_n261(3A-3O) DC_66A_n261(3A-4O) DC_66A_n261(3A-5O) DC_66A_n261(3A-D) DC_66A_n261(3A-G-O) DC_66A_n261(3A-O) DC_66A_n261(3G-O) DC_66A_n261(3G) DC_66A_n261(3O) DC_66A_n261(4A-2O) DC_66A_n261(4A-3O) DC_66A_n261(4A-4O) DC_66A_n261(4A-G) DC_66A_n261(4A-O) DC_66A_n261(4G) DC_66A_n261(4O) DC_66A_n261(5A-2O) DC_66A_n261(5A-3O) DC_66A_n261(5A-O) DC_66A_n261(5A) DC_66A_n261(5O) DC_66A_n261(6A-2O) DC_66A_n261(6A-O) DC_66A_n261(6A) DC_66A_n261(6O) DC_66A_n261(7A-O) DC_66A_n261(7A) DC_66A_n261(7O) DC_66A_n261(8A) DC_66A_n261(A-2G-O) DC_66A_n261(A-2O) DC_66A_n261(A-3G) DC_66A_n261(A-3O) DC_66A_n261(A-5O) DC_66A_n261(A-6O) DC_66A_n261(A-D-G) DC_66A_n261(A-D-O) DC_66A_n261(A-E) DC_66A_n261(A-G-2O) DC_66A_n261(A-G-O) DC_66A_n261(A-O-P) DC_66A_n261(A-O-Q) DC_66A_n261(A-O) DC_66A_n261(A-P-Q) DC_66A_n261(A-P) DC_66A_n261(A-Q) DC_66A_n261(D-2O) DC_66A_n261(G-2O) DC_66A_n261(G-O) DC_66A_n261(O-P) DC_66A_n261(O-Q) DC_66A_n261(P-Q) | DC_66A_n261A DC_66A_n261G DC_66A_n261H DC_66A_n261I |
| DC_66A-66A_n261A DC_66A-66A_n261G DC_66A-66A_n261H DC_66A-66A_n261I DC_66A-66A_n261J DC_66A-66A_n261K DC_66A-66A_n261L DC_66A-66A_n261M | DC_66A_n261A DC_66A_n261G DC_66A_n261H DC_66A_n261I DC_66A_n261J DC_66A_n261K DC_66A_n261L DC_66A_n261M |
| DC_66A-66A_n261(2A) DC_66A-66A_n261(2G) DC_66A-66A_n261(3A) DC_66A-66A_n261(4A) DC_66A-66A_n261(A-G) DC_66A-66A_n261(A-G-H) DC_66A-66A_n261(A-G-I) DC_66A-66A_n261(A-2G) DC_66A-66A_n261(A-H) DC_66A-66A_n261(A-I) DC_66A-66A_n261(A-J) DC_66A-66A_n261(A-K) DC_66A-66A_n261(A-L) DC_66A-66A_n261(G-H) DC_66A-66A_n261(G-I) DC_66A-66A_n261(G-J) DC_66A-66A_n261(H-I) DC_66A-66A_n261(2H) DC_66A-66A_n261(2A-G) DC_66A-66A_n261(2A-H) DC_66A-66A_n261(2A-I) DC_66A-66A_n261(3A-G) | DC_66A_n261A DC_66A_n261G DC_66A_n261H DC_66A_n261I |
| DC_71A_n257A DC_71A_n257G DC_71A_n257H DC_71A_n257I DC_71A_n257J DC_71A_n257K DC_71A_n257L DC_71A_n257M | DC_71A_n257A DC_71A_n257G DC_71A_n257H DC_71A_n257I DC_71A_n257J DC_71A_n257K DC_71A_n257L DC_71A_n257M |
| DC_71A_n258A DC_71A_n258G DC_71A_n258H DC_71A_n258I DC_71A_n258J DC_71A_n258K DC_71A_n258L DC_71A_n258M | DC_71A_n258A DC_71A_n258G DC_71A_n258H DC_71A_n258I DC_71A_n258J DC_71A_n258K DC_71A_n258L DC_71A_n258M |
| DC_71A_n260A DC_71A_n260G DC_71A_n260H DC_71A_n260I DC_71A_n260J DC_71A_n260K DC_71A_n260L DC_71A_n260M DC_71A_n260O DC_71A_n260P DC_71A_n260Q | DC_71A_n260A DC_71A_n260G DC_71A_n260H DC_71A_n260I DC_71A_n260J DC_71A_n260K DC_71A_n260L DC_71A_n260M DC_71A_n260O DC_71A_n260P DC_71A_n260Q |
| DC_71A_n261A DC_71A_n261G DC_71A_n261H DC_71A_n261I DC_71A_n261J DC_71A_n261K DC_71A_n261L DC_71A_n261M DC_71A_n261O DC_71A_n261P DC_71A_n261Q | DC_71A_n261A DC_71A_n261G DC_71A_n261H DC_71A_n261I DC_71A_n261J DC_71A_n261K DC_71A_n261L DC_71A_n261M DC_71A_n261O DC_71A_n261P DC_71A_n261Q |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability for all of the above combinations. NOTE 3: Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band for Band 46 is paired with the uplink operating band (external E-UTRA band) of the carrier aggregation configuration that is supporting the configured Pcell. |  |

<< Next Changes >>

6.2B.1.3	Inter-band EN-DC within FR1

For inter-band EN-DC of E-UTRA and NR in FR1, the following UE Power Classes define the maximum output power for any transmission bandwidth within the aggregated channel bandwidth. The maximum output power is measured as the sum of the maximum output power at each UE antenna connector. The period of measurement shall be at least one sub frame (1ms). UE maximum output power shall be measured over all component carriers from different bands. If each band has separate antenna connectors, maximum output power is measured as the sum of maximum output power at each UE antenna connector.

Table 6.2B.1.3-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_1A_n3A |  |  | 23 | +2/-3 |
| DC_1A_n5A |  |  | 23 | +2/-3 |
| DC_1A_n7A |  |  | 23 | +2/-3 |
| DC_1A_n8A |  |  | 23 | +2/-3 |
| DC_1A_n20A |  |  | 23 | +2/-3 |
| DC_1A_n26A |  |  | 23 | +2/-3 |
| DC_1A_n28A |  |  | 23 | +2/-3 |
| DC_1A_n38A |  |  | 23 | +2/-3 |
| DC_1A_n40A |  |  | 23 | +2/-3 |
| DC_1A_n41A |  |  | 23 | +2/-3 |
| DC_1A_n50A |  |  | 23 | +2/-3 |
| DC_1A_n51A |  |  | 23 | +2/-3 |
| DC_1A_n71A |  |  | 23 | +2/-3 |
| DC_1A_n77A DC_1A_n84A_ULSUP-TDM_n77A | 266,8 | +2/-3 | 23 | +2/-3 |
| DC_1A_n78A DC_1A_n84A_ULSUP-TDM_n78A | 266,8 | +2/-3 | 23 | +2/-3 |
| DC_1A_n79A | 266,8 | +2/-3 | 23 | +2/-3 |
| DC_1A_n84A_ULSUP-TDM_n79A |  |  | 23 | +2/-3 |
| DC_1A_n80A |  |  | 23 | +2/-3 |
| DC_1A_n105A |  |  | 23 | +2/-3 |
| DC_2A_n5A |  |  | 23 | +2/-3 |
| DC_2A_n7A |  |  | 23 | +2/-3 |
| DC_2A_n12A |  |  | 23 | +2/-3 |
| DC_2A_n25A |  |  | N/A | N/A |
| DC_2A_n28A |  |  | 23 | +2/-3 |
| DC_2A_n30A |  |  | 23 | +2/-3 |
| DC_2A_n38A |  |  | 23 | +2/-3 |
| DC_2A_n41A | 266,8 | +2/-3 | 23 | +2/-3 |
| DC_2A_n46A |  |  | 23 | +2/-3 |
| DC_2A_n48A |  |  | 23 | +2/-3 |
| DC_2A_n66A |  |  | 23 | +2/-3 |
| DC_2A_n71A |  |  | 23 | +2/-3 |
| DC_2A_n77A | 266,8 | +2/-3 | 23 | +2/-3 |
| DC_2A_n78A |  |  | 23 | +2/-3 |
| DC_3A_n1A DC_3C_n1A |  |  | 23 | +2/-3 |
| DC_3A_n5A DC_3C_n5A |  |  | 23 | +2/-3 |
| DC_3A_n7A DC_3A_n7B DC_3C_n7A |  |  | 23 | +2/-3 |
| DC_3A_n8A |  |  | 23 | +2/-3 |
| DC_3A_n20A |  |  | 23 | +2/-3 |
| DC_3A_n26A DC_3C_n26A |  |  | 23 | +2/-3 |
| DC_3A_n28A DC_3C_n28A |  |  | 23 | +2/-3 |
| DC_3A_n38A |  |  | 23 | +2/-3 |
| DC_3A_n40A |  |  | 23 | +2/-3 |
| DC_3A_n41A, DC_3C_n41A, DC_3C_n41A, | 266,8 | +2/-3 | 23 | +2/-3 |
| DC_3A_n50A |  |  | 23 | +2/-3 |
| DC_3A_n51A |  |  | 23 | +2/-3 |
| DC_3A_n71A |  |  | 23 | +2/-3 |
| DC_3A_n77A DC_3C_n77A | 266,8 | +2/-3 | 23 | +2/-3 |
| DC_3A_n78A DC_3C_n78A | 266,8 | +2/-3 | 23 | +2/-3 |
| DC_3A_n79A DC_3C_n79A | 266,8 | +2/-3 | 23 | +2/-3 |
| DC_3A_n80A_ULSUP-TDM_n41 DC_3C_n80A_ULSUP-TDM_n41 |  |  | 23 | +2/-3 |
| DC_3A_n80A_ULSUP-TDM_n77A |  |  | 23 | +2/-3 |
| DC_3A_n80A_ULSUP-TDM_n78A |  |  | 23 | +2/-3 |
| DC_3A_n80A_ULSUP-TDM_n79A |  |  | 23 | +2/-3 |
| DC_3A_n82A |  |  | 23 | +2/-3 |
| DC_3A_n84A |  |  | 23 | +2/-3 |
| DC_3A_n105A |  |  | 23 | +2/-3 |
| DC_4A_n2A |  |  | 23 | +2/-3 |
| DC_4A_n5A |  |  | 23 | +2/-3 |
| DC_4A_n7A |  |  | 23 | +2/-3 |
| DC_4A_n28A |  |  | 23 | +2/-3 |
| DC_4A_n38A |  |  | 23 | +2/-3 |
| DC_4A_n41A |  |  | 23 | +2/-3 |
| DC_4A_n78A |  |  | 23 | +2/-3 |
| DC_5A_n2A |  |  | 23 | +2/-3 |
| DC_5A_n7A |  |  | 23 | +2/-3 |
| DC_5A_n12A |  |  | 23 | +2/-3 |
| DC_5A_n25A |  |  | 23 | +2/-3 |
| DC_5A_n30A |  |  | 23 | +2/-3 |
| DC_5A_n38A |  |  | 23 | +2/-3 |
| DC_5A_n40A |  |  | 23 | +2/-3 |
| DC_5A_n41A |  |  | 23 | +2/-3 |
| DC_5A_n48A |  |  | 23 | +2/-3 |
| DC_5A_n66A |  |  | 23 | +2/-3 |
| DC_5A_n71A |  |  | 23 | +2/-3 |
| DC_5A_n77A | 266,8 | +2/-3 | 23 | +2/-3 |
| DC_5A_n78A | 266,8 | +2/-3 | 23 | +2/-3 |
| DC_5A_n79A |  |  | 23 | +2/-3 |
| DC_7A_n1A |  |  | 23 | +2/-3 |
| DC_7A_n2A |  |  | 23 | +2/-3 |
| DC_7A_n3A |  |  | 23 | +2/-3 |
| DC_7A_n5A DC_7C_n5A |  |  | 23 | +2/-3 |
| DC_7A_n8A |  |  | 23 | +2/-3 |
| DC_7A_n12A |  |  | 23 | +2/-3 |
| DC_7A_n20A |  |  | 23 | +2/-3 |
| DC_7A_n25A |  |  | 23 | +2/-3 |
| DC_7A_n26A DC_7C_n26A |  |  | 23 | +2/-3 |
| DC_7A_n28A |  |  | 23 | +2/-3 |
| DC_7A_n40A |  |  | 23 | +2/-3 |
| DC_7A_n51A |  |  | 23 | +2/-3 |
| DC_7A_n66A |  |  | 23 | +2/-3 |
| DC_7A_n71A |  |  | 23 | +2/-3 |
| DC_7A_n77A |  |  | 23 | +2/-3 |
| DC_7A_n78A DC_7C_n78A | 266,8 | +2/-3 | 23 | +2/-3 |
| DC_7A_n79A |  |  | 23 | +2/-3 |
| DC_7A_n80A |  |  | 23 | +2/-3 |
| DC_7A_n105A |  |  | 23 | +2/-3 |
| DC_8A_n1A |  |  | 23 | +2/-3 |
| DC_8A_n2A |  |  | 23 | +2/-3 |
| DC_8A_n3A |  |  | 23 | +2/-3 |
| DC_8A_n7A |  |  | 23 | +2/-3 |
| DC_8A_n20A |  |  | 23 | +2/-3 |
| DC_8A_n28A |  |  | 23 | +2/-3 |
| DC_8A_n34A |  |  | 23 | +2/-3 |
| DC_8A_n38A |  |  | 23 | +2/-3 |
| DC_8A_n39A |  |  | 23 | +2/-3 |
| DC_8A_n40A |  |  | 23 | +2/-3 |
| DC_8A_n41A, |  |  | 23 | +2/-3 |
| DC_8A_n77A |  |  | 23 | +2/-3 |
| DC_8A_n78A | 266,8 | +2/-3 | 23 | +2/-3 |
| DC_8B_n78A |  |  | 23 | +2/-3 |
| DC_8A_n79A DC_8A_n79C |  |  | 23 | +2/-3 |
| DC_8A_n80A |  |  | 23 | +2/-3 |
| DC_8A_n81A_ULSUP-TDM_n41 |  |  | 23 | +2/-3 |
| DC_8A_n81A_ULSUP-TDM_n78A |  |  | 23 | +2/-3 |
| DC_8A_n81A_ULSUP-TDM_n79A |  |  | 23 | +2/-3 |
| DC_11A_n1A |  |  | 23 | +2/-3 |
| DC_11A_n3A |  |  | 23 | +2/-3 |
| DC_11A_n28A |  |  | 23 | +2/-3 |
| DC_11A_n41A |  |  | 23 | +2/-3 |
| DC_11A_n77A |  |  | 23 | +2/-3 |
| DC_11A_n78A |  |  | 23 | +2/-3 |
| DC_11A_n79A |  |  | 23 | +2/-3 |
| DC_12A_n2A |  |  | 23 | +2/-3 |
| DC_12A_n5A |  |  | 23 | +2/-3 |
| DC_12A_n7A |  |  | 23 | +2/-3 |
| DC_12A_n25A |  |  | 23 | +2/-3 |
| DC_12A_n30A |  |  | 23 | +2/-3 |
| DC_12A_n38A |  |  | 23 | +2/-3 |
| DC_12A_n41A |  |  | 23 | +2/-3 |
| DC_12A_n66A |  |  | 23 | +2/-3 |
| DC_12A_n71A7 |  |  | 23 | +2/-3 |
| DC_12A_n77A | 266,8 | +2/-3 | 23 | +2/-3 |
| DC_12A_n78A |  |  | 23 | +2/-3 |
| DC_13A_n2A |  |  | 23 | +2/-3 |
| DC_13A_n5A |  |  | 23 | +2/-3 |
| DC_13A_n7A |  |  | 23 | +2/-3 |
| DC_13A_n25A |  |  | 23 | +2/-3 |
| DC_13A_n48A |  |  | 23 | +2/-3 |
| DC_13A_n66A |  |  | 23 | +2/-3 |
| DC_13A_n71A |  |  | 23 | +2/-3 |
| DC_13A_n77A | 266,8 | +2/-3 | 23 | +2/-3 |
| DC_13A_n78A |  |  | 23 | +2/-3 |
| DC_14A_n2A |  |  | 23 | +2/-3 |
| DC_14A_n5A |  |  | 23 | +2/-3 |
| DC_14A_n30A |  |  | 23 | +2/-3 |
| DC_14A_n66A |  |  | 23 | +2/-3 |
| DC_14A_n77A | 266,8 | +2/-3 | 23 | +2/-3 |
| DC_18A_n3A |  |  | 23 | +2/-3 |
| DC_18A_n28A |  |  | 23 | +2/-3 |
| DC_18A_n41A |  |  | 23 | +2/-3 |
| DC_18A_n77A |  |  | 23 | +2/-3 |
| DC_18A_n78A |  |  | 23 | +2/-3 |
| DC_18A_n79A |  |  | 23 | +2/-3 |
| DC_19A_n1A |  |  | 23 | +2/-3 |
| DC_19A_n77A |  |  | 23 | +2/-3 |
| DC_19A_n78A |  |  | 23 | +2/-3 |
| DC_19A_n79A | 266,8 | +2/-3 | 23 | +2/-3 |
| DC_20A_n1A |  |  | 23 | +2/-3 |
| DC_20A_n3A |  |  | 23 | +2/-3 |
| DC_20A_n7A |  |  | 23 | +2/-3 |
| DC_20A_n8A |  |  | 23 | +2/-3 |
| DC_20A_n38A |  |  | 23 | +2/-3 |
| DC_20A_n28A |  |  | 23 | +2/-3 |
| DC_20A_n41A |  |  | 23 | +2/-3 |
| DC_20A_n50A |  |  | 23 | +2/-3 |
| DC_20A_n51A |  |  | 23 | +2/-3 |
| DC_20A_n77A |  |  | 23 | +2/-3 |
| DC_20A_n80A |  |  | 23 | +2/-3 |
| DC_20A_n78A |  |  | 23 | +2/-3 |
| DC_20A_n82A_ULSUP-TDM_n78A |  |  | 23 | +2/-3 |
| DC_20A_n83A |  |  | 23 | +2/-3 |
| DC_21A_n1A |  |  | 23 | +2/-3 |
| DC_21A_n28A |  |  | 23 | +2/-3 |
| DC_21A_n77A | 266,8 | +2/-3 | 23 | +2/-3 |
| DC_21A_n78A | 266,8 | +2/-3 | 23 | +2/-3 |
| DC_21A_n79A | 266,8 | +2/-3 | 23 | +2/-3 |
| DC_25A_n41A |  |  | 23 | +2/-3 |
| DC_25A_n77A |  |  | 23 | +2/-3 |
| DC_25A_n78A |  |  | 23 | +2/-3 |
| DC_26A_n25A |  |  | 23 | +2/-3 |
| DC_26A_n41A |  |  | 23 | +2/-3 |
| DC_26A_n77A |  |  | 23 | +2/-3 |
| DC_26A_n78A |  |  | 23 | +2/-3 |
| DC_26A_n79A |  |  | 23 | +2/-3 |
| DC_28A_n1A |  |  | 23 | +2/-3 |
| DC_28A_n2A |  |  | 23 | +2/-3 |
| DC_28A_n3A |  |  | 23 | +2/-3 |
| DC_28A_n5A |  |  | 23 | +2/-3 |
| DC_28A_n7A DC_28A_n7B |  |  | 23 | +2/-3 |
| DC_28A_n8A |  |  | 23 | +2/-3 |
| DC_28A_n20A |  |  | 23 | +2/-3 |
| DC_28A_n38A |  |  | 23 | +2/-3 |
| DC_28A_n40A |  |  | 23 | +2/-3 |
| DC_28A_n41A |  |  | 23 | +2/-3 |
| DC_28A_n50A |  |  | 23 | +2/-3 |
| DC_28A_n51A |  |  | 23 | +2/-3 |
| DC_28A_n66A |  |  | 23 | +2/-3 |
| DC_28A_n77A |  |  | 23 | +2/-3 |
| DC_28A_n78A | 266,8 | +2/-3 | 23 | +2/-3 |
| DC_28A_n79A |  |  | 23 | +2/-3 |
| DC_28A_n83A_ULSUP-TDM_n41A |  |  | 23 | +2/-3 |
| DC_28A_n83A_ULSUP-TDM_n78A |  |  | 23 | +2/-3 |
| DC_30A_n2A |  |  | 23 | +2/-3 |
| DC_30A_n5A |  |  | 23 | +2/-3 |
| DC_30A_n66A |  |  | 23 | +2/-3 |
| DC_30A_n77A | 266,8 | +2/-3 | 23 | +2/-3 |
| DC_38A_n1A |  |  | 23 | +2/-3 |
| DC_38A_n3A |  |  | 23 | +2/-3 |
| DC_38A_n8A |  |  | 23 | +2/-3 |
| DC_38A_n28A |  |  | 23 | +2/-3 |
| DC_38A_n78A |  |  | N/A | N/A |
| DC_38A_n79A |  |  | 23 | +2/-3 |
| DC_39A_n40A |  |  | 23 | +2/-3 |
| DC_39A_n41A DC_39C_n41A | 265,8 | +2/-3 | 23 | +2/-3 |
| DC_39A_n78A |  |  | 23 | +2/-3 |
| DC_39A_n79A | 265,8 | +2/-3 | 23 | +2/-3 |
| DC_40A_n1A |  |  | 23 | +2/-3 |
| DC_40A_n41A DC_40C_n41A |  |  | 23 | +2/-3 |
| DC_40A_n77A |  |  | N/A | N/A |
| DC_40A_n78A |  |  | 23 | +2/-3 |
| DC_40C_n78A |  |  | 23 | +2/-3 |
| DC_40A_n79A |  |  | 23 | +2/-3 |
| DC_41A_n1A DC_41C_n1A |  |  | 23 | +2/-3 |
| DC_41A_n3A DC_41C_n3A |  |  | 23 | +2/-3 |
| DC_41A_n28A DC_41C_n28A |  |  | 23 | +2/-3 |
| DC_41A_n77A DC_41C_n77A |  |  | 23 | +2/-3 |
| DC_41A_n78A DC_41C_n78A |  |  | 23 | +2/-3 |
| DC_41A_n79A DC_41C_n79A | 265,8 | +2/-3 | 23 | +2/-3 |
| DC_42A_n1A DC_42C_n1A |  |  | 23 | +2/-3 |
| DC_42A_n3A DC_42C_n3A |  |  | 23 | +2/-3 |
| DC_42A_n28A DC_42C_n28A |  |  | 23 | +2/-3 |
| DC_42A_n51A |  |  | 23 | +2/-3 |
| DC_42A_n77A |  |  | N/A | N/A |
| DC_42A_n78A |  |  | N/A | N/A |
| DC_42A_n79A |  |  | N/A | N/A |
| DC_48A_n2A |  |  | 23 | +2/-3 |
| DC_48A_n5A |  |  | 23 | +2/-3 |
| DC_48A_n12A |  |  | 23 | +2/-3 |
| DC_48A_n25A |  |  | 23 | +2/-3 |
| DC_2A_n46A |  |  | 23 | +2/-3 |
| DC_48A_n66A |  |  | 23 | +2/-3 |
| DC_48A_n71A |  |  | 23 | +2/-3 |
| DC_66A_n2A |  |  | 23 | +2/-3 |
| DC_66A_n5A |  |  | 23 | +2/-3 |
| DC_66A_n7A |  |  | 23 | +2/-3 |
| DC_66A_n12A |  |  | 23 | +2/-3 |
| DC_66A_n25A |  |  | 23 | +2/-3 |
| DC_66A_n28A |  |  | 23 | +2/-3 |
| DC_66A_n30A |  |  | 23 | +2/-3 |
| DC_66A_n38A |  |  | 23 | +2/-3 |
| DC_66A_n41A | 266,8 | +2/-3 | 23 | +2/-3 |
| DC_66A_n46A |  |  | 23 | +2/-3 |
| DC_66A_n48A |  |  | 23 | +2/-3 |
| DC_66A_n71A |  |  | 23 | +2/-3 |
| DC_66A_n77A | 266,8 | +2/-3 | 23 | +2/-3 |
| DC_66A_n78A DC_66A-66A_n78A |  |  | 23 | +2/-3 |
| DC_66A_n86A_ULSUP-TDM_n78A |  |  | 23 | +2/-3 |
| DC_71A_n2A |  |  | 23 | +2/-3 |
| DC_71A_n5A |  |  | 23 | +2/-3 |
| DC_71A_n7A |  |  | 23 | +2/-3 |
| DC_71A_n12A7 |  |  | 23 | +2/-3 |
| DC_71A_n25A |  |  | 23 | +2/-3 |
| DC_71A_n38A |  |  | 23 | +2/-3 |
| DC_71A_n41A |  |  | 23 | +2/-3 |
| DC_71A_n48A |  |  | 23 | +2/-3 |
| DC_71A_n66A |  |  | 23 | +2/-3 |
| DC_71A_n78A |  |  | 23 | +2/-3 |
| NOTE 1: An uplink DC configuration in which at least one of the bands has NOTE 3 in Table 6.2.1-1 in TS 38.101-1 or NOTE 2 in Table 6.2.2-1 in TS 36.101 is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of at least one of the bands is confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high. NOTE 2: PPowerClass, EN-DC is the maximum UE power specified without taking into account the tolerance NOTE 3: For inter-band EN-DC the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). NOTE 4: Power Class 3 is the default power class unless otherwise stated. NOTE 5: The UE is not required to support PC2 within each individual cell group. Power class support within each individual cell group is signaled separately by the UE. NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signaled separately by the UE. NOTE 7: Only single switched UL is supported. NOTE 8: The UE that supports PC3 within a TDD or FDD band and supports PC2 within a second TDD band may signal a higherPowerLimit-r17 capability whereby the maximum output power indicated in the table may be exceeded in accordance with sub-clause 6.2B.4.1.3. |  |  |  |  |

If a UE supports a different power class than the default UE power class for an E-UTRA TDD and NR TDD Inter-band EN-DC band combination and the supported power class enables higher maximum output power than that of the default power class:

–	if the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than 30% (The exact evaluation period is no less than one radio frame); or

–	if the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is not absent and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 as defined in TS38.331 (The exact evaluation period is no less than one radio frame); or

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is provided and set to the maximum output power of the default power class or lower;

–	shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified sub-clause 6.2B.4;

–	Else if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 as defined in TS 38.331; or

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to 30% when maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent. (The exact evaluation period is no less than one radio frame):

–	shall apply all requirements for the supported power class and set the configured transmitted power class as specified in sub-clause 6.2B.4.

If a UE supports a different power class than the default UE power class for an E-UTRA FDD and NR TDD EN-DC band combination and the supported power class enables higher maximum output power than that of the default power class:

If UE indicating the two capabilities maxUplinkDutyCycle-FDD-TDD-EN-DC1 and maxUplinkDutyCycle-FDD-TDD-EN-DC2:

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of EUTRA uplink symbols transmitted in a certain evaluation period is between 40% and 70%, and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal tomaxUplinkDutyCycle-FDD-TDD-EN-DC1as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); or

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of EUTRA uplink symbols transmitted in a certain evaluation period is no larger than 40%, and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to maxUplinkDutyCycle-FDD-TDD-EN-DC2 as defined in TS 38.331 (The exact evaluation period is no less than one radio frame)

–	shall apply all requirements for the supported power class and set the configured transmitted power class as specified in sub-clause 6.2B.4.

–	else

–	shall apply all requirements for the default power class and set the configured transmitted power as specified sub-clause 6.2B.4;

else

–	shall apply all requirements for the supported power class and set the configured transmitted power as specified sub-clause 6.2B.4;

6.2B.1.3a	Inter-band NE-DC within FR1

For inter-band NE-DC of E-UTRA and NR in FR1, the following UE power classes define the maximum output power for any transmission bandwidth within the aggregated channel bandwidth. The maximum output power is measured as the sum of the maximum output power at each UE antenna connector. The period of measurement shall be at least one sub frame (1 ms). UE maximum output power shall be measured over all component carriers from different bands. If each band has separate antenna connectors, maximum output power is measured as the sum of maximum output power at each UE antenna connector.

Table 6.2B.1.3a-1: Maximum output power for inter-band NE-DC (two bands)

| NE-DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_n1A_28A | 23 | +2/-3 |
| DC_n3A_1A | 23 | +2/-3 |
| DC_n3A_8A | 23 | +2/-3 |
| DC_n8A_1A | 23 | +2/-3 |
| DC_n8A_3A | 23 | +2/-3 |
| DC_n28A_3A DC_n28A_3C | 23 | +2/-3 |
| DC_n28A_8A | 23 | +2/-3 |
| DC_n28A_20A | 23 | +2/-3 |
| DC_n28A_34A | 23 | +2/-3 |
| DC_n28A_39A | 23 | +2/-3 |
| DC_n28A_40A DC_n28A_40C | 23 | +2/-3 |
| DC_n41A_3A | 23 | +2/-3 |
| DC_n41A_8A | 23 | +2/-3 |
| DC_n41A_34A | 23 | +2/-3 |
| DC_n41A_39A | 23 | +2/-3 |
| DC_n41A_40A | 23 | +2/-3 |
| DC_n77A_1A | 23 | +2/-3 |
| DC_n77A_3A | 23 | +2/-3 |
| DC_n77A_8A | 23 | +2/-3 |
| DC_n78A_1A | 23 | +2/-3 |
| DC_n78A_3A | 23 | +2/-3 |
| DC_n78A_5A | 23 | +2/-3 |
| DC_n78A_7A | 23 | +2/-3 |
| DC_n78A_8A | 23 | +2/-3 |
| DC_n78A_26A | 23 | +2/-3 |

<< Next Changes >>

6.2B.2.1a	Intra-band contiguous NE-DC

Unless otherwise stated, for intra-band contiguous NE-DC, UE maximum output power reduction specified in TS 36.101 [4] and TS 38.101-1 [2] apply for E-UTRA and NR respectively.

<< Next Changes >>

6.2B.4.1.1a	Intra-band contiguous NE-DC

The following requirements apply for one component carrier per CG configured for synchronous DC.

For intra-band dual connectivity with one uplink serving cell per CG on E-UTRA and NR respectively, the UE is allowed to set its configured maximum output power PCMAX,c(i),i for serving cell c(i) of CG i, i = 1,2, and its total configured maximum transmission power for NE-DC operation PTotalNE-DC= 10log10(PtotalNE-DC) with PtotalNE-DC as specified in clause 7.6.1A of TS 38.213 [10].

The configured maximum output power PCMAX_ E-UTRA,c (p) in sub-frame p for the configured E-UTRA uplink carrier shall be set within the bounds:

PCMAX_L_ E-UTRA,c (p) ≤ PCMAX_ E-UTRA,c (p) ≤  PCMAX H _ E-UTRA,c (p)

where PCMAX_L_ E-UTRA,c and PCMAX H _ E-UTRA,c are the limits for a serving cell c as specified in TS 36.101 [4] clause 6.2.5 modified by PLTE as follows:

PCMAX_L_ E-UTRA,c = MIN { MIN(PEMAX, NE-DC , PEMAX,c , PLTE) – tC_E-UTRA, c, (PPowerClass, NE-DC – ΔPPowerClass, NE-DC), (PPowerClass,E-UTRA – ΔPPowerClass,E-UTRA) – MAX(MPRc + A-MPRc + ΔTIB,c  + TC_ E-UTRA, c + TProSe, P-MPRc)}

PCMAX H _ E-UTRA,c = MIN {PEMAX,c,  PEMAX, NE-DC , (PPowerClass, NE-DC – ΔPPowerClass,NE-DC), PLTE, (PPowerClass,E-UTRA – ΔPPowerClass,E-UTRA)}

with exception that

-	if no symbol of slot  of the NR that is indicated as uplink or flexible by TDD-UL-DL-ConfigurationCommon or TDD-UL-DL-ConfigDedicated overlaps with subframe  of the E-UTRA; or

-	if NR slot(s) that is indicated as downlink by TDD-UL-DL-ConfigurationCommon or TDD-UL-DL-ConfigDedicated does not overlap with subframe  of the E-UTRA; then

PCMAX_L_ E-UTRA,c = MIN { MIN(PEMAX, NE-DC , PEMAX,c) – tC_E-UTRA, c, (PPowerClass, NE-DC – ΔPPowerClass, NE-DC), (PPowerClass,E-UTRA – ΔPPowerClass,E-UTRA) – MAX(MPRc + A-MPRc + ΔTIB,c  + TC_ E-UTRA, c + TProSe, P-MPRc)}

PCMAX H _ E-UTRA,c = MIN {PEMAX,c,  PEMAX, NE-DC , (PPowerClass, NE-DC – ΔPPowerClass,NE-DC), (PPowerClass,E-UTRA – ΔPPowerClass,E-UTRA)}

The configured maximum output power PCMAX,f,c,NR (q) in physical-channel q for the configured NR carrier shall be set within the bounds:

PCMAX_L,f,c,NR (q) ≤  PCMAX,f,c,NR (q) ≤  PCMAX_H,f,c,NR (q)

where PCMAX_L,f,c,NR and PCMAX_H,f,c,NR are the limits for a serving cell c as specified in clause 6.2.4 of TS 38.101-1 [2] modified by PNR as follows:

PCMAX_L,f,c,NR = MIN { MIN(PEMAX, NE-DC , PEMAX,c , PNR) – tC_NR, c, (PPowerClass, NE-DC – ΔPPowerClass,NE-DC ), (PPowerClass,NR – ΔPPowerClass,NR) – MAX(MPRc + A-MPRc+ ΔTIB,c + TC_NR, c + ∆TRxSRS,  P-MPRc) }

PCMAX_H,f,c,NR = MIN {PEMAX,c, PEMAX, NE-DC, PNR, (PPowerClass, NE-DC – ΔPPowerClass,NE-DC ), PPowerClass,NR – ΔPPowerClass,NR}

-	PEMAX,NE-DC signalled by RRC as p-UE-FR1 in TS 38.331 [9];

-	PLTE signalled by RRC as p-MaxEUTRA in TS 36.331 [8];

-	PNR signalled by RRC as p-NR-FR1 defined in TS 38.331 [9];

-	ΔTc_E-UTRA,c = 1.5dB when NOTE 2 in Table 6.2.2-1 in TS 36.101 [4] applies for a serving cell c, otherwise TC_ E-UTRA,c = 0dB;

-	TC_NR,c = 1.5dB when NOTE 3 in Table 6.2.1-1 in TS 38.101-1 [2] applies for a serving cell c, otherwise TC_NR,c = 0dB;

-	ΔTIB,c is specified in clause 6.2B.4.2;

-	PPowerClass, NE-DC is defined in clause 6.2B.1.1a for intra-band contiguous NE-DC;

-	ΔPPowerClass,NE-DC = 3 dB for a power class 2 capable NE-DC UE when requirements of default power class had been applied as specified in sub-clause 6.2B.1; otherwise ΔPPowerClass,NE-DC = 0 dB;

-	PPowerClass,NR is the nominal UE power of the power class that the UE supports for the NR band of the NE-DC combination as defined in clause 6.2.1 of 38.101-1 [2]; in case IE [powerClassNRPart] as defined in TS 38.331 [9] is indicated, PPowerClass,NR should use that value instead.

-	ΔPPowerClass,NR is 3 dB or 0 dB according to clause 6.2.4 of TS 38.101-1 [2] for a UE that supports power class 2 in the NR band of the EN-DC combination as defined in clause 6.2.1 of TS 38.101-1 [2];

-	PPowerClass,E-UTRA is the nominal UE power of the power class that the UE supports for the E-UTRA band of the NE-DC combination as defined in clause 6.2.2 of 36.101 [4];

-	ΔPPowerClass,E-UTRA is 3 dB or 0 dB according to clause 6.2.5 of TS 36.101 [4] for a UE that supports power class 2 in the E-UTRA band of the EN-DC combination as defined in clause 6.2.2 of TS 36.101 [4];

If the transmissions from NR and E-UTRA do not overlap, then the complete clauses for configured transmitted power for E-UTRA and NR respectively from their own specifications apply with the modifications specified above. The lower value between PPowerClass, NE-DC or PEMAX, NE-DC shall not be exceeded at any time by UE.

PTotalNE-DC = 10log10(PtotalNE-DC) with PTotalNE-DC the configured maximum transmission power for NE-DC operation as specified in clause 7.6 of TS 38.213 [10].

The total configured maximum transmission power is

PTotalNE-DC= MIN { PEMAX, NE-DC ,PPowerClass, NE-DC – ΔPPowerClass, NE-DC }

If the UE does not support dynamic power sharing,

PTotalNE-DC = MIN { PEMAX, NE-DC , PPowerClass, NE-DC - ΔPPowerClass,EN-DC } + 0.3 dB

If the NE-DC UE is not supporting dynamic power sharing, then the complete clauses for configured transmitted power for E-UTRA and NR respectively from their own specifications TS 36.101 [4] and TS 38.101-1 [2] respectively apply with the modifications specified above and PTotalNE-DC applies.

<< Next Changes >>

6.2B.4.2.3	Inter-band EN-DC within FR1

6.2B.4.2.3.1	ΔTIB,c for EN-DC two bands

Table 6.2B.4.2.3.1-1: ΔTIB,c due to EN-DC(two bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)7 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |
| DC_1_n3 | 0.3 | 0.3 |
| DC_1_n5 | 0.3 | 0.3 |
| DC_1_n7 DC_1-1_n7 | 0.5 | 0.6 |
| DC_1_n8 | 0.3 | 0.3 |
| DC_1_n20 | 0.3 | 0.3 |
| DC_1_n26 | 0.3 | 0.3 |
| DC_1_n28 | 0.3 | 0.6 |
| DC_1_n38 | 0.5 | 0.5 |
| DC_1_n40 | 0.5 | 0.5 |
| DC_1_n41 | 0.5 | 0.5 |
| DC_1_n50 | 0.5 | 0.5 |
| DC_1_n51 | 0.6 | 0.6 |
| DC_1_n71 | 0.3 | 0.3 |
| DC_1_n77 | 0.6 | 0.8 |
| DC_1_n78 | 0.3 | 0.8 |
| DC_1_n105 | 0.3 | 0.6 |
| DC_2_n5 DC_2-2_n5 | 0.3 | 0.3 |
| DC_2_n7 DC_2-2_n7 | 0.5 | 0.5 |
| DC_2_n12 | 0.3 | 0.3 |
| DC_2_n28 | 0.3 | 0.3 |
| DC_2_n30 DC_2-2_n30 | 0.5 | 0.3 |
| DC_2_n38 DC_2-2_n38 | 0.5 | 0.9 |
| DC_2_n41 DC_2-2_n41 | 0.5 | 0.41 / 0.92 |
| DC_2_n48 | 0.6 | 0.8 |
| DC_2_n66 DC_2-2_n66 | 0.5 | 0.5 |
| DC_2_n71 DC_2-2_n71 | 0.3 | 0.3 |
| DC_2_n77 DC_2-2_n77 | 0.6 | 0.8 |
| DC_2_n78 DC_2-2_n78 | 0.6 | 0.8 |
| DC_3_n1 | 0.3 | 0.3 |
| DC_3_n5 | 0.3 | 0.3 |
| DC_3_n8 DC_3-3_n8 | 0.3 | 0.3 |
| DC_3_n7 DC_3-3_n7 | 0.5 | 0.5 |
| DC_3_n20 | 0.3 | 0.3 |
| DC_3_n26 | 0.3 | 0.3 |
| DC_3_n28 | 0.3 | 0.3 |
| DC_3_n34 | 0.5 | 0.5 |
| DC_3_n38 | 0.5 | 0.5 |
| DC_3_n40 | 0.5 | 0.5 |
| DC_3-n41 | 0.5 | 0.33 / 0.84 |
| DC_3_n50 | 0.5 | 0.5 |
| DC_3_n51 | 0.3 | 0.3 |
| DC_3_n71 | 0.3 | 0.3 |
| DC_3_n77 DC_3-3_n77 | 0.6 | 0.8 |
| DC_3_n78 DC_3-3_n78 | 0.6 | 0.8 |
| DC_3_n105 | 0.3 | 0.6 |
| DC_4_n2 | 0.5 | 0.5 |
| DC_4_n5 | 0.3 | 0.3 |
| DC_4_n7 | 0.5 | 0.5 |
| DC_4_n28 | 0.3 | 0.6 |
| DC_4_n38 | 0.5 | 0.8 |
| DC_4_n41 | 0.5 | 0.81 / 1.32 |
| DC_4_n78 | 0.6 | 0.8 |
| DC_5_n1 | 0.3 | 0.3 |
| DC_5_n2 DC_5-5_n2 | 0.3 | 0.3 |
| DC_5_n3 | 0.3 | 0.3 |
| DC_5_n7 | 0.3 | 0.3 |
| DC_5_n12 | 0.8 | 0.4 |
| DC_5_n25 | 0.3 | 0.3 |
| DC_5_n30 | 0.3 | 0.3 |
| DC_5_n38 | 0.3 | 0.3 |
| DC_5_n40 | 0.3 | 0.3 |
| DC_5_n41 | 0.6 | 0.3 |
| DC_5_n48 | 0.3 | 0.3 |
| DC_5_n66 DC_5-5_n66 | 0.3 | 0.3 |
| DC_5_n71 | 0.5 | 0.5 |
| DC_5_n77 | 0.6 | 0.8 |
| DC_5_n78 | 0.6 | 0.8 |
| DC_7_n1 DC_7-7_n1 | 0.6 | 0.5 |
| DC_7_n2 | 0.5 | 0.5 |
| DC_7_n3 | 0.5 | 0.5 |
| DC_7_n5 DC_7-7_n5 | 0.3 | 0.3 |
| DC_7_n8 DC_7-7_n8 | 0.3 | 0.6 |
| DC_7_n12 | 0.3 | 0.3 |
| DC_7_n20 | 0.3 | 0.3 |
| DC_7_n25 | 0.5 | 0.5 |
| DC_7_n26 | 0.3 | 0.3 |
| DC_7_n28, DC_7-7_n28 | 0.3 | 0.3 |
| DC_7_n40 DC_7-7_n40 | 0.5 | 0.6 |
| DC_7_n51 | 0.3 | 0.3 |
| DC_7_n71 | 0.3 | 0.6 |
| DC_7_n66 DC_7-7_n66 | 0.5 | 0.5 |
| DC_7_n77 DC_7-7_n77 | 0.5 | 0.8 |
| DC_7_n78 DC_7-7_n78 | 0.5 | 0.8 |
| DC_7_n79 | 0.5 | 0.8 |
| DC_7_n105 | 0.3 | 0.6 |
| DC_8_n1 | 0.3 | 0.3 |
| DC_8_n2 | 0.3 | 0.3 |
| DC_8_n3 | 0.3 | 0.3 |
| DC_8_n7 | 0.6 | 0.3 |
| DC_8_n20 | 0.4 | 0.4 |
| DC_8_n28 | 0.6 | 0.5 |
| DC_8_n34 | 0.3 | 0.3 |
| DC_8_n38 | 0.6 | 0.3 |
| DC_8_n39 | 0.3 | 0.3 |
| DC_8_n40 | 0.3 | 0.3 |
| DC_8_n41 | 0.3 | 0.3 |
| DC_8_n77 | 0.6 | 0.8 |
| DC_8_n78 | 0.6 | 0.8 |
| DC_11_n1 | 0.3 | 0.3 |
| DC_11_n3 | 0.8 | 0.9 |
| DC_11_n28 | 0.4 | 0.6 |
| DC_11_n41 | 0.3 | 0.3 |
| DC_11_n77 | 0.4 | 0.8 |
| DC_11_n78 | 0.4 | 0.8 |
| DC_12_n2 | 0.3 | 0.3 |
| DC_12_n5 | 0.4 | 0.8 |
| DC_12_n7 | 0.3 | 0.3 |
| DC_12_n25 | 0.3 | 0.3 |
| DC_12_n30 | 0.3 | 0.3 |
| DC_12_n38 | 0.3 | 0.3 |
| DC_12_n41 | 0.3 | 0.3 |
| DC_12_n66 | 0.8 | 0.3 |
| DC_12_n71 | 1 | 1 |
| DC_12_n77 | 0.5 | 0.8 |
| DC_12_n78 | 0.5 | 0.8 |
| DC_13_n2 | 0.3 | 0.3 |
| DC_13_n5 | 0.5 | 0.5 |
| DC_13_n7 | 0.5 | 0.5 |
| DC_13_n25 | 0.3 | 0.3 |
| DC_13_n48 | 0.3 | 0.3 |
| DC_13_n66 | 0.3 | 0.3 |
| DC_13_n71 | 0.5 | 0.5 |
| DC_13_n77 | 0.5 | 0.8 |
| DC_13_n78 | 0.5 | 0.8 |
| DC_14_n2 | 0.3 | 0.3 |
| DC_14_n5 | 0.5 | 0.5 |
| DC_14_n30 | 0.3 | 0.3 |
| DC_14_n66 | 0.3 | 0.3 |
| DC_14_n77 | 0.5 | 0.8 |
| DC_18_n3 | 0.3 | 0.3 |
| DC_18_n28 | 0.5 | 0.5 |
| DC_18_n41 | 0.3 | 0.33 |
| DC_18_n77 | 0.3 | 0.8 |
| DC_18_n78 | 0.3 | 0.8 |
| DC_19_n1 | 0.3 | 0.3 |
| DC_19_n77 | 0.3 | 0.8 |
| DC_19_n78 | 0.3 | 0.8 |
| DC_20_n1 | 0.3 | 0.3 |
| DC_20_n3 | 0.3 | 0.3 |
| DC_20_n7 | 0.3 | 0.3 |
| DC_20_n8 | 0.4 | 0.4 |
| DC_20_n28 | 0.5 | 0.5 |
| DC_20_n38 | 0.5 | 0.3 |
| DC_20_n41 | 0.3 | 0.3 |
| DC_20_n50 | 0.3 | 0.4 |
| DC_20_n51 | 0.5 | 0.5 |
| DC_20_n77 | 0.6 | 0.8 |
| DC_20_n78 | 0.6 | 0.8 |
| DC_21_n1 | 0.3 | 0.3 |
| DC_21_n28 | 0.4 | 0.3 |
| DC_21_n77 | 0.4 | 0.8 |
| DC_21_n78 | 0.4 | 0.8 |
| DC_25_n41, DC_25-25_n41 | 0.5 | 0.41 / 0.92 |
| DC_25_n77 DC_25-25_n77 | 0.6 | 0.8 |
| DC_25_n78 DC_25-25_n78 | 0.6 | 0.8 |
| DC_26_n25 | 0.3 | 0.3 |
| DC_26_n41 | 0.3 | 0.3 |
| DC_26_n77 | 0.3 | 0.8 |
| DC_26_n78 | 0.3 | 0.8 |
| DC_28_n1 | 0.6 | 0.3 |
| DC_28_n2 | 0.3 | 0.3 |
| DC_28_n3 | 0.3 | 0.3 |
| DC_28_n5 | 0.5 | 0.5 |
| DC_28_n7 | 0.3 | 0.3 |
| DC_28_n8 | 0.5 | 0.6 |
| DC_28_n20 | 0.5 | 0.5 |
| DC_28_n38 | 0.3 | 0.3 |
| DC_28_n40 | 0.3 | 0.3 |
| DC_28_n41 | 0.3 | 0.3 |
| DC_28_n50 | 0.3 | 0.4 |
| DC_28_n51 | 0.5 | 0.5 |
| DC_28_n66 | 0.6 | 0.3 |
| DC_28_n77 | 0.5 | 0.8 |
| DC_28_n78 | 0.5 | 0.8 |
| DC_30_n2 | 0.3 | 0.5 |
| DC_30_n5 | 0.3 | 0.3 |
| DC_30_n66 | 0.5 | 0.8 |
| DC_30_n77 | 0.5 | 0.8 |
| DC_38_n1 | 0.5 | 0.5 |
| DC_38_n3 | 0.5 | 0.5 |
| DC_38_n8 | 0.6 | 0.3 |
| DC_38_n28 | 0.3 | 0.3 |
| DC_38_n78 | - | 0.5 |
| DC_38_n79 | 0.3 | 0.8 |
| DC_39-n41 | 0.5 | 0.5 |
| DC_39_n78 | 0.3 | 0.8 |
| DC_39_n79 | 0.3 | 0.8 |
| DC_40_n1 | 0.5 | 0.5 |
| DC_40_n415 | 0.5 | 0.5 |
| DC_40_n77 | - | 0.5 |
| DC_40_n78 | - | 0.56 |
| DC_40_n79 | 0.3 | 0.8 |
| DC_41_n1 | 0.5 | 0.5 |
| DC_41_n3 | 0.33 / 0.84 | 0.5 |
| DC_41_n28 | 0.3 | 0.3 |
| DC_41_n77 | 0.3 | 0.8 |
| DC_41_n78 | 0.3 | 0.8 |
| DC_41_n79 | 0.3 | 0.8 |
| DC_42_n1 | 0.8 | 0.3 |
| DC_42_n3 | 0.8 | 0.6 |
| DC_42_n28 | 0.5 | 0.8 |
| DC_42_n51 | 0.6 | 0.8 |
| DC_48_n2 | 0.8 | 0.6 |
| DC_48_n5 | 0.3 | 0.3 |
| DC_48_n12 | 0.3 | 0.3 |
| DC_48_n25 | 0.8 | 0.6 |
| DC_48_n46 | 0.8 | - |
| DC_48_n66 | 0.8 | 0.6 |
| DC_48_n71 DC_48-48_n71 DC_48-48-48_n71 | 0.3 | 0.3 |
| DC_66_n2 DC_66-66_n2 DC_66-66-66_n2 | 0.5 | 0.5 |
| DC_66_n5, DC_66-66_n5, DC_66-66-66_n5 | 0.3 | 0.3 |
| DC_66_n7 DC_66-66_n7 | 0.5 | 0.5 |
| DC_66_n12 | 0.8 | 0.3 |
| DC_66_n25 | 0.5 | 0.5 |
| DC_66_n28 | 0.3 | 0.6 |
| DC_66_n30 DC_66-66_n30 | 0.5 | 0.8 |
| DC_66_n38 DC_66-66_n38 | 0.5 | 0.5 |
| DC_66_n41 | 0.5 | 0.81 / 1.32 |
| DC_66_n48 DC_66-66_n48 | 0.6 | 0.8 |
| DC_66_n71 DC_66-66_n71 | 0.3 | 0.3 |
| DC_66_n77 DC_66-66_n77 DC_66-66-66_n77 | 0.6 | 0.8 |
| DC_66_n78 DC_66-66_n78 | 0.6 | 0.8 |
| DC_71_n2 | 0.3 | 0.3 |
| DC_71_n5 | 0.5 | 0.5 |
| DC_71_n7 | 0.6 | 0.3 |
| DC_71_n12 | 1 | 1 |
| DC_71_n25 | 0.3 | 0.3 |
| DC_71_n38 | 0.6 | 0.3 |
| DC_71_n41 | 0.6 | 0.3 |
| DC_71_n48 | 0.3 | 0.3 |
| DC_71_n66 | 0.3 | 0.3 |
| DC_71_n77 | 0.5 | 0.8 |
| DC_71_n78 | 0.5 | 0.8 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545 MHz. NOTE 3: Applicable for the frequency range of 2515 – 2690 MHz. NOTE 4: Applicable for the frequency range of 2496 - 2515 MHz. NOTE 5: Applicable for UE supporting inter-band EN-DC without simultaneous Rx/Tx. NOTE 6: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. NOTE 7: “-” denotes ΔTIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively. |  |  |

<< Next Changes >>

6.3B.2	Output power dynamics for intra-band EN-DC without dual PA capability

For intra-band contiguous and intra-band non-contiguous EN-DC configurations without dual PA capability, maximum UL switching time is defined as 120 us and DL reception interruption is allowed during UL switching. Time masks in Figure 6.3B.2-1 and Figure 6.3B.2-2 shall apply.

Figure 6.3B.2-1: E-UTRA to NR switching time mask for intra-band EN-DC without dual PA capabilitywhen single UL is allowed

Figure 6.3B.2-2: NR to E-UTRA switching time mask for intra-band EN-DC without dual PA capabilitywhen single UL is allowed

6.3B.2a	Output power dynamics for intra-band NE-DC without dual PA capability

Unless otherwise specified, same requirements in clause 6.3B.2 are applicable for intra-band NE-DC configurations without dual PA capability.

6.3B.3	Output power dynamics for intra-band EN-DC with dual PA capability

For both intra-band contiguous and non-contiguous EN-DC with dual PA capability, time masks in Figure 6.3B.3-1 and Figure 6.3B.3-2 shall apply.

Figure 6.3B.3-1: E-UTRA to NR switching time mask for intra-band EN-DC with dual PA capability

Figure 6.3B.3-2: NR to E-UTRA switching time mask for intra-band EN-DC with dual PA capability

6.3B.3a	Output power dynamics for intra-band NE-DC with dual PA capability

Unless otherwise specified, same requirements in clause 6.3B.3 are applicable for intra-band NE-DC configurations with dual PA capability.

<< Next Changes >>

6.4B.1.1	Frequency error for Intra-band contiguous EN-DC

For intra-band contiguous EN-DC, the requirement shall apply on each component carrier as defined in clause 6.5.1 in TS 36.101 [4] and in clause 6.4.1 in TS 38.101-1 [2], respectively.

6.4B.1.1a	Frequency error for Intra-band contiguous NE-DC

For intra-band contiguous NE-DC, the requirement shall apply on each component carrier as defined in clause 6.5.1 in TS 36.101 [4] and in clause 6.4.1 in TS 38.101-1 [2], respectively.

<< Next Changes >>

7.3B.2.3	Inter-band EN-DC within FR1

7.3B.2.3.0	General

Reference sensitivity exceptions are specified for the condition when there is uplink transmission only in the aggressor band.

7.3B.2.3.1	Reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

Sensitivity degradation is allowed for a band if it is impacted by UL harmonic interference from another band part of the same EN-DC configuration. Reference sensitivity exceptions for the victim band (high) are specified in Table 7.3B.2.3.1-1 with uplink configuration of the aggressor band (low) specified in Table 7.3B.2.3.1-2.

Table 7.3B.2.3.1-1: Reference sensitivity exceptions (MSD) due to UL harmonic for EN-DC in NR FR1

|  | E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 1, 3 | n772,13 |  | 23.9 | 22.1 | 20.9 | 19.8 | 19.0 |  | 17.9 | 16.8 | 16.0 | 15.4 | 14.8 | 14.3 | 13.8 |
|  | n773 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |  |
| 2 | n482,13 | 27.3 | 24.4 | 22.4 | 21.2 |  | 19.0 |  | 18 | 17.1 | 16.3 | 15.4 | 15 | 14.5 | 14 |
|  | n483 | 1.9 | 1.4 | 0.9 | 0.4 |  | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| n2 | 482, 13 | 27.3 | 24.4 | 22.4 | 21.2 |  | 19.0 |  | 18.0 | 17.1 | 16.3 | 15.4 | 15.0 | 14.5 | 14 |
|  | 483 | 1.9 | 1.4 | 0.9 | 0.4 |  | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | n772, 13 |  | 23.9 | 22.1 | 20.9 | 19.8 | 19.0 |  | 17.9 | 16.8 | 16.0 | 15.5 | 14.8 | 14.3 | 13.8 |
|  | n773 |  | 1.1 | 0.8 | 0.3 | 0.1 | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | n782,13 |  | 23.9 | 22.1 | 20.9 | 19.8 | 19.0 |  | 17.9 | 16.8 | 16.0 | 15.4 | 14.8 | 14.3 | 13.8 |
|  | n783 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |  |
| 3 | n782,13 |  | 23.9 | 22.1 | 20.9 | 19.8 | 19.0 |  | 17.9 | 16.8 | 16.0 | 15.4 | 14.8 | 14.3 | 13.8 |
|  | n783 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |  |
| n3 | 422, 13 | 27.3 | 24.3 | 22.5 | 21.3 |  |  |  |  |  |  |  |  |  |  |
|  | 423 | 1.9 | 1.3 | 1.0 | 0.5 |  |  |  |  |  |  |  |  |  |  |
| 4 | n782,13 |  | 23.9 | 22.1 | 20.9 | 19.8 | 19.0 |  | 17.9 | 16.8 | 16.0 | 15.4 | 14.8 | 14.3 | 13.8 |
|  | n783 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |  |
| 5 | n418,9 | N/A | 10.3 | 8.4 | 7.4 | 6.5 | 5.5 |  | 5 | 4.6 | 4.3 | 3.9 | 3.1 |  | 2.7 |
| 5 | n776, 7, 17 |  | 10.5 | 8.9 | 7.8 | 7.2 | 6.5 |  | 5.1 | 4.2 | 3.5 | 2.8 | 2.3 | 2.1 | 1.4 |
|  | n774, 5, 17 |  | 10.4 | 8.9 | 7.8 | 6.7 | 6 |  | 4.7 | 3.7 | 3 | 2.3 | 1.7 | 1.2 | 0.7 |
| 5 | n786,7 |  | 10.5 | 8.9 | 7.8 | 7.1 | 6.5 |  | 5.4 | 4.2 | 3.5 | 2.9 | 2.3 | 2.1 | 1.4 |
| 7 | n793 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |  |
| 8 | n314 | N/A | N/A | N/A | N/A |  |  |  |  |  |  |  |  |  |  |
| 8 | n78,9 | 10 | 7.6 | 6.2 | 5.3 | 4.3 | 3.2 |  | 2.3 | 1.3 |  |  |  |  |  |
| 8 | n418,9 | N/A | 13 | 11.3 | 10.1 |  | 8.3 |  | 7.0 | 6.1 | 5.5 | 4.9 | 4.3 | 3.9 | 3.5 |
| 8 | n776,7 n786,7 |  | 10.8 | 9.1 | 8 | 7.2 | 6.5 |  | 5.1 | 4.2 | 3.5 | 2.9 | 2.3 | 2.1 | 1.4 |
| 8 | n794,5 |  |  |  |  |  |  |  | 6.8 | 6.2 | 5.6 |  | 4.9 |  | 4.4 |
| n8 | 314 | N/A | N/A | N/A | N/A |  |  |  |  |  |  |  |  |  |  |
| n8 | 78,9,10 | 10 | 7.6 | 6.2 | 5.3 |  |  |  |  |  |  |  |  |  |  |
| 12 | n668,9,10 | 10 | 7.5 | 6.2 | 5.5 | 4.5 | 3.7 |  | 2.4 |  |  |  |  |  |  |
| 12 | n774,5 |  | 10.4 | 8.9 | 7.8 | 6.7 | 5.7 |  | 4.7 | 3.7 | 3 | 2.3 | 1.7 | 1.2 | 0.7 |
| 12 | n784,5 |  | 10.4 | 8.9 | 7.8 | 7.1 | 6.5 |  | 4.7 | 3.7 | 3 | 2.3 | 1.7 | 1.2 | 0.7 |
| n12 | 484,5 | 13 | 10.4 | 8.9 | 7.8 |  |  |  |  |  |  |  |  |  |  |
| n12 | 668,9,10 | 10 | 7.5 | 6.2 | 5.5 |  |  |  |  |  |  |  |  |  |  |
| 13 | n774, 5 |  | 10.4 | 8.9 | 7.8 | 6.7 | 5.7 |  | 4.7 | 3.7 | 3 | 2.3 | 1.7 | 1.2 | 0.7 |
| 14 | n774, 5 |  | 10.4 | 8.9 | 7.8 | 6.7 | 5.7 |  | 4.7 | 3.7 | 3 | 2.3 | 1.7 | 1.2 | 0.7 |
| 18，19 | n774,5 n784,5 |  | 10.4 | 8.9 | 7.8 | 7.1 | 6.5 |  | 4.7 | 3.7 | 3 | 2.3 | 1.7 | 1.2 | 0.7 |
| n28 | 32 | 28.1 | 25.3 | 24.0 | 22.8 |  |  |  |  |  |  |  |  |  |  |
| 28 | n502,13 | 27.8 | 24.6 | 22.8 | 21.6 |  | 19.5 |  | 18.5 | 17.5 | 16.7 |  | 15.4 |  |  |
|  | n503 | 1.9 | 1.4 | 0.9 | 0.4 |  |  |  |  |  |  |  |  |  |  |
| 28 | n512,13 | 27.8 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n513 | 1.9 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 28 | n668,9,10 | 10.2 | 7.6 | 6.2 | 5.3 | 4.0 | 3.2 |  | 2 |  |  |  |  |  |  |
| 28 | n774,5 n784,5 |  | 10.4 | 8.9 | 7.8 | 7.1 | 6.5 |  | 4.7 | 3.7 | 3 | 2.3 | 1.7 | 1.2 | 0.7 |
| 20 | n388,9 | 12.9 | 10.3 | 8.4 | 7.4 | 6.7 | 6.1 |  | 5 |  |  |  |  |  |  |
| 20 | n41 | 12.9 | 10.3 | 8.4 | 7.4 |  | 6.1 |  | 5 | 4.3 | 3.9 | 3.5 | 3.1 | 2.7 | 2.1 |
| 20 | n776,7 n786,7 |  | 10.8 | 9.1 | 8 | 7.3 | 6.8 |  | 6 | 4.0 | 3.2 | 2.5 | 2.0 | 1.5 | 1.0 |
| n28 | 2116 | N/A | N/A | N/A |  |  |  |  |  |  |  |  |  |  |  |
| 25 | n772, 13 |  | 23.9 | 22.1 | 20.9 | 19.8 | 19.0 |  | 17.9 | 16.8 | 16.0 | 15.5 | 14.8 | 14.3 | 13.8 |
|  | n773 |  | 1.1 | 0.8 | 0.3 | 0.1 | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 25 | n782, 13 |  | 23.9 | 22.1 | 20.9 | 19.8 | 19.0 |  | 17.9 | 16.8 | 16.0 | 15.5 | 14.8 | 14.3 | 13.8 |
|  | n783 |  | 1.1 | 0.8 | 0.3 | 0.1 | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| n25 | 482,13 | 27.3 | 24.4 | 22.4 | 21.2 |  |  |  |  |  |  |  |  |  |  |
|  | 483 | 1.9 | 1.4 | 0.9 | 0.4 |  |  |  |  |  |  |  |  |  |  |
| 26 | n418,9 |  | 10.3 | 8.4 | 7.4 |  | 6.1 |  | 5 | 4.3 | 3.9 | 3.5 | 3.1 | 2.9 | 2.7 |
| 26 | n776,7 n786,7 |  | 10.8 | 9.1 | 8 | 7.3 | 6.8 |  | 6 | 4.0 | 3.2 | 2.5 | 2.0 | 1.5 | 1.0 |
| 28 | n18,9,10 | 10.2 | 7.6 | 6.2 | 5.3 | 4.0 | 3.2 |  | 2 | 1.0 |  |  |  |  |  |
| n28 | 18,9,10 | 10.2 | 7.6 | 6.2 | 5.3 |  |  |  |  |  |  |  |  |  |  |
| n28 | 48,9,10 | 10.2 | 7.6 | 6.2 | 5.3 |  |  |  |  |  |  |  |  |  |  |
| 28 | n75 | 28.1 | 25.3 | 24.0 | 22.8 | 21.6 | 20.4 |  | 19.2 | 18.0 |  |  |  |  |  |
| n28 | 112,10,13 | 24.8 | 21.8 |  |  |  |  |  |  |  |  |  |  |  |  |
| n28 | 424,5,10 | 14.1 | 10.4 | 8.9 | 7.9 |  |  |  |  |  |  |  |  |  |  |
| n28 | 668,9,10 | 10.2 | 7.6 | 6.2 | 5.3 |  |  |  |  |  |  |  |  |  |  |
| 71 | n211 | 4.6 | 1.0 | 0.7 | 0.6 |  |  |  |  |  |  |  |  |  |  |
|  | n212 | 1.7 | 1.0 | 0.7 | 0.6 |  |  |  |  |  |  |  |  |  |  |
| n71 | 211 | 4.6 | 1.0 | 0.7 | 0.6 |  |  |  |  |  |  |  |  |  |  |
|  | 212 | 1.7 | 1.0 | 0.7 | 0.6 |  |  |  |  |  |  |  |  |  |  |
| 71 | n76,7 | 14.6 | 11.7 | 10.1 | 9 | 8 | 7.1 | 6.3 | 5.5 | 4 |  |  |  |  |  |
| n71 | 76,7 | 14.6 | 11.7 | 10.1 | 9 |  |  |  |  |  |  |  |  |  |  |
| n71 | n2511 | 4.6 | 1.0 | 0.7 | 0.6 | 0.5 | 0.3 |  | 0.2 |  |  |  |  |  |  |
|  | n2512 | 1.7 | 1.0 | 0.7 | 0.6 | 0.5 | 0.3 |  | 0.2 |  |  |  |  |  |  |
| 71 | n416,7 |  | 10.8 | 9.1 | 8.0 |  | 6.1 |  | 5.1 | 4.2 | 3.5 | 2.7 | 2.3 | 2.1 | 1.4 |
| 66 | n482,13 | 27.3 | 24.4 | 22.4 | 21.2 |  | 19.0 |  | 18 | 17.1 | 16.3 | 15.4 | 15 | 14.5 | 14 |
|  | n483 | 1.9 | 1.4 | 0.9 | 0.4 |  | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 66 | n772, 13 |  | 23.9 | 22.1 | 20.9 | 19.8 | 19.0 |  | 17.9 | 16.8 | 16.0 | 15.3 | 14.8 | 14.3 | 13.8 |
|  | n773 |  | 1.1 | 0.8 | 0.3 | 0.1 | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 66 | n782,13 |  | 23.9 | 22.1 | 20.9 | 19.8 | 19.0 |  | 17.9 | 16.8 | 16.0 | 15.4 | 14.8 | 14.3 | 13.8 |
|  | n783 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |  |
| n66 | 482,13 | 27.3 | 24.4 | 22.4 | 21.2 |  |  |  |  |  |  |  |  |  |  |
|  | 483 | 1.9 | 1.4 | 0.9 | 0.4 |  |  |  |  |  |  |  |  |  |  |
| 71 | n774, 5 |  | 10.4 | 8.9 | 7.8 | 7.1 | 6.5 |  | 4.7 | 3.7 | 3 | 2.3 | 1.7 | 1.2 | 0.7 |
| 71 | n784,5 |  | 10.4 | 8.9 | 7.8 | 7.1 | 6.5 |  | 4.7 | 3.7 | 3 | 2.3 | 1.7 | 1.2 | 0.7 |
| n105 | 13 | 1.4 | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |  |
| n105 | 76,7 | 14.6 | 11.7 | 10.1 | 9 |  |  |  |  |  |  |  |  |  |  |
|  | NOTE 1: Void NOTE 2: The requirements should be verified for UL EARFCN or NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements are only applicable to channel bandwidths no larger than 20 MHz and with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 4: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 5: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 6: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 4th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 7: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 8: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 9 The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LBsuch that in MHz and with the carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the low band. NOTE 10: Void NOTE 11: These requirements apply when the lower edge frequency of the 5 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 12: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 13: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range ∆FHD above and below the edge of this downlink transmission bandwidth. The value ∆FHD depends on the EN-DC band combination: ∆FHD = 10 MHz for DC_1_n77, DC_2_n48, DC_2_n77, DC_42_n3, DC_48_n25, DC_48_n66, DC_66_n48, DC_66_n77, DC_3_n77, DC_3_n78, DC_11_n28 and DC_28_n50, DC_28_n51, DC_66_n78, DC_25_n77, DC_25_n78. NOTE 14: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity for all active downlink component carriers is only verified when this is not the case (the requirements specified in clause 7.3.1 from TS 36.101-1 apply unless otherwise specified). NOTE 15: MSD test point can be chosen according to supported BW and lowest SCS supported by the UE. NOTE 16: The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL. This band is subject to 2nd harmonic fall in B21 also which MSD is not specified. NOTE 17: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.1-2: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 35 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 70 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |  |
| 1 | n77 | 15 |  | 25 | 36 | 50 | 64 | 80 |  | 100 | 100 | 100 | 100 | 100 | 100 | 100 |  |
| 2 | n48 | 15 | 12 | 25 | 36 | 50 |  | 80 |  | 100 | 100 | 100 | 100 | 100 | 100 | 100 |  |
| n2 | 48 | 15 | 25 | 50 | 75 | 100 |  | 100 |  | 100 | 100 | 100 |  | 100 | 100 | 100 |  |
| 2 | n77 | 15 |  | 25 | 36 | 50 | 50 | 50 |  | 50 | 50 | 50 | 50 | 50 | 50 | 50 |  |
| 2 | n78 | 15 |  | 25 | 36 | 50 | 50 | 50 |  | 50 | 50 | 50 | 50 | 50 | 50 | 50 |  |
| 3 | n77, n78 | 15 |  | 25 | 36 | 50 | 64 | 80 |  | 50 | 50 | 50 | 100 | 50 | 50 | 50 |  |
| n3 | 42 | 15 | 12 | 25 | 36 | 50 |  |  |  |  |  |  |  |  |  |  |  |
| 4 | n78 | 15 |  | 25 | 36 | 50 | 64 | 80 |  | 100 | 100 | 100 | 100 | 100 | 100 | 100 |  |
| 5 | n41 | 15 | - | 16 | 25 | 25 | 25 | 25 |  | 25 | 25 | 25 | 25 | 25 |  | 25 |  |
| 5 | n77 | 15 |  | 16 | 25 | 25 | 25 | 25 |  | 25 | 25 | 25 | 25 | 25 | 25 | 25 |  |
| 5 | n78 | 15 | 8 | 16 | 25 | 25 | 25 | 25 |  | 25 | 25 | 25 | 25 | 25 | 25 | 25 |  |
| 7 | n79 | 15 |  | 25 | 36 | 50 |  |  |  |  |  |  |  |  |  |  |  |
| 8 | n7 | 15 | 8 | 16 | 25 | 25 | 25 | 25 |  | 25 | 25 |  |  |  |  |  |  |
| 8 | n41 | 15 |  | 16 | 25 | 25 |  | 25 |  | 25 | 25 | 25 | 25 | 25 | 25 | 25 |  |
| 8 | n77 n78 | 15 |  | 16 | 25 | 25 | 25 | 25 |  | 25 | 25 | 25 | 25 | 25 | 25 | 25 |  |
| 8 | n79 | 15 |  | 10 |  | 20 |  | 25 |  | 25 | 25 | 25 | 25 | 25 | 25 | 25 |  |
| n8 | 7 | 15 | 8 | 16 | 25 | 25 |  |  |  |  |  |  |  |  |  |  |  |
| 12 | n66 | 15 | 8 | 16 | 20 | 20 | 20 | 20 |  | 20 |  |  |  |  |  |  |  |
| 12 | n77 | 15 |  | 10 | 15 | 20 | 20 | 20 |  | 20 | 20 | 20 | 20 | 20 | 20 | 20 |  |
| 12 | n78 | 15 |  | 10 | 15 | 20 | 25 | 25 |  | 25 | 25 | 25 | 25 | 25 | 25 | 25 |  |
| n12 | 48 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
| n12 | 66 | 15 | 8 | 16 | 20 | 20 |  |  |  |  |  |  |  |  |  |  |  |
| 13 | n77 | 15 |  | 15 | 20 | 20 | 20 | 20 |  | 20 | 20 | 20 | 20 | 20 | 20 | 20 |  |
| 14 | n77 | 15 |  | 15 | 15 | 15 | 15 | 15 |  | 15 | 15 | 15 | 15 | 15 | 15 | 15 |  |
| 18 | n77, n78 | 15 |  | 16 | 25 | 25 | 25 | 25 |  | 25 | 25 | 25 | 25 | 25 | 25 | 25 |  |
| 19 | n77, n78 | 15 |  | 16 | 25 | 25 | 25 | 25 |  | 25 | 25 | 25 | 25 | 25 | 25 | 25 |  |
| 20 | n38 | 15 | 8 | 16 | 25 | 25 | 25 | 25 |  | 25 |  |  |  |  |  |  |  |
| 20 | n41 | 15 | 8 | 16 | 25 | 25 |  | 25 |  | 25 | 25 | 25 | 25 | 25 | 25 | 25 |  |
| 20 | n77, n78 | 15 |  | 16 | 25 | 25 | 25 | 25 |  | 25 | 25 | 25 | 25 | 25 | 25 | 25 |  |
| 25 | n77 | 15 |  | 25 | 36 | 50 | 50 | 50 |  | 50 | 50 | 50 | 50 | 50 | 50 | 50 |  |
| 25 | n78 | 15 |  | 25 | 36 | 50 | 50 | 50 |  | 50 | 50 | 50 | 50 | 50 | 50 | 50 |  |
| 26 | n41 | 15 |  | 16 | 25 | 25 |  | 25 |  | 25 | 25 | 25 | 25 | 25 | 25 | 25 |  |
| 26 | n77, n78 | 15 |  | 16 | 25 | 25 | 25 | 25 |  | 25 | 25 | 25 | 25 | 25 | 25 | 25 |  |
| 28 | n1 | 15 | 8 | 16 | 25 | 25 | 25 | 25 |  | 25 | 25 |  |  |  |  |  |  |
| n28 | 1 | 15 | 8 | 16 | 25 | 25 |  |  |  |  |  |  |  |  |  |  |  |
| n28 | 4 | 15 | 8 | 16 | 25 | 25 |  |  |  |  |  |  |  |  |  |  |  |
| 28 | n75 | 15 | 8 | 16 | 25 | 25 | 25 | 25 |  | 25 | 25 |  |  |  |  |  |  |
| 28 | n50 | 15 | 12 | 25 | 25 | 25 |  | 25 |  | 25 | 25 | 25 |  | 25 |  |  |  |
| 28 | n51 | 15 | 12 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 28 | n66 | 15 | 8 | 16 | 25 | 25 | 25 | 25 |  | 25 |  |  |  |  |  |  |  |
| n28 | 11 | 15 | 12 | 25 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n28 | 42 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
| n28 | 66 | 15 | 8 | 16 | 25 | 25 |  |  |  |  |  |  |  |  |  |  |  |
| n28 | 32 | 12 | 25 | 25 | 25 |  |  |  |  |  |  |  |  |  |  |  |  |
| 28 | n77, n78 | 15 |  | 10 | 15 | 20 | 25 | 25 |  | 25 | 25 | 25 | 25 | 25 | 25 | 25 |  |
| 66 | n48 | 15 | 12 | 25 | 36 | 50 |  | 80 |  | 100 | 100 | 100 | 100 | 100 | 100 | 100 |  |
| 66 | n77 | 15 |  | 25 | 36 | 50 | 64 | 80 |  | 100 | 100 | 100 | 100 | 100 | 100 | 100 |  |
| 66 | n78 | 15 |  | 25 | 36 | 50 | 64 | 80 |  | 100 | 100 | 100 | 100 | 100 | 100 | 100 |  |
| n66 | 48 | 15 | 12 | 25 | 36 | 50 |  |  |  |  |  |  |  |  |  |  |  |
| 71 | n2 | 15 | 254 85 | 254 85 | 204 85 | 204 85 |  |  |  |  |  |  |  |  |  |  |  |
| 71 | n7 | 15 | 8 | 16 | 25 | 25 | 25 | 25 | 25 | 25 | 25 |  |  |  |  |  |  |
| n71 | 2 | 15 | 254 85 | 254 85 | 204 85 | 204 85 |  |  |  |  |  |  |  |  |  |  |  |
| 71 | n25 | 15 | 254 85 | 254 85 | 204 85 | 204 85 | 204 85 | 204 85 |  | 204 85 |  |  |  |  |  |  |  |
| n71 | 7 | 15 | 8 | 16 | 25 | 25 |  |  |  |  |  |  |  |  |  |  |  |
| 71 | n41 | 15 |  | 16 | 25 | 25 |  | 25 |  | 25 | 25 | 25 | 25 | 25 | 25 | 25 |  |
| 71 | n77 | 15 |  | 10 | 15 | 20 | 25 | 25 |  | 25 | 25 | 25 | 25 | 25 | 25 | 25 |  |
| 71 | n78 | 15 |  | 10 | 15 | 20 | 25 | 25 |  | 25 | 25 | 25 | 25 | 25 | 25 | 25 |  |
| n105 | 1 | 15 | 8 | 16 | 25 | 25 |  |  |  |  |  |  |  |  |  |  |
| n105 | 7 | 15 | 8 | 16 | 25 | 25 |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: The UL configuration applies regardless of the channel bandwidth of the UL band unless the UL resource blocks exceed that specified in Table 7.3.1-2 in TS 36.101 [4] or Table 7.3.2-3 in TS 38.101-1 [2] for the uplink bandwidth in which case the allocation according to Table 7.3.1-2 in TS 36.101 [4] or Table 7.3.2-3 in TS 38.101-1 [2] applies NOTE 2: Void NOTE 3: Unless stated otherwise, UL resource blocks shall be centred within the transmission bandwidth configuration for the channel bandwidth. NOTE 4: These requirements apply when the lower edge frequency of the 5 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 5: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 6: If the aggressor band is NR band, the test SCS and UL RB can be adjusted according to supported BW and lowest SCS supported by the UE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.1-3: Reference sensitivity QPSK PREFSENS (EN-DC with n46)

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 2 | n461 |  |  |  | N/A |  | N/A |  | N/A | N/A |  |  |
| n46 | 22,3 | 28 | 28 | 28 | 28 |  |  |  |  |  |  |  |
| n46 | 482,4 | 22.6 | 19.5 | 17.8 | 16.6 |  |  |  |  |  |  |  |
| 66 | n46 |  |  |  | N/A |  | N/A |  | N/A | N/A |  |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the downlink (victim) transmission bandwidth which falls into the reference sensitivity exclusion region as specified in Table 6.x.1.7-2 and Table 6.x.1.7-3. NOTE 2: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (higher) band and when the frequency range of relative higher band’s uplink channel bandwidth or uplink 1st adjacent channel bandwidth is fully or partially overlapped with the downlink transmission bandwidth of a victim (lower) band. NOTE 3: The requirements for a victim (lower) band apply for UL EARFCN of the aggressor (higher) band (superscript HB) such that in MHz with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. Note 4: The requirements should be verified for UL NR-ARFCN of the aggressor (high) band (superscript HB) such that in MHz and with carrier frequency in the victim (lower) band in MHz and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.1-4: n46 Reference sensitivity measurement exclusion region in MHz

| Licensed Component Carriers / E-UTRA Band / Harmonic order / Channel BW in UL |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Band | Harmonic order | 5MHz | 10MHz | 15MHz | 20MHz |
| 2 | 3 | +/- 15 | +/- 23 | +/- 35 | +/- 45 |
| 66 | 3 | +/- 15 | +/- 23 | +/- 35 | +/- 45 |
| NOTE 1: Even though UL harmonic does not fall directly into n46 the exclusion region still applies. NOTE 2: The center of the exclusion region is obtained by multiplying the uplink channel center frequency by the harmonic order. |  |  |  |  |  |

Table 7.3B.2.3.1-5: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for EN-DC paring with n46

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n46 | 2 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |
| n46 | 48 | 15 | 12 | 25 | 36 | 50 |  |  |  |  |  |  |  |

7.3B.2.3.2	Reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

Sensitivity degradation is allowed for a band if it is impacted by receiver harmonic mixing due to another band part of the same EN-DC configuration. Reference sensitivity exceptions for the victim band (low) are specified in Table 7.3B.2.3.2-1 with uplink configuration of the aggressor band (high) specified in Table 7.3B.2.3.2-2.

Table 7.3B.2.3.2-1: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 1 | n714 | 26.8 | 23.6 | 21.2 | 15.6 |  |  |  |  |  |  |  |
| 1 | n1054 | 26.8 | 23.6 | 21.2 | 15.6 |  |  |  |  |  |  |  |
| 2 | n714 | 26.8 | 23.6 | 21.2 | 15.6 |  |  |  |  |  |  |  |
| n2 | 714 | 26.8 | 23.6 | 21.2 | 15.6 |  |  |  |  |  |  |  |
| 7 | n264,12 | [2.0] |  |  |  |  |  |  |  |  |  |  |
| 7 | n105 | 5.7 | 4.0 | 3.0 | 2.7 |  |  |  |  |  |  |  |
| n25 | 714 | 26.8 | 23.6 | 21.2 | 15.6 |  |  |  |  |  |  |  |
| n38 | 59 | N/A | N/A |  |  |  |  |  |  |  |  |  |
| n40 | 284 | 37.8 | 34.8 | 33 | 30.3 |  |  |  |  |  |  |  |
| n41 | 54 | 24.3 | 24.3 |  |  |  |  |  |  |  |  |  |
| 48 | n122 | 31 | 28 |  |  |  |  |  |  |  |  |  |
| n41 | 189 | N/A | N/A | N/A |  |  |  |  |  |  |  |  |
| n41 | 264 | 24.3 | 24.3 | 22.5 | N/A |  |  |  |  |  |  |  |
| n77 | 2 | 6.1 | 5.0 | 4.0 | 3.7 |  |  |  |  |  |  |  |
| n77 | 3 | 5.7 | 4.0 | 3.0 | 2.7 |  |  |  |  |  |  |  |
| n78 | 3 | 5.7 | 4.0 | 3.0 | 2.7 |  |  |  |  |  |  |  |
| n77 | 78 | 10.4 | 10.4 | 10.4 | 10.4 |  |  |  |  |  |  |  |
| n77 | 122 | 31 | 28 |  |  |  |  |  |  |  |  |  |
| n77 | 132 | 31 | 28 |  |  |  |  |  |  |  |  |  |
| n77 | 142 | 31 | 28 |  |  |  |  |  |  |  |  |  |
| n77 | 418 | 10.4 | 10.4 | 10.4 | 10.4 |  |  |  |  |  |  |  |
| n77 | 25 | [6.1] | [5.0] | [4.0] | [3.7] |  |  |  |  |  |  |  |
| n77 | 282 | 28 | 25 | 23.2 | 22 |  |  |  |  |  |  |  |
| n7711 | 292 | 31 | 28 |  |  |  |  |  |  |  |  |  |
| n78 | 8 | 5.7 | 4.0 | 3.0 | 2.7 |  |  |  |  |  |  |  |
| n78 | 122 | 31 | 28 |  |  |  |  |  |  |  |  |  |
| n78 | 132 | 31 | 28 |  |  |  |  |  |  |  |  |  |
| n78 | 408 | 10.4 | 10.4 | 10.4 | 10.4 |  |  |  |  |  |  |  |
| n78 | 418 | 10.4 | 10.4 | 10.4 | 10.4 |  |  |  |  |  |  |  |
| n79 | 114 | 39.3 | 36.3 | 34.5 |  |  |  |  |  |  |  |  |
| n79 | 192 | 29.5 | 26.5 | 24.7 |  |  |  |  |  |  |  |  |
| n79 | 214 | 39.3 | 36.3 | 34.5 |  |  |  |  |  |  |  |  |
| n79 | 262 | 27 | 24 | 22.2 |  |  |  |  |  |  |  |  |
| n79 | 82 | 25 | 22 |  |  |  |  |  |  |  |  |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (higher) band for which the mixing product due to harmonic of victim (lower) band LO with leakage of aggressor (higher) band is within the downlink transmission bandwidth of a victim (lower) band. NOTE 2: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 3: Void. NOTE 4: The requirements should be verified for DL EARFCN or NR ARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 5: Void NOTE 6: Void NOTE 7: Void NOTE 8: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1 with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 9: No requirements apply for the case that there is at least one individual RE within the uplink transmission bandwidth of the relative higher band and when the frequency range of relative higher band’s uplink channel bandwidth or uplink 1st adjacent channel bandwidth is fully or partially overlapped with the 3 times of the frequency range of the relative lower band’s downlink channel bandwidth. The reference sensitivity is only verified when this is not the case. NOTE 10: MSD test point can be chosen according to supported BW and lowest SCS supported by the UE. NOTE 11: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. NOTE 12: The requirements should be verified for the lowest NR ARFCN of the affected DL (lower) band and for the highest NR ARFCN of the UL (higher) band |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.2-1a: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for PC2 EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n77 | 2 | 9.1 | 8.0 | 7.0 | 6.7 |  |  |  |  |  |  |  |
| n77 | 3 | 8.1 | 7.0 | 6.0 | 5.7 |  |  |  |  |  |  |  |
| n77 | 121 | 34 | 31 |  |  |  |  |  |  |  |  |  |
| n77 | 131 | 34 | 31 |  |  |  |  |  |  |  |  |  |
| n77 | 141 | 34 | 31 |  |  |  |  |  |  |  |  |  |
| n772 | 291 | 34 | 31 |  |  |  |  |  |  |  |  |  |
| n78 | 282 | 31 | 28 | 26.2 | 25 |  |  |  |  |  |  |  |
| n79 | 191 | 32.5 | 29.5 | 27.7 |  |  |  |  |  |  |  |  |
| n79 | 213 | 42.3 | 39.3 | 37.5 |  |  |  |  |  |  |  |  |
| NOTE 1: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 2: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. . NOTE 3: The requirements should be verified for DL EARFCN or NR ARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.2-2: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the aggressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| 1 | n71 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |
| 1 | n105 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |
| 2 | n71 | 15 | 25 | 50 | 50 | 50 |  |  |  |  |  |  |  |
| n2 | 71 | 15 | 25 | 50 | 50 | 50 |  |  |  |  |  |  |  |
| 7 | n26 | 15 |  |  |  | 256 |  |  |  |  |  |  |  |
| 7 | n105 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |
| n25 | 71 | 15 | 25 | 50 | 50 | 50 |  |  |  |  |  |  |  |
| n40 | 28 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |
| n41 | 5 | 15 | 25 | 50 |  |  |  |  |  |  |  |  |  |
| n41 | 26 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |
| 48 | n12 | 15 | 25 | 50 |  |  |  |  |  |  |  |  |  |
| n77 | 2 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |
| n77 | 3 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |
| n78 | 3 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |
| n77 | 7 | 15 | 12 | 25 | 36 | 50 |  |  |  |  |  |  |  |
| n77 | 12 | 15 | 25 | 50 |  |  |  |  |  |  |  |  |  |
| n77 | 13 | 15 | 25 | 50 |  |  |  |  |  |  |  |  |  |
| n77 | 14 | 15 | 25 | 50 |  |  |  |  |  |  |  |  |  |
| n77 | 25 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |
| n77 | 28 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |
| n77 | 29 | 15 | 25 | 50 |  |  |  |  |  |  |  |  |  |
| n77 | 41 | 15 | 12 | 25 | 36 | 50 |  |  |  |  |  |  |  |
| n78 | 8 | 15 | 25 | 25 | 20 | 20 |  |  |  |  |  |  |  |
| n78 | 12 | 15 | 25 | 50 |  |  |  |  |  |  |  |  |  |
| n78 | 13 | 15 | 25 | 50 |  |  |  |  |  |  |  |  |  |
| n78 | 28 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |
| n78 | 29 | 15 | 25 | 50 |  |  |  |  |  |  |  |  |  |
| n78 | 40 | 15 | 12 | 25 | 36 | 50 |  |  |  |  |  |  |  |
| n78 | 41 | 15 | 12 | 25 | 36 | 50 |  |  |  |  |  |  |  |
| n79 | 11 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |
| n79 | 19 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |
| n79 | 21 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |
| n79 | 26 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |
| n79 | 8 | 15 | 25 | 50 |  |  |  |  |  |  |  |  |  |
| NOTE 1: Void NOTE 2: Void NOTE 3: The UL configuration applies regardless of the channel bandwidth of the UL band. UL resource blocks allocation in the table shall be further limited to that specified in Table 7.3.1-2 in TS 36.101 [4] or Table 7.3.2-3 in TS 38.101-1 [2]. NOTE 4: Unless otherwise stated, the UL resource blocks allocation is applied at the center of the channel bandwidth. The note applies to the entire table. NOTE 5: If the aggressor band is NR band, the test SCS and UL RB can be adjusted according to supported BW and lowest SCS supported by the UE. NOTE 6: RBstart = 75 |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.3B.2.3.3	Void

7.3B.2.3.4	Reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1

Sensitivity degradation is allowed for a band if it is impacted by UL of another band part of the same EN-DC configuration due to cross band isolation issues. Reference sensitivity exceptions for the victim band are specified in Table 7.3B.2.3.4-1 and Table 7.3B.2.3.4-1a with uplink configuration of the agressor band specified in Table 7.3B.2.3.4-2.

Table 7.3B.2.3.4-1: Reference sensitivity exceptions (MSD) due to cross band isolation for PC3 EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n13 | 3 | 3 | 2.3 | 2 | 1.8 |  |  |  |  |  |  |  |  |  |
| n1 | 40 | 6.6 | 6.6 | 6.6 | 6.6 |  |  |  |  |  |  |  |  |  |
| n1 | 41 | 6.1 | 6.1 | 6.1 | 6.1 |  |  |  |  |  |  |  |  |  |
| 13 | n3 | 3 | 2.2 | 1.9 | 1.7 | 1.6 | 1.5 | 1.4 | 1.3 |  |  |  |  |  |
| 1 | n40 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 |
| 1 | n41 |  | 6.1 | 6.1 | 6.1 |  | 6.1 | 6.1 | 6.1 | 6.1 | 6.1 | 6.1 | 6.1 | 6.1 |
| n3 | 11 | 6.4 | 6.1 |  |  |  |  |  |  |  |  |  |  |  |
| 3 | n41 |  | 0.7 | 0.7 | 0.7 |  | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 |
| 3 | n51 | 6.4 |  |  |  |  |  |  |  |  |  |  |  |  |
| 30 | n66 | 8.3 | 8.3 | 8.3 | 8.3 | 8.3 | 8.3 | 8.3 |  |  |  |  |  |  |
| n3 | 41 | 0.7 | 0.7 | 0.7 | 0.7 |  |  |  |  |  |  |  |  |  |
| n5 | 28 | [17.5] | [15.8] | [14.0] | [11.7] |  |  |  |  |  |  |  |  |  |
| 7 | n40 | 3.7 | 3.4 | 3.2 | 3.1 | 3.1 | 3.1 | 3.1 | 3.1 | 3.1 | 3.1 | 3.1 | 3.1 | 3.1 |
| n12 | 71 | 3.8 |  |  |  |  |  |  |  |  |  |  |  |  |
| 18 | n287 | 31.3 | 28.7 | 26.9 | 24.3 |  | 12.4 |  |  |  |  |  |  |  |
| n25 | 2 | 33 | 33 | 33 | 33 |  |  |  |  |  |  |  |  |  |
| n34 | 3 | 3 | 2.2 | 1.9 | 1.7 |  |  |  |  |  |  |  |  |  |
| n38 | 1 | 1.9 | 1.9 | 1.9 | 1.9 |  |  |  |  |  |  |  |  |  |
| n38 | 2 | 0.6 | 0.6 | 0.6 | 0.6 |  |  |  |  |  |  |  |  |  |
| n38 | 4 | 1.9 | 1.9 | 1.9 | 1.9 |  |  |  |  |  |  |  |  |  |
| n38 | 66 | 1.9 | 1.9 | 1.9 | 1.9 |  |  |  |  |  |  |  |  |  |
| 38 | n1 | 1.9 | 1.9 | 1.9 | 1.9 | 1.9 | 1.9 | 1.9 | 1.9 |  |  |  |  |  |
| n1 | 38 | 2.9 | 2.9 | 2.9 | 2.9 |  |  |  |  |  |  |  |  |  |
| n40 | 1 | 8.3 | 8.3 | 8.3 | 8.3 |  |  |  |  |  |  |  |  |  |
| n41 | 4 | 3.5 | 3.5 | 3.5 | 3.5 |  |  |  |  |  |  |  |  |  |
| 40 | n1 | 8.3 | 8.3 | 8.3 | 8.3 | 8.3 | 8.3 | 8.3 | 8.3 |  |  |  |  |  |
| n40 | 7 | 3.7 | 3.7 | 3.7 | 3.7 |  |  |  |  |  |  |  |  |  |
| n41 | 1 | 9.1 | 9.1 | 9.1 | 9.1 |  |  |  |  |  |  |  |  |  |
| n41 | 2 | 0.6 | 0.6 | 0.6 | 0.6 |  |  |  |  |  |  |  |  |  |
| n41 | 3 | 0.6 | 0.6 | 0.6 | 0.6 |  |  |  |  |  |  |  |  |  |
| 41 | n1 | 9.1 | 9.1 | 9.1 | 9.1 | 9.1 | 9.1 | 9.1 | 9.1 |  |  |  |  |  |
| 41 | n3 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 |  |  |  |  |  |
| n41 | 661 | 3.5 | 3.5 | 3.5 | 3.5 |  |  |  |  |  |  |  |  |  |
| n41 | 25 | 0.6 | 0.6 | 0.6 | 0.6 |  |  |  |  |  |  |  |  |  |
| n50 | 3 | 2.5 | 1.9 | 1.6 | 1.5 |  |  |  |  |  |  |  |  |  |
| n71 | 12 | 2.3 | 2.3 |  |  |  |  |  |  |  |  |  |  |  |
| 71 | n12 | 8.2 |  |  |  |  |  |  |  |  |  |  |  |  |
| n77 | 71 | 4.5 | 4.5 | 4.5 | 4.5 |  |  |  |  |  |  |  |  |  |
| n77 | 411 | 4.5 | 4.5 | 4.5 | 4.5 |  |  |  |  |  |  |  |  |  |
| 41 | n77 |  | 8.3 | 8.3 | 8.3 | 7.3 | 6.5 | 6.3 | 5.3 | 4.5 | 4.3 | 4.0 | 3.9 | 3.8 |
| n78 | 71 | 4.5 | 4.5 | 4.5 | 4.5 |  |  |  |  |  |  |  |  |  |
| n78 | 38 | 3.3 | 3.3 | 3.3 | 3.3 |  |  |  |  |  |  |  |  |  |
| n78 | 401 | 4.5 | 4.5 | 4.5 | 4.5 |  |  |  |  |  |  |  |  |  |
| n78 | 411 | 4.5 | 4.5 | 4.5 | 4.5 |  |  |  |  |  |  |  |  |  |
| n78 | 46 |  |  |  | 7 |  |  |  |  |  |  |  |  |  |
| 41 | n78 |  | 8.3 | 8.3 | 8.3 | 7.3 | 6.5 | 6.3 | 5.3 | 4.5 | 4.3 | 4.0 | 3.9 | 3.8 |
| n79 | 426 | 2.6 | 2.6 | 2.6 | 2.6 |  |  |  |  |  |  |  |  |  |
| n843 | 3 | 3 | 2.3 | 2 | 1.8 |  |  |  |  |  |  |  |  |  |
| 48 | n46 | - | - | - | 7 | - | - | 5.7 | - | 5.1 | - | 4.7 | - | - |
| n46 | 48 | 13.3 | 10.4 | 8.8 | 7.8 | - | - | 7.8 | 7 | 6.5 | - | 5.7 | 5.4 | 5.1 |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: The B41 requirements are modified by -0.5dB when carrier frequency of the assigned E-UTRA channel bandwidth is within 2515 – 2690 MHz. NOTE 3: These requirements apply when the uplink is active in Band n1, n84 and the separation between the lower edge of the uplink channel in Band n1, n84 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in Band 3, the requirement applies regardless of channel bandwidth in Band n1, n84. NOTE 4: The DL victim band should be configured using the lowest SCS that is compatible with the highest CBW for which an MSD is specified. NOTE 5: MSD test point can be chosen according to supported BW and lowest SCS supported by the UE. NOTE 6: The requirements only apply for UEs supporting inter-band DC_42_n79 ENDC with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band 42 with a n77 implementation only. These restrictions are applicable to related higher order configurations. NOTE 7: The MSD exceptions are applicable to the case that interference of UL band 3rd order IMD product falls into the affected DL channels. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.4-1a: Reference sensitivity exceptions (MSD) due to cross band isolation for PC2 EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 3 | n41 |  | 0.7 | 0.7 | 0.7 |  | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 |
| n41 | 3 | 2.3 | 2.3 | 2.3 | 2.3 |  |  |  |  |  |  |  |  |  |
| n78 | 7 | 6.4 | 6.4 | 6.4 | 6.4 |  |  |  |  |  |  |  |  |  |
| n41 | 2 | 1.6 | 1.6 | 1.6 | 1.6 |  |  |  |  |  |  |  |  |  |
| n41 | 66 | 5.4 | 5.4 | 5.4 | 5.4 |  |  |  |  |  |  |  |  |  |
| n77 | 2 | 1.0 | 1.0 | 1.0 | 1.0 |  |  |  |  |  |  |  |  |  |
| n77 | 30 | 1.0 | 1.0 |  |  |  |  |  |  |  |  |  |  |  |
| n77 | 66 | 1.0 | 1.0 | 1.0 | 1.0 |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.4-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 70 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n1 | 3 | 15 | 25 | 25 | 25 | 25 |  |  |  |  |  |  |  |  |  |
| n1 | 40 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |  |  |
| n1 | 41 | 15 | 100 | 100 | 100 | 100 |  |  |  |  |  |  |  |  |  |
| 1 | n3 | 15 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 |  |  |  |  |  |
| 1 | n40 | 15 | 25 | 50 | 75 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| 1 | n41 | 15 |  | 100 | 100 | 100 |  | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| n3 | 11 | 15 | 25 | 50 |  |  |  |  |  |  |  |  |  |  |  |
| 3 | n41 | 15 |  | 50 | 50 | 50 |  | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |
| 3 | n51 | 15 | 25 |  |  |  |  |  |  |  |  |  |  |  |  |
| 30 | n66 | 15 | 25 | 25 | 25 | 25 | 25 | 25 | 25 |  |  |  |  |  |  |
| n3 | 41 | 15 | 25 | 502 | 502 | 502 |  |  |  |  |  |  |  |  |  |
| n5 | 28 | 15 | 20 | 20 | 20 | 20 |  |  |  |  |  |  |  |  |  |
| 7 | n40 | 15 | 25 | 50 | 75 | 75 | 75 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| n12 | 71 | 15 |  |  | 20 |  |  |  |  |  |  |  |  |  |  |
| 18 | n286 | 15 | 25 | 25 | 25 | 25 |  | 25 |  |  |  |  |  |  |  |
| n25 | 2 | 15 | 40 | 40 | 40 | 40 |  |  |  |  |  |  |  |  |  |
| n34 | 3 | 15 | 25 | 25 | 25 | 25 |  |  |  |  |  |  |  |  |  |
| n38 | 1 | 15 | 100 | 100 | 100 | 100 |  |  |  |  |  |  |  |  |  |
| n38 | 2 | 15 | 100 | 100 | 100 | 100 |  |  |  |  |  |  |  |  |  |
| n38 | 4 | 15 | 100 | 100 | 100 | 100 |  |  |  |  |  |  |  |  |  |
| n38 | 66 | 15 | 100 | 100 | 100 | 100 |  |  |  |  |  |  |  |  |  |
| 38 | n1 | 15 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |  |  |  |  |  |
| n1 | 38 | 15 | 100 | 100 | 100 | 100 |  |  |  |  |  |  |  |  |  |
| n40 | 1 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |  |  |
| n41 | 4 | 30 | 128 | 128 | 128 | 128 |  |  |  |  |  |  |  |  |  |
| 40 | n1 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |  |  |
| n40 | 7 | 30 | 216 | 216 | 216 | 216 |  |  |  |  |  |  |  |  |  |
| n41 | 1 | 30 | 128 | 128 | 128 | 128 |  |  |  |  |  |  |  |  |  |
| n41 | 2 | 30 | 160 | 160 | 160 | 160 |  |  |  |  |  |  |  |  |  |
| n41 | 3 | 30 | 160 | 160 | 160 | 160 |  |  |  |  |  |  |  |  |  |
| 41 | n1 | 15 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |  |  |  |  |  |
| 41 | n3 | 15 | 25 | 50 | 75 | 100 | 100 | 100 | 100 | 100 |  |  |  |  |  |
| n41 | 66 | 30 | 128 | 128 | 128 | 128 |  |  |  |  |  |  |  |  |  |
| n41 | 25 | 30 | 160 | 160 | 160 | 160 |  |  |  |  |  |  |  |  |  |
| n50 | 3 | 30 | 160 | 160 | 160 | 160 |  |  |  |  |  |  |  |  |  |
| n71 | 12 | 15 | 20 | 20 |  |  |  |  |  |  |  |  |  |  |  |
| 71 | n12 | 15 |  |  |  | 20 |  |  |  |  |  |  |  |  |  |
| n77 | 2 | 30 | 270 | 270 | 270 | 270 |  |  |  |  |  |  |  |  |  |
| n77 | 7 | 30 | 270 | 270 | 270 | 270 |  |  |  |  |  |  |  |  |  |
| n77 | 30 | 30 | 270 | 270 |  |  |  |  |  |  |  |  |  |  |  |
| n77 | 41 | 30 | 270 | 270 | 270 | 270 |  |  |  |  |  |  |  |  |  |
| n77 | 66 | 30 | 270 | 270 | 270 | 270 |  |  |  |  |  |  |  |  |  |
| 41 | n77 | 15 |  | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| n78 | 7 | 30 | 270 | 270 | 270 | 270 |  |  |  |  |  |  |  |  |  |
| n78 | 38 | 30 | 270 | 270 | 270 | 270 |  |  |  |  |  |  |  |  |  |
| n78 | 40 | 30 | 270 | 270 | 270 | 270 |  |  |  |  |  |  |  |  |  |
| n78 | 41 | 30 | 270 | 270 | 270 | 270 |  |  |  |  |  |  |  |  |  |
| n78 | 46 | 30 |  |  |  | 270 |  |  |  |  |  |  |  |  |  |
| 41 | n78 | 15 |  | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| n79 | 42 | 30 | 2705 | 2705 | 2705 | 2705 |  |  |  |  |  |  |  |  |  |
| n84 | 3 | 15 | 25 | 25 | 25 | 25 |  |  |  |  |  |  |  |  |  |
| 48 | n46 | 15 |  |  |  | 216 |  |  | 216 |  | 216 |  | 216 |  |  |
| n46 | 48 | 30 | 216 | 216 | 216 | 216 |  |  | 216 | 216 | 216 |  | 216 | 216 | 216 |
| NOTE 1: The UL configuration applies regardless of the channel bandwidth of the UL band. UL resource blocks allocation in the table shall be further limited to that specified in Table 7.3.1-2 in TS 36.101 [4] or Table 7.3.2-3 in TS 38.101-1 [2]. NOTE 2: The UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth. NOTE 3: When the maximum UL RB allocation “LCRB” value is less than the maximum transmission bandwidth configuration “NRB” defined in Table 5.3.2-1 in 38.101-1 [2] for the specified UL band SCS, the UL band should be configured using the lowest CBW that is compatible with the maximum specified LCRB value. NOTE 4: If the aggressor band is NR band, the test SCS and UL RB can be adjusted according to supported BW and lowest SCS supported by the UE. NOTE 5: The requirements only apply for UEs supporting inter-band ENDC with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band 42 with a n77 implementation only. These restrictions are applicable to related higher order configurations. NOTE 6: The UL configuration are applicable to the case that interference of UL band 3rd order IMD product falls into the affected DL channels. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.3B.2.3.5	MSD for intermodulation interference due to dual uplink operation for EN-DC in NR FR1

7.3B.2.3.5.0	General

For EN-DC configurations in NR FR1 the UE may indicate capability of not supporting simultaneous dual uplink operation due to possible intermodulation interference overlapping in frequency to its own primary downlink channel bandwidth if

-	the intermodulation order is 2;

-	the intermodulation order is 3 when both operating bands are between 450 MHz – 960 MHz or between 1427 MHz – 2690 MHz

In the case for EN-DC configurations in NR FR1 for which the intermodulation products caused by dual uplink operation do not interfere with its own primary downlink channel bandwidth as defined in Annex I the UE is mandated to operate in dual and triple uplink mode.

For EN-DC configurations in NR FR1 with uplink and downlink assigned to E-UTRA and NR FR1 bands given in Table 7.3B.2.3.5.1-1, Table 7.3B.2.3.5.1-1a, Table 7.3B.2.3.5.2-0 and Table 7.3B.2.3.5.2-1 the reference sensitivity is defined only for the specific uplink and downlink test points specified in Table 7.3B.2.3.5.1-1, Table 7.3B.2.3.5.1-1a, Table 7.3B.2.3.5.2-0 and Table 7.3B.2.3.5.2-1. For these test points the reference sensitivity levels specified in clause 7.3.1 in TS 36.101 [4] and 7.3.2 of TS 38.101-1 [2] for the corresponding channel bandwidths or in clause 7.3.1 of TS 36.101 [4] are relaxed by the amount of the parameter MSD given in Table 7.3B.2.3.5.1-1, Table 7.3B.2.3.5.1-1a, Table 7.3B.2.3.5.2-0 and Table 7.3B.2.3.5.2-1.

The throughput on each of the CGs shall be ≥ 95% of the maximum throughput of the respective reference measurement channels as specified in Annex A of TS 38.101-1 [2] and Annex A of TS 36.101 [4], with parameters specified in Table 7.3B.2.3.5.1-1, Table 7.3B.2.3.5.1-1a, Table 7.3B.2.3.5.2-0 and Table 7.3B.2.3.5.2-1 with dual UL transmissions overlapping in time unless otherwise stated.

7.3B.2.3.5.1	MSD test points for intermodulation interference due to dual uplink operation for PC3 EN-DC in NR FR1 involving two bands

Table 7.3B.2.3.5.1-1: MSD test points for PCell due to dual uplink operation for PC3 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1_n3 | 1 | 1950 | 5 | 25 | 2140 | 23 | IMD3 |
|  | n3 | 1760 | 5 | 25 | 1855 | N/A | N/A |
| DC_1C_n3 | 1C | 1950 1970 | 20 20 | 1 (RBstart=0) 1 (RBstart=67) | 2140 2160 | N/A | N/A |
|  | n3 | N/A | 5 | N/A | 1877.5 | 36 | IMD5 |
| DC_1A_n8A | 1 | 1965 | 5 | 25 | 2155 | 6.0 | IMD4 |
|  | n8 | 887.5 | 5 | 25 | 932.5 | N/A | N/A |
| DC_1A_n71A DC_1A_n71B | 1 | 1958 | 5 | 25 | 2148 | N/A | N/A |
|  | n71 | 668 | 5 | 25 | 622 | 15.1 | IMD3 |
| DC_1A_n77A, DC_1A_SUL_n77A-n84A, DC_1A_n77(2A), | 1 | 1950 | 5 | 25 | 2140 | 29.8 | IMD23 |
|  | n77 | 4090 | 10 | 50 | 4090 | N/A | N/A |
| DC_1A_n77A, DC_1A_SUL_n77A-n84A, DC_1A_n77(2A), DC_1A_n77(3A), DC_1A_n78A, DC_1A_SUL_n78A-n84A, DC_1A_n78(2A) DC_1A_n78(A-C) | 1 | 1950 | 5 | 25 | 2140 | 8.0 | IMD43 |
|  | n77, n78 | 3710 | 10 | 50 | 3710 | N/A | N/A |
| DC_2A_n46A | 2 | 1880 | 5 | 25 | 1960 | 12.0 | IMD3 |
|  | n46 | 5720 | 20 | 100 | 5720 | N/A | N/A |
| DC_2A_n48A | 2 | 1852.5 | 5 | 25 | 1932.5 | 12 | IMD4 |
|  | n48 | 3625 | 20 | 100 | 3625 | N/A | N/A |
| DC_2A_n66A, DC_2A-2A_n66A DC_2A_n66(2A) | 2 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |
|  | n66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
| DC_2A_n66A, DC_2A-2A_n66A DC_2A_n66(2A) | 2 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | 4 | IMD5 |
| DC_2A_n77A DC_2A_n77(2A) DC_2A-2A_n77A DC_2A_n77(2A) DC_2A-2A_n77(2A) | 2 | 1855 | 5 | 25 | 1935 | 26 | IMD2 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | N/A |
|  | 2 | 1900 | 5 | 25 | 1980 | 8.0 | IMD4 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |
|  | 2 | 1885 | 5 | 25 | 1965 | 5 | IMD5 |
|  | n77 | 3810 | 10 | 50 | 3810 | N/A | N/A |
| DC_2A_n78A DC_2A_n78(2A) DC_2A-2A_n78(2A) | 2 | 1855 | 5 | 25 | 1935 | 26 | IMD23 |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | N/A |
|  | 2 | 1885 | 5 | 25 | 1965 | 8.0 | IMD43 |
|  | n78 | 3690 | 10 | 50 | 3690 | N/A | N/A |
| DC_3_n1 | 3 | 1760 | 5 | 25 | 1855 | N/A | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | 23 | IMD3 |
| DC_3_n5 | 3 | 1771 | 10 | 50 | 1866 | 4 | IMD4 |
|  | n5 | 838 | 5 | 25 | 883 | N/A | N/A |
|  | 3 | 1721 | 10 | 50 | 1816 | N/A | N/A |
|  | n5 | 838 | 5 | 25 | 883 | 24 | IMD23 |
| DC_3A_n7A DC_3C_n7A | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n7 | 2535 | 10 | 50 | 2655 | 10.2 | IMD4 |
| DC_3_n8 | n8 | 900 | 5 | 25 | 945 | 8 | IMD43 |
|  | 3 | 1755 | 10 | 50 | 1850 | N/A | N/A |
|  | n8 | 897.5 | 5 | 25 | 942.5 | N/A | N/A |
|  | 3 | 1747.5 | 10 | 50 | 1842.5 | 6.4 | IMD5 |
| DC_3A_n20A DC_3C_n20A | 3 | 1775 | 5 | 25 | 1870 | 4 | IMD4 |
|  | n20 | 840 | 5 | 25 | 799 | N/A | N/A |
|  | 3 | 1735 | 5 | 25 | 1830 | N/A | N/A |
|  | n20 | 847 | 5 | 25 | 806 | 9 | IMD4 |
| DC_3A_n26A | 3 | 1771 | 10 | 50 | 1866 | 4 | IMD4 |
|  | n26 | 838 | 5 | 25 | 883 | N/A | N/A |
|  | 3 | 1721 | 10 | 50 | 1816 | N/A | N/A |
|  | n26 | 838 | 5 | 25 | 883 | 24 | IMD23 |
| DC_3C_n26A | 3 | 1720 | 20 | 1 (RBSTART=0) | 1815 | N/A | N/A |
|  |  | 1739.8 | 20 | 1 (RBSTART=99) | 1834.8 | N/A |  |
|  | n26 | 841.5 | 15 | 25(RBSTART=54) | 886.5 | 18.9 | IMD3 |
| DC_3C_n28A | n28 | 715.5 | 25 | 25(RBSTART=108) | 770.5 | 11 | 1st order triple beat α (TX22TX1) i.e. IMD3 |
|  | 3 | 1720 | 20 | 1 (RBSTART=0) | 1815 | N/A | N/A |
| DC_3C_n20A | n20 | 840 | 5 | 25 | 799 | N/A | N/A |
|  | 3 | 1735 | 5 | 25 | 1830 | N/A | N/A |
|  | n20 | 847 | 5 | 25 | 806 | 9 | IMD4 |
| DC_3A_n38A | 3 | 1712.8 | 5 | 25 | 1807.8 | 8.2 | IMD4 |
|  | n38 | 2616.7 | 10 | 50 | 2616.7 | N/A | N/A |
| DC_3A_n41A DC_3C_n41A DC_3A_SUL_n41A-n80A, DC_3C_SUL_n41A-n80A | 3 | 1740 | 5 | 25 | 1835 | 8.2 | IMD4 |
|  | n41 | 2657.5 | 10 | 50 | 2657.5 | N/A | N/A |
| DC_3A_n77A, DC_3A_n77(2A), DC_3A_n77(3A), DC_3A_SUL_n77A-n80A, DC_3A_n78A, DC_3A_SUL_n78A-n80A, DC_3A_n78(2A), DC_3A_n78(A-C) DC_3C_n78A DC_3C_n78(2A) | 3 | 1740 | 5 | 25 | 1835 | 26 | IMD23 |
|  | n77, n78 | 3575 | 10 | 50 | 3575 | N/A | N/A |
| DC_3A_n77A, DC_3A_n77(2A), DC_3C_n77A, DC_3C_n77(2A), DC_3A_SUL_n77A-n80A, DC_3A_n78A, DC_3A_SUL_n78A-n80A, DC_3A_n78(2A), DC_3C_n78A DC_3C_n78(2A) | 3 | 1765 | 5 | 25 | 1860 | 8.0 | IMD43 |
|  | n77, n78 | 3435 | 10 | 50 | 3435 | N/A | N/A |
| DC_4A_n2A | 2 | 1860 | 20 | 502 | 1940 | 5 | IMD3 |
|  | 4 | 1752.5 | 5 | 25 | 2152.5 | N/A | N/A |
|  | 2 | 1868.3 | 5 | 25 | 1948.3 | N/A | N/A |
|  | 4 | 1735 | 5 | 25 | 2135 | 5 | IMD5 |
| DC_4A_n5A | n5 | 838 | 5 | 25 | 883 | 30 | IMD23 |
|  | 4 | 1721 | 5 | 25 | 2121 | N/A | N/A |
| DC_4A_n7A | 4 | 1730 | 5 | 25 | 2130 | N/A | N/A |
|  | n7 | 2535 | 10 | 50 | 2655 | 15 | IMD4 |
| DC_5A_n3A | 5 | 838 | 5 | 25 | 883 | N/A | N/A |
|  | n3 | 1771 | 10 | 50 | 1866 | 4 | IMD4 |
|  | 5 | 838 | 5 | 25 | 883 | 24 | IMD23 |
|  | n3 | 1721 | 10 | 50 | 1816 | N/A | N/A |
| DC_5_n7 | n7 | 2547 | 10 | 50 | 2667 | N/A | N/A |
|  | 5 | 834 | 5 | 25 | 879 | 12 | IMD33 |
| DC_5_n38 | 5 | 844 | 5 | 25 | 889 | 12 | IMD33 |
|  | n38 | 2577 | 10 | 50 | 2577 | N/A | N/A |
| DC_5A_n41A | 5 | 839 | 5 | 25 | 884 | 15.6 | IMD33 |
|  | n41 | 2562 | 10 | 50 | 2562 | N/A | N/A |
| DC_5A_n66A | 5 | 838 | 5 | 25 | 883 | 30 | IMD23 |
|  | n66 | 1721 | 5 | 25 | 2121 | N/A | N/A |
| DC_5A_n77A8 DC_5A_n77(2A)8 DC_5A_n77(3A)8 | 5 | 844 | 5 | 25 | 889 | 8.3 | IMD4 |
|  | n77 | 3421 | 10 | 50 | 3421 | N/A | N/A |
|  | 5 | 826.5 | 5 | 25 | 871.5 | 5.5 | IMD5 |
|  | n77 | 4177.5 | 10 | 50 | 4177.5 | N/A | N/A |
| DC_5A_n78A DC_5A_n78(2A) DC_5A_n78(A-C) DC_5A_n78C | 5 | 844 | 5 | 25 | 889 | 8.3 | IMD4 |
|  | n78 | 3421 | 10 | 50 | 3421 | N/A | N/A |
| DC_7_n3 | 7 | 2535 | 10 | 50 | 2655 | 13 | IMD4 |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
| DC_7_n5 | 7 | 2547 | 10 | 50 | 2667 | N/A | N/A |
|  | n5 | 834 | 5 | 25 | 879 | 12 | IMD33 |
| DC_7A_n20A | 7 | 2512 | 10 | 50 | 2632 | N/A | N/A |
|  | n20 | 851 | 5 | 25 | 810 | 12 | IMD33 |
| DC_7A_n26A DC_7C_n26A | 7 | 2547 | 10 | 50 | 2667 | N/A | N/A |
|  | n26 | 834 | 5 | 25 | 879 | 12 | IMD33 |
|  | 7 | 2567.5 | 5 | 25 | 2687.5 | 2.5 | IMD5 |
|  | n26 | 816.5 | 5 | 25 | 861.5 | N/A | N/A |
| DC_7A_n40A DC_7A-7A_n40A | 7 | 2510 | 5 | 25 | 2630 | 23 | IMD3 |
|  | n40 | 2390 | 5 | 25 | 2390 | N/A | N/A |
| DC_7A_n66A DC_7A-7A_n66A DC_7C_n66A | 7 | 2535 | 10 | 50 | 2655 | 15 | IMD4 |
|  | n66 | 1730 | 5 | 25 | 2130 | N/A | N/A |
| DC_7A_n77A DC_7A-7A_n77(2A) DC_7A-7A_n77(3A) DC_7A_n77(2A) DC_7A_n77(3A) DC_7C_n77A DC_7C_n77(2A) | 7 | 2540 | 5 | 25 | 2660 | 7.1 | IMD4 |
|  | n77 | 3870 | 10 | 50 | 3870 | N/A | N/A |
| DC_7_n79 | 7 | 2510 | 5 | 25 | 2630 | [8] | IMD4 |
|  | n79 | 4900 | 40 | 216 | 4900 | N/A | N/A |
| DC_8A_n1A | 8 | 887.5 | 5 | 25 | 932.5 | N/A | N/A |
|  | n1 | 1965 | 5 | 25 | 2155 | 6 | IMD4 |
| DC_8A_n3A | 8 | 900 | 5 | 25 | 945 | 8 | IMD43 |
|  | n3 | 1755 | 10 | 50 | 1850 | N/A | N/A |
|  | 8 | 897.5 | 5 | 25 | 942.5 | N/A | N/A |
|  | n3 | 1747.5 | 10 | 50 | 1842.5 | 6.4 | IMD5 |
| DC_8A_n20A | n20 | 849.5 | 5 | 25 | 808.5 | 25 | IMD33 |
|  | 8 | 890.5 | 5 | 25 | 935.5 | N/A | N/A |
|  | n20 | 847.5 | 5 | 25 | 806.5 | N/A | N/A |
|  | 8 | 892.5 | 5 | 25 | 937.5 | 25 | IMD33 |
| DC_8A_n38A | 8 | 887.5 | 5 | 25 | 932.5 | 8.1 | IMD5 |
|  | n38 | 2617.5 | 5 | 25 | 2617.5 | N/A | N/A |
| DC_8A_n41A DC_8A_SUL_n41A-n81A | 8 | 882.5 | 5 | 25 | 927.5 | 12.1 | IMD33 |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | N/A |
| DC_8A_n77A, DC_8A_n78A, DC_8B_n78A DC_8A_n78(2A), DC_8A_n77(3A), DC_8A_SUL_n78A-n81A | 8 | 897.5 | 5 | 25 | 942.5 | 8.3 | IMD4 |
|  | n77, n78 | 3635 | 10 | 50 | 3635 | N/A | N/A |
| DC_8A_n79A, DC_8A_n79C, DC_8A_SUL_n79A-n81A | 8 | 897.5 | 5 | 25 | 942.5 | 4.8 | IMD5 |
|  | n79 | 4532.5 | 40 | 216 | 4532.5 | N/A | N/A |
| DC_11A_n28A | 11 | 1430.5 | 5 | 25 | 1478.5 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | 10.4 | IMD4 |
| DC_12A_n77A DC_12A_n77(2A) | 12 | 702 | 5 | 20 | 732 | 5.5 | IMD5 |
|  | n77 | 3540 | 10 | 50 | 3540 | N/A | N/A |
| DC_12A_n78A | 12 | 710 | 5 | 25 | 740 | 5.5 | IMD5 |
|  | n78 | 3580 | 10 | 50 | 3580 | N/A | N/A |
| DC_13A_n5A | 13 | 783 | 5 | 25 | 752 | N/A | N/A |
|  | n5 | 828 | 5 | 25 | 873 | 25 | IMD3 |
| DC_13A_n7A DC_13A_n7(2A) | 13 | 784.5 | 5 | 25 | 753.5 | N/A | N/A |
|  | n7 | 2520 | 40 | 216 | 2640 | 2.5 | IMD5 |
| DC_13A_n77A | 13 | 784.5 | 5 | 20 | 753.5 | 5.5 | IMD5 |
|  | n77 | 3891.5 | 10 | 50 | 3891.5 | N/A | N/A |
| DC_14A_n5A | 14 | 791 | 5 | 25 | 761 | N/A | N/A |
|  | n5 | 836 | 5 | 25 | 881 | 25 | IMD3 |
|  | 14 | 795.5 | 5 | 25 | 765.5 | 25 | IMD3 |
|  | n5 | 826.5 | 5 | 25 | 871.5 | N/A | N/A |
| DC_14A_n77A DC_14A_n77(2A) | 14 | 795.5 | 5 | 15 | 765.5 | 5.5 | IMD5 |
|  | n77 | 3947.5 | 10 | 50 | 3947.5 | N/A | N/A |
| DC_18A_n3A | 18 | 823 | 5 | 25 | 868 | N/A | N/A |
|  | n3 | 1721 | 5 | 25 | 1816 | 4 | IMD4 |
| DC_18A_n77A DC_18A_n78A | 18 | N/A | N/A | N/A | N/A | N/A | IMD4 |
|  | n77, n78 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_19A_n78A | 19 | N/A | N/A | N/A | N/A | N/A | IMD4 |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_20A_n3A | 20 | 840 | 5 | 25 | 799 | N/A | N/A |
|  | n3 | 1775 | 5 | 25 | 1870 | 4 | IMD4 |
|  | 20 | 847 | 5 | 25 | 806 | 9 | IMD4 |
|  | n3 | 1735 | 5 | 25 | 1830 | N/A | N/A |
| DC_20A_n38A | 20 | N/A | N/A | N/A | N/A | N/A | IMD5 |
|  | n38 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_20_n7 | 20 | 851 | 5 | 25 | 810 | 12 | IMD33 |
|  | n7 | 2512 | 10 | 50 | 2632 | N/A | N/A |
| DC_20A_n8A | 20 | 849.5 | 5 | 25 | 808.5 | 25 | IMD3 |
|  | n8 | 892.5 | 5 | 25 | 937.5 | 25 | IMD3 |
| DC_20_n41 | 20 | 851 | 5 | 25 | 810 | 12.1 | IMD3 |
|  | n41 | 2512 | 10 | 50 | 2512 | N/A | N/A |
| DC_20_n41 | 20 | 841 | 5 | 25 | 800 | 8.1 | IMD5 |
|  | n41 | 2564 | 10 | 50 | 2564 | N/A | N/A |
| DC_20A_n77A, DC_20A_n78A DC_20A_n78C7, DC_20A_n78(2A), DC_20A_SUL_n78A-n82A | 20 | 850 | 5 | 25 | 809 | 11 | IMD4 |
|  | n77, n78 | 3359 | 10 | 50 | 3359 | N/A | N/A |
| DC_20A_n77A | 20 | 840 | 5 | 25 | 799 | 6.5 | IMD5 |
|  | n77 | 4159 | 10 | 50 | 4159 | N/A | N/A |
| DC_21A_n28A7 | 21 | 1450.4 | 5 | 25 | 1498.4 | 2.5 | IMD5 |
|  | n28 | 735.5 | 5 | 25 | 790.5 | N/A | N/A |
| DC_21A_n79A | 21 | 1457.5 | 5 | 25 | 1505.5 | 18.4 | IMD3 |
|  | n79 | 4420.5 | 40 | 216 | 4420.5 | N/A | N/A |
| DC_25A_n77A DC_25A-25A_n77A | 25 | 1855 | 5 | 25 | 1935 | 26 | IMD2 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | N/A |
|  | 25 | 1900 | 5 | 25 | 1980 | 8 | IMD4 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |
|  | 25 | 1885 | 5 | 25 | 1965 | 5 | IMD5 |
|  | n77 | 3810 | 10 | 50 | 3810 | N/A | N/A |
| DC_25A_n78A DC_25A-25A_n78A | 25 | 1855 | 5 | 25 | 1935 | 26 | IMD2 |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | N/A |
|  | 25 | 1885 | 5 | 25 | 1965 | 8 | IMD4 |
|  | n78 | 3690 | 10 | 50 | 3690 | N/A | N/A |
|  | 25 | 1875 | 5 | 25 | 1955 | 5 | IMD5 |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | N/A |
| DC_26A_n41A | 26 | 839 | 5 | 25 | 884 | 15.6 | IMD33 |
|  | n41 | 2562 | 10 | 50 | 2562 | N/A | N/A |
| DC_28_n50 | 28 | 730 | 10 | 50 | 775 | 15.3 | IMD 2 |
|  | n50 | 1500 | 10 | 50 | 1500 | N/A | N/A |
|  | 28 | 740 | 10 | 50 | 785 | 6 | IMD 4 |
|  | n50 | 1500 | 10 | 50 | 1500 | N/A | N/A |
|  | 28 | 740 | 10 | 50 | 785 | 0.5 | IMD 5 |
|  | n50 | 1500 | 10 | 50 | 1500 | N/A | N/A |
| DC_28A_n51A | 28 | 742.3 | 5 | 25 | 797.3 | 5 | IMD4 |
|  | n51 | 1429.5 | 5 | 25 | 1429.5 | N/A | N/A |
| DC_26A_n77A, DC_26A_n78A, DC_26A_n78(2A) | 26 | 836.5 | 5 | 25 | 881.5 | 11.1 | IMD4 |
|  | n77, n78 | 3391 | 10 | 50 | 3391 | N/A | N/A |
| DC_28A_n77A, DC_28A_n78A, DC_28A_n78(2A), DC_28A_SUL_n78A-n83A | 28 | 705.5 | 5 | 25 | 760.5 | 5.5 | IMD5 |
|  | n77, n78 | 3582.5 | 10 | 50 | 3582.5 | N/A | N/A |
| DC_30A_n77A DC_30A_n77(2A) | 30 | 2310 | 5 | 25 | 2355 | 8.0 | IMD4 |
|  | n77 | 3487.5 | 10 | 50 | 3487.5 | N/A | N/A |
| DC_38A_n3A | n3 | 1713 | 5 | 25 | 1808 | 8.2 | IMD4 |
|  | 38 | 2617 | 5 | 25 | 2617 | N/A | N/A |
| DC_38A_n8A | 38 | 2617.5 | 5 | 25 | 2617.5 | N/A | N/A |
|  | n8 | 887.5 | 5 | 25 | 932.5 | 8.1 | IMD5 |
| DC_41A_n3A DC_41C_n3A | n3 | 1740 | 5 | 25 | 1835 | 8.2 | IMD4 |
|  | 41 | 2657.5 | 5 | 25 | 2657.5 | N/A | N/A |
| DC_42_n3 | 42 | 3575 | 10 | 50 | 3575 | N/A | N/A |
|  | n3 | 1740 | 5 | 25 | 1835 | 26 | 2nd3 |
|  | 42 | 3435 | 10 | 50 | 3435 | N/A | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | 8.0 | IMD4 |
| DC_42_n28 | 42 | 3582.5 | 10 | 50 | 3582.5 | N/A | N/A |
|  | n28 | 705.5 | 5 | 25 | 760.5 | 5.5 | IMD5 |
| DC_48A_n2A DC_48C_n2A DC_48D_n2A DC_48E_n2A | 48 | 3625 | 20 | 100 | 3625 | N/A | N/A |
|  | n2 | 1852.5 | 5 | 25 | 1932.5 | 12 | IMD4 |
| DC_48A_n12A | 48 | 3557.5 | 10 | 50 | 3557.5 | N/A | N/A |
|  | n12 | 705.5 | 5 | 25 | 735.5 | 5.5 | IMD5 |
| DC_48A_n25A DC_48C_n25A DC_48D_n25A | 48 | 3625 | 20 | 100 | 3625 | N/A | N/A |
|  | n25 | 1852.5 | 5 | 25 | 1932.5 | 12 | IMD4 |
| DC_48A_n66A DC_48C_n66A DC_48D_n66A | 48 | 3630 | 20 | 100 | 3630 | N/A | N/A |
|  | n66 | 1715 | 5 | 25 | 2115 | 4 | IMD5 |
| DC_66A_n2A, DC_66A_n2(2A) DC_66A-66A_n2A | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | n2 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |
|  | 66 | 1750 | 5 | 25 | 2150 | 4 | IMD5 |
|  | n2 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
| DC_66A_n5A | n5 | 838 | 5 | 25 | 883 | 30 | IMD23 |
|  | 66 | 1721 | 5 | 25 | 2121 | N/A | N/A |
| DC_66A_n7A DC_66A-66A_n7A DC_66A_n7(2A) DC_66A-66A_n7(2A) | 66 | 1730 | 5 | 25 | 2130 | N/A | N/A |
|  | n7 | 2535 | 10 | 50 | 2655 | 15 | IMD4 |
| DC_66A_n25A | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | n25 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |
|  | 66 | 1712.5 | 5 | 25 | 2112.5 | 23 | IMD3 |
|  | n25 | 1912.5 | 5 | 25 | 1992.5 | N/A | N/A |
|  | 66 | 1750 | 5 | 25 | 2150 | 4 | IMD5 |
|  | n25 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
| DC_66A_n46A | 66 | 1735 | 5 | 25 | 2135 | 12.0 | IMD3 |
|  | n46 | 5605 | 20 | 100 | 5605 | N/A | N/A |
| DC_66A_n48A | 66 | 1715 | 5 | 25 | 2115 | 4 | IMD5 |
|  | n48 | 3630 | 20 | 100 | 3630 | N/A | N/A |
| DC_66A_n71A | 66 | 1750 | 5 | 25 | 2150 | 5 | IMD4 |
|  | n71 | 675 | 5 | 25 | 629 | N/A | N/A |
| DC_66A_n77A DC_66A_n77(2A) DC_66A-66A_n77A DC_66A-66A_n77(2A) DC_66A-66A-66A_n77A DC_66A-66A-66A_n77(2A) | 66 | 1775 | 5 | 25 | 2175 | 31.0 | IMD2 |
|  | n77 | 3950 | 10 | 50 | 3950 | N/A | N/A |
|  | 66 | 1760 | 5 | 25 | 2160 | 5.0 | IMD5 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |
| DC_66A_n78A | 66 | 1730 | 5 | 25 | 2150 | 5.0 | IMD5 |
|  | n78 | 3660 | 10 | 50 | 3660 | N/A | N/A |
| DC_71A_n38A | 71 | 665 | 5 | 25 | 619 | 11 | IMD4 |
|  | n38 | 2614 | 10 | 50 | 2614 | N/A | N/A |
| DC_71A_n41A | 71 | 666 | 5 | 25 | 620 | 11 | IMD4 |
|  | n41 | 2618 | 5 | 25 | 2618 | N/A | N/A |
| DC_71A_n66A | 71 | 675 | 5 | 25 | 629 | N/A | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | 5 | IMD4 |
| DC_71A_n77A8 DC_71A_n77(2A)8 | 71 | 671 | 5 | 25 | 625 | 5.5 | IMD5 |
|  | n77 | 3309 | 10 | 50 | 3309 | N/A | N/A |
| DC_71A_n78A | 71 | 681.5 | 5 | 25 | 635.5 | 5.5 | IMD5 |
| DC_71A_n78(2A) | n78 | 3361.5 | 10 | 50 | 3361.5 | N/A | N/A |
| NOTE 1: E-UTRA carrier shall be set to min(+20 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+20 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. NOTE 2: RBstart = 0 NOTE 3: This band is subject to IMD5 also which MSD is not specified. NOTE 4: Void NOTE 5: Void NOTE 6: For NR band, UL/DL BW and UL LCRB can be adjusted according to the supported BW and lowest SCS supported by the UE. NOTE 7: The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL and 783 - 793 MHz for the DL. This band is subject to IMD2, IMD4 and IMD5 fall in n28 also which MSD is not specified. In addition, this band is subject to IMD4 fall in B21 also which MSD is not specified. NOTE 8: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |

Table 7.3B.2.3.5.1-1a: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A_n77A | 1 | 1950 | 5 | 25 | 2140 | 35.8 | IMD21 |
|  | n77 | 4090 | 10 | 50 | 4090 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 17.8 | IMD41 |
|  | n77 | 3710 | 10 | 50 | 3710 | N/A | N/A |
| DC_3A_n41A | 3 | 1740 | 5 | 25 | 1835 | 18.4 | IMD4 |
|  | n41 | 2657.5 | 10 | 50 | 2657.5 | N/A | N/A |
| DC_3A_n78A | 3 | 1740 | 5 | 25 | 1835 | 31.9 | IMD2 |
| DC_3A-3A_n78A | n78 | 3575 | 10 | 50 | 3575 | N/A | N/A |
| DC_3A_n78A | 3 | 1765 | 5 | 25 | 1860 | 18.5 | IMD4 |
| DC_3A-3A_n78A | n78 | 3435 | 10 | 50 | 3435 | N/A | N/A |
| DC_1A_n78A | 1 | 1950 | 5 | 25 | 2140 | 17.8 | IMD4 |
|  | n78 | 3710 | 10 | 50 | 3710 | N/A | N/A |
| DC_8A_n78A | 8 | 897.5 | 5 | 25 | 942.5 | 15.5 | IMD4 |
|  | n78 | 3635 | 10 | 50 | 3635 | N/A | N/A |
| DC_2A_n77A DC_2A-2A_n77A DC_2A_n77C DC_2A-2A_n77C DC_2A_n77(2A) DC_2A-2A_n77(2A) | 2 | 1855 | 5 | 25 | 1935 | 32.10 | IMD2 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | N/A |
|  | 2 | 1900 | 5 | 25 | 1980 | 19.10 | IMD41 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |
| DC_3A_n77A | 3 | 1740 | 5 | 25 | 1835 | 31.9 | IMD21 |
|  | n77 | 3575 | 10 | 50 | 3575 | N/A | N/A |
|  | 3 | 1765 | 5 | 25 | 1860 | 18.5 | IMD41 |
|  | n77 | 3435 | 10 | 50 | 3435 | N/A | N/A |
| DC_5A_n77A3 DC_5A_n77C3 DC_5A_n77(2A)3 | 5 | 844 | 5 | 25 | 889 | 18.60 | IMD41 |
|  | n77 | 3421 | 10 | 50 | 3421 | N/A | N/A |
| DC_13A_n77A DC_13A_n77C | 13 | 782 | 5 | 20 | 751 | 15.37 | IMD5 |
|  | n77 | 3879 | 10 | 50 | 3879 | N/A | N/A |
| DC_66A_n77A DC_66A-66A_n77A DC_66A-66A-66A_n77A DC_66A_n77C DC_66A-66A_n77C DC_66A-66A-66A_n77C DC_66A_n77(2A) DC_66A-66A_n77(2A) DC_66A-66A-66A_n77(2A) | 66 | 1775 | 5 | 25 | 2175 | 34.33 | IMD2 |
|  | n77 | 3950 | 10 | 50 | 3950 | N/A | N/A |
|  | 66 | 1760 | 5 | 25 | 2160 | 11.27 | IMD5 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |
| DC_5A_n78A | 5 | 844 | 5 | 25 | 889 | 17.5 | IMD4 |
|  | n78 | 3421 | 10 | 52 | 3421 | N/A | N/A |
| DC_12A_n77A DC_12A_n77(2A) | 12 | 702 | 5 | 20 | 732 | 11.7 | IMD5 |
|  | n77 | 3540 | 10 | 50 | 3540 | N/A | N/A |
| DC_14A_n77A DC_14A_n77(2A) | 14 | 795.5 | 5 | 15 | 765.5 | 11.7 | IMD5 |
|  | n77 | 3947.5 | 10 | 50 | 3947.5 | N/A | N/A |
| DC_30A_n77A DC_30A_n77(2A) | 30 | 2310 | 5 | 25 | 2355 | 17.6 | IMD4 |
|  | n77 | 3487.5 | 10 | 50 | 3487.5 | N/A | N/A |
| DC_28A_n78A | 28 | 705.5 | 5 | 25 | 760.5 | 11.7 | IMD5 |
|  | n78 | 3582.5 | 10 | 50 | 3582.5 | N/A | N/A |
| DC_21A_n79A | 21 | 1457.5 | 5 | 25 | 1505.5 | 33.4 | IMD3 |
|  | n79 | 4420.5 | 10 | 50 | 4420.5 | N/A | N/A |
| DC_71A_n77A3 | 71 | 681.5 | 5 | 25 | 635.5 | 11.4 | IMD5 |
|  | n77 | 3361.5 | 10 | 50 | 3361.5 | N/A | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: Void NOTE 3: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 4: E-UTRA carrier shall be set to min(+23 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+23 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. |  |  |  |  |  |  |  |

<< Next Changes >>

7.3B.3.3	Inter-band EN-DC within FR1

7.3B.3.3.1	ΔRIB,c for EN-DC in two bands

Table 7.3B.3.3.1-1: ΔRIB,c due to EN-DC(two bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)6 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |
| DC_1_n28 | - | 0.2 |
| DC_1_n51 | - | 0.1 |
| DC_1_n77 | 0.2 | 0.5 |
| DC_1_n78 | - | 0.5 |
| DC_1_n105 | 0.3 | 0.6 |
| DC_2_n30 DC_2-2_n30 | 0.4 | 0.5 |
| DC_2_n48 | 0.2 | 0.5 |
| DC_2_n66 DC_2-2_n66 | 0.3 | 0.3 |
| DC_2_n77 DC_2-2_n77 | 0.2 | 0.5 |
| DC_2_n78 DC_2-2_n78 | 0.2 | 0.5 |
| DC_3_n41 | - | 03 / 0.54 |
| DC_3_n51 | 0.2 | 0.2 |
| DC_3_n77 DC_3-3_n77 | 0.2 | 0.5 |
| DC_3_n78 DC_3-3_n78 | 0.2 | 0.5 |
| DC_3_n105 | - | 0.3 |
| DC_4_n2 | 0.3 | 0.3 |
| DC_4_n28 | - | 0.2 |
| DC_4_n38 | 0.5 | 0.5 |
| DC_4_n41 | 0.5 | 0.51 / 12 |
| DC_4_n78 | 0.2 | 0.5 |
| DC_5_n3 | 0.2 | 0.2 |
| DC_5_n12 | 0.5 | 0.3 |
| DC_5_n41 | 0.2 | - |
| DC_5_n77 | 0.2 | 0.5 |
| DC_5_n78 | 0.2 | 0.5 |
| DC_7_n8 DC_7-7_n8 | - | 0.2 |
| DC_7_n40 DC_7-7_n40 | - | 0.5 |
| DC_7_n51 | - | 0.2 |
| DC_7_n66 DC_7-7_n66 | 0.5 | 0.5 |
| DC_7_n71 | - | 0.2 |
| DC_7_n77 DC_7-7_n77 | - | 0.5 |
| DC_7_n78 DC_7-7_n78 | - | 0.5 |
| DC_7_n79 | - | 0.5 |
| DC_7_n105 | - | 0.2 |
| DC_8_n7 | - | 0.2 |
| DC_8_n28 | 0.2 | 0.1 |
| DC_8_n77 | 0.2 | 0.5 |
| DC_8_n78 | 0.2 | 0.5 |
| DC_11_n3 | 0.3 | 0.5 |
| DC_11_n28 | - | 0.2 |
| DC_11_n77 | - | 0.5 |
| DC_11_n78 | - | 0.5 |
| DC_12_n5 | 0.3 | 0.5 |
| DC_12_n66 | 0.5 | - |
| DC_12_n71 | 0.8 | 0.8 |
| DC_12_n77 | 0.2 | 0.5 |
| DC_12_n78 | 0.2 | 0.5 |
| DC_13_n7 | 0.5 | 0.5 |
| DC_13_n77 | 0.2 | 0.5 |
| DC_13_n78 | 0.2 | 0.5 |
| DC_14_n77 | 0.2 | 0.5 |
| DC_18_n41 | - | 03 |
| DC_18_n77 | - | 0.5 |
| DC_19_n77 | - | 0.5 |
| DC_19_n78 | - | 0.5 |
| DC_20_n38 | 0.2 | - |
| DC_20_n51 | - | 0.2 |
| DC_20_n77 | - | 0.5 |
| DC_20_n78 | - | 0.5 |
| DC_21_n77 | - | 0.5 |
| DC_21_n78 | - | 0.5 |
| DC_25_n41 DC_25-25_n41 | - | 01 / 0.52 |
| DC_25_n77 DC_25-25_n77 DC_25-25_n77 | 0.2 | 0.5 |
| DC_25_n78 DC_25-25_n78 | 0.2 | 0.5 |
| DC_26_n77 | - | 0.5 |
| DC_26_n78 | - | 0.5 |
| DC_28_n1 | 0.2 | - |
| DC_28_n8 | 0.1 | 0.2 |
| DC_28_n51 | - | 0.2 |
| DC_28_n66 | 0.2 | - |
| DC_28_n77 | 0.2 | 0.5 |
| DC_28_n78 | 0.2 | 0.5 |
| DC_30_n66 | 0.5 | 0.4 |
| DC_30_n77 | - | 0.5 |
| DC_38_n78 | 0.4 | 0.5 |
| DC_38_n79 | - | 0.5 |
| DC_39_n40 | 0.3 | 0.3 |
| DC_39_n41 | 0.2 | 0.2 |
| DC_39_n78 | - | 0.5 |
| DC_39_n79 | - | 0.5 |
| DC_40_n77 | 0.4 | 0.5 |
| DC_40_n78 | 0.45 | 0.55 |
| DC_40_n79 | - | 0.5 |
| DC_41_n3 | 03 / 0.54 | - |
| DC_41_n77 | - | 0.5 |
| DC_41_n78 | - | 0.5 |
| DC_41_n79 | - | 0.5 |
| DC_42_n1 | 0.5 | - |
| DC_42_n3 | 0.5 | 0.2 |
| DC_42_n28 | 0.2 | 0.5 |
| DC_42_n51 | - | 0.2 |
| DC_48_n2 | 0.5 | 0.2 |
| DC_48_n25 | 0.5 | 0.2 |
| DC_48_n46 | 0.5 | - |
| DC_48_n66 | 0.5 | 0.2 |
| DC_66_n2 DC_66-66_n2 DC_66-66-66_n2 | 0.3 | 0.3 |
| DC_66_n7 DC_66-66_n7 | 0.5 | 0.5 |
| DC_66_n12 | 0.5 | - |
| DC_66_n25 | 0.3 | 0.3 |
| DC_66_n28 | - | 0.2 |
| DC_66_n30 DC_66-66_n30 | 0.5 | 0.4 |
| DC_66_n38 DC_66-66_n38 | 0.5 | 0.5 |
| DC_66_n41 | 0.5 | 0.51 / 12 |
| DC_66_n48 DC_66-66_n48 | 0.2 | 0.5 |
| DC_66_n77 DC_66-66_n77 DC_66-66-66_n77 | 0.2 | 0.5 |
| DC_66_n78 DC_66-66_n78 | 0.2 | 0.5 |
| DC_71_n7 | 0.2 | - |
| DC_71_n12 | 0.8 | 0.8 |
| DC_71_n38 | 0.2 | - |
| DC_71_n41 | 0.2 | - |
| DC_71_n77 | 0.2 | 0.5 |
| DC_71_n78 | 0.2 | 0.5 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 – 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 – 2545 MHz. NOTE 3: Applicable for the frequency range of 2515 – 2690 MHz. NOTE 4: Applicable for the frequency range of 2496 – 2515 MHz. NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. NOTE 6: “-” denotes ΔRIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_1_n77 the band order from left to right is 1 and n77. |  |  |

<< End of changes >>