3GPP TSG-RAN4 Meeting #107	R4-2309376

# Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0384 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on FRC for PDSCH CA requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz-Perf |  |  |  |  |  | Date: |  |  | 2023-05-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The FRC for PDSCH CA requirements are not the same as single carrier reqt – - For FR1+FR2-2 with 120kHz SCS, PDSCH is scheduled in slot 0,1,2,3,5,6,7,8 in every 40 slots of FR2-2 considering 8 HARQ processes is assumed. HARQ-ACK is transmitted in slot 8 every 10 slots of FR1. - For FR1+FR2-2 with 480kHz SCS, PDSCH is scheduled in slot 0~14 and slot 20 in every 160 slots of FR2-2 considering 16 HARQ processes is assumed. HARQ-ACK is transmitted in slot 8 every 10 slots of FR1. The FRC for PDCCH for 480KHz SCS is in the wrong section |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added FRCs for PDSCH CA requirements and updated the requirements table with correct FRC. Moved PDCCH FRC for 480KHz to correct section |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PDSCH CA requirements definition will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2A.1.1, A.3.2.2.5, A.3.2.2.8, A.3.3.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------------------------------Start of Change 1 --------------------------------------------------------

#### 9.2A.1.1	NR CA between FR1 and FR2-2

The performance requirements for SCell on FR2-2 band are specified in Table 9.2A.1-5. The test parameters for SCell are specified in Table 7.2.2.2.1-2 with additional change of PDSCH resource allocation type specified in Table 9.2A.1-1:

Table 9.2A.1-1: PDSCH resource allocation type

| Test numer | Allocation Type | Start RB | LRBs |
| --- | --- | --- | --- |
| 1-1 | Type 1 | 98 | 66 |
| 1-2 |  |  |  |
| 1-3 |  |  |  |
| 1-4 |  |  |  |
| 1-5 |  | 116 | 32 |
| 1-7 | ` | 24 | 20 |
| 1-6Note1 | Type 0 | N/A | N/A |
| Note 1: Full BWP is allocated for PDSCH |  |  |  |

The test parameters for PCell in Table 9.2A.1-3 and the downlink physical channel setup according to Annex C.3.1. In this test, Pcell is in FR1 and Scell is in FR2-2 and only requirements for Scell should be verified.

The test purposes are specified in Table 9.2A.1-2.

Table 9.2A.1-2: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH mapping Type A normal performance in FR2-2 Scell CC in CA between FR1 and FR2-2 under 2 receive antenna conditions and with different channel models, MCSs and number of MIMO layers | 1-1,1-2,1-3,1-4,1-5,1-6 |

Table 9.2A.1-3: Test parameters for PCell

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Bandwidth |  | MHz | 40 |
| Subcarrier spacing |  | kHz | 30 |
| TDD pattern |  |  | FR1.30-1 |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Dmrs-AdditionalPosition |  | pos1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |
| PUCCH format for HARQ-ACK feedback |  |  | PUCCH format 3 |

Table 9.2A.1-4: K1 values for each CC

| Cells | K1 values |
| --- | --- |
| Pcell CC | {8,7,6,5,4,3,2} |
| SCell CC with 120kHz | {8,8,8,8,7,7,7,7} |
| SCell CC with 480kHz (Note 1) | {8,8,8,7} |
| Note 1: One k1 value applies for 4 TBs scheduled by one DCI |  |

Table 9.2A.1-5: Minimum performance for Rank 1 (FRC) in Scell CC

| Test num | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of max through-put (%) | SNRBB (dB) |
| 1-1 | R.PDSCH.5-1.3 TDD | 400 / 120 | QPSK, 0.30 | FR2.120-1 | TDLA30-650 | 2x2 ULA Low | 70 | [0.9] |
| 1-2 | R.PDSCH.5-2.4 TDD | 400 / 120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-200 | 2x2 ULA Low | 70 | [9.0] |
| 1-3 | R.PDSCH.5-2.4 TDD | 400 / 120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-650 | 2x2 ULA Low | 30 | [2.7] |
| 1-4 | R.PDSCH.5-3.3 TDD | 400 / 120 | 64QAM, 0.43 | FR2.120-1 | TDLD30-200 | 2x2 ULA Low | 70 | [11.6] |
| 1-5 | R.PDSCH.8-1.2 TDD | 400 / 480 | QPSK, 0.30 | FR2.480-1 | TDLA10-200 | 2x2 ULA Low | 70 | [1.0] |
| 1-6 | R.PDSCH.8-2.2 TDD | 400 / 480 | 16QAM 0.48 | FR2.480-1 | TDLD10-200 | 2x2 ULA Low | 70 | [8.3] |

--------------------------------------End of Change 1---------------------------------------------------------

--------------------------------------Start of Change 2 --------------------------------------------------------

#### A.3.2.2.5	Reference measurement channels for SCS 120 kHz FR2

Table A.3.2.2.5-1: PDSCH Reference Channel for TDD UL-DL pattern FR2.120-1 and FR2.120-1A (QPSK)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.5-1.1 TDD | R.PDSCH.5-1.2 TDD | R.PDSCH.5-1.3 TDD |  |  |
| Channel bandwidth | MHz | 100 | 100 | 100 |  |  |
| Subcarrier spacing | kHz | 120 | 120 | 120 |  |  |
| Allocated resource blocks | PRBs | 66 | 66 | 66 |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,8, 40,…,48, 80,…,88, 120,…,128} |  | 9 | 2 | 9 |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {9,…,39,49,…,79,89,…,119,129,…159} |  | 9 | 2 | N/A |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…, 8, 40,…,48, 80,…,88, 120,…,128} |  | 13 | 2 | 13 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {9,…,39,49,…,79,89,…,119, 129,…159} |  | 13 | 2 | N/A |  |  |
| Allocated slots per 2 frames |  | 127 | 127 | 31 |  |  |
| MCS table |  | 64QAM | 64QAM | 64QAM |  |  |
| MCS index |  | 4 | 4 | 4 |  |  |
| Modulation |  | QPSK | QPSK | QPSK |  |  |
| Target Coding Rate |  | 0.30 | 0.30 | 0.30 |  |  |
| Number of MIMO layers |  | 1 | 1 | 1 |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 8, 40,…,48, 80,…,88, 120,…,128} |  | 12 | 6 | 12 |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {9,…,39,49,…,79,89,…,119,129,…159} |  | 12 | 6 | N/A |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…, 8, 40,…,48, 80,…,88, 120,…,128} |  | 12 | 6 | 12 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {9,…,39,49,…,79,89,…,119, 129,…159} |  | 12 | 6 | N/A |  |  |
| Overhead for TBS determination |  | 6 | 0 | 6 |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,8, 40,…,48, 80,…,88, 120,…,128} | Bits | 3624 | 736 | 3624 |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {9,…,39,49,…,79,89,…,119,129,…159} | Bits | 3624 | 736 | N/A |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,8, 40,…,48, 80,…,88, 120,…,128} | Bits | 5504 | 736 | 5504 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {9,…,39,49,…,79,89,…,119, 129,…159} | Bits | 5504 | 736 | N/A |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,8, 40,…,48, 80,…,88, 120,…,128} | Bits | 16 | 16 | 16 |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {9,…,39,49,…,79,89,…,119,129,…159} | Bits | 16 | 16 | N/A |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…, 8, 40,…,48, 80,…,88, 120,…,128} | Bits | 24 | 16 | 24 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {9,…,39,49,…,79,89,…,119, 129,…159} | Bits | 24 | 16 | N/A |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | CBs | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,8, 40,…,48, 80,…,88, 120,…,128} | CBs | 1 | 1 | 1 |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {9,…,39,49,…,79,89,…,119,129,…159} | CBs | 1 | 1 | N/A |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…, 8, 40,…,48, 80,…,88, 120,…,128} | CBs | 1 | 1 | 1 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {9,…,39,49,…,79,89,…,119, 129,…159} | CBs | 1 | 1 | N/A |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A |  |  |
| For Slots i = 80, 81 | Bits | 17490 | 2310 | 17490 |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,8, 40,…,48, 80,…,88, 120,…,128} | Bits | 12210 | 2310 | 12210 |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {9,…,39,49,…,79,89,…,119,129,…159} | Bits | 12210 | 2310 | N/A |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,8, 40,…,48, 82,…,88, 120,…,128} | Bits | 18282 | 2310 | 18282 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {9,…,39,49,…,79,89,…,119, 129,…159} | Bits | 18282 | 2310 | N/A |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 31.942 | 4.673 | 7.7792 |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

Table A.3.2.2.5-2: PDSCH Reference Channel for TDD UL-DL pattern FR2.120-1 (16QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.5-2.1 TDD | R.PDSCH.5-2.2 TDD | R.PDSCH.5-2.3 TDD | R.PDSCH.5-2.4 TDD |  |
| Channel bandwidth | MHz | 100 | 100 | 200 | 100 |  |
| Subcarrier spacing | kHz | 120 | 120 | 120 | 120 |  |
| Allocated resource blocks | PRBs | 66 | 66 | 132 | 66 |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,8, 40,…,48, 80,…,88, 120,…,128} |  | 9 | 9 | 9 | 9 |  |
| For Slot i, if mod(i, 5) = 3 for i from {9,…,39,49,…,79,89,…,119,129,…159} |  | 9 | 9 | 9 | N/A |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,8, 40,…,48, 80,…,88, 120,…,128} |  | 13 | 13 | 13 | 13 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {9,…,39,49,…,79,89,…,119, 129,…159} |  | 13 | 13 | 13 | N/A |  |
| Allocated slots per 2 frames |  |  | 127 |  | 127 |  | 127 |  | 31 |  |  |  |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM |  |
| MCS index |  | 13 | 13 | 13 | 13 |  |
| Modulation |  | 16QAM | 16QAM | 16QAM | 16QAM |  |
| Target Coding Rate |  | 0.48 | 0.48 | 0.48 | 0.48 |  |
| Number of MIMO layers |  | 1 | 2 | 2 | 1 |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 5) = {0,1,2,3} for i from {1,…, 8, 40,…,48, 80,…,88, 120,…,128} |  | 12 | 12 | 12 | 12 |  |
| For Slot i, if mod(i, 5) = {0,1,2,3} for i from {9,…,39,49,…,79,89,…,119, 129,…159} |  | 12 | 12 | 12 | N/A |  |
| Overhead for TBS determination |  | 6 | 6 | 6 | 6 |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,8, 40,…,48, 80,…,88, 120,…,128} | Bits | 11272 | 22536 | 45096 | 11272 |  |
| For Slot i, if mod(i, 5) = 3 for i from {9,…,39,49,…,79,89,…,119, 129,…159} | Bits | 11272 | 22536 | 45096 | N/A |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,8, 40,…,48, 80,…,88, 120,…,128} | Bits | 17424 | 34816 | 69672 | 17424 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {9,…,39,49,…,79,89,…,119, 129,…159} | Bits | 17424 | 34816 | 69672 | N/A |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,8, 40,…,48, 80,…,88, 120,…,128} | Bits | 24 | 24 | 24 | 24 |  |
| For Slot i, if mod(i, 5) = 3 for i from {9,…,39,49,…,79,89,…,119, 129,…159} | Bits | 24 | 24 | 24 | N/A |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,8, 40,…,48, 80,…,88, 120,…,128} | Bits | 24 | 24 | 24 | 24 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {9,…,39,49,…,79,89,…,119, 129,…159} | Bits | 24 | 24 | 24 | N/A |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | CBs | N/A | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,8, 40,…,48, 80,…,88, 120,…,128} | CBs | 2 | 3 | 6 | 2 |  |
| For Slot i, if mod(i, 5) = 3 for i from {9,…,39,49,…,79,89,…,119, 129,…159} | CBs | 2 | 3 | 6 | N/A |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,8, 40,…,48, 80,…,88, 120,…,128} | CBs | 3 | 5 | 9 | 3 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {9,…,39,49,…,79,89,…,119, 129,…159} | CBs | 3 | 5 | 9 | N/A |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A | N/A |  |
| For Slots i = 80, 81 | Bits | 36564 | 69960 | 139920 | 36564 |  |
| For Slots i = 82 | Bits | 34980 | 73128 | 146256 | 34980 |  |
| For Slots i = 83 | Bits | 22308 | 48840 | 97680 | 22308 |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 8, 40,…,48, 80,…,88, 120,…,128} | Bits | 24420 | 48840 | 97680 | 24420 |  |
| For Slot i, if mod(i, 5) = 3 for i from {9,…,39,49,…,79,89,…,119,129,…159} | Bits | 24420 | 48840 | 97680 | N/A |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…, 8, 40,…,48, 84,…,88, 120,…,128} | Bits | 36564 | 73128 | 146256 | 36564 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {9,…,39,49,…,79,89,…,119, 129,…159} | Bits | 36564 | 73128 | 146256 | N/A |  |
| Max. Throughput averaged over 2 frames | Mbps | 100.799 | 201.434 | 403.096 | 24.5464 |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

Table A.3.2.2.5-3: PDSCH Reference Channel for TDD UL-DL pattern FR2.120-1 (64QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.5-3.1 TDD | R.PDSCH.5-3.2 TDD | R.PDSCH.5-3.3 TDD |  |  |
| Channel bandwidth | MHz | 100 | 100 | 100 |  |  |
| Subcarrier spacing | kHz | 120 | 120 | 120 |  |  |
| Allocated resource blocks | PRBs | 66 | 66 | 66 |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A |  |  |
| For Slots i = 80, 81 |  | 9 | N/A | N/A |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,8, 40,…,48, 80,…,88, 120,…,128} |  | 9 | 9 | 9 |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {9,…,39,49,…,79,89,…,119, 129,…159} |  | 9 | 9 | N/A |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…, 8, 40,…,48, 82,…,88, 120,…,128} |  | 13 | 13 | 13 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {9,…,39,49,…,79,89,…,119, 129,…159} |  | 13 | 13 | N/A |  |  |
| Allocated slots per 2 frames |  |  |  | 127 |  | 125 |  | 29 |  |  |  |  |
| MCS table |  | 64QAM | 64QAM | 64QAM |  |  |
| MCS index |  | 18 | 17 | 17 |  |  |
| Modulation |  | 64QAM | 64QAM | 64QAM |  |  |
| Target Coding Rate |  | 0.46 | 0.43 | 0.43 |  |  |
| Number of MIMO layers |  | 1 | 1 | 1 |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A |  |  |
| For Slots i = 80, 81 |  | 12 | N/A | N/A |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,8, 40,…,48, 80,…,88, 120,…,128} |  | 12 | 12 | 12 |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {9,…,39,49,…,79,89,…,119, 129,…159} |  | 12 | 12 | N/A |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…, 8, 40,…,48, 82,…,88, 120,…,128} |  | 12 | 12 | 12 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {9,…,39,49,…,79,89,…,119, 129,…159} |  | 12 | 12 | N/A |  |  |
| Overhead for TBS determination |  | 6 | 6 | 6 |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | Bits |  | N/A |  | N/A |  | N/A |  |  |  |  |
| For Slots i = 80, 81 | Bits | 25104 | N/A | N/A |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,8, 40,…,48, 80,…,88, 120,…,128} | Bits | 16136 | 15112 | 15112 |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {9,…,39,49,…,79,89,…,119, 129,…159} | Bits | 16136 | 15112 | N/A |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…, 8, 40,…,48, 82,…,88, 120,…,128} | Bits | 25104 | 23568 | 23568 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {9,…,39,49,…,79,89,…,119, 129,…159} | Bits | 25104 | 23568 | N/A |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | Bits |  | N/A |  | N/A |  | N/A |  |  |  |  |
| For Slots i = 80, 81 | Bits | 24 | N/A | N/A |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 8, 40,…,48, 80,…,88, 120,…,128} | Bits | 24 | 24 | 24 |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {9,…,39,49,…,79,89,…,119, 129,…159} | Bits | 24 | 24 | N/A |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…, 8, 40,…,48, 82,…,88, 120,…,128} | Bits | 24 | 24 | 24 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {9,…,39,49,…,79,89,…,119, 129,…159} | Bits | 24 | 24 | N/A |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | CBs | N/A | N/A | N/A |  |  |
| For Slots i = 80, 81 | CBs | 3 | N/A | N/A |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 8, 40,…,48, 80,…,88, 120,…,128} | CBs | 2 | 2 | 2 |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {9,…,39,49,…,79,89,…,119, 129,…159} | CBs | 2 | 2 | N/A |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…, 8, 40,…,48, 82,…,88, 120,…,128} | CBs | 3 | 3 | 3 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {9,…,39,49,…,79,89,…,119, 129,…159} | CBs | 3 | 3 | N/A |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | Bits |  | N/A |  | N/A |  | N/A |  |  |  |  |
| For Slots i = 80, 81 | Bits | 52470 | N/A | N/A |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 8, 40,…,48, 80,…,88, 120,…,128} | Bits | 36630 | 35640 | 35640 |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {9,…,39,49,…,79,89,…,119, 129,…159} | Bits | 36630 | 35640 | N/A |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…, 8, 40,…,48, 82,…,88, 120,…,128} | Bits | 54846 | 54648 | 54648 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {9,…,39,49,…,79,89,…,119, 129,…159} | Bits | 54846 | 54648 | N/A |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 145.062 | 136.1272 | 30.7912 |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

--------------------------------------End of Change 2 ---------------------------------------------------------

--------------------------------------Start of Change 3 --------------------------------------------------------

#### A.3.2.2.8	Reference measurement channels for SCS 480 kHz FR2-2

Table A.3.2.2.8-1: PDSCH Reference Channel for TDD UL-DL pattern FR2.480-1 (QPSK)

| Parameter |  | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  |  | R.PDSCH.8-1.1 TDD | R.PDSCH.8-1.2 TDD |  |  |  |
| Channel bandwidth |  |  | MHz |  | 400 |  | 400 |  |  |  |  |  |  |
| Subcarrier spacing |  |  | kHz |  | 480 |  | 480 |  |  |  |  |  |  |
| Allocated resource blocks |  |  | PRBs |  | 66 |  | 66 |  |  |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 20) = {15,16,17,18,19} for i from {0,…,639} |  |  |  |  | N/A |  | N/A |  |  |  |  |  |  |
| For Slots i = 320, 321 |  |  | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 20) = 14 for i from {0,…, 639} |  |  |  |  | 11 |  | 11 |  |  |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1…13} for i from {1,…,639} |  |  |  |  | 13 |  | 13 |  |  |  |  |  |  |
| Allocated slots per 2 frames |  |  |  |  | 477 |  | 477 |  |  |  |  |  |  |
| MCS table |  |  |  |  | 64QAM |  | 64QAM |  |  |  |  |  |  |
| MCS index |  |  |  |  | 4 |  | 4 |  |  |  |  |  |  |
| Modulation |  |  |  |  | QPSK |  | QPSK |  |  |  |  |  |  |
| Target Coding Rate |  |  |  |  | 0.30 |  | 0.30 |  |  |  |  |  |  |
| Number of MIMO layers |  |  |  |  | 1 |  | 1 |  |  |  |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 20) = {15,16,17,18,19} for i from {0,…,639} |  |  |  |  | N/A |  | N/A |  |  |  |  |  |  |
| For Slots i = 320, 321 |  |  | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 20) = 14 for i from {0,…, 639} |  |  |  |  | 12 |  | 12 |  |  |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1…13} for i from {1,…,639} |  |  |  |  | 12 |  | 12 |  |  |  |  |  |  |
| Overhead for TBS determination |  |  |  |  | 6 |  | 6 |  |  |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 20) = {15,16,17,18,19} for i from {0,…,639} |  |  | Bits |  | N/A |  | N/A |  |  |  |  |  |  |
| For Slots i = 320, 321 | Bits |  | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 20) = 14 for i from {0,…, 639} |  |  | Bits |  | 4480 |  | 4480 |  |  |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1…13} for i from {1,…,639} |  |  | Bits |  | 5504 |  | 5504 |  |  |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 20) = {15,16,17,18,19} for i from {0,…,639} |  |  | Bits |  | N/A |  | N/A |  |  |  |  |  |  |
| For Slots i = 320, 321 | Bits |  | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 20) = 14 for i from {0,…, 639} |  |  | Bits |  | 24 |  | 24 |  |  |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1…13} for i from {1,…,639} |  |  | Bits |  | 24 |  | 24 |  |  |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 20) = {15,16,17,18,19} for i from {0,…,639} |  |  | CBs |  | N/A |  | N/A |  |  |  |  |  |  |
| For Slots i = 320, 321 | CBs |  | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 20) = 14 for i from {0,…, 639} |  |  | CBs |  | 1 |  | 1 |  |  |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1…13} for i from {1,…,639} |  |  | CBs |  | 1 |  | 1 |  |  |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 20) = {15,16,17,18,19} for i from {0,…,639} |  |  | Bits |  | N/A |  | N/A |  |  |  |  |  |  |
| For Slots i = 320, 321 | Bits |  | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 20) = 14 for i from {0,…, 639} |  |  | Bits |  | 15048 |  | 15048 |  |  |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1…13} for i from {1,…,639} |  |  | Bits |  | 18216 |  | 18216 |  |  |  |  |  |  |
| Max. Throughput averaged over 2 frames |  |  | Mbps |  | 129.632 |  | 129.632 |  |  |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |  |

Table A.3.2.2.8-2: PDSCH Reference Channel for TDD UL-DL pattern FR2.480-1 (16QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.8-2.1 TDD | R.PDSCH.8-2.2 TDD |  |  |  |
| Channel bandwidth | MHz | 400 | 400 |  |  |  |
| Subcarrier spacing | kHz | 480 | 480 |  |  |  |
| Allocated resource blocks | PRBs | 20 | 20 |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 20) = {15,16,17,18,19} for i from {0,…,639} |  | N/A | N/A |  |  |  |
| For Slots i = 320, 321 |  | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 20) = 14 for i from {0,…, 639} |  | 11 | 11 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1…13} for i from {1,…,639} |  | 12 | 12 |  |  |  |
| Allocated slots per 2 frames |  | 477 | 477 |  |  |  |
| MCS table |  | 64QAM | 64QAM |  |  |  |
| MCS index |  | 13 | 13 |  |  |  |
| Modulation |  | 16QAM | 16QAM |  |  |  |
| Target Coding Rate |  | 0.48 | 0.48 |  |  |  |
| Number of MIMO layers |  | 1 | 1 |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 20) = {15,16,17,18,19} for i from {0,…,639} |  | N/A | N/A |  |  |  |
| For Slots i = 320, 321 |  | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 20) = 14 for i from {0,…, 639} |  | 12 | 12 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1…13} for i from {1,…,639} |  | 12 | 12 |  |  |  |
| Overhead for TBS determination |  | 6 | 6 |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 20) = {15,16,17,18,19} for i from {0,…,639} | Bits | N/A | N/A |  |  |  |
| For Slots i = 320, 321 | Bits | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 20) = 14 for i from {0,…, 639} | Bits | 3904 | 3904 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1…13} for i from {1,…,639} | Bits | 4736 | 4736 |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 20) = {15,16,17,18,19} for i from {0,…,639} | Bits | N/A | N/A |  |  |  |
| For Slots i = 320, 321 | Bits | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 20) = 14 for i from {0,…, 639} | Bits | 24 | 24 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1…13} for i from {1,…,639} | Bits | 24 | 24 |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 20) = {15,16,17,18,19} for i from {0,…,639} | CBs | N/A | N/A |  |  |  |
| For Slots i = 320, 321 | CBs | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 20) = 14 for i from {0,…, 639} | CBs | 1 | 1 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1…13} for i from {1,…,639} | CBs | 1 | 1 |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 20) = {15,16,17,18,19} for i from {0,…,639} | Bits | N/A | N/A |  |  |  |
| For Slots i = 320, 321 | Bits | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 20) = 14 for i from {0,…, 639} | Bits | 8160 | 8160 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1…13} for i from {1,…,639} | Bits | 10080 | 10080 |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 111.6224 | 111.6224 |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

A.3.3.2.6	Reference measurement channels for SCS 480 kHz FR2-2

Table A.3.3.2.6-1: PDCCH Reference Channels (Time domain allocation 1 symbol)

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDCCH.6-1.1 TDD |  |  |  |  |  |
| Subcarrier spacing | kHz | 480 |  |  |  |  |  |
| CORESET frequency domain allocation |  | 60 |  |  |  |  |  |
| CORESET time domain allocation |  | 1 |  |  |  |  |  |
| Aggregation level |  | 8 |  |  |  |  |  |
| DCI Format |  | 1_1 |  |  |  |  |  |
| Payload (without CRC) | Bits | 56 |  |  |  |  |  |

--------------------------------------End of Change 3 ---------------------------------------------------------

--------------------------------------Start of Change 4 --------------------------------------------------------

#### A.3.3.2.5	Reference measurement channels for SCS 120 kHz FR2

Table A.3.3.2.5-1: PDCCH Reference Channels (Time domain allocation 1 symbol)

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDCCH.5-1.1 TDD | R.PDCCH.5-1.2 TDD | R.PDCCH.5-1.3 TDD | R.PDCCH. 5-1.4 TDD |  |  |
| Subcarrier spacing | kHz | 120 | 120 | 120 | 120 |  |  |
| CORESET frequency domain allocation |  | 60 | 60 | 60 | 60 |  |  |
| CORESET time domain allocation |  | 1 | 1 | 1 | 1 |  |  |
| Aggregation level |  | 2 | 4 | 8 | 8 |  |  |
| DCI Format |  | 1_0 | 1_1 | 1_1 | 2_6 |  |  |
| Payload (without CRC) | Bits | 40 | 56 | 56 | 12 |  |  |

Table A.3.3.2.5-2: PDCCH Reference Channel (Time domain allocation 2 symbols)

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDCCH.5-2.1 TDD |  |  |  |  |  |
| Subcarrier spacing | kHz | 120 |  |  |  |  |  |
| CORESET frequency domain allocation |  | 60 |  |  |  |  |  |
| CORESET time domain allocation |  | 2 |  |  |  |  |  |
| Aggregation level |  | 16 |  |  |  |  |  |
| DCI Format |  | 1_0 |  |  |  |  |  |
| Payload (without CRC) | Bits | 40 |  |  |  |  |  |

#### A.3.3.2.6	Reference measurement channels for SCS 480 kHz FR2-2

Table A.3.3.2.6-1: PDCCH Reference Channels (Time domain allocation 1 symbol)

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDCCH.6-1.1 TDD |  |  |  |  |  |
| Subcarrier spacing | kHz | 480 |  |  |  |  |  |
| CORESET frequency domain allocation |  | 60 |  |  |  |  |  |
| CORESET time domain allocation |  | 1 |  |  |  |  |  |
| Aggregation level |  | 8 |  |  |  |  |  |
| DCI Format |  | 1_1 |  |  |  |  |  |
| Payload (without CRC) | Bits | 56 |  |  |  |  |  |

--------------------------------------End of Change 4 ---------------------------------------------------------