3GPP TSG-RAN4 Meeting #107	R4-2309424

Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

Source: 	Qualcomm Incorporated

Title: 	Discussion on requirements of inter-RAT measurement without gaps.

Agenda item:	8.10.3.2

Document for:	Discussion

# Introduction

In this paper, we share our view on the remaining issues for remaining issue for inter-RAT measurement without gap.

# Discussion

## <Scope discussion>

- Issue 2-1-1: Differentiate scenarios for Case b-2 < Way forward/Agreement >: - FFS on whether to differentiate the following scenarios in case b-2. - Scenario 1: Inter-RAT LTE measurement for LTE CRS rate-matching feature - Scenario 2: Inter-RAT LTE measurement for CRS-IM receiver

- Issue 2-2-2: UE capability to support the inter-RAT LTE measurement requirements when LTE CRS to be measured is contained in UE’s active BWP(Case b-2) < Way forward >: - A new per-UE capability to support Case b-2 should be defined. - FFS on indication of “no gap with interruption” because of AGC,T/F offset issue

Some companies propose to differentiate scenario 1 and scenario 2 in case b-2. However, we do not see which requirement can be different from each scenario. From our understanding, requirements for inter-RAT measurement without gap when LTE CRS in active BWP is applied in general. Thus, RAN4 does not need to differentiate the scenarios.

Proposal: RAN4 does not differentiate scenarios under case b-2.

Since active BWP can be dynamically switched, similar mechanism for R16 inter-frequency without gap can be reused to support inter-RAT LTE measurement when LTE CRS in active BWP. From our understanding, for UE who support interFrequencyMeas-Nogap-r16, UE does not need to indicate ‘no-gap’ in reporting. Similarly, UE does not need to indicate ‘no-gap’ if RAN4 agree to define UE capability for case b-2 similar to interFrequencyMeas-Nogap-r16.

Proposal: if RAN4 agree to define per-UE capability to support case b-2 similar to interFrequencyMeas-Nogap-r16, UE does not need to indicate ‘no-gap with/without interruption’ for case b-2.

- Issue 2-3-2: Searcher limitation < Way forward/Agreement >: - Option 1: - Inter-RAT LTE measurement without gap(case b-2) can be performed in parallel with NR measurement without searcher limitation - Option 2: - Performing inter-RAT measurement and NR measurements in parallel without searcher limitation is NOT supported

Proposal : Performing inter-RAT measurement and NR measurements in parallel without searcher limitation is NOT supported

- Issue 2-4-2: Measurement period requirements for case a-1 with “no gap but interruption allowed” < Way forward/Agreement >: - FFS on measurement period requirements for case a-1 with “no gap but interruption allowed” - Issue 2-4-3: Measurement period requirements for case a-1 with “no gap no interruption” < Way forward/Agreement >: - FFS on measurement period requirements for case a-1 with “no gap no interruption”

Since RAN4 agreed to use same signalling for inter-RAT NR measurement, we think the requirements for inter-frequency measurement can be reused for inter-RAT NR measurement without gap with/without interruption. By assuming the definition of Tcycle = max (80ms, SMTC, DRX), the requirements for inter frequency measurement without gap with/without interruption can be applied.

Proposal : if RAN4 adopt Tcycle = max (80ms, SMTC, DRX), the requirements for inter-frequency measurement without gap with/without interruption can be reused for inter-RAT NR measurement without gap with/without interruption.

- Issue 2-4-5: Measurement period requirements for case b-1 no gap no interruption allowed < Way forward/Agreement >: - The measurement requirement for case b-1 and case b-2 can be defined by the framework below. TIdentify, interRAT_EUTRAN_gapless=TBasicIdentifya??480TInter1a??CSSFinterRAT msTIdentify, interRAT_EUTRAN_gapless=TBasicIdentify*480TInter1*CSSFinterRAT ms - FFS on update definition of TBasicIdentify,Tinter1 and CSSFinterRAT - Issue 2-4-9: Effiective measurement window < Way forward >: - Option 1: vivo, MTK, Huawei, Ericsson - RAN4 to introduce an effective measurement window for inter-RAT E-UTRAN measurement without gap. - Option 1a: Ericsson - The effective measurement window can be defined based on measurement duration, measurement periodicity and offset. - Option 1b: MTK - FFS on the overlap between NR SMTC and LTE SMTC-lte (i.e. the effective measurement window).

Some companies support Effective measurement window to apply scheduling restriction on LTE CRS. If NW configure the effective measurement window duration, period, and offset, we understand NW can scheduling restriction need to be applied. From our understanding, the effective measurement window can be configured by NW similar to SMTC configuration. Since the purpose of the effective measurement window is to apply scheduling restriction around LTE CRS, we do not expect the window overlaps with other SMTC, SSB, and gaps.

Proposal : The effective measurement window can be defined with including window periodicity, offset, and duration.

Proposal : The effective measurement window should not be overlapped with SSB, SMTC, and other gaps.

Proposal : Multiple patterns of effective measurement window can be defined such as

- Pattern #1 : effective measurement window periodicity 40ms, EFW duration 6ms,

- Pattern #2 : effective measurement window periodicity 80ms, EFW duration 6ms

- Other patterns,

Proposal : TbasicIdentify is 480ms, CSSFinterRAT is number of inter-RAT MOs measured outside of measurement gap. Tinter1 is defined based on configured effective measurement window patterns, such as

- Tinter1 = 60ms for EFW pattern #1

- Tinter1 = 30ms for EFW pattern #2,

- Others

- Issue 2-5-2: Scheduling restriction for inter-RAT LTE measurements < Way forward/Agreement >: - Scheduling restriction due to inter-RAT LTE measurement are applicable when - UE does not support simultaneous Tx and Rx on the serving cell and target band - FFS on Serving cell and target MO have mixed SCS and they are in the same band

Observation : mixed SCS between serving Cell and target MO implies EN-DC. The scheduling restriction for UE does not support simultaneousRxTxInterBandENDC in TDD band include the mixed SCS scenario.

Proposal : No need to define additional scheduling restriction for mixed SCS between serving Cell and target MO case.

Proposal : If RAN4 adopt the effective measurement window, For UE who does not support [simultaneousRxTxInterBandENDC], the UE is not expected to transmit PUCCH/PUSCH/SRS on all symbols during the effective measurement window.

# Conclusions

Proposal 1: RAN4 does not differentiate scenarios under case b-2.

Proposal 2: if RAN4 agree to define per-UE capability to support case b-2 similar to interFrequencyMeas-Nogap-r16, UE does not need to indicate ‘no-gap with/without interruption’ for case b-2.

Proposal 3: Performing inter-RAT measurement and NR measurements in parallel without searcher limitation is NOT supported

Proposal 4: if RAN4 adopt Tcycle = max (80ms, SMTC, DRX), the requirements for inter-frequency measurement without gap with/without interruption can be reused for inter-RAT NR measurement without gap with/without interruption.

Proposal 5: The effective measurement window can be defined with including window periodicity, offset, and duration.

Proposal 6: The effective measurement window should not be overlapped with SSB, SMTC, and other gaps.

Proposal 7: Multiple patterns of effective measurement window can be defined such as

- Pattern #1 : effective measurement window periodicity 40ms, EFW duration 6ms,

- Pattern #2 : effective measurement window periodicity 80ms, EFW duration 6ms

- Other patterns,

Proposal 8: TbasicIdentify is 480ms, CSSFinterRAT is number of inter-RAT MOs measured outside of measurement gap. Tinter1 is defined based on configured effective measurement window patterns, such as

- Tinter1 = 60ms for EFW pattern #1

- Tinter1 = 30ms for EFW pattern #2,

- Others

Observation : mixed SCS between serving Cell and target MO implies EN-DC. The scheduling restriction for UE does not support simultaneousRxTxInterBandENDC in TDD band include the mixed SCS scenario.

Proposal 9: No need to define additional scheduling restriction for mixed SCS between serving Cell and target MO case.

Proposal 10: If RAN4 adopt the effective measurement window, For UE who does not support [simultaneousRxTxInterBandENDC], the UE is not expected to transmit PUCCH/PUSCH/SRS on all symbols during the effective measurement window.

# References

[1] R4-2306331, WF on Rel-18 NR MG enhancements (part 2), Intel, RAN #106bis-e.