3GPP TSG-RAN4 Meeting #107	R4-2309428

Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

Source: 	Qualcomm Incorporated

Title: 	Discussion on RRM requirements of SSBless Scell for inter-band CA

Agenda item:	8.35.4

Document for:	Discussion

# Introduction

During the last meeting, RAN4 discussed about the scenario scope and the side conditions to operate SSBless SCell for inter-band CA. We keep sharing our view about scenario scope and RRM requirements.

# Discussion

## <Scope discussion>

- Agreements - The following scenarios are considered for inter-band SSB-less SCell requirements definition - Scenario 1: No SSB but with TRS transmission configured on the SSB-less SCell - FFS whether to consider requirements based on TRS transmission during and/or after SCell activation - Scenario 2: No SSB and no TRS transmission configured on the SSB-less SCell - Scenario 2a: No DL transmission but with UL reception at the NW side on the SSB-less SCell. - FFS if requirements will be defined for all or a subset of scenarios subject to feasibility analysis Whether to consider PCell first as the reference cell. - Candidate Options: - Option 1 (Qualcomm, Apple, Nokia): RAN4 shall consider PCell first as the reference cell transmitting SSB. FFS: another Scell can be used as the reference cell.

The most important requirement to operate SSBless Scell whether reference cell can provide initial T/F sync and coarse AGC to SSBless SCell. Once UE acquire initial T/F sync and coarse AGC for SSBless SCell, UE can perform fine AGC based on TRS on SSBless SCell. Furthermore, UE can track T/F on SSBless SCell with using both both SSB on reference cell and TRS on SSBless SCell.

Proposal : There is at least one active serving cell as reference cell which can provide SSB for T/F sync, AGC for the SSB-less Scell. The reference cell shall keep being activated.

- RAN4 consider PCell first as the reference cell transmitting SSB. FFS: another Scell can be used as the reference cell.

- SSBless SCell for interband CA is considered as known SCell from Scell activation latency perspective. (e.g, UE can skip coarse AGC, measurement and report for TCI activation can be skipped)

Observation: After UE acquire coarse AGC info, initial T/F from the reference cell, UE can perform fine AGC from TRS on SSBless SCell. UE can track T/F for SSBless SCell with using both SSB on reference and TRS on SSBless SCell.

Proposal: Periodic TRS is available on the SSBless Scell.

Observation: The scenario 2 for TRS is used during Scell activation and no TRS after Scell activation is not valid.

Proposal: RAN4 deprioritizes scenario 2 (No SSB, No TRS transmission on SSBless SCell).

## <Requirements for SSBless Cell inter-band CA>

- Conditions for SSB-less inter-band CA SCell operation for scenario-1 - Received time difference (RTD) between the SSB-less SCell and the FR1 inter-band active serving cell - Option 1-1: 260ns - Option 1-2: 3us - Option 1-3: smaller than 3us - Other values are not precluded - The difference of the reception power with the FR1 inter-band active serving cell - Option 2-1: 6dB - Option 2-2: 6dB+offset - Other values are not precluded - FFS: TCI state (using TCI state information of inter-band co-located SSB less SCell from active serving cell) - FFS: frequency domain separation between the SSB-less SCC and the FR1 inter-band active serving CC; or Certain band combinations. - FFS if requirements will be defined subject to feasibility analysis - Note: different conditions and values may be applicable scenario-1, scenario-2 and scenario-2a

From our understanding, the most important condition to perform the SSBless SCell for inter-band CA operation is how UE obtain TCI state information. If NW cannot provide TCI state information for the SSBless SCell from the measurement report with SSB index, UE cannot decode DL signal on SSBless SCell.

Observation: Obtain TCI state information for SSBless Scell is the most important condition to operate SSBless SCell for inter-band CA.

Proposal: NW shall be able to provide TCI state information for the SSBless SCell based on measurement and report conducted from the reference Cell.

Observation : Separate RF chain is considered for inter-band CA SSBless SCell scenario. With separate RF chain assumption, it is difficult to achieve RTD < 260ns for inter-band CA scenario.

MRTD requirements for intra-band non-contiguous CA, co-located deployment is already 3 us. For inter-band CA, co-located deployment, RTD condition for inter-band CA SSBless SCell is at least same as intra-band non-contiguous CA MRTD requirements which is 3us.

Proposal : RTD condition is 3us for inter-band CA SSBless SCell co-located deployment.

Unlike to intra-band SSBless SCell, it is not guarantee that NW transmit with same power between reference Cell and SSBless SCell.

Observation : it is not guarantee that NW transmit with same power between reference and SSBless SCell. If transmit power is not same between reference Cell and the SSBless SCell, UE cannot perform L1/L3 measurements based on the reference Cell.

Proposal : NW can provide transmit power offset between reference Cell and SSBless SCell, where the transmit power offset = Transmit power of RS on reference Cell – Transmit power of RS on SSBless SCell.

When NW provide transmit power offset information, it is beneficial to UE to determine AGC for SSBless SCell. UE can apply ± 6dB reception power difference after compensating the signalled transmit power to reference Cell power. UE can apply AGC setting from reference Cell to SSBless SCell if reception power difference is ±6dB after compensating the power offset from the reference Cell. It can be represented by either

Observation : If NW can provide transmit power offset information, UE can apply coarse AGC setting from reference Cell to SSBless Scell when differences of the reception power with reference Cell after compensating transmit power offset is within 6dB.

Proposal: During coarse AGC, UE can apply the AGC setting from the reference cell to SSBless SCell when UE meet the following conditions: reception power difference between the reference cell with applying transmit power compensation and SSBless SCell is within 6dB.

- FFS for how to capture in Spec. (e.g : difference reception power with the FR1 inter-band active serving cell <= 6dB + |power offset|)

Not all inter-band combination can support SSBless Scell with inter-band CA. UE capability is required to indicate the bands of reference cell and the SSBless SCell.

Proposal : UE capability for SSBless SCell should be defined per band combo where the band combo indicate the bands of reference cell and the SSBless SCell.

Issue 2-4-1: Scenario for L1 measurement operation on the SSB-less SCell apart from SCell activation procedure - Candidate Options: - Option 1: No SSB but with CSI-RS resource for L1 measurement on the inter-band SSB-less SCell - Option 2: No SSB and No CSI-RS resource for L1 measurement on the inter-band SSB-less SCell Issue 2-4-2: Scenario for L3 measurement operation on the SSB-less SCell apart from SCell activation procedure - Candidate Options: - Option 1: No SSB but with CSI-RS resource for L3 measurement on the inter-band SSB-less SCell - Option 2: No SSB and No CSI-RS resource for L3 measurement on the inter-band SSB-less SCell Issue 2-4-3: L1/L3 measurement operation on the SSB-less SCell - Candidate Options: - Option 1: given that the SSB-less SCell can fully leverage the information from an already activated serving cell, the corresponding L1/L3 measurements can be skipped. - Option 2: For UE deriving SSB-less SCell L1/L3 measurements results by performing L1/L3 measurements on PCell or another SCell, whether and how to assist UE to determine the coverage of SSB-less SCell should be further investigated. - Option 3: The UE is not required to perform SSB-based L1/L3 measurements on the SSB-less SCells. The CSI-RS based L1/L3 measurement requirements need to be specified for SSB-less SCell operation.

Since reference cell is always active, NW can infer L3 RSRP of SSBless SCell from measurement reports on a reference cell. CSI-RS based L3 measurement on SSBless inter-band Scell may not be needed.

Observation: CSI-RS based L3 measurement on SSBless inter-band SCell may not be needed as NW can infer L3 RSRP of SSBless SCell from measurement reports on a reference cell.

Proposal : RAN4 can study whether and how to perform RLM/BFD/CBD on the SSBless SCell based on reference Cell measurement.

# Conclusions

Proposal : There is at least one active serving cell as reference cell which can provide SSB for T/F sync, AGC for the SSB-less Scell. The reference cell shall keep being activated.

- RAN4 consider PCell first as the reference cell transmitting SSB. FFS: another Scell can be used as the reference cell.

- SSBless SCell for interband CA is considered as known SCell from Scell activation latency perspective. (e.g, UE can skip coarse AGC, measurement and report for TCI activation can be skipped)

Observation: After UE acquire coarse AGC info, initial T/F from the reference cell, UE can perform fine AGC from TRS on SSBless SCell. UE can track T/F for SSBless SCell with using both SSB on reference and TRS on SSBless SCell.

Proposal: Periodic TRS is available on the SSBless Scell.

Observation: The scenario 2 for TRS is used during Scell activation and no TRS after Scell activation is not valid.

Proposal: RAN4 deprioritizes scenario 2 (No SSB, No TRS transmission on SSBless SCell).

Observation: Obtain TCI state information for SSBless Scell is the most important condition to operate SSBless SCell for inter-band CA.

Proposal: NW shall be able to provide TCI state information for the SSBless SCell based on measurement and report conducted from the reference Cell.

Observation: Separate RF chain is considered for inter-band CA SSBless SCell scenario. With separate RF chain assumption, it is difficult to achieve RTD < 260ns for inter-band CA scenario.

Proposal : RTD condition is 3us for inter-band CA SSBless SCell co-located deployment.

Observation : it is not guarantee that NW transmit with same power between reference and SSBless SCell. If transmit power is not same between reference Cell and the SSBless SCell, UE cannot perform L1/L3 measurements based on the reference Cell.

Proposal : NW can provide transmit power offset between reference Cell and SSBless SCell, where the transmit power offset = Transmit power of RS on reference Cell – Transmit power of RS on SSBless SCell.

Observation : If NW can provide transmit power offset information, UE can apply coarse AGC setting from reference Cell to SSBless Scell when differences of the reception power with reference Cell after compensating transmit power offset is within 6dB.

Proposal: During coarse AGC, UE can apply the AGC setting from the reference cell to SSBless SCell when UE meet the following conditions: reception power difference between the reference cell with applying transmit power compensation and SSBless SCell is within 6dB.

- FFS for how to capture in Spec. (e.g : difference reception power with the FR1 inter-band active serving cell <= 6dB + |power offset|)

Proposal : UE capability for SSBless SCell should be defined per band combo where the band combo indicate the bands of reference cell and the SSBless SCell.

Observation: CSI-RS based L3 measurement on SSBless inter-band SCell may not be needed as NW can infer L3 RSRP of SSBless SCell from measurement reports on a reference cell.

Proposal : RAN4 can study whether and how to perform RLM/BFD/CBD on the SSBless SCell based on reference Cell measurement.

# References

[1] R4-2306369, WF on RRM requirements for NR network energy saving, Huawei, RAN #106bis-e