3GPP TSG-RAN4 Meeting #107	R4-2309430

Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

Source: 	Qualcomm Incorporated

Title: 	Discussion on remaining issues on L3 part enhancements for FR2 Unknown SCell activation

Agenda item:	8.9.2.1

Document for:	Discussion

# Introduction

In this paper, we share our view on the remaining issues for FR2 unknown FR2 Scell activation.

# Discussion

Issue 1-1-6: If measurement results are available, the UE will report them to the NW. How to determine the measurement result is available? FFS: - Option 1 (Apple, OPPO, Huawei, Xiaomi, vivo): - No need to define criteria to determine the L3 measurement result is available or not for FR2 unknown SCell activation enhancement. - Option 2 (Nokia, Ericsson): - The L3 measurement report is considered as valid only if it fulfils the measurement requirement for a deactivated Scell as specified in TS38.133 Table 9.2.5.2-3 (for FR1) and Table 9.2.5.2-4 (for FR2). - Option 3 (Intel): - If UE already has available L3 measurement result when Scell activation command is triggered, the measurement result can be reported only if it satisfied the following conditions: - Measurement should satisfy the requirement for a deactivated Scell as specified in TS38.133 Table 9.2.5.2-3 (for FR1) and Table 9.2.5.2-4 (for FR2) - the time duration between measurement and Scell activation command is less than [X]s. - Option 4 (MTK): - RAN4 to define a new window [W]s to determine the freshness of the available L3-RSRP measurements at the UE prior to the reception time of Scell activation command. - Proposal 6: If L3-RSRP measurements are within [W], then the UE can send L3-RSRP report. - Option 6 (vivo): - The criteria to determine whether a measurement result is valid (i.e. available), including the signal quality thresholds and the maximum time-delta of the measurement before the reporting, are configured by network - Option 7 (CMCC, ZTE): - for L3 measurement report after Scell activation command, the reported SS-RSRP, SS-RSRQ, and SS-SINR measurements need to meet the accuracy requirements spedified in Clause 10. - Option 8 (ZTE): - To address the concern of the quick report is valid or not, the verification lies in two aspects: - 1)Whether the measurement report is fresh enough. - 2)Whether the quality of measurement result in the report is good or available enough. - The 1st aspect can be ignored since same issue exists in legacy L3 report. The 2nd aspect should be considered to guarantee the NW can obtain useful RSRP and SSB index info from the report. - Option 9 (Apple, CMCC, LGE, CTC, QC, OPPO): - Option 1 and the valid reporting must meet the existing measurement delay/accuracy requirement.

When UE report L3 measurement after receiving SCell activation command for unknown FR2 Scell, there is no new procedure or requirement from measurement perspective. UE can follow existing measurement requirements. If UE has measurement results to report but it has not been reported, UE can report the measurement result after receiving the SCell activation command for the unknown FR2 SCell.

Proposal1: No need to define criteria to determine the L3 measurement results is available. Existing measurement delay/accuracy requirements are reused.

Issue 1-1-7: FFS on necessity of L3 measurement reporting if UE has no valid measurement results? FFS: - Option 1 (Apple, ZTE, CTC, OPPO, Ericsson, Xiaomi, HW, vivo, QC): No need to report L3 measurement reporting after receiving SCell activation command if UE has no valid measurement results. - Option 2 (Nokia, Intel, vivo, Ericsson): The UE may continue the measurement and report a valid L3 measurement result when it is ready, if there is no valid L3 measurement result at the time of Scell activation. - Option 3 (LGE, CTC, MTK): If a UE has no valid measurement results, UE needs to report even the lowest RSRP reported value to notice whether a UE has valid measurement results to the network. - Option 4 (vivo, MTK): If the reporting of the results is carried by uplink L1L2 signaling, and UE has no valid measurement results at the L1L2 reporting occasion, UE may report -Inf value, e.g. ‘RSRP<-156dBm’. - Option 5 (MTK): when report triggering conditions are met, but UE has no fresh L3-RSRP measurements within [W] period, then instead of sending the report UE indicates unknown status for the target Scell. Issue 1-1-8: FFS: If valid L3 measurement results are reported, L3 and L1 parts can be skipped, i.e., network can perform TCI activation after valid L3 measurement results are reported. FFS: - Option 1 (Apple, Nokia, Intel, QC, MTK, LGE, Xiaomi, vivo, CMCC, ZTE, CTC, OPPO): - if UE reports valid L3 measurement result, L3 and L1 parts can be skipped, i.e., network can perform TCI activation after valid L3 measurement results are reported; - Proposal 1a (Apple, CTC, QC): after receiving SCell activation command, - if UE reports valid L3 measurement result before n+X slot (n is the slot when UE received SCell activation command), L3 and L1 parts can be skipped, i.e., network can perform TCI activation after valid L3 measurement results are reported; - otherwise, it shall be treated as legacy unknown case. - Proposal 1b(QC, CTC): - RAN4 shall define the reporting delay requirements from MAC-CE to trigger report and actual measurement report. If UE do not report measurement results during the reporting delay requirements, the new measurement configuration shall be expired. - Proposal 1c(vivo): - Send LS to RAN2, asking RAN2 to design a timer, so as to supervise the longest period between gNB triggering the reporting and the actual result being sent by the UE in PUSCH. The RRM requirements are not enhanced in R18 for the case when a UE does no report a valid measurement result before this timer expires. - Proposal 1d(OPPO): - FFS the side conditions. - Proposal 1e(Nokia): - A-TRS for fast SCell activation can be triggered based on the valid L3 measurement report after SCell activation command. - Proposal 1f(Xiaomi): - If no valid result to be reported, the UE needs to perform L3 measurement and L1 measurement with/without enhanced requirement according to UE capability. - Option 2 (HW, vivo, Apple (bullet #2)): (moderator: this option is also up to the conclusion in issue 1-1-1) - When the new triggered measurement reporting is reported before SCell activation command, it can be considered as known cases. - When the new triggered measurement reporting is reported after SCell activation, it can be considered as unknown case where the uncertainty for TCI can be defined as the time period between TCI configuration/activation relative to the first valid L1-RSRP reporting and the new triggered L3 measurement report which occurs earlier. - Option 3 (Ericsson): - RAN4 to agree that L3 part can be skipped and whether L1 part can be skipped or not pending on whether NW sends TCI command or not. - Option 4 (moderator): - use option 1 as starting point, - FFS on the detailed requirement design, e.g. option 2.

UE expect to report L3 measurement results if available within certain time period after the reporting is triggered. If NW does not receive measurement results, NW expect UE to perform SCell activation procedure from L3 measurement to L1 measurement and reporting. To make same understanding between UE and NW about reporting and following procedure, RAN4 shall define reporting delay requirements from receiving/decoding MAC-CE for Scell activation command with tiggering L3 measurement reports.

Proposal 2 : RAN4 shall define reporting delay requirements from receiving/decoding MAC-CE for SCell activation command to L3 measurement reporting.

- UE shall report L3 measurement report within [X] ms from receiving [L3 report triggering command] if measurement result is available.

- UE does not report L3 measurement report after exceeding reporting delay requirements.

Note: L3 report trigger command can be included at Scell activation command per RAN4 #106bis-e agreement.

Even if UE report L3 measurement results with SSB index, it is up to NW decision whether reported measurement result can be used for TCI activation because Scell was originally considered as unknown when NW transmit Scell activation command. If UE does not receive TCI activation command within certain period after transmitting L3 measurement report, UE can perform either legacy Scell activation or enhanced SCell activation.

Proposal 3: UE expect to receive TCI activation command after reporting L3 measurement report with SSB index.

- If UE does not receive TCI activation command within certain time period after reporting L3 measurement, UE can perform either legacy SCell activation process or enhanced Scell activation.

- FFS whether to use SSB periodicity instead of SMTC periodicity for FR2 unknown Scell activation - Option 1: - Do not change the definition/usage of SMTC periodicity for FR2 unknown Scell activation requirements. - Option 2: - For enhanced unknown FR2 Scell activation requirement, RAN4 to use SSB periodicity instead of SMTC periodicity. - Such enhancement is only applied for FR2 unknown SCell activation.

Upon Scell activation procedure, UE is able to monitor SSB of the target Scell in every occasion. Some companies have concern that SSB periodicity is not always shorter than SMTC periodicity. Such concern can be resolved by adopting min(SSM periodicity, SMTC periodicity) in activation delay requirement.

Proposal 4: min(SSB periodicity, SMTC periodicity) is used for enhanced FR2 SCell activation delay requirements.

# Conclusion

Proposal1: No need to define criteria to determine the L3 measurement results is available. Existing measurement delay/accuracy requirements are reused.

Proposal 2 : RAN4 shall define reporting delay requirements from receiving/decoding MAC-CE for SCell activation command to L3 measurement reporting.

- UE shall report L3 measurement report within [X] ms from receiving [L3 report triggering command] if measurement result is available.

- UE does not report L3 measurement report after exceeding reporting delay requirements.

Note: [L3 report trigger command] can be included at Scell activation command per RAN4 #106bis-e agreement.

Proposal 3: UE expect to receive TCI activation command after reporting L3 measurement report with SSB index.

- If UE does not receive TCI activation command within certain time period after reporting L3 measurement, UE can perform either legacy SCell activation process or enhanced Scell activation.

Proposal 4: min(SSB periodicity, SMTC periodicity) is used for enhanced FR2 SCell activation delay requirements.

# References

[1] R4-2306315, WF on R18 RRM enhancement - FR2 SCell activation delay reduction, Apple, RAN4 #106bis-e