3GPP TSG-RAN4 Meeting #107	R4-2309432

Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

Source: 	Qualcomm Incorporated

Title: 	Discussion on remaining issues on other enhancements for FR2 Unknown SCell activation

Agenda item:	8.9.2.3

Document for:	Discussion

# 1.	Introduction

In this paper, we discuss remaining issue about RRM requirements RRM requirements about FR2 unknown Scell activation enhancements from the last RAN4 meeting [1].

# 2. 	Discussion

From the last meeting, RAN4 agreed to report L3 measurement report for unknown FR2 Scell.

- Sub-topic 3-1 Other possible enhancement for FR2 SCell activation Issue 3-1-1: Scheduling enhancement for FR2 unknown SCell activation FFS: - Option 1 (Apple, QC, OPPO, HW, Ericsson, ZTE): the early scheduling is network implementation and it’s not precluded in existing SCell activation requirement. No enhancement is needed. - Option 1a(ZTE): - For FR2, the initial scheduling can be decided by the NW after receiving the beam info and signal quality info after L1-RSRP report. For FR1, once the signal quality info derived by NW, the initial scheduling is possible. - Option 2 (Nokia): - The potential to enable earlier data transmission within the activation period shall be studied. - The UE shall be scheduled by the network immediately after L3 or L1-RSRP reporting. - Starting from receiving the TCI activation command, the UE shall report CSI, instead of OoR, if the UE has available uplink resources to report CQI for the SCell.

From our understanding, without receiving valid CSI report from UE, NW may not apply right MCS or beam forming which can introduce UE performance loss and consume more energy.

Proposal: No need to consider enhancement on earlier data transmission within the activation period for FR2 unknown Scell.

# 3. Conclusions

Proposal: No need to consider enhancement on earlier data transmission within the activation period for FR2 unknown Scell.

# 4. References

[1] R4-2303228, Way forward on R18 RRM enhancement (part 1), Apple, RAN4 #106