3GPP TSG-RAN WG4 Meeting #107	R4-2309451
Incheon, KR, May 22 – May 26, 2023

Agenda item:	8.12.2

Source:	Huawei, Hisilicon

Title:	Enabling Network to change UE reception to switch between collocated and non-collocated modes

Document for:	Approval

# Introduction

In the RAN4 106-bis meeting the following points were left for the further discussion in the WF[1]

< Issue 2-1-2: Whether Type 1 shall be default when no request comes from BS > Way Forward: Type 1 is default as baseline to continue the following discussions. How UE behaves in case of UE is configured change from default (Type 1 UE) to Type 2. Whether to need signalling from BS to UE operating as Type 1 for configuring Type 2 as the standardized solution. FFS about the definition of Type1 in the next meeting.

In this tdoc we express our view on these issues.

# How a network could switch between type 1 and  type 2 UEs?

The difference between these two UE types is based on their MRTD and power imbalance. Type 1{MRTD=3us, Rx power imbalance<=6dB hence suitable for collocation}, Type 2{MRTD=33us, Rx power imbalance<=25dB, hence suitable for non-collocation}. Currently the networks are deployed based on a collocated configuration and type 1 UEs are being used in NR CA. With the arrival of type 2 UEs, operators would want to ensure a control over the type of the UEs in their network. In other words, they prefer to have control on whether a UE should operate in a collocated NR CA configuration or a non-collocated one, depending on their network deployment scenarios. Based on the supported MRTD and power imbalance, one may observe that type 2 UEs can also be used in a collocated scenario without a problem. After all,  type 2 architecture for UEs is optional and type 1 architecture is the mandatory one.

Observation 1: Type 1{MRTD=3us, Rx power imbalance<=6dB hence suitable for collocation}, Type 2{MRTD=33us, Rx power imbalance<=25dB, hence suitable for non-collocation}

Observation 2: Based on the supported MRTD and power imbalance, one may observe than type 2 UEs can also be used in a collocated scenario.

Observation 3: Type 2 architecture for UEs is optional and type 1 architecture is mandatory.

Observation 4: A type 2 UE can operate in collocated scenario.

The other difference between type 1 and type 2 UEs is the number of MIMO layers that they support. On the reception side, type 1 and type 2 UEs support 4 and 2 layers/CC, respectively. So the network can use the number of DL layers as to switch a UE between Type 2 (non-collocated with collocation possibility) and type 1 (purely collocated).  When a UE reports the capability [intraBandNonColocatedCA-Rel18] then the network can configure the UE to operate in a non-collocated mode. At RRC reconfiguration the network can send PDSCH-ServingCellConfig.maxMIMO-Layers to the UEs to configure the maximum number of DL layers in that serving cell. It should be mentioned that this IE is common for all BWPs of the serving cell. The IE supports up to 8 MIMO layers. As An example if  a UE reports [intraBandNonColocatedCA-Rel18] to the network (i.e. type 2 UE) and the operator would want to ensure that the UEs in the serving cell is set to type 1 UEs(collocated scenario), it can set  ServingCellConfig.maxMIMO-Layers= 4. Therefore the type 2 UE (which supports type 1 architecture as a mandatory feature) will know to switch to type 1. In the other way around, if the network would want to configure a non-collocated NR CA DL communication, then when it drops the collocated Scell and configures the new non-collocated Scell, it may choose ServingCellConfig.maxMIMO-Layers= 2 on the new Scell so the UE will know that a type 2 architecture is required. Based on the truncated version of Table 12.1-1 in TS 38.331, which is depicted below, RRC reconfiguration (collocated scenario) and RRC reconfiguration (scell addition/release) (non-collocated scenario ) will need a delay of 10us and 16us, respectively, which should be enough for UEs to switch from architecture type 1 to type 2 or vice versa.

Proposal 1:   Network can switch between type 1 and type 2 using by setting ServingCellConfig.maxMIMO-Layers= 4, and 2, respectively. Hence a separate dedicated signalling is not needed

Proposal  2:   As ServingCellConfig can be distributed during RRC reconfiguration and RRC reconfiguration (scell addition/release) a delay of 10us and 16us, respectively, is considered , which should give enough time for UEs to switch between the two architectures

# Conclusions

Observation 1: Type 1{MRTD=3us, Rx power imbalance<=6dB hence suitable for collocation}, Type 2{MRTD=33us, Rx power imbalance<=25dB, hence suitable for non-collocation}

Observation 2: Based on the supported MRTD and power imbalance, one may observe than type 2 UEs can also be used in a collocated scenario.

Observation 3: Type 2 architecture for UEs is optional and type 1 architecture is mandatory.

Observation 4: A type 2 UE can operate in collocated scenario.

Proposal 1:   Network can switch between type 1 and type 2 using by setting ServingCellConfig.maxMIMO-Layers= 4, and 2, respectively. Hence a separate dedicated signalling is not needed

Proposal 2:   As ServingCellConfig can be distributed during RRC reconfiguration and RRC reconfiguration (scell addition/release) a delay of 10us and 16us, respectively, is considered, which should give enough time for UEs to switch between the two architectures

# References

[1]	“R4-2306605, WF on UE RF requirements for intra-band non-collocated EN-DC/NR-CA deployment, KDDI.”