3GPP TSG-RAN WG4 Meeting #107 	R4-2309489

Incheon, Korea, May 22 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on LTM General aspects and Scenarios

Agenda item:	8.25.2.1

Document for:	Discussion

Introduction

In this contribution we discuss some general aspects of LTM handovers. We start the discussion with the clarification about the supported cell switch scenarios in the WI and continue with discussion about the known cell condition.

Discussion

Supported scenarios

LTM scenarios according to the agreed WID are:

Note 3: The procedure of L1/L2 based inter-cell mobility are applicable to the following scenarios: • Standalone, CA and NR-DC case with serving cell change within one CG • Intra-DU case and intra-CU inter-DU case (applicable for Standalone and CA: no new RAN interfaces are expected) • Both intra-frequency and inter-frequency • Both FR1 and FR2 Source and target cells may be synchronized or non-synchronized

In the RAN2 running LS (R2-2302039), the supported scenarios that are listed are:

The following scenarios are supported: - PCell change in non-CA scenario, - PCell change without SCell change in CA scenario, - PCell change with SCell change(s) in CA scenario, including the following cases: a) The target PCell/target SCell(s) is not a current serving cell (CA-to-CA scenario with PCell change) b) The target PCell is a current SCell c) The target SCell is the current PCell. - Dual connectivity scenario, at least for the PSCell change without MN involvement case, i.e. intra-SN.

Furthermore, RAN4 has agreed the following further details about the supported scenarios:

Issue 1-3-1: Whether to cover inter-frequency cell switch < Agreement >: - Introduce requirements for inter-frequency cell switch - Which scenarios to define cell switch requirements is up to RAN4 discussion. - FFS: whether to support inter-frequency L1-RSRP measurement. Issue 2-5-1: Whether to consider FR2-2 Offline agreement < Agreement>: - Not consider FR2-2 in R18 LTM. Issue 4-1-1: Whether define cell switch delay requirements for the case “PCell change with PSCell change” Ad hoc agreement < Agreement >: - Only define requirements for serving cell change within one CG, e.g., not define cell switch delay requirements for the case “PCell change with PSCell change”. Issue 4-1-2: Define cell switch delay requirements for SpCell change without SCell change Offline agreement < Agreement >: - Define cell switch delay requirements for: - PCell change without SCell change - PSCell change without SCell change Issue 3-1-2: Whether to define cell switch delay requirements for SCell addition (without activation) at SpCell change < Agreement>: - No need to define requirements for SCell during SCell addition (without activation) at SpCell change Issue 3-1-3: Whether to define requirements for Direct SCell activation at SpCell change (target SCell is not current PCell) < Agreement>: - No more discussion until RAN2 concludes to support direct SCell activation at SpCell change Issue 3-1-4: Whether to define requirements for Direct SCell activation at SpCell change (target SCell is current PCell) < Agreement>: - Specify cell switch requirements for the following scenarios: - Target Pcell/SCell is current SCell/PCell.

Summarized, the scenarios for which RAN4 has agreed to define requirements within this WI are:

- PCell change without SCell change

- PSCell change without SCell change

- Role switch scenario: Target PCell/SCell is current SCell/PCell

Clarifying the role switch scenario, we interpret Note 3 in the WID so that in DC scenario, role switch can only happen between PCell and SCell in MCG, not with an SCell in SCG (“Standalone, CA and NR-DC case with serving cell change within one CG”).

According to the WID, serving cell change can happen within one CG, which means that the role switch scenario between PCell and SCell is more specifically between PCell and SCell in MCG.

Following Note 3 in the WID, it is agreed to support both FR1 and FR2 in this WI. However, RAN4 has not yet made a clear agreement about the supported FR scenarios. Therefore, we propose RAN4 to agree:

- RAN4 to define LTM requirements for the following scenarios (source à target):

- FR1 à FR1

- FR1 à FR2

- FR2 à FR2

- FR2 à FR1

More specifically, the scenarios that are supported according to the current agreements are listed in Table 1.

- RAN4 to support the following (detailed) scenarios for LTM:

Table 1: Supported LTM scenarios.

| Cell switch scenario | CA/DC setup before cell switch | FR scenario (intra- and inter-frequency) |
| --- | --- | --- |
| PCell switch to a neighbor cell | SA: PCell configured CA: PCell and one or more SCells configured DC: PCell and PSCell configured (and one or more SCells in SCG and/or MCG) | FR1 PCell à FR1 cell FR1 PCell à FR2 cell FR2 PCell à FR1 cell FR2 PCell à FR2 cell |
| PSCell switch to a neighbor cell | DC: PCell and PSCell configured (and one or more SCells in SCG and/or MCG) | FR1 PSCell à FR1 cell FR1 PSCell à FR2 cell FR2 PSCell à FR1 cell FR2 PSCell à FR2 cell |
| Role switch between PCell and one of the configured SCells in MCG | CA: PCell and one or more SCells configured DC: PCell, PSCell and at least one SCell in MCG configured | FR1 PCell ↔ FR1 MCG SCell FR1 PCell ↔ FR2 MCG SCell FR2 PCell ↔ FR1 MCG SCell FR2 PCell ↔ FR2 MCG SCell |

Additionally, based on the agreements about the scenarios it is not clear whether the scenario where the target cell is a current SCell, but role switch scenario is not in question, is supported. This applies to both PCell and PSCell switch. RAN4 should clarify the support of these scenarios in this WI.

- RAN4 to discuss whether the following scenarios are supported in the Rel-18 NR_Mob_Enh2 WI:
1) PCell switch to a current SCell in MCG (without PCell becoming a new SCell)
2) PSCell switch to a current SCell in SCG (without PSCell becoming a new SCell)

Known cell condition – 5 seconds

As stated in NR 38.133, section 9.13, for L1-RSRP measurement of a cell with different PCI than the serving cell, the following known cell condition is specified:

- The UE has sent a valid L3 measurement report during the last 5 seconds, and - The SSB from the cell with different PCI remains detectable according to the cell identification requirements specified in clause 9.2. Otherwise, the cell is unknown.

The above known condition is defined for ICBM, which is a procedure that involves serving cell and assistant cell/s for enhancing data transmission reliability. In ICBM, no mobility is involved, UE will only listen to the common search space of the serving cell and listen to the broadcast channel of the serving cell. Hence, these will not be received from the cell with different PCI. ICBM focus on the TCI state of the assistant cells is for the purpose of data reception.

LTM procedure is different from ICBM as it addresses mobility between 2 cells while no data reception from the 2 cells is assumed. It can be seen as a complementary handover procedure that goes along side with L3 handover by utilizing lower layer measurements. Hence, if RAN4 decide to use the requirements in section 9.13, RAN4 need to discuss the very basic applicability conditions.

The network can configure the UE with event-based reporting which will inform the network when a cell fulfils an event decided by ‘enter’ conditions. Network can also use event-based reporting to ensure that network is informed when a cell no longer fulfils an event decided by ‘leave’ condition.

Hence, by use of ‘enter’ and ‘leave’ conditions the network will know if an LTM target cell is a potential candidate for further LTM measurements. By use of event-based measurement report triggers, the network can avoid configuring the UE to send periodic measurement reports.

Network may of course use periodic measurement reporting, but this is often seen as bringing additional, and in many situation unnecessary, measurement reporting overhead.

However, if RAN4 keeps the ‘The UE has sent a valid L3 measurement report during the last 5 seconds’ condition for the target LTM cell to be known, the network would have to configure the UE to transmit measurement reports regularly. Only this way can the network ensure that it will receive L3 measurement report from the UE to fulfill the known condition. And only this way would the network be sure that at least that condition is seen as fulfilled on UE side.

As a result, we find that it is redundant to have the condition “The UE has sent a valid L3 measurement report during the last 5 seconds” as part of having an LTM cell being considered as known.

The 5 seconds does not guarantee that the channel is usable for the network or UE, especially in mobility and/or in FR2 scenarios. It is more important to have real conditions that inform the network about real UE state rather than an arbitrary time limitation.

- Not to specify “during 5 seconds” in known conditions

Conclusion

A couple of sentences on what was analyzed in the paper.

Observation 1: According to the WID, serving cell change can happen within one CG, which means that the role switch scenario between PCell and SCell is more specifically between PCell and SCell in MCG.

Proposal 1: RAN4 to define LTM requirements for the following scenarios (source  target):

-	FR1  FR1

-	FR1  FR2

-	FR2  FR2

-	FR2  FR1

Proposal 2: RAN4 to support the following (detailed) scenarios for LTM:

| Cell switch scenario | CA/DC setup before cell switch | FR scenario (intra- and inter-frequency) |
| --- | --- | --- |
| PCell switch to a neighbor cell | SA: PCell configured CA: PCell and one or more SCells configured DC: PCell and PSCell configured (and one or more SCells in SCG and/or MCG) | FR1 PCell à FR1 cell FR1 PCell à FR2 cell FR2 PCell à FR1 cell FR2 PCell à FR2 cell |
| PSCell switch to a neighbor cell | DC: PCell and PSCell configured (and one or more SCells in SCG and/or MCG) | FR1 PSCell à FR1 cell FR1 PSCell à FR2 cell FR2 PSCell à FR1 cell FR2 PSCell à FR2 cell |
| Role switch between PCell and one of the configured SCells in MCG | CA: PCell and one or more SCells configured DC: PCell, PSCell and at least one SCell in MCG configured | FR1 PCell ↔ FR1 MCG SCell FR1 PCell ↔ FR2 MCG SCell FR2 PCell ↔ FR1 MCG SCell FR2 PCell ↔ FR2 MCG SCell |

Proposal 3: RAN4 to discuss whether the following scenarios are supported in the Rel-18 NR_Mob_Enh2 WI: 
1) PCell switch to a current SCell in MCG (without PCell becoming a new SCell) 
2) PSCell switch to a current SCell in SCG (without PSCell becoming a new SCell)

Proposal 4: Not to specify “during the last 5 seconds” in known or unknown conditions