3GPP TSG-RAN WG4 Meeting #107 	R4-2309492

Incheon, Korea, May 22 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on NR-DC with selective activation of cell groups via L3 enhancements

Agenda item:	8.25.3

Document for:	Discussion

Introduction

In 3GPP meetings held in April 2023, RAN4 has reached the following agreements about selective activation:

Issue 1-1: starting point of subsequent CPC in RRM requirements - Agreements: - Starting point of subsequent CPC in RRM requirements is the time when UE transmits SN RRCReconfigurationcomplete message for the previous PSCell addition or change. Issue 1-2: ending point of subsequent CPC in RRM requirements - Agreements: - Ending point of subsequent CPC in RRM requirements is the transmission of PRACH preamble towards the target PSCell. Issue 1-3: subsequent CPC delay requirements - Agreements: - For subsequent CPC delay requirements: if starting point is the time when UE completes the previous CPC/CPA, Tconfig_PSCell_Subsequent_Change_Conditional = TEvent_DU + Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms and the definition of each component is the same as 8.11B.2 in TS38.133except that TEvent_DU needs to be updated. Issue 1-4: TEvent_DU in subsequent CPC delay requirements - Agreements: - TEvent_DU is the delay uncertainty which is the time from [when UE transmits SN RRCReconfigurationcomplete message for the previous PSCell addition or change] until a condition exists at the measurement reference point which will trigger the subsequent conditional PSCell change.

In 3GPP meeting held in April in 2023, RAN2 has reached the following agreement for selective activation:

Baseline procedure to support subsequent secondary cell group change Step 1: when the execution condition of a CPC candidate PScell is met, a UE performs the execution of CPC towards this candidate PScell. Step 2: After finishing the PSCell addition or change, the UE doesn’t release conditional configuration of other candidate PSCells for subsequent CPC, the UE continues evaluating the execution conditions of other candidate PScells. Step 3: When the execution condition of a candidate PScell is met, the UE performs the execution of CPC towards this candidate PSCell.

In this contribution, we focus on the remaining open issues in selective activation.

Discussion

NR-DC with selective activation of cell groups via L3 enhancements

Issue 1-6: whether to consider UE is not evaluating the execution condition of other candidate PSCells while executing CPC - Candidate solutions: - Option 1: Yes. Event_DU for the subsequent change can be set to zero, if the execution condition for another PSCell is fulfilled while the UE is completing the PSCell change. - Option 2: No. According to TS37.340 and RAN2 agreements, UE is not evaluating the execution condition of other candidate PSCells while executing CPC, and the evaluation is continued after finishing the PSCell addition or change.

Based on the RAN2 agreement, UE shall not release conditional configuration of other candidate PScells for subsequent CPC. Therefore, UE is required to keep the measurements for the retained conditional configuration. Depending on UE capability, the continuation of measurements on other candidate PScells could continue even during executing CPC. Hence, we interpret the RAN2 agreement as such that UE doesn’t reset the measurement but continue the measurement based on retained configurations. The evaluation of subsequent CPCs continues based on these measurements and further measurements after finishing CPC.

UE shall not reset the measurement but continue the measurement based on the retained configurations during executing CPC

The evaluation of subsequent CPCs continues based on these measurements and further measurements after finishing CPC

Conclusion

In the paper, the following Observations and Proposals were made:

Proposal 1: UE shall not reset the measurement but continue the measurement based on the retained configurations during executing CPC

Proposal 2: The evaluation of subsequent CPCs continues based on these measurements and further measurements after finishing CPC