3GPP TSG-RAN WG4 Meeting #107	R4-2309553

Incheon, South Korea, May 22 – May 26, 2023

Agenda Item:	8.26.2.3

Source:	MediaTek Inc.

Title:	Discussion on NW A RRM requirements for MUSIM

Document for:	Discussion

# Introduction

In NR Rel-17 specification, RAN4 has introduced gap patterns particularly for MUSIM purpose. However, their corresponding RRM requirements were not specified. In Rel-18 MUSIM WI, RAN4 has discussed the RRM requirements for MUSIM and the open issues are captured in the WF [1]. In this paper, NW A requirements for MUSIM are further discussed.

# Discussion

Based on the current signaling framework, UE can request from NW A up to four MUSIM gaps (three periodic and one aperiodic as defined in Table 9.1.10-1 of 38.133 [2]) to monitor NW B activities. Then NW A may comply with the UE request and provide the required configurations for MUSIM gaps.

The configured MUSIM gaps may collide with L3/L1 measurements in NW A while the UE is in RRC_CONNECTED state in NW A. This collision will impact the RRM requirements for NW A. As a result, new requirements are required to be introduced for L3 and L1 measurements in NW A.

For example, the scaling factor Kp and Kgap for L3 measurements should be defined to consider MUSIM gaps impact, which can apply the same principle of concurrent measurements gaps, as follows:

- Kp for intra/inter-frequency measurements (without gap):

  - Kp = Ntotal / Navailable, where

- Ntotal is the total number of SMTC occasions within the window W, including those overlapped with MGs and/or MUSIM gaps within the window.

- Navailable is the number of SMTC occasions that are not overlapped with any non-dropped MG occasion or non-dropped MUSIM gap occasion within the window W.

- W is the largest periodicity among MGs, MUSIM gaps and SMTC.

- Kgap for intra/inter-frequency measurements (with gap):

  - Kgap = Ntotal / Navailable, where

- Ntotal is the total number of SMTC occasions that are covered by instances of the associated MG within the window W, including those overlapped with other MGs and/or MUSIM gaps within the window.

- Navailable is the number of SMTC occasions that are covered by instances of the non-dropped associated MG within the window W.

- W is the largest periodicity among MGs, MUSIM gaps and SMTC.

Proposal 1: Reuse the same principle of Rel-17 concurrent gaps WI to define network A L1/L3 measurement requirements when MUSIM gaps are configured, i.e., introduce a scaling factor like Kx = Ntotal /Navailable for network A requirements when MUSIM gaps are configured.

In the same way, the definition of the parameters Kgap_EUTRA , Kp_CSI-RS , Kp_PRS , CSSF (for Intra, Inter and inter-RAT) and P scaling factor for L1 measurements can be updated to account the impact of MUSIM gaps. In this meeting, RAN4 needs at least to agree on what parameters to update in the specs, the exact wording can be discussed during CR drafting stage.

Proposal 2: Update the definition of the following L3/L1 parameters to account the impact of MUSIM gaps:

- Kp for intra/inter-frequency measurements (without gap)

- Kgap for intra/inter-frequency measurements (with gap)

- Kgap_EUTRA Kgap_EUTRA , Kp_CSI-RS and Kp_PRS

- CSSF for intra/inter and inter-RAT measurements

- P scaling factor for L1 measurements

# Summary

In this contribution, the RRM requirements for NW A are discussed, where new requirements for L3/L1 measurements need to be defined to address the impact of MUSIM gaps. The following proposals were approached:

Proposal 1: Reuse the same principle of Rel-17 concurrent gaps WI to define network A L1/L3 measurement requirements when MUSIM gaps are configured, i.e., introduce a scaling factor like Kx = Ntotal /Navailable for network A requirements when MUSIM gaps are configured.

Proposal 2: Update the definition of the following L3/L1 parameters to account the impact of MUSIM gaps:

- Kp for intra/inter-frequency measurements (without gap)

- Kgap for intra/inter-frequency measurements (with gap)

- Kgap_EUTRA Kgap_EUTRA , Kp_CSI-RS and Kp_PRS

- CSSF for intra/inter and inter-RAT measurements

- P scaling factor for L1 measurements

# References

- R4-2306357, WF on NR Dual TxRx Multi-SIM, vivo.

- 3GPP TS 38.133, V17.7.0, “NR; Requirements for support of radio resource management”.