3GPP TSG-RAN WG4 Meeting #107	R4-2309558

Incheon, South Korea, May 22 – May 26, 2023

Agenda Item:	8.23.3.4

Source:	MediaTek Inc.

Title:	Discussion on Positioning for RedCap UEs

Document for:	Discussion

# Introduction

In the last meeting, RAN4 continued the discussion on the RRM core requirements for RedCap Positioning in R18 for RedCap UEs. Some agreements were made as captured in the WF [1]. In this paper we further discuss the RRM requirements for positioning measurements for RedCap UEs.

# Discussion

RedCap UEs are reduced capability devices that support limited frequency bandwidth, 20 MHz in FR1 and 100 MHz in FR2. In addition, RedCap UEs can be designed with 2 Rx or reduced 1 Rx receiver antenna. Enabling positioning measurements in RedCap UEs in R18 should take into account these reduced capabilities in the UE design. Given the limited BW of RedCap UEs, PRS/SRS resources used for positioning measurements that occur over a larger BW (>20 MHz in FR1 and >100 MHz in FR2) will require frequency hopping in order to cover the resources outside UE’s BW coverage.

Therefore, the RRM core requirements for RedCap Positioning can be studied under the two scenarios, with and without frequency hopping. When no frequency hopping is needed (i.e., PRS/SRS resources are already covered by the UE capability), the existing requirements for positioning in R16/R17 can be used as a baseline for RedCap Positioning. In the last meeting RAN4 has discussed the following issues as captured in the WF [1]:

Issue 2-1-3: Relation with Rel-16/Rel-17 positioning: Agreements: - PRS requirements without frequency hopping (FH): - Option 1: HW, E///, QC, CATT, OPPO, Nokia - PRS requirements for both 1Rx and 2Rx RedCap UE without FH shall be defined for all the Rel-16/Rel-17 positioning features/techniques. - Option 2: MTK - PRS requirements for 1Rx RedCap UE without FH shall be defined for all the Rel-16 positioning features/techniques. - PRS requirements for 2 Rx RedCap UE without FH shall be defined for all the Rel-16/Rel-17 positioning features/techniques. - PRS requirements with FH: - The Rel-16/Rel-17 positioning features/techniques for which the PRS requirements for RedCap UE with FH will be defined can be decided on case-by-case basis and based on RAN1 agreements on the FH. Details are FFS. - NOTE: - Rel-16 features/techniques refer to: measurement with gaps in RRC connected, 4-samples measurement in RRC connected. - Rel-17 features/techniques refer to: measurement without gaps in RRC connected, measurement in RRC inactive, reduced-sample measurement in RRC connected/inactive, Tx/Rx timing mitigation with TEG in RRC connected/inactive, FR2 with reduced Rx beam sweeping factor in RRC connected/inactive.

For Issue 2-1-3, the intention of Option 2 is to point out that all the enhancement related to FR2 is not applicable to 1Rx (e.g., reduced Rx beam sweeping factor). In other words, it is not very accurate to say that all Rel-17 positioning features/techniques shall be defined for 1Rx RedCap UE. Therefore, the wording in Option 1 can be revised or clarified by adding a note saying that: all the enhancement related to FR2 is not applicable to 1Rx.

Proposal 1: PRS requirements for both 1Rx and 2Rx RedCap UE without FH shall be defined for all the Rel-16/Rel-17 positioning features/techniques. (Note: all the enhancement related to FR2 is not applicable to 1Rx).

RAN4 has also discussed the side conditions for PRS measurements for RedCap without FH. In the last meeting, it was agreed that, for 2Rx RedCap UE, the side conditions defined in R17 can be reused for defining corresponding requirements for 2Rx RedCap UE (as shown below [1]). However, the side conditions for 1Rx RedCap UE without FH are still under discussion.

Issue 2-2-1: Side conditions for 2Rx without FH: Agreements: - The side conditions (PRS Es/Iot) defined in Rel-17 are reused for defining corresponding PRS requirements for 2Rx RedCap UE without FH. Issue 2-2-2: Side conditions for 1Rx without FH: Agreements: - Side conditions for 1Rx without FH - For AWGN channel, re-use the Rel-17 side conditions, and relax accuracy requirements - The agreement applies for 4 measurement samples case. FFS if it applies for lower number of samples. - FFS for fading channel - Option 1: Reuse Rel-17 side conditions and relax accuracy requirements - Option 2: Reuse approximately Rel-17 accuracy requirements and relax the side condition Note: Updated simulation assumptions to address open issues are discussed under issue 2-2-8

For Issue 2-2-2, for AWGN channel, we think Rel-17 side conditions can be reused with relaxing the accuracy requirements for both 4 and 1 measurement samples. However, for fading channel our simulation result cannot pass based on the existing side conditions. Therefore, the side conditions for fading channel need to be relaxed as well as the accuracy. As a result, we prefer Option 2, and the exact value can be further studied.

Proposal 2: The side conditions for fading channel for 1Rx RedCap UE without FH: relax Rel-17 side conditions and the accuracy requirements.

# Summary

In this contribution, the following proposals are concluded:

Proposal 1: PRS requirements for both 1Rx and 2Rx RedCap UE without FH shall be defined for all the Rel-16/Rel-17 positioning features/techniques. (Note: all the enhancement related to FR2 is not applicable to 1Rx).

Proposal 2: The side conditions for fading channel for 1Rx RedCap UE without FH: relax Rel-17 side conditions and the accuracy requirements.

# References

- R4-2306349, WF on RRM requirements for RedCap positioning and PRS/SRS BW aggregation, Ericsson.