3GPP TSG-RAN WG4 Meeting #107 	R4-2309570

Incheon, Korea meeting, May 22nd – 26th, 2023

Agenda Item: 8.11.2.3

Source: MediaTek inc.

Title: Discussion on BWP operation without BW restrictions – Option C

Document for: Discussion and decision

# Introduction

RAN plenary discussed the topic BWP operation without restrictions in Meeting RAN#99 and defined a new WI with objectives captured below [1]:

## Objective ### Objective Core part WI ### The work item includes following objectives: - For Option C - Specify support of BM/RLM/BFD based on NCD-SSB within active BWP for non-RedCap UEs (RAN4, RAN2, RAN1)

Therefore, in this contribution paper, we provide analysis and discussion on the completeness of Option C (NCD-SSB).

# Discussion on RRM requirements impact/workload in RAN4

RAN4 has discussed the requirements applicability for Option C in the previous meeting R4#106-bis-e RAN4 reached the following way forward:

## Issue 4-1: Applicable of existing RLM/BFD/BM requirements for supporting Option C <Agreement >: - The existing requirements for SSB-based RLM/BFD/BM for non-RedCap UE are applicable to option C. - FFS details or wording of applicable condition. ## Issue 4-2: L1 measurement requirements to be made applicable for supporting option C <Agreement >: - Existing SSB based RLM/BFD/CBD/BM measurement requirements in clause 8.1, 8.5, 9.5 and 9.8 are to be made applicable for supporting option C. ## Issue 4-4: Impact to intra-frequency measurement requirements for supporting option C During GTW on 17th April, following agreements were reached. <Agreement >: - It is a common RAN4 understanding that support of L3 measurements based on NCD-SSB for Option C can be beneficial, but it is not explicitly included in the WI objectives. Whether to support L3 measurements based on NCD-SSB is up to RAN decision. - RAN4 work will focus on enabling L1 measurements based on NCD-SSB in Q2’2023. ## Issue 4-6: Whether to define requirements for transition between CD-SSB and NCD-SSB for supporting option C <Agreement >: - Re-use the requirements for transition between CD-SSB and NCD-SSB for RLM and BFD from RedCap. ## Issue 4-3: Clarification on existing timing requirements for supporting Option C <Way forward >: - Proposals - Option 1: (Ericsson, Nokia, Intel, CATT, MediaTek, OPPO) - The current SSB-based timing requirement can be also applied to non-redCap UE with NCD-SSB support. - Option 2: (vivo, Huawei, Spreadtrum) - It is clarified in the spec that existing timing requirements for non-RedCap UE shall be met provided at least one SSB is available during 160ms, which can also be NCD-SSB within active BWP if UE supports option C.

Regarding Issue 4-3:

The existing RAN4 requirements specify that the UE is not required to perform BM/RLM/BFD outside the active BWP. However, in the previous RAN4 meeting we agree that ‘the existing requirements for SSB-based RLM/BFD/BM for non-RedCap UE are applicable to option C’. In addition, RAN4 also discussed the timing requirements and whether to define additional requirements for option C. In our view, the current Section 7.1.2 is already general enough to cover both cases when SSB is within and outside the active BWP and to cover CD-SSB and NCD-SSB. Therefore, we do not see a need for further clarification for Option C.

- No further clarification is needed on existing timing requirements in TS38.133 for Option C.

- The current SSB-based timing requirement can be also applied to non-redCap UE with NCD-SSB support.

# Summary

In this contribution, we have the following proposal:

Proposal 1: No further clarification is needed on existing timing requirements in TS38.133 for Option C.

Proposal 2: The current SSB-based timing requirement can be also applied to non-redCap UE with NCD-SSB support.

# References

- RP-230805, “New WID: Complete the specification support for BandWidth Part operation without restriction in NR”, Vodafone, vivo, Rotterdam, Netherlands, March 2023.

- R4-2306397, “WF on completion of specification support for BWP operation without restriction”, vivo, e-meeting, April 2023.