3GPP TSG-RAN WG4 Meeting #107 	R4-2309573

Incheon, Korea meeting, May 22nd – 26th, 2023

Agenda Item: 8.15.5

Source: MediaTek inc.

Title: Discussion on issues for less than 5MHz

Document for: Discussion and decision

# Introduction

The work item (WI) for less than 5 MHz has the following objectives [1]:

The following objectives shall be included for dedicated FDD spectrum in FR1: - Identify and specify necessary changes to NR physical layer with minimum specification impact to operate in spectrum allocations from approximately 3 MHz up to below 5 MHz [RAN1]: - Restrict to subcarrier spacing of 15kHz and the use of normal cyclic prefix. - For SSB: - Reuse PSS/SSS specification without puncturing. - PBCH based on current design - Identify and specify necessary minimum changes to PDCCH, CSI-RS/TRS, PUCCH, and PRACH for functional support based on existing design, without optimization. - Specify necessary RAN4 requirements to support deploying NR in spectrum allocations from approximately 3 MHz up to below 5 MHz [RAN4], including in bands n100, n8, n26 and n28: - Specify system parameters (including channel and sync rasters) for the associated dedicated spectrum. - Minimize impact on RF requirements: - Reuse 5 MHz channel bandwidth at least for FRMCS use case (assuming co-located NR and GSM-R with same operator). - Specify the required RF requirements for optional 3 MHz channel bandwidth in bands n100, n8, n26 and n28. - Specify RRM requirements while minimizing specification impact to support operation in dedicated spectrum allocations from approximately 3 MHz up to below 5 MHz.

Further, RAN plenary has sent an LS to RAN1 with further updates on PBCH design for band n100:

RAN Plenary has discussed the possible transmission bandwidth options for 3 MHz and 5 MHz channel bandwidths for the spectrum allocations on the bands of interest in this work item, and concluded the following: - For the 3MHz channel bandwidth in band n100 (max channel utilization 15 PRBs as already agreed in RAN1/RAN4): - PBCH transmission bandwidth is 12 PRBs - CORESET#0 transmission bandwidth is to be decided by RAN1 - RAN1 is requested to consider whether the above also applies for other bands with 3MHz channel bandwidth, or whether the PBCH transmission bandwidth is 15 PRBs for such bands. - For the 5MHz channel bandwidth: - PBCH transmission bandwidth is 20 PRBs - CORESET#0 transmission bandwidth is to be decided by RAN1 - Other details (including sync raster details) are to be progressed in the WGs.

Furthermore, the open issues captured from the previous meeting is captured in the way forward (WF) for core requirements [3] and WF for simulation assumptions [4].

# Simulation performance

In this section, the simulation performance of the MIB detection is provided with two SSB BW of 20 PRB and 12 PRB. Also, two UE speeds are used, which is 3 km/h for TDL channels, while using 500 km/h for HST channel model with 1-tap. The MIB detection performance of one-shot detection is given in the form of detection rate, which is given in percentage. While the soft combining performance results are given in number of SSB samples needed to complete detection rates 99%, 95%, and 90%.

## Simulation performance with one-shot detection

The simulation performance with 20 PRB for TDL-A, TDL-B and TDL-C channel models are given in Table 1. The performance is given in detection percentage rate with one-shot detection for MIB payload.

Table 1: MIB detection rate for 20 PRB BW in the percentage form

|  | MIB detection rate for 20 PRB BW in the % form |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SINR | 0 | -1 | -2 | -3 | -4 | -5 | -6 | -7 | -8 | -9 | -10 |
| TDL-A | 99.2 | 98.8 | 98.4 | 97.6 | 95.6 | 90.8 | 86.8 | 81.6 | 73.6 | 59.6 | 44.8 |
| TDL-B | 100 | 100 | 99.6 | 99.2 | 99.2 | 97.2 | 94 | 87.2 | 81.6 | 68.4 | 54.8 |
| TDL-C | 100 | 100 | 99.6 | 99.2 | 98.4 | 96.8 | 93.2 | 87.6 | 76.8 | 63.6 | 48.8 |

The simulation performance with 12 PRB for TDL-A, TDL-B, TDL-C and 1-tap HST channel models are given in Table 2. The performance is given in detection percentage rate with one-shot detection for MIB payload.

Table 2: MIB detection rate for 12 PRB BW in the percentage form

|  | MIB detection rate for 12 PRB BW in the % form |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SINR | 0 | -1 | -2 | -3 | -4 | -5 | -6 | -7 | -8 | -9 | -10 |
| TDL-A | 96 | 94 | 92.4 | 86 | 79.6 | 70.4 | 60.4 | 50 | 40 | 29.2 | 19.2 |
| TDL-B | 98 | 95.6 | 93.6 | 89.6 | 82.4 | 75.2 | 61.2 | 52 | 39.2 | 28.8 | 18 |
| TDL-C | 97.6 | 94.8 | 90.8 | 84.8 | 78.8 | 69.2 | 58.4 | 46.8 | 34.8 | 18.8 | 12.4 |
| 1-tap HST | 100 | 100 | 100 | 99.2 | 96.4 | 94 | 86.4 | 68.8 | 38.4 | 17.2 | 1.2 |

- One-shot detection is not sufficient to detect MIB with both 20 PRB and 12 PRB bandwidth.

## Simulation performance with soft combining

The simulation performance for MIB detection rates 99%, 95% and 90% using soft combining for up to 4 SSB samples are provided in the tables below.

Performance results in tables 3 and 4 with target 99%:

Table 3: MIB detection rate for 20 PRB BW using soft combining given in number of needed SSB samples for target detection rate 99%

|  | Number of needed SSB samples for MIB detection with 20 PRB BW |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SINR | 0 | -1 | -2 | -3 | -4 | -5 | -6 | -7 | -8 | -9 | -10 |
| TDL-A | 1 | 2 | 2 | 2 | 3 | 3 | 3 | 4 | NA | NA | NA |
| TDL-B | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 3 | 3 | 4 | NA |
| TDL-C | 1 | 1 | 1 | 1 | 2 | 3 | 3 | 3 | 4 | NA | NA |

Table 4: MIB detection rate for 12 PRB BW using soft combining given in number of needed SSB samples for target detection rate 99%

|  | Number of needed SSB samples for MIB detection with 12 PRB BW |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SINR | 0 | -1 | -2 | -3 | -4 | -5 | -6 | -7 | -8 | -9 | -10 |
| TDL-A | 2 | 3 | 3 | 3 | 4 | 4 | NA | NA | NA | NA | NA |
| TDL-B | 2 | 2 | 2 | 2 | 3 | 4 | 4 | NA | NA | NA | NA |
| TDL-C | 2 | 3 | 3 | 4 | 4 | NA | NA | NA | NA | NA | NA |
| 1-tap HST | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 3 | 4 | NA |

Performance results in tables 5 and 6 with target 95%:

Table 5: MIB detection rate for 20 PRB BW using soft combining given in number of needed SSB samples for target detection rate 95%

|  | Number of needed SSB samples for MIB detection with 20 PRB BW |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SINR | 0 | -1 | -2 | -3 | -4 | -5 | -6 | -7 | -8 | -9 | -10 |
| TDL-A | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 3 | 3 | 4 | NA |
| TDL-B | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 3 | 4 |
| TDL-C | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 3 | 3 | 4 |

Table 6: MIB detection rate for 12 PRB BW using soft combining given in number of needed SSB samples for target detection rate 95%

|  | Number of needed SSB samples for MIB detection with 12 PRB BW |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SINR | 0 | -1 | -2 | -3 | -4 | -5 | -6 | -7 | -8 | -9 | -10 |
| TDL-A | 1 | 2 | 2 | 2 | 3 | 4 | 4 | NA | NA | NA | NA |
| TDL-B | 1 | 1 | 2 | 2 | 2 | 2 | 3 | 4 | NA | NA | NA |
| TDL-C | 1 | 2 | 2 | 2 | 3 | 3 | 4 | NA | NA | NA | NA |
| 1-tap HST | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 3 | NA |

Performance results in tables 7 and 8 with target 90%:

Table 7: MIB detection rate for 20 PRB BW using soft combining given in number of needed SSB samples for target detection rate 90%

|  | Number of needed SSB samples for MIB detection with 20 PRB BW |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SINR | 0 | -1 | -2 | -3 | -4 | -5 | -6 | -7 | -8 | -9 | -10 |
| TDL-A | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 3 | 4 |
| TDL-B | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 3 |
| TDL-C | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 3 |

Table 8: MIB detection rate for 12 PRB BW using soft combining given in number of needed SSB samples for target detection rate 90%

|  | Number of needed SSB samples for MIB detection with 12 PRB BW |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SINR | 0 | -1 | -2 | -3 | -4 | -5 | -6 | -7 | -8 | -9 | -10 |
| TDL-A | 1 | 1 | 1 | 2 | 2 | 2 | 3 | 4 | 4 | NA | NA |
| TDL-B | 1 | 1 | 1 | 2 | 2 | 2 | 3 | 3 | 4 | NA | NA |
| TDL-C | 1 | 1 | 1 | 2 | 2 | 2 | 3 | 4 | NA | NA | NA |
| 1-tap HST | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 3 | NA |

- In order to achieve MIB detection rate 99% soft combining is needed to enhance the performance compared to using one-shot detection.

- The minimum SINR threshold to achieve 99% MIB detection rate with 12 PRB bandwidth is equal to -4 dB.

# Summary

In this contribution, we have the following proposals:

Proposal 1: One-shot detection is not sufficient to detect MIB with both 20 PRB and 12 PRB bandwidth.

Proposal 2: In order to achieve MIB detection rate 99% soft combining is needed to enhance the performance compared to using one-shot detection.

Proposal 3: The minimum SINR threshold to achieve 99% MIB detection rate with 12 PRB bandwidth is equal to -4 dB.

# References

- RP-213603, “New WI: NR support for dedicated spectrum less than 5MHz for FR1”, Nokia, December 2021.

- RP-230780, “Response to LS from RAN1 on transmission bandwidths for NR on dedicated spectrum less than 5 MHz”, RAN, March 2023.

- R4-2306346, “WF on RRM requirements for NR_FR1_lessthan_5MHz_BW”, Nokia, e-meeting, April 2023.

- R4-2306400, “WF on simulation assumptions for NR_FR1_lessthan_5MHz_BW”, Nokia, e-meeting, April 2023.