3GPP TSG-RAN4 Meeting #107		R4-2309574

Incheon, South Korea, May 22nd – 26th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 3355 | rev |  | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Formal CR to Rel-17 TS 38.133: on RedCap maintenance in TS 38.133 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap-Core |  |  |  |  |  | Date: |  |  | 2023-05-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Provides some corrections and adding missing clauses for RedCap Rel-17 in TS 38.133 for IDLE/INACTIVE for RedCap |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes are: Change 1: - The conditions for absolute priorities were appearing under the condition of equal priority, hence it is fixed to appear as alternative condition. Change 2, 3, 4, 6, 7, and 9: - Remove the duplicated and/or un-needed offset for 1Rx UE. - Adding the missing low mobilty configuration for intra-frequency, inter-frequency and inter-RAT. - Fixing the following conditions for inter-frequency and inter-RAT - Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ - Srxlev > SnonIntraSearchP or Squal > SnonIntraSearchQ Change 5 and 8: - RAN4 has the following agreement [R4-2207105]: ‘UE is allowed to meet the requirements that are the most relaxed out of Rel-16 and Rel-17 requirements’. For the scenario of UE fulfilling low mobility and not-at-cell edge criterion and stationary criteria, we observe the following: - For the case: Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ - The requirements for low mobility and not-at-cell edge criterion are more relaxed compared to stationary, which is 1 hour. - Srxlev > SnonIntraSearchP or Squal > SnonIntraSearchQ - The requirements for stionary criterion are more relaxed compared to low mobility and not-at-cell edge criterion, which is K2=240. Change 10: - Fixing typo mistakes in reference to other section in clause 5.2B.2 and scheduling restriction clause - Reference section changed from 5.2B.2 5.5.2. - Reference section changed from 5.1B.2.4 5.5.4. - Reference section changed from 5.1.2.4 5.1B.2.4. - Reference section changed from 5.1.2.5 5.1B.2.5. - Clauses (i) TA validation, (ii) scheduling restriction, and (iii) Applicability condition for SDT are promoted as sub-clauses instead of sub-sub-clause under the requirements on UE synchornization. Change 11: - Adding missing requirements for RA-SDT for RedCap. - Adding applicability condition for RA-SDT for RedCap. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be incomplete and incorrect requirements or typo-mistakes in the existing specifications in TS 38.133. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2B.2.4, 4.2B.2.9, 4.2B.2.10, 4.2B.2.11, 5.2B, 5.7B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------START OF CHANGE 1----------------------------

#### 4.2B.2.4	Measurements of inter-frequency NR cells for RedCap UE

The UE shall be able to identify new inter-frequency cells and perform SS-RSRP or SS-RSRQ measurements of identified inter-frequency cells if carrier frequency information is provided by the serving cell, even if no explicit neighbour list with physical layer cell identities is provided.

If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7.

If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the UE shall search for and measure inter-frequency layers of higher, equal or lower priority in preparation for possible reselection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority layers shall be the same as that defined below in this clause.

The UE shall be able to evaluate whether a newly detectable inter-frequency cell meets the reselection criteria defined in TS 38.304 [1] within Kcarrier_RedCap * Tdetect,NR_Inter_RedCap  if at least carrier frequency information is provided for inter-frequency neighbour cells by the serving cells when Treselection = 0 provided that the reselection criteria is met

For 2 Rx RedCap by a margin of at least

5 dB in FR1 or 6.5 dB in FR2 for reselections based on ranking or

6 dB in FR1 or 7.5 dB in FR2 for SS-RSRP reselections based on absolute priorities or

4 dB in FR1 and 4 dB in FR2 for SS-RSRQ reselections based on absolute priorities

For 1 Rx RedCap by a margin of at least

6 dB in FR1 for reselections based on ranking or

7 dB in FR1 for SS-RSRP reselections based on absolute priorities or

5 dB in FR1 for SS-RSRQ reselections based on absolute priorities.

The parameter Kcarrier_RedCap is the number of NR inter-frequency carriers indicated by the serving cell. An inter-frequency cell is considered to be detectable according to the conditions defined in Annex B.1.5 for a corresponding Band. When higher priority cells are found by the higher priority search, they shall be measured at least every Tmeasure,NR_Inter_RedCap. If, after detecting a cell in a higher priority search, it is determined that reselection has not occurred then the UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of reselection. However, the minimum measurement filtering requirements specified later in this clause shall still be met by the UE before it makes any determination that it may stop measuring the cell. If the UE detects on a NR carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE shall measure SS-RSRP or SS-RSRQ at least every Kcarrier_RedCap * Tmeasure,NR_Inter_RedCap for identified lower or equal priority inter-frequency cells. If the UE detects on a NR carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE shall filter SS-RSRP or SS-RSRQ measurements of each measured higher, lower and equal priority inter-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,NR_Inter/2.

The UE shall not consider a NR neighbour cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell.

For an inter-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the inter-frequency cell has met reselection criterion defined TS 38.304 [1] within Kcarrier * Tevaluate,NR_Inter_RedCap when Treselection = 0 provided that the reselection criteria is met by

-	the condition when performing equal priority reselection and

-	when rangeToBestCell is not configured:

-	the cell is at least 5dB better ranked in FR1 or 6.5dB better ranked in FR2 for 2 Rx RedCap.

-	the cell is at least [6dB] better ranked in FR1 for 1 Rx RedCap.

-	when rangeToBestCell is configured:

-	the cell has the highest number of beams above the threshold absThreshSS-BlocksConsolidation among all detected cells whose cell-ranking criterion R value defined in TS38.304 [1] is within rangeToBestCell of the cell-ranking criterion R value of the highest ranked cell.

-	if there are multiple such cells, the cell has the highest rank among them

-	the cell is at least 5dB better ranked in FR1 or 6.5dB better ranked in FR2 if the current serving cell is among them, or 6dB in FR1 or 7.5dB in FR2 for SS-RSRP reselections based on absolute priorities for 2 Rx RedCap or 4dB in FR1 or 4dB in FR2 for SS-RSRQ reselections based on absolute priorities for 2 Rx RedCap.

-	the cell is at least [6dB] better ranked in FR1 if the current serving cell is among them, or [7dB] in FR1 for SS-RSRP reselections based on absolute priorities or [5dB] in FR1 for SS-RSRQ reselections based on absolute priorities for 1 Rx RedCap.

The 1 Rx RedCap UE applies absThreshSS-BlocksConsolidation as the signaled value of absThreshSS-BlocksConsolidation [2] + 1 dB.

When evaluating cells for reselection, the SSB side conditions apply to both serving and inter-frequency cells.

If Treselection timer has a non-zero value and the inter-frequency cell is satisfied with the reselection criteria, the UE shall evaluate this inter-frequency cell for the Treselection time. If this cell remains satisfied with the reselection criteria within this duration, then the UE shall reselect that cell.

The UE is not expected to meet the measurement requirements for an inter-frequency carrier under DRX cycle=320 ms defined in Table 4.2B.2.4-1 for both 1Rx RedCap and 2Rx RedCap under the following conditions:

-	TSMTC_intra = TSMTC_inter = 160 ms; where TSMTC_intra and TSMTC_inter are periodicities of the SMTC occasions configured for the intra-frequency carrier and the inter-frequency carrier respectively, and

-	SMTC occasions configured for the inter-frequency carrier occur up to 1 ms before the start or up to 1 ms after the end of the SMTC occasions configured for the intra-frequency carrier, and

-	SMTC occasions configured for the intra-frequency carrier and for the inter-frequency carrier occur up to 1 ms before the start or up to 1 ms after the end of the paging occasion defined in TS38.304 [1].

For 1 Rx RedCap and 2 Rx RedCap not configured with eDRX_IDLE cycle, Tdetect,NR_Inter_RedCap, Tmeasure,NR_ Inter _RedCap and Tevaluate,NR_ Inter _RedCap are specified in Table 4.2B.2.4.1-1.

For 1 Rx RedCap and 2 Rx RedCap configured with eDRX_IDLE cycle, Tdetect,NR_ Inter _RedCap, Tmeasure,NR_ Inter _RedCap and Tevaluate,NR_ Inter _RedCap are specified in Table 4.2B.2.4-2 and Table 4.2B.2.4-3 for FR1 and FR2 respectively. The requirements apply provided that the serving cell is configured with eDRX_IDLE and is the same in all PTWs during any of Tdetect,NR_ Inter _RedCap, Tmeasure,NR_ Inter _RedCap and Tevaluate,NR_ Inter _RedCap when multiple PTWs are used.

Table 4.2B.2.4-1: Tdetect,NR_Inter_RedCap, Tmeasure,NR_Inter_RedCap and Tevaluate,NR_Inter_RedCap

| DRX cycle length [s] | Scaling Factor (N1) |  | Tdetect,NR_Inter_RedCap [s] (number of DRX cycles) | Tmeasure,NR_Inter_RedCap [s] (number of DRX cycles) | Tevaluate,NR_Inter_RedCap [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2Note1 |  |  |  |
| 0.32 | 1 | 8 | 11.52 x N1 x 1.5 (36 x N1 x 1.5) | 1.28 x N1 x 1.5 (4 x N1 x 1.5) | 5.12 x N1 x 1.5 (16 x N1 x 1.5) |
| 0.64 |  | 5 | 17.92x N1 (28 x N1) | 1.28 x N1 (2 x N1) | 5.12 x N1 (8 x N1) |
| 1.28 |  | 4 | 32 x N1 (25 x N1) | 1.28 x N1 (1 x N1) | 6.4 x N1 (5 x N1) |
| 2.56 |  | 3 | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
| Note 1: Applies for RedCap UE of all FR2 power class. |  |  |  |  |  |

Table 4.2B.2.4-2: Tdetect,NR_Inter_RedCap, Tmeasure,NR_Inter_RedCap and Tevaluate,NR_Inter_RedCap for UE configured with eDRX_IDLE cycle (Frequency range FR1)

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Tdetect,NR_Inter_RedCap [s] (number of DRX cycles or eDRX cycles Note 3) | Tmeasure,NR_Inter_RedCap [s] (number of DRX cycles or eDRX cycles Note 3) | Tevaluate,NR_Inter_RedCap [s] (number of DRX cycles or eDRX cycles Note 3) |
| --- | --- | --- | --- | --- | --- |
| 2.56 | - | - | 58.88 (23) | 2.56 (1) | 7.68 (3) |
| 5.12 | - | - | 117.76 (23) | 5.12 (1) | 10.24 (2) |
| 10.24 | - | - | 235.52 (23) | 10.24 (1) | 20.48 (2) |
| 20.48 ≤ eDRX_IDLE cycle length ≤10485.76 | 0.32 | ≥1.28 (1) | eDRX_cycle_length×23PTW/DRX_cycle_length (23) | 0.32 x 1.5 (1 x 1.5) | 0.64 x 1.5 (2 x 1.5) |
|  | 0.64 | ≥1.28 (1) |  | 0.64 (1) | 1.28 (2) |
|  | 1.28 | ≥2.56 (2) |  | 1.28 (1) | 2.56 (2) |
|  | 2.56 | ≥5.12 (4) |  | 2.56 (1) | 5.12 (2) |
| NOTE 1: The number of DRX cycles in this table is given for the DRX cycles within PTWs. NOTE 2: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [34]. NOTE 3: Number of eDRX cycles when eDRX_IDLE cycle length equals 2.56s, 5.12s and 10.24s. Otherwise, number of DRX cycles. NOTE 4: The lower bound of PTW length is derived based on Tevaluate,NR_Inter_RedCap*DRX_cycle1.28*1.28. |  |  |  |  |  |

Table 4.2B.2.4-3: Tdetect,NR_Inter_RedCap, Tmeasure,NR_Inter_RedCap and Tevaluate,NR_Inter_RedCap for UE configured with eDRX_IDLE cycle (Frequency range FR2)

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Scaling Factor (N1) Note1 | Tdetect,NR_Inter_RedCap [s] (number of DRX cycles or eDRX cycles Note 3) | Tmeasure,NR_Inter_RedCap [s] (number of DRX cycles or eDRX cycles Note 3) |  | Tevaluate,NR_Inter_RedCap [s] (number of DRX cycles or eDRX cycles Note 3) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2.56 | - | - | 3 | 58.88 x N1 (23 x N1) |  | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
| 5.12 | - | - | 3 | 117.76 x N1 (23 x N1) |  | 5.12 x N1 (1 x N1) | 10.24 x N1 (2 x N1) |
| 10.24 | - | - | 3 | 235.52 x N1 (23 x N1) |  | 10.24 x N1 (1 x N1) | 20.48 x N1 (2 x N1) |
| 20.48 ≤ eDRX_IDLE cycle length ≤10485.76 | 0.32 | ≥5.12 (4) | 8 | eDRX_cycle_length×23×N1PTW/DRX_cycle_length (23 x N1) |  | 0.32 x N1 (1 x N1) | 0.64 x N1 (2 x N1) |
|  | 0.64 | ≥6.4 (5) | 5 |  |  | 0.64 x N1 (1 x N1) | 1.28 x N1 (2 x N1) |
|  | 1.28 | ≥10.24 (8) | 4 |  |  | 1.28 x N1 (1 x N1) | 2.56 x N1 (2 x N1) |
|  | 2.56 | ≥15.36 (12) | 3 |  |  | 2.56 x N1 (1 x N1) | 5.12 x N1 (2 x N1) |
| NOTE 1: Applies for RedCap UE of all power class. NOTE 2: The number of DRX cycles in this table is given for the DRX cycles within PTWs. NOTE 3: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [34]. NOTE 4: Number of eDRX cycles when eDRX_IDLE cycle length equals 2.56s, 5.12s and 10.24s. Otherwise, number of DRX cycles. NOTE 5: The lower bound of PTW length is derived based on Tevaluate,NR_Inter_RedCap*DRX_cycle1.28*1.28. NOTE 6: When eDRX=20.48s and DRX=0.32s, UE is allowed to perform cell evaluation within PTW in every 2 eDRX cycles. |  |  |  |  |  |  |  |

For any requirement in this section, when the UE transitions between any two states when being configured with eDRX_IDLE, being configured with eDRX_IDLE cycle, changing eDRX_IDLE cycle length, or changing PTW configuration, the UE shall meet the transition requirement, which is the less stringent requirement of the two requirements corresponding to the first state and the second state, during the transition time interval which is the time corresponding to the transition requirement. After the transition time interval, the UE shall meet the requirement corresponding to the second state.

----------------------END OF CHANGES 1----------------------------

----------------------START OF CHANGE 2----------------------------

#### 4.2B.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion for RedCap

##### 4.2B.2.9.1	Introduction

This clause contains the requirements for measurements on intra-frequency NR cells when Srxlev ≤ SIntraSearchP or Squal ≤ SIntraSearchQ and when the UE is configured any of the following relaxed measurement critera:

-	Relaxed measurement criterion for a stationary UE defined in clause 5.2.4.9.3 in [1],

-	Relaxed measurement criteria for a stationary UE and not-at-cell edge defined in clause 5.2.4.9.4 in [1],

-	Both low mobility criterion and stationary criterion as defined in clause 5.2.4.9.1 and 5.2.4.9.3 or 5.2.4.9.4 in [1] respectively.

-	Relaxed measurement criterion for UE with low mobility defined in clause 5.2.4.9.1 in [1],

-	Relaxed measurement criterion for UE not-at-cell edge defined in clause 5.2.4.9.2 in [1],

-	Both low mobility criterion and not-at-cell edge criterion as defined in clauses 5.2.4.9.1 and 5.2.4.9.2 in [1] respectively.

----------------------END OF CHANGES 2----------------------------

----------------------START OF CHANGE 3----------------------------

#### 4.2B.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion

##### 4.2B.2.10.1	Introduction

This clause contains the requirements for measurements on inter-frequency NR cells when the UE is configured any of the following relaxed measurement critera:

-	Relaxed measurement criterion for a stationary UE defined in clause 5.2.4.9.3 in [1],

-	Relaxed measurement criterion for a stationary UE not at cell edge defined in clause 5.2.4.9.4 in [1],

-	Both low mobility criterion and stationary criterion as defined in clause 5.2.4.9.1 and 5.2.4.9.3 or 5.2.4.9.4 in [1] respectively.

-	Relaxed measurement criterion for UE with low mobility defined in clause 5.2.4.9.1 in [1],

-	Relaxed measurement criterion for UE not-at-cell edge defined in clause 5.2.4. 9.2 in [1],

-	Both low mobility criterion and not-at-cell edge criterion as defined in clauses 5.2.4. 9.1 and 5.2.4.9.2 in [1] respectively.

##### 4.2B.2.10.2	Measurements for UE fulfilling stationary criterion

This clause contains requirements for measurements on inter-frequency NR cells provided that:

-	UE is configured with stationaryMobilityEvaluation [2] criterion and UE has fulfilled that criterion, or

-	UE is configured with both stationaryMobilityEvaluation [2] criterion and cellEdgeEvaluationWhileStationary [2] criterion and combineRelaxedMeasCondition2 [2] not configured, and UE has fulfilled only the stationaryMobilityEvaluation [2] criterion, and

When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the requirements defined in clause 4.2B.2.4 apply for this clause except that:

-	Tdetect,NR_Inter_RedCap_Relax as specified in Table 4.2B.2.10.2-1 and Table 4.2B.2.10.2-2 for 1 Rx RedCap and 2 Rx RedCap respectively.

-	Tmeasure,NR_Inter_RedCap_Relax as specified in Table 4.2B.2.10.2-1 and Table 4.2B.2.10.2-2 for 1 Rx RedCap and 2 Rx RedCap respectively.

-	Tevaluate,NR_Inter_RedCap_Relax as specified in Table 4.2B.2.10.2-1 and Table 4.2B.2.10.2-2 for 1 Rx RedCap and 2 Rx RedCap respectively.

-

If the UE is configured with eDRX_IDLE cycle then the requirements in Table 4.2B.2.10.2-3 and Table 4.2B.2.10.2-4 are applicable for eDRX cycle up to 10.24 s in FR1 and FR2 respectively.

If the UE is configured with eDRX_IDLE cycle greater than 10.24 s in FR1 and FR2, then the requirements in Table Table 4.2B.2.10.2-5 and Table 4.2B.2.10.2-6 respectively apply provided that eDRX cycle is ≤ [163.84] sec and evaluation/measurement time with relaxation on one carrier is not greater than single PTW window length.

When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-frequency layers of higher priority at least every K2*Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7 and, K2 = 240.

Table 4.2B.2.10.2-1: Tdetect,NR_Inter_RedCap_Relax, Tmeasure,NR_Inter_RedCap_Relax and Tevaluate,NR_Inter_RedCap_Relax for 1 Rx RedCap UE

| DRX cycle length [s] | Tdetect,NR_Inter_RedCap_Relax [s] (number of DRX cycles) | Tmeasure,NR_Inter_RedCap_Relax [s] (number of DRX cycles) | Tevaluate,NR_Inter_RedCap_Relax [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 11.52 x 1.5 x K4(36 x 1.5 x K4) | 1.28 x 1.5 x K4 (4 x 1.5 x K4) | 5.12 x 1.5 x K4 (16 x 1.5 x K4) |
| 0.64 | 17.92 x K4 (28 x K4) | 1.28x K4 (2 x K4) | 5.12 x K4 (8 x K4) |
| 1.28 | 32 x K4 (25 x K4) | 1.28x K4 (1 x K4) | 6.4 x K4 (5 x K4) |
| 2.56 | 58.88 x K4 (23 x K4) | 2.56 x K4 (1 x K4) | 7.68 x K4 (3 x K4) |
| Note 1: K4 = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. |  |  |  |

Table 4.2B.2.10.2-2: Tdetect,NR_Inter_RedCap_Relax, Tmeasure,NR_Inter_RedCap_Relax and Tevaluate,NR_Inter_RedCap_Relax for 2 Rx RedCap UE

| DRX cycle length [s] | Scaling Factor (N1) |  | Tdetect,NR_Inter_Relax [s] (number of DRX cycles) | Tmeasure,NR_Inter_Relax [s] (number of DRX cycles) | Tevaluate,NR_Inter_Relax [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2Note1 |  |  |  |
| 0.32 | 1 | 8 | 11.52 x N1 x 1.5 x K4 (36 x N1 x 1.5 x K4) | 1.28 x N1 x 1.5 x K4 (4 x N1 x 1.5 x K4) | 5.12 x N1 x 1.5 x K4 (16 x N1 x 1.5 x K4) |
| 0.64 |  | 5 | 17.92x N1 x K4 (28 x N1 x K4) | 1.28 x N1 x K4 (2 x N1 x K4) | 5.12 x N1 x K4 (8 x N1 x K4) |
| 1.28 |  | 4 | 32 x N1 x K4 (25 x N1 x K4) | 1.28 x N1 x K4 (1 x N1 x K4) | 6.4 x N1 x K4 (5 x N1 x K4) |
| 2.56 |  | 3 | 58.88 x N1 x K4 (23 x N1 x K4) | 2.56 x N1 x K4 (1 x N1 x K4) | 7.68 x N1 x K4 (3 x N1 x K4) |
| Note 1: Applies for RedCap UE of all supporting power class. Note 2: K4 = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. |  |  |  |  |  |

Table 4.2B.2.10.2-3: Tdetect, NR_Inter _RedCap_Relax, Tmeasure,NR_,NR_Inter _RedCap_Relax and Tevaluate, NR_Inter _RedCap_Relax for UE configured with eDRX_IDLE cycle (Frequency range FR1)

| eDRX_IDLE cycle length [s] | Tdetect, NR_Inter_RedCap_Relax [s] (number of DRX cycles) | Tmeasure, NR_Inter_RedCap_Relax [s] (number of DRX cycles) | Tevaluate, NR_Inter_RedCap_Relax [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 2.56 | 58.88 x K3 (23 x K3) | 2.56 x K3 (1 x K3) | 5.12 x K3 (2 x K3) |
| 5.12 | 117.76 x K3 (23 x K3) | 5.12 x K3 (1 x K3) | 10.24 x K3 (2 x K3) |
| 10.24 | 235.52 x K3 (23 x K3) | 10.24 x K3 (1 x K3) | 20.48 x K3 (2 x K3) |
| Note 1: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. If different SMTC periodicities are configured for different cells, the SMTC periodicity in this note is the one used by the cell being identified. During PSS/SSS detection, the periodicity of the SMTC configured for the intra-frequency carrier is assumed, and if the actual SSB transmission periodicity is greater than the SMTC configured for the intra-frequency carrier, longer Tdetect, NR_intra is expected. Note 2: K3 = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. |  |  |  |

Table 4.2B.2.10.2-4: Tdetect,NR_Inter_RedCap_Relax, Tmeasure,NR_Inter_RedCap_Relax and Tevaluate,NR_Inter_RedCap_Relax for UE configured with eDRX_IDLE cycle (Frequency range FR2) for eDRX_IDLE cycle upto 10.24 s

| eDRX_IDLE cycle length [s] | Tdetect,NR_Inter_RedCap_Relax [s] (number of DRX cycles) | Tmeasure,NR_Inter_RedCap_Relax [s] (number of DRX cycles) | Tevaluate,NR_Inter_RedCap_Relax [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 2.56 | 58.88 x N1 x K3 (23 x N1 x K3) | 2.56 x N1 x K3 (1 x N1 x K3) | 7.68 x N1 x K3 (3 x N1 x K3) |
| 5.12 | 117.76 x N1 x K3 (23 x N1 x K3) | 5.12 x N1 x K3 (1 x N1 x K3) | 10.24 x N1 x K3 (2 x N1 x K3) |
| 10.24 | 235.52 x N1 x K3 (23 x N1 x K3) | 10.24 x N1 x K3 (1 x N1 x K3) | 20.48 x N1 x K3 (2 x N1 x K3) |
| Note 1: K3 = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. |  |  |  |

Table 4.2B.2.10.2-5: Tdetect,NR_Inter_RedCap_Relax, Tmeasure,NR_ Inter _RedCap_Relax and Tevaluate,NR_ Inter _RedCap_Relax for UE configured with eDRX_IDLE cycle (Frequency range FR1) for eDRX_IDLE cycle larger than 10.24 s

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Tdetect,NR_Inter_RedCap_Relax [s] (number of DRX cycles or eDRX cycles Note 3) | Tmeasure,NR_Inter_RedCap_Relax [s] (number of DRX cycles or eDRX cycles Note 3) | Tevaluate,NR_Inter_RedCap_Relax [s] (number of DRX cycles or eDRX cycles Note 3) |
| --- | --- | --- | --- | --- | --- |
| 20.48 ≤ eDRX_IDLE cycle length ≤[163.84] | 0.32 | ≥[6.4] ([5]) | eDRX_cycle_length×23PTW/DRX_cycle_lengthx K3 (23 x K3) | 0.32 x M2 x K3 (1 x M2 x K3) | 0.64 x M2 x K3 (2 x M2 x K3) |
|  | 0.64 | ≥[12.8] ([10]) |  | 0.64 x K3 (1 x K3) | 1.28 x K3 (2 x K3) |
|  | 1.28 | ≥[15.36] ([12]) |  | 1.28 x K3 (1 x K3) | 2.56 x K3 (2 x K3) |
|  | 2.56 | ≥[30.72] ([4]) |  | 2.56 x K3 (1 x K3) | 5.12 x K3 (2 x K3) |
| Note 1: The number of DRX cycles in this table is given for the DRX cycles within PTWs. Note 2: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [34]. Note 3: The lower bound of PTW length is derived based on Tevaluate,NR_Inter_RedCap*DRX_cycle1.28*1.28. Note 4: K3 = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. |  |  |  |  |  |

Table 4.2B.2.10.2-6: Tdetect,NR_Inter_RedCap_Relax, Tmeasure,NR_Inter_RedCap_Relax and Tevaluate,NR_Inter_RedCap_Relax for UE configured with eDRX_IDLE cycle (Frequency range FR2) for eDRX_IDLE cycle larger than 10.24 s

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Scaling Factor (N1) Note1 | Tdetect,NR_Inter_RedCap_Relax [s] (number of DRX cycles or eDRX cycles Note 3) | Tmeasure,NR_Inter_RedCap_Relax [s] (number of DRX cycles or eDRX cycles Note 3) |  | Tevaluate,NR_Inter_RedCap_Relax [s] (number of DRX cycles or eDRX cycles Note 3) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 20.48 ≤ eDRX_IDLE cycle length ≤[163.84] | 0.32 | ≥30.72 (24) | 8 | K3 x eDRX_cycle_length×23×N1PTW/DRX_cycle_length (23 x N1 x K3) |  | 0.32 x N1 x K3 (1 x N1 x K3) | 0.64 x N1 x K3 (2 x N1 x K3) |
|  | 0.64 | ≥38.4 (30) | 5 |  |  | 0.64 x N1 x K3 (1 x N1 x K3) | 1.28 x N1 x K3 (2 x N1 x K3) |
|  | 1.28 | 40.96 (32) | 4 |  |  | 1.28 x N1 x K3 (1 x N1 x K3) | 2.56 x N1 x K3 (2 x N1 x K3) |
|  | 2.56 | 40.96 (32) | 3 |  |  | 20.48 | 40.96 |
| Note 1: Applies for RedCap UE of all supporting FR2 power classes. Note 2: The number of DRX cycles in this table is given for the DRX cycles within PTWs. Note 3: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [34]. Note 4: The lower bound of PTW length is derived based on Tevaluate,NR_Inter_RedCap*DRX_cycle1.28*1.28. Note 5: For DRX cycle length is 0.32s, 0.64s and 2.56s, K3 = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. For DRX cycle length is 1.28s, K3 = 4 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. |  |  |  |  |  |  |  |

----------------------END OF CHANGES 3----------------------------

----------------------START OF CHANGE 4----------------------------

##### 4.2B.2.10.3A	Measurements for a UE fulfilling stationary and Rel-16 not at cell edge criterion

This clause contains requirements for measurements on inter-frequency NR cells provided that:

-	UE is configured with both stationaryMobilityEvaluation [2] criterion and cellEdgeEvaluation [2] criterion, and-	has also fulfilled both criteria, or

-	UE is configured with cellEdgeEvaluation [2] criterion and with both stationaryMobilityEvaluation [2] criterion and cellEdgeEvaluationWhileStationary [2] criterion and combineRelaxedMeasCondition2 [2] not configured, and UE has fulfilled cellEdgeEvaluation and stationaryMobilityEvaluation [2] criteria

When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the requirements defined in clause 4.2B.2.4 apply for this clause except that:

-	Tdetect,NR_Inter_RedCap_Relax as specified in Table 4.2B.2.10.2-1 and Table 4.2B.2.10.2-2 for 1 Rx RedCap and 2 Rx RedCap respectively.

-	Tmeasure,NR_Inter_RedCap_Relax as specified in Table 4.2B.2.10.2-1 and Table 4.2B.2.10.2-2 for 1 Rx RedCap and 2 Rx RedCap respectively.

-	Tevaluate,NR_Inter_RedCap_Relax as specified in Table 4.2B.2.10.2-1 and Table 4.2B.2.10.2-2 for 1 Rx RedCap and 2 Rx RedCap respectively.

If the UE is configured with eDRX_IDLE cycle then the requirements in Table 4.2B.2.10.2-3 and Table 4.2B.2.10.2-4 are applicable for eDRX cycle up to 10.24 s in FR1 and FR2 respectively.

If the UE is configured with eDRX_IDLE cycle greater than 10.24 s in FR1 and FR2, then the requirements in Table Table 4.2B.2.10.2-5 and Table 4.2B.2.10.2-6 respectively apply provided that eDRX cycle is ≤ [163.84] sec and evaluation/measurement time with relaxation on one carrier is not greater than single PTW window length.

When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-frequency layers of higher priority at least every K2*Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7 and, K2 = 240.

----------------------END OF CHANGES 4----------------------------

----------------------START OF CHANGE 5----------------------------

##### 4.2B.2.10.8	Measurements for a UE fulfilling low mobility not-at-cell edge criterion and stationary criteria

This clause contains requirements for measurements on inter-frequency NR cells provided that:

-	UE is configured with both lowMobilityEvaluation [2] criterion and cellEdgeEvaluation [2] criterion, and has fulfilled both criteria, and

-	UE is configured with stationaryMobilityEvaluation [2] criterion and has also fulfilled that criterion, or UE is configured with both stationaryMobilityEvaluation [2] criterion and cellEdgeEvaluationWhileStationary [2] criterion and combineRelaxedMeasCondition2 [2] not configured, and UE has fulfilled stationaryMobilityEvaluation [2] criterion.

In this case the UE is not required to meet Tdetect,NR_Inter_RedCap, Tmeasure,NR_Inter_RedCap and Tevaluate,NR_Inter_RedCap as defined in clause 4.2B.2.4.

When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, the UE shall search for, measure and evaluate inter-frequency layers of higher, equal or lower priority at least every 1 hour.

When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the UE shall search for inter-frequency layers of higher priority at least every K2*Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7 and K2=240.

----------------------END OF CHANGES 5----------------------------

----------------------START OF CHANGE 6----------------------------

##### 4.2B.2.10.11	Measurements for UE fulfilling low mobility and not-at-cell edge criterion

This clause contains requirements for measurements on inter-frequency NR cells provided that:

-	UE is configured with both lowMobilityEvaluation [2] criterion and cellEdgeEvaluation [2] criterion, and

-	Has also fulfilled both criteria, and

-	less than 1 hour have passed since measurements for cell reselection were last performed

In this case the UE is not required to meet Tdetect,NR_Inter_RedCap, Tmeasure,NR_Inter_RedCap and Tevaluate,NR_Inter_RedCap as defined in clause 4.2B.2.4.

When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, the UE shall search for, measure and evaluate inter-frequency layers of higher, equal or lower priority at least every 1 hours.

When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the UE shall search for inter-frequency layers of higher priority at least every K2*Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7 and K2=60.

The requirments defined in this clause apply regardless of eDRX_IDLE configurations.

----------------------END OF CHANGES 6----------------------------

----------------------START OF CHANGE 7----------------------------

#### 4.2B.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion

##### 4.2B.2.11.1	Introduction

This clause contains the requirements for measurements on inter-RAT E-UTRAN cells when the UE is configured any of the following relaxed measurement critera:

-	Relaxed measurement criterion for a stationary UE defined in clause 5.2.4.9.X in [1],

-	Relaxed measurement criterion for a stationary UE not at cell edge defined in clause 5.2.4.9.Y in [1],

-	Both low mobility criterion and stationary criterion as defined in clause 5.2.4.9.1 and 5.2.4.9.X in [1] respectively.

-	Relaxed measurement criterion for UE with low mobility defined in clause 5.2.4.9.1 in [1],

-	Relaxed measurement criterion for UE not-at-cell edge defined in clause 5.2.4.9.2 in [1],

-	Both low mobility criterion and not-at-cell edge criterion as defined in clauses 5.2.4.9.1 and 5.2.4.9.2 in [1] respectively.

##### 4.2B.2.11.2	Measurements for UE fulfilling stationary criterion

This clause contains requirements for measurements on inter-RAT E-UTRAN cells provided that:

-	UE is configured with stationaryMobilityEvaluation [2] criterion and UE has fulfilled that criterion, or

-	UE is configured with both stationaryMobilityEvaluation [2] criterion and cellEdgeEvaluationWhileStationary [2] criterion and combineRelaxedMeasCondition2 [2] not configured, and UE has fulfilled only the stationaryMobilityEvaluation [2] criterion, and

When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the requirements defined in clause 4.2B.2.5 apply for this clause except that:

-	Tdetect,EUTRAN_RedCap Relax as specified in Table 4.2B.2.11.2-1 and Table 4.2B.2.11.2-2 for 1 Rx RedCap and 2 Rx RedCap respectively.

-	Tmeasure,EUTRAN RedCap Relax as specified in Table 4.2B.2.11.2-1 and Table 4.2B.2.11.2-2 for 1 Rx RedCap and 2 Rx RedCap respectively.

-	Tevaluate,EUTRAN RedCap Relax as specified in Table 4.2B.2.11.2-1 and Table 4.2B.2.11.2-2 for 1 Rx RedCap and 2 Rx RedCap respectively.

If the UE is configured with eDRX_IDLE cycle then the requirements in Table 4.2B.2.11.2-3 are applicable for eDRX cycle < 10.24 s.

If the UE is configured with eDRX_IDLE cycle ≥ 10.24 s, then the requirements in Table 4.2B.2.11.2-4 apply provided that filtering of a measurement is done within a single PTW and provided that the eDRX cycle is ≤ [163.84] sec and evaluation/measurement time with relaxation on one carrier is not greater than single PTW window length.

When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for E-UTRA inter-RAT frequency layers of higher priority at least every K2*Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7 and, K2 = 240.

Table 4.2B.2.11.2-1: Tdetect,EUTRAN_RedCap_Relax, Tmeasure,EUTRAN_RedCap_Relax, and Tevaluate,EUTRAN_RedCap_Relax for 1 Rx RedCap

| DRX cycle length [s] | Tdetect,EUTRAN_Relax [s] (number of DRX cycles) | Tmeasure,EUTRAN_Relax [s] (number of DRX cycles) | Tevaluate,EUTRAN_Relax [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 11.52 x K5 (36 x K5) | 1.28 x K5 (4 x K5) | 5.12 x K5 (16 x K5) |
| 0.64 | 17.92 x K5 (28 x K5) | 1.28 x K5 (2 x K5) | 5.12 x K5 (8 x K5) |
| 1.28 | 32 x K5 (25 x K5) | 1.28 x K5 (1 x K5) | 6.4 x K5 (5 x K5) |
| 2.56 | 58.88 x K5 (23 x K5) | 2.56 x K5 (1 x K5) | 7.68 x K5 (3 x K5) |
| Note 1: K5 = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. |  |  |  |

Table 4.2B.2.11.2-2: Tdetect,EUTRAN_Relax, Tmeasure,EUTRAN_Relax, and Tevaluate,EUTRAN_Relax for 2 Rx RedCap

| DRX cycle length [s] | Tdetect,EUTRAN_Relax [s] (number of DRX cycles) | Tmeasure,EUTRAN_Relax [s] (number of DRX cycles) | Tevaluate,EUTRAN_Relax [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 11.52 x K5 (36 x K5) | 1.28 x K5 (4 x K5) | 5.12 x K5 (16 x K5) |
| 0.64 | 17.92 x K5 (28 x K5) | 1.28 x K5 (2 x K5) | 5.12 x K5 (8 x K5) |
| 1.28 | 32 x K5 (25 x K5) | 1.28 x K5 (1 x K5) | 6.4 x K5 (5 x K5) |
| 2.56 | 58.88 x K5 (23 x K5) | 2.56 x K5 (1 x K5) | 7.68 x K5 (3 x K5) |
| Note 1: K5 = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. |  |  |  |

Table 4.2B.2.11.2-3: Tdetect,E-UTRAN _RedCap_Relax, Tmeasure,NR_,E-UTRAN _RedCap_Relax and Tevaluate,NR_,E-UTRAN _RedCap_Relax for UE configured with eDRX_IDLE cycle

| eDRX_IDLE cycle length [s] | Tdetect,NR_E-UTRAN_RedCap_Relax [s] (number of DRX cycles) | Tmeasure,NR_E-UTRAN_RedCap_Relax [s] (number of DRX cycles) | Tevaluate,NR_E-UTRAN_RedCap_Relax [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 5.12 | 117.76 x K3 (23 x K3) | 5.12 x K3 (1 x K3) | 10.24 x K3 (2 x K3) |
| Note 1: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. If different SMTC periodicities are configured for different cells, the SMTC periodicity in this note is the one used by the cell being identified. During PSS/SSS detection, the periodicity of the SMTC configured for the intra-frequency carrier is assumed, and if the actual SSB transmission periodicity is greater than the SMTC configured for the intra-frequency carrier, longer Tdetect, NR_intra is expected. Note 2: K3 = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. |  |  |  |

Table 4.2B.2.11.2-4: Tdetect,E-UTRAN _RedCap_Relax, Tmeasure,NR_,E-UTRAN _RedCap_Relax and Tevaluate,NR_,E-UTRAN _RedCap_Relax for UE configured with eDRX_IDLE cycle

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Tdetect,EUTRAN_RedCap_Relax [s] (number of DRX or eDRX cycles Note 3) | Tmeasure,EUTRAN_RedCap_Relax [s] (number of DRX or eDRX cycles Note 3) | Tevaluate,E-UTRAN_RedCap_Relax [s] (number of DRX or eDRX cycles Note 3) |
| --- | --- | --- | --- | --- | --- |
| 10.24 ≤ eDRX_IDLE cycle length ≤ 2621.444 | 0.32 | ≥1.28 (1) | K3 x (23 x K3) | 0.32 x K3 (1 x K3) | 0.64 x K3 (2 x K3) |
|  | 0.64 | ≥1.28 (1) |  | 0.64 x K3 (1 x K3) | 1.28 x K3 (2 x K3) |
|  | 1.28 | ≥2.56 (2) |  | 1.28 x K3 (1 x K3) | 2.56 x K3 (2 x K3) |
|  | 2.56 | ≥5.12 (4) |  | 2.56 x K3 (1 x K3) | 5.12 x K3 (2 x K3) |
| NOTE 1: The number of DRX cycles in this table is given for the DRX cycles within PTWs. NOTE 2: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [34]. NOTE 3: Number of eDRX cycles when eDRX_IDLE cycle length equals 5.12s, number of DRX cycles otherwise. NOTE 4: The lower bound of PTW length is derived based on Tevaluate,E-UTRAN_RedCap*DRX_cycle1.28*1.28. NOTE 5: K3 = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion. |  |  |  |  |  |

##### 4.2B.2.11.3	Measurements for a UE fulfilling stationary not at cell edge criterion

This clause contains requirements for measurements on inter-RAT E-UTRAN cells provided that:

-	UE is configured with both stationaryMobilityEvaluation [2] criterion and cellEdgeEvaluationWhileStationary [2] criterion, and

-	has also fulfilled both criteria, and

-	less than 4 hours have passed since measurements for cell reselection were last performed, and

In this case the UE is not required to meet Tdetect,EUTRAN, Tmeasure,EUTRAN and Tevaluate,EUTRAN as defined in clause 4.2B.2.5.

When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, the UE shall search for, measure and evaluate inter-RAT E-UTRAN layers of higher or lower priority at least every 4 hour.

When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the UE shall search for inter-RAT E-UTRAN of higher priority at least every K2*Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7 and K2=240.

The requirments defined in this clause apply regardless of eDRX_IDLE configurations.

##### 4.2B.2.11.3A	Measurements for a UE fulfilling stationary and Rel-16 not at cell edge criterion

This clause contains requirements for measurements on inter-RAT E-UTRAN cells provided that:

-	UE is configured with both stationaryMobilityEvaluation [2] criterion and cellEdgeEvaluation [2] criterion, and has also fulfilled both criteria, or

-	UE is configured with cellEdgeEvaluation [2] criterion and with both stationaryMobilityEvaluation [2] criterion and cellEdgeEvaluationWhileStationary [2] criterion and combineRelaxedMeasCondition2 [2] not configured, and UE has fulfilled cellEdgeEvaluation and stationaryMobilityEvaluation [2] criterion.

When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the requirements defined in clause 4.2B.2.5 apply for this clause except that:

-	Tdetect,EUTRAN_Relax as specified in Table 4.2B.2.11.2-1 and Table 4.2B.2.11.2-2 for 1 Rx RedCap and 2 Rx RedCap respectively.

-	Tmeasure,EUTRAN_Relax as specified in Table 4.2B.2.11.2-1 and Table 4.2B.2.11.2-2 for 1 Rx RedCap and 2 Rx RedCap respectively.

-	Tevaluate,EUTRAN_Relax as specified in Table 4.2B.2.11.2-1 and Table 4.2B.2.11.2-2 for 1 Rx RedCap and 2 Rx RedCap respectively.

If the UE is configured with eDRX_IDLE cycle then the requirements in Table 4.2B.2.11.2-3 are applicable for eDRX cycle < 10.24 s.

If the UE is configured with eDRX_IDLE cycle ≥ 10.24 s, then the requirements in Table 4.2B.2.11.2-4 apply provided that filtering of a measurement is done within a single PTW and provided that the eDRX cycle is ≤ [163.84] sec and evaluation/measurement time with relaxation on one carrier is not greater than single PTW window length.

When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the UE shall search for inter-RAT E-UTRAN of higher priority at least every K2*Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7 and K2=240.

----------------------END OF CHANGES 7----------------------------

----------------------START OF CHANGE 8----------------------------

##### 4.2B.2.11.8	Measurements for a UE fulfilling low mobility not-at-cell edge criterion and stationary criteria

This clause contains requirements for measurements on inter-RAT E-UTRAN cells provided that:

-	UE is configured with both lowMobilityEvaluation [2] criterion and cellEdgeEvaluation [2] criterion, and has fulfilled both criteria, and

-	UE is configured with stationaryMobilityEvaluation [2] criterion and has also fulfilled that criterion, or UE is configured with both stationaryMobilityEvaluation [2] criterion and cellEdgeEvaluationWhileStationary [2] criterion and combineRelaxedMeasCondition2 [2] not configured, and UE has fulfilled stationaryMobilityEvaluation [2] criteria.

In this case the UE is not required to meet Tdetect,NR_Inter_RedCap, Tmeasure,NR_Inter_RedCap and Tevaluate,NR_Inter_RedCap as defined in clause 4.2B.2.5.

When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, the UE shall search for, measure and evaluate inter-RAT E-UTRAN layers of higher, equal or lower priority at least every 1 hour.

When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the UE shall search for inter-RAT E-UTRAN layers of higher priority at least every K2*Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7 and K2=240.

----------------------END OF CHANGES 8----------------------------

----------------------START OF CHANGE 9----------------------------

##### 4.2B.2.11.11	Measurements for UE fulfilling low mobility and not-at-cell edge criterion

This clause contains requirements for measurements on inter-RAT E-UTRAN cells provided that:

-	UE is configured with both lowMobilityEvalutation [2] criterion and cellEdgeEvaluation [2] criterion, and

-	has also fulfilled both criteria, and

-	less than 1 hour have passed since measurements for cell reselection were last performed,

In this case the UE is not required to meet Tdetect,EUTRAN_RedCap, Tmeasure,EUTRAN_RedCap  and Tevaluate,EUTRAN_RedCap as defined in clause 4.2B.2.5.

When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, the UE shall search for, measure and evaluate inter-RAT E-UTRAN layers of higher, equal or lower priorivty at least every 1 hour.

When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the UE shall search for inter-RAT E-UTRAN layers of higher priority at least every K2*Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7 and K2=60.

The requirments defined in this clause apply regardless of eDRX_IDLE configurations.

----------------------END OF CHANGES 9----------------------------

----------------------START OF CHANGE 10----------------------------

## 5.2B	Configured Grant based Small Data Transmissions (CG-SDT) for RedCap

### 5.2B.1	Introduction

This section contains the requirements for Small Data Transmissions (SDT) for 1 Rx RedCap and 2 Rx RedCap.

The 1 Rx RedCap UE for determining whether to perform SDT procedure defined in clause 5.27 [7] applies:

-	sdt-RSRP-Threshold-r17 as the signaled value of sdt-RSRP-Threshold-r17 [2] + 1 dB.

### 5.2B.2	Requirements on UE synchronization for small data transmissions for RedCap

The requirements in clause 5.5.2 shall apply for RedCap UEs.

#### 5.2B.2.1	Void

### 5.2B.3	TA validation requirements for RedCap

When cg-SDT-RSRP-ChangeThreshold [TS 38.331] is configured for TA validation based on the RSRP change criterion according to clause 5.27.2 in [TS 38.321], the UE is allowed to transmit using CG-SDT using the timing derived using the latest available NTA value as specified in subclause 7.1 provided that

-	the first RSRP (RSRP1) measurement and the second RSRP (RSRP2) measurements used in the TA validation are valid measurements and,

-	timing alignment validation for transmission using CG-SDT is valid according to the validation criteria in clause 5.27.2 in [TS 38.321].

RSRP1 and RSRP2 are considered valid provided that the conditions in Table 5.2B.2-1 are met for FR1 when UE is not configured with any eDRX cycle by serving gNB or core network.

RSRP1 and RSRP2 are considered valid provided that the conditions in Table 5.2B.2-2 are met for FR2-1 regardless of whether the UE is configured with eDRX cycle by serving gNB or core network (AMF).

RSRP1 and RSRP2 are considered valid provided that the conditions in Table 5.2B.2-3 are met for FR1 when UE is configured:

-	only with RAN eDRX cycle (TeDRX-RAN) by the serving gNB, or

-	with both TeDRX-RAN by serving gNB and CN eDRX cycle (TeDRX-CN) by the core network (AMF)

Table 5.2B.3-1 Valid measurement for FR1 without eDRX cycle

| Measurement | FR1 |
| --- | --- |
| RSRP1 | (T1 – min(640ms, M1*TDRX)) ≤ T1’ ≤ (T1 + min(640ms, M1*TDRX)) |
| RSRP2 | (T2 – min(640ms, M1*TDRX)) ≤ T2’ ≤ T2 |

Table 5.2B.3-2 Valid measurement for FR2-1

| Measurement | FR2-1 |
| --- | --- |
| RSRP1 | (T1 – max{480ms, 8*SMTC periodicity}) ≤ T1’ ≤ (T1 + max{480ms, 8*SMTC periodicity}) |
| RSRP2 | (T2 – max{480ms, 8*SMTC periodicity}) ≤ T2’ ≤ T2 |

Table 5.2B.3-3 Valid measurement for FR1 with eDRX cycle

| Measurement | FR1 |
| --- | --- |
| RSRP1 | (T1 – min(640ms, M1* T)) ≤ T1’ ≤ (T1 + min(640ms, M1* T)) |
| RSRP2 | (T2 – min(640ms, M1* TeDRX-RAN)) ≤ T2’ ≤ T2 |
| Note 1: T is determined according to clause 7.1 in [1]. |  |

If at least one of RSRP1 and RSRP2 is considered to be invalid based on the above conditions, then the UE shall not validate the CG-SDT using RSRP1 and RSRP2 and shall not transmit using CG-SDT. Additionally, for the initial CG-SDT transmission, the UE shall not transmit in an CG-SDT occasion that occurs more than 640 ms after T2.

Where:

-	T1 is the time when NTA

-	RRCRelease with CG-SDT configuration (TS 38.331 [2]) is received, or

-	the latest TA is received if TA is received while in RRC_INACITVE state.

-	T1’ is the time when the UE has completed RSRP1.

-	T2 is the time when the UE performs TA validation as defined in clause 5.27.2  in [TS 38.321] for transmission using CG-SDT.

-	T2’ is the time when the UE has completed RSRP2.

-	TDRX is the DRX cycle length in ms.

-	M1 the scaling factor as defined in clause 4.2.2.2.

#### 5.2B.2.2	Void

#### 5.2B.2.3	Void

### 5.2B.4	Scheduling restriction

The requirements in clause 5.5.4 shall apply for RedCap UEs.

### 5.2B.5	Applicability conditions for CG-SDT for RedCap

The UE is not required to meet the following measurement requirements during subsequent SDT transmissions:

-	Measurements of inter-frequency NR cells in clause 5.1B.2.4

-	Measurements of inter-RAT E-UTRAN cells in clause 5.1B.2.5

----------------------END OF CHANGES 10----------------------------

----------------------START OF CHANGE 11----------------------------

## 5.7B	Random access based Small Data Transmissions (RA-SDT) for RedCap

### 5.7B.1	Introduction

The requirements in this clause are applicable for the UE performing small data transmissions 1 Rx RedCap and 2 Rx RedCap UE using 2-step RA or 4-step RA procedures [3].

The 1 Rx RedCap UE for determining whether to perform SDT procedure defined in clause 5.27 [7] applies:

-	sdt-RSRP-Threshold-r17 as the signaled value of sdt-RSRP-Threshold-r17 [2] + 1 dB.

### 5.7B.2	Requirements for small data transmissions based on 2-step RA

The requirements in clause 6.2.2.3 shall apply.

### 5.7B.3	Requirements for small data transmissions based on 4-step RA

The requirements in clause 6.2.2.2 shall apply.

### 5.7B.4	Applicability conditions for RA-SDT for RedCap

The UE is not required to meet the following measurement requirements during SDT transmissions:

-	Measurements of inter-frequency NR cells in clause 5.1B.2.4

-	Measurements of inter-RAT E-UTRAN cells in clause 5.1B.2.5

----------------------END OF CHANGES 11----------------------------