3GPP TSG-RAN WG4 Meeting # 107 										   R4-2309581

# Incheon, 22 May 2023 – 26 May 2023

Title:	Discussion on FeMIMO maintenance

Agenda item:			5.2.3.1

Source:	Ericsson

Document for:	Discussion

# Introduction

In this paper we disucss the remaining open issues in FeMIMO

# Discussion

Whether UE need to track UL time/frequency if source RS in UL TCI state is not in the DL active TCI list

This issue was discussed for many meetings and in last meeting follwing is tentatively agreed. Since his is maintenance stage of the WI, we are fine with tentative agreement.

[Do not define requirements for the case if source RS in UL TCI state is not in DL active TCI state list]

- RAN4 not to define requirements for the case source RS in UL TCI state is not in DL active TCI state list.

MAC-CE based UL TCI state switching delay when SSB is indicated as PL-RS in UL TCI state for FR2

- Proposals

  - Proposal 1(Apple, Huawei, Samsung):

    - When PL-RS in UL TCI state switch is SSB in FR2, longer delay is expected.

    - If no consensus can be achieved in RAN4, no requirements are defined for this case.

  - Proposal 2(MTK,vivo):

    - Reuse the existing delay requirement of MAC CE based UL TCI state switch.

  - Proposal 3(ZTE):

    - Prefer to reuse the existing delay requirement of MAC CE based UL TCI state switch. However to move forward, a compromised solution is needed, e.g. allowing a clear but not too long additional latency.

  - Proposal 4(Nokia):

    - There is no need for beam sweeping for PL-RS measurements in FR2 if the PL-RS is SSB (assuming UE is having no more than 4 different PL-RS).

    - RAN4 does not discuss UE requirements for the scenario where the UE is configured with more than 4 different PL-RS for all active UL (or joint) TCI states.

    - There is no need for any additional measurements or beam sweeping at UL TCI state switch if the associated PL-RS is maintained by the UE.

    - The number of samples will not always be fixed as 5 samples when PL-RS is not maintained.

    - RAN4 to add [] around the number of samples needed if NM=1.

We think having RX beam sweeping for PL-RS measurements is not necessary as there is no testing for power control using PL-RS. Transmission power control is contiguous process and UE can adjust the power if it finds another best RX beam later. There is no chance of beam failure even if UE do not perform the RX beam sweeping during the PL-RS measurement. Considering this we think option 2 is the best way to go.

- RAN4 to reuse the existing delay requirement of MAC CE based UL TCI state switch

Whether need clarification or update in RAN4 spec when PDCCH/PDSCH and SSB associated with different PCI are overlapped on the same RE

- To be confirmed:

- [If the TCI state of the PDSCH /PDCCH is associated to the SSB of the cell with different PCI,

-	UE is not expected to receive PDSCH/PDCCH on the symbols corresponding to the SSB indexes configured for L1-RSRP measurement on the serving cell].

This restriction seems inspired from FR2, and we think same restriction is not applicable to FR1. In FR1, TCI state do not indicate spatial relation info. That means the above restriction of SSB of NSC being associated to TCI state do not necessarily mean UE cannot receive simultaneously from SC and NSC. If the concern is UE not able to demodulate both of them due to interference, this has to be discussed in Demod session.

- RAN4 not agree on ‘if the TCI state of the PDSCH /PDCCH is associated to the SSB of the cell with different PCI, UE is not expected to receive PDSCH/PDCCH on the symbols corresponding to the SSB indexes configured for L1-RSRP measurement on the serving cell’

## Sharing factor when TSSB_CDP = TSMTCperiod

While defining requirements for L1-RSRP of cell with different PCI (CDP), RAN4 agreed that L3 mobility should not be impacted due to introdcution of L1-RSRP requirements of CDP. Due to this agreement, RAN4 did not defiend requirements for the case TSSB_CDP = TSMTCperiod.

However, most practical or configuration used in NR networks is configuring all SSB occasions are also configured as SMTC occasions. When we consider this as the most common configuration or bassline configuration, if we do not define the requirements for baseline or common configuration (e.g., let us assume SSB and SMTC periodicity of 20ms), NW would be forced to configure one of the following two configurations.

Config 1: SSB periodicity =20 ms and SMTC periodicty=40ms.

Config 2: SSB periodicty=10ms and SMTC periodicty=20ms.

When config 1 is used to allow SSB to be present outside SMTC, NW has to increase the SMTC periodcity and this results in poor L3 mobility performance compared to baseline scenarios. This conflicts the reason why we did not introduce the requirements for baseline or common configuration.

If config 2 is used to allow SSB to be present outside SMTC, NW has to transmit more frequenct SSB ocassions than the necessary. This results in high SSB overhead and thereby reducing the efficiency of NR system.

Since config 1 and config 2 are not good for system performnce, we think RAN4 should introdcue requirement for common case or baseline case.

- RAN4 to introdcue L1-RRSP requirements for TSSB_CDP = TSMTCperiod

Since this is maintenance stage of the WI, we think equal sharing between serving cell’s L1-RSRP and CDP L1-RSRP may be reasonble WF.

- RAN4 to introduce equal sharing between SC and NSC L1-RSRP.

# Summary and conclusions

Follwing proposals are made.

- RAN4 not to define requirements for the case source RS in UL TCI state is not in DL active TCI state list.

- RAN4 to reuse the existing delay requirement of MAC CE based UL TCI state switch

- RAN4 to not agree on ‘if the TCI state of the PDSCH /PDCCH is associated to the SSB of the cell with different PCI, UE is not expected to receive PDSCH/PDCCH on the symbols corresponding to the SSB indexes configured for L1-RSRP measurement on the serving cell’

- RAN4 to introdcue L1-RRSP requirements for TSSB_CDP = TSMTCperiod

- RAN4 to introduce equal sharing between SC and NSC L1-RSRP