3GPP TSG RAN WG4 Meeting #107	R4-2309589

Incheon, 22 May 2023 – 26 May 2023

Agenda Item:	8.9.2.3

Source: 	Ericsson

Title:	Discussion on other potential enhancement for FR2 SCell activation

Document for:	Discussion

# Introduction

In this contribution, we provide our views on the other enhancements, other than L1 and L3 parts.

# Other enhancements

Following WF is agreed in last meeting.

Issue 3-1-1: Scheduling enhancement for FR2 unknown SCell activation

FFS:

- Option 1 (Apple, QC, OPPO, HW, Ericsson, ZTE): the early scheduling is network implementation and it’s not precluded in existing SCell activation requirement. No enhancement is needed.

  - Option 1a(ZTE):

    - For FR2, the initial scheduling can be decided by the NW after receiving the beam info and signal quality info after L1-RSRP report. For FR1, once the signal quality info derived by NW, the initial scheduling is possible.

- Option 2 (Nokia):

  - The potential to enable earlier data transmission within the activation period shall be studied.

  - The UE shall be scheduled by the network immediately after L3 or L1-RSRP reporting.

  - Starting from receiving the TCI activation command, the UE shall report CSI, instead of OoR, if the UE has available uplink resources to report CQI for the SCell.

NW needs to know about the rough beam information from the UE to start scheduling after L3 measurements. If the L3 measurements results are not available, then RAN4 need to introduce a new report so that UE can report the l3 measurement results after L3 part completion. If such report is introduced, NW can optionally start scheduling the UE after receiving the L3 report as NW can know the beam information and TCI information can be derived based on the beam information. Further, NW may need to start scheduling with lower MCS values if NW starts scheduling UE before CQI report reception. This may depend on whether NW like to schedule UE before receiving the CQI report or not. Therefore, in our understanding, this should not be a mandatory from NW point of view.

- Scheduling enhancement for FR2 unknown SCell may be a NW controlled optional feature. That means, NW may indicate the starting point of the scheduling to UE.

# Summary and Conclusion

In this contribution we have provided our views on the FR2 SCell activation delay reduction and made following proposals:

- Scheduling enhancement for FR2 unknown SCell may be a NW controlled optional feature. That means, NW may indicate the starting point of the scheduling to UE.

# References

- RP-220977 “WID on Even Further RRM enhancement for NR and MR-DC”, Apple Oppo