3GPP TSG RAN WG4 Meeting #107	R4-2309644

Incheon, Korea, May 22-26, 2023

Agenda Item:	8.34.4

Source: 	Ericsson

Title:	Further analysis of RRM requirements for mobile IAB

Document for:	Discussion

# Introduction

In the last RAN4 meeting a WF on mobile IAB RRM was approved containing several agreements [1]. There are still few open issues which require further analysis and discussion [1].

In this paper, we analyze the remaining RRM issues identified in the last meeting.

# Analysis of RRM requirements for Mobile IAB-MT

## General aspects

In the last RAN4 meeting, the following remaining issue related to the general principle/aspects of the requirement was identified for the mIAB-MT for further investigation [1]:

- RRM requirements to be based on the UE requirements

  - FFS which requirements are to be re-used for mobile IAB since many UE requirements would not be applicable/neeed to mobile IABs

The above agreement means that the necessary RRM requirements for the IAB-MT will be based on the corresponding RRM requirements defined for the UE in TS 38.133.

Several scenarios/features (e.g. NR-U, RedCap, positioning etc) for which RRM requirements are defined in TS 38.133 are not applicable for the IAB. Therefore, as a general principle, only the RRM requirements which were explicitly agreed in the previous meetings or being agreed for the mIAB-MT should be defined for the mIAB-MT. Any other RRM requirement in TS 38.133 should not be defined for the mIAB-MT.

## RRC Connection Mobility Control

In the last RAN4 meeting, the following remaining issue related to the RRC connection mobility control requirement was identified for the mIAB-MT for further investigation [1]:

- Introduce requirements for RACH

- FFS if requirements for connection release with redirection are needed

The main open issue is whether to specify the RRC connection release with redirection requirements defined in clause 6.2.3 of TS 38.133 (for the UE) also for the mIAB-MT.

The RRC connection release with redirection is performed on the target NR cell on the indicated carrier frequency, which is non-serving carrier frequency. This procedure is analoguous to an inter-frequency handover. It was agreed to define only intra-frequency handover requirements. We therefore suggest not to define the RRC connection release with redirection requirements for the mIAB-MT

## Radio link monitoring

In the last RAN4 meeting, the following issue related to the radio link monitoring (RLM) requirement was identified for the mIAB-MT for further investigation [1]:

- Introduce RLM requirements based on SSB and CSI-RS

  - FFS if all requirements are needed (e.g. requirements at transitions probably not needed since we will not define DRX requirements)

To identify which RLM requirements in clause 8.1 of TS 38.133 should be supported for the mIAB-MT, the following principles need to be considered:

- It was agreed that the RRM requirements for the mIAB-MT will be defined in non-DRX and without considering any measurement relaxations for power savings [1].

- The applicability of mIAB-MT for high-speed train scenarios is controversial and is separately discussed.

- The dual connectivity related requirements for mIAB-MT should not be defined.

Based on the above principles, the following aspects of the RLM requirements in clause 8.1 of TS 38.133 should NOT be supported for the mIAB-MT:

- When highSpeedMeasFlagFR2-r17 is configured

- Aspects related to measurement gaps

- Requirements for RLM on PSCell

- Requirements for RLM when the PSCell is deactivated

- UE transitions between DRX and no DRX or when DRX cycle periodicity changes

The RLM requirements clause 8.1 of TS 38.133 other than those listed above can be specified for the mIAB-MT.

## Link recovery procedure

In the last RAN4 meeting, the following issue related to the link recovery procedure (LRP) requirement was identified for the mIAB-MT for further investigation [1]:

- Introduce requirements for the link recovery procedure

  - FFS which requirements to be introduced since not all the requirements defined for UE are needed

To identify which LRP requirements in clause 8.5 of TS 38.133 should be supported for the mIAB-MT, the same principles as listed in section 2.2 (for RLM) need to be considered.

Based on the above principles, the following aspects of the LRP requirements in clause 8.5 of TS 38.133 should NOT be supported for the mIAB-MT:

- When highSpeedMeasFlagFR2-r17 is configured

- Aspects related to measurement gaps

- Requirements for LRP on PSCell and SCell

- Requirements for LRP when the PSCell is deactivated

- UE transitions between DRX and no DRX or when DRX cycle periodicity changes

The LRP requirements clause 8.5 of TS 38.133 other than those listed above can be specified for the mIAB-MT.

## CA related RRM requirements

In the last RAN4 meeting, the following issue related to the CA requirement for the mIAB-MT was identified for further investigation [1]:

- Further check if any CA related requirements are needed to support mobility (e.g. SCell handling at handover)

It was agreed in the last meeting that only intra-frequency handover requirements will be defined for the IAB-MT [1]. The intra-frequency handover involves changing of the PCell of the IAB-MT. Even if the IAB-MT was configured with additional serving cells/carriers before the handover, the additional carriers are reconfigured after the handover by the new serving PCell. We therefore do not see the need for defining any CA related requirement to support mobility of the the mIAB-MT.

## HST related requirements

In the last RAN4 meeting, the following issue related to applicability of HST requirement for the mIAB-MT was identified for further investigation [1]:

- FFS whether HST requirements are needed

  - Even if introduced, these will be optional since not all mobile IABs will be deployed in such environments

It has been argued by the proponent of specifying the HST requirement for the mIAB-MT that the HST scenario is an important use case for the mIAB.

The mIAB WID undoubtedly does not state that the HST scenario is within the scope of the WI [3]. On the contrary according to the WID the main scenario comprises of the mIAB nodes mounted on vehicles as mentioned in the justification part [3]:

- The work on Mobile IAB in Rel-18 should focus on the scenario of mobile-IAB-nodes mounted on vehicles providing 5G coverage/capacity enhancement to onboard and/or surrounding UEs.

Therefore, in our view the HST scenario is NOT in the scope of mIAB WI scope. Applicability of the mIAB to the HST will require upscoping of the WID and therefore plenary level agreements. Furthermore, the deployment scenario for the IAB in HST scenario may not be the same as used for the UEs and would require considerable discussion. Therefore, in our view the existing HST in FR1, which are defined for mobile UEs inside the train and in FR2, which are defined for roof-top mounted UE on the train cannot be simply made applicable to the mIAB.

# Summary

The following are the observations and the proposals related to RRM requirements for different aspects/procedures related to the mobile IAB-MT (mIAB-MT):

General aspects of RRM requirements:

- Observation #1: Several scenarios/features for which RRM requirements are defined in TS 38.133 are not applicable for the mIAB.

- Proposal #1: A list of RRM requirements explicitly identified for mIAB-MT are defined for the mIAB-MT.

RRC Connection Mobility Control:

- Observation #2: The RRC connection release with redirection to target NR carrier is analoguous to an inter-frequency handover.

- Observation #3: It has been agreed to define only intra-frequency handover requirements since intra-frequency mobility is the most important and basic scenario for the mIAB.

- Proposal #2: Do not define RRC connection release with redirection requirements for mIAB-MT.

RLM:

- Observation #4: It was agreed that in general RRM requirements for the mIAB-MT will be defined in non-DRX and without considering any measurement relaxations for power savings.

- Observation #5: The measurement gaps will not be used by the mIAB-MT for the RRM measurements.

- Proposal #3: The RLM requirements except the following in clause 8.1 of TS 38.133 are defined for the mIAB-MT:

  - Proposal 3A:

    - When highSpeedMeasFlagFR2-r17 is configured

  - Proposal 3B:

    - Aspects related to measurement gaps

  - Proposal 3C:

    - Requirements for RLM on PSCell

  - Proposal 3D:

    - Requirements for RLM when the PSCell is deactivated

  - Proposal 3E:

    - UE transitions between DRX and no DRX or when DRX cycle periodicity changes

Link recovery procedure (LRP):

- Proposal #4: The LRP requirements except the following in clause 8.5 of TS 38.133 are defined for the mIAB-MT:

  - Proposal 4A:

    - When highSpeedMeasFlagFR2-r17 is configured

  - Proposal 4B:

    - Aspects related to measurement gaps

  - Proposal 4C:

    - Requirements for LRP on PSCell and SCell

  - Proposal 4D:

    - Requirements for LRP when the PSCell is deactivated

  - Proposal 4E:

    - UE transitions between DRX and no DRX or when DRX cycle periodicity changes

CA related requirements:

- Observation #6: Only intra-frequency handover/mobility requirements will be defined and does not impact any other serving cell as it involves changing of the PCell of the IAB-MT.

- Proposal #5: No CA related requirement to support mobility of the mIAB-MT is required.

HST scenario:

- Observation #7: The mIAB WID does not explicitly state that the HST scenario is within the scope of the WI.

- Observation #8: According to the WID the main scenario comprises of the mIAB nodes mounted on vehicles.

- Observation #9: mIAB applicability under HST scenario will require upscoping of the WID and therefore plenary level agreements.

- Observation #10: Deployment scenario for the IAB in HST scenario may not be the same as used for the UEs and would require considerable discussion mIAB to the HST.

- Proposal #6: HST scenario is NOT within the scope of mIAB WI.

- Proposal #7: Do not specify requirements for mIAB under HST scenario.

# References

- R4-2306368, WF on Mobile IAB RRM requirements, Qualcomm.

- R4-2306259, Topic summary for [106-bis-e][228] NR_mobile_IAB, Qualcomm.

- RP-222671, Mobile IAB (Integrated Access and Backhaul) for NR, Revised WID (RAN#98-e).