3GPP TSG RAN WG4 Meeting #107	R4-2309646

Incheon, Korea, May 22-26, 2023

Agenda Item:	8.11.2.1

Source: 	Ericsson

Title:	Further analysis of BWP operation without restriction option A

Document for:	Discussion

# Introduction

In the last RN4 meeting it was concluded that the existing requirements for intra-frequency measurement with gap shall apply for UE supporting option A and no update is needed in this regard [1]. However, there was no consensus whether any clarification is needed on existing timing requirements for option A [1] i.e. when CD-SSB is outside active BWP.

In this paper we further analyze the impact on the existing timing requirements when option A is used for the BM/RLM/BFD based on CSI-RS within active BWP

# Analysis of timing requirements for option A

The following was the outcome regarding the need for the clarification on existing timing requirements when CD-SSB is outside active BWP [1]:

- Option 1: (Ericsson, Apple, Nokia, Intel, Huawei, CATT, MediaTek, OPPO)

  - No clarifications on existing timing requirements are needed.

- Option 2: (vivo)

  - It is clarified in the spec that existing timing requirements for non-RedCap UE are applicable regardless of whether SSB is within active BWP or not.

  - A note is added for timing requirements that when SSB is outside active BWP, availability of SSB is at least relevant to configuration of measurement gap, number of measurement objects and gap sharing factor.

- Option 3: (Ericsson, Intel)

  - The condition to configure gaps to meet the existing UE transmission timing error requirements in clause 7.1 of TS 38.133, when the UE is performing BM/RLM/BFD based on option A, is NOT needed.

  - A possible compromise is to clarify in clause 7.1.2 of TS 38.133, that the availability of the SSB at the UE is for the purpose of acquiring the timing of the reference cell

As analyzed in our previous contribution, the SSB (or more specifically CD-SSB) is always available in the serving cell. The reference cell for the timing acquisition is the PCell or one of the serving cells in CA/DC scenario. The UE performs at least the SSB based measurements on the serving cell. There is no specific or dedicated SSB configured for the UE timing acquisition. Therefore, the UE will use the same SSB configured for the serving cell measurements for performing the timing acquisition.

When performing BM/RLM/BFD according to option A, the SSB can be outside the active BWP. If the SSB is outside the active BWP, then the measurement gaps will be provided by the network to enable the UE to perform the SSB based measurements on the serving cell. This means, even if the SSB is outside the active BWP as in case of option A, the UE will be able to measure the SSB within the measurement gaps. To meet the timing error requirements obviously the SSB from the serving/reference cell must be available at the UE at least once every 160 ms. It is the responsibility of the network to provide the measurement configuration (e.g. gaps, number of frequency layers etc) that would ensure that the SSB of the serving/reference cell is available at the UE at least once every 160 ms. Otherwise, the UE is not expected to meet the timing error requirements. In our view no additional clarification or side condition is needed for the UE to meet the existing UE transmission timing error requirements based on the SSB defined in clause 7.1 of TS 38.133, when that UE is performing BM/RLM/BFD using option A. At least no such condition to mandate configuration of gaps in the legacy requirements exists, and in practice (e.g. in the field) we have not observed any issues with with current specification text. Therefore, we do not see any reason to mandate the condition to configure gaps for meeting the timing requirements. However, to move forward one possibility is to clarify in clause 7.1.2, TS 38.133, that the availability of the SSB at the UE should be for the purpose of acquiring the timing of the reference cell e.g.

- When the UL SCS is 120 kHz or smaller, the UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available at the UE for acquiring the frame timing of the reference cell during the last 160 ms. When the UL SCS is 480 kHz the UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available at the UE for acquiring the frame timing of the reference cell in the last 80 ms. When the UL SCS is 960 kHz the UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available at the UE for acquiring the frame timing of the reference cell in the last 40 ms.

The above clarification explicitly clarifies that the UE is mandated to meet transmission timing error requirements provided that the SSB is available at the UE for the purpose of the timing acquisition.

# Summary

This paper has further analyzed the applicability of the UE transmission timing requirements for the UE performing BM/RLM/BFD based on option A i.e. when the CSI-RS is within the active BWP. The following are the observations and proposals:

- Observation #1: The SSB (i.e. CD-SSB) is always available in the serving cell since the UE performs at least the SSB based measurements on the serving cell.

- Observation #2: If the SSB is outside the active BWP then the measurement gaps will be provided by the network to enable the UE to perform the SSB based measurements on at least the serving cell.

- Observation #3: Even if the SSB is outside the active BWP, the UE will be able to measure the SSB within the measurement gaps and acquire the timing from the serving/reference cell.

- Observation #4: Regardless of the measurement configuration (e.g. gaps, number of frequency layers etc) the SSB of the serving/reference cell should be provided by the network at the UE at least once every e.g. 160 ms for SCS ≤  120 kHz. Otherwise, the UE is not expected to meet the UE transmission timing error requirements.

Proposal #1: Option 1 is preferred but option 2 can be compromise solution:

  - Option 1:

    - The condition to configure gaps to meet the existing UE transmission timing error requirements in clause 7.1 of TS 38.133, when the UE is performing BM/RLM/BFD based on option A, is NOT needed.

  - Option 2:

    - A possible compromise is to clarify in clause 7.1.2 of TS 38.133, that the availability of the SSB at the UE is for the purpose of acquiring the timing of the reference cell as shown below:

  - When the UL SCS is 120 kHz or smaller, the UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available at the UE for acquiring the frame timing of the reference cell during the last 160 ms. When the UL SCS is 480 kHz the UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available at the UE for acquiring the frame timing of the reference cell in the last 80 ms. When the UL SCS is 960 kHz the UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available at the UE for acquiring the frame timing of the reference cell in the last 40 ms.

# References

- R4-2306397, WF on BWP operation without restriction, vivo

- R4-2306393, Topic summary for [106-bis-e][208] NR_BWP_wor, vivo.