3GPP TSG-RAN WG4 Meeting # 107	R4-2309665

Incheon, KR, May 22 – May 26, 2023

Source: 	Qualcomm Incorporated

Title: 	FR2 HST Enh. UE Demod with Tunnel Deployment

Agenda item:	8.13.5.4

Document for:	Discussion

- Introduction

In this paper, we share our considerations on the items on UE Demodulation requirements with Tunnel Deployment for FR2 HST Enhancements, based on the WF [1].

# FR2 HST Requirements for Tunnel Deployment

In the past meeting, RAN4 has discussed the necessity to introduce new UE demodulation requirements for the deployment of FR2 HST RRHs in tunnel. There are two aspects to consider for this work item, with the first being whether or not a new channel is necessary to model real world deployment. After that, we should consider whether additional performance tests will significantly increase coverage or not

## Channel Model

Many companies have supported that in their view the tunnel deployment does not provide a sufficiently rich scattering environment to provide significant multipath components, and we share this view based on the geometry of the tunnel and the principles of radio propagation.

Observation 1: Propagation in the case of Tunnel Deployment for FR2 HST follows mainly a single path;

Proposal 1: RAN4 should not consider multipath channel to model propagation in the case of Tunnel Deployment for FR2 HST;

If RAN4 agrees that multipath should not be considered, then we recommend using the existing FR2 HST DPS channel model as reference, setting the Dmin parameter to 1m following the same agreements in RRM.

Proposal 2: To study the scope of FR2 HST Tunnel Deployments, RAN4 should consider the existing FR2 HST DPS channel model as baseline, Dmin = 1m to model reduced distance between Track and RRH;

The consequence of assuming a smaller Dmin on channel model is to reduce the difference between relative velocities between train and RRHs at the switching point between RRH, thus resulting in a smaller the doppler trajectory jump compared to tests with larger Dmin.

Observation 2: A smaller Dmin results in a smaller ‘jump’ in Doppler shift when switching across RRH during the train path along the tracks;

## Test Scope

It is our view that considering the conclusions reached in the previous section, introducing UE Demod tests for Tunnel Deployment with the same single path channel and smaller Doppler jump compared to existing requirement will result in no additional test coverage.

Proposal 3: RAN4 should agree to not introduce dedicated Demodulation requirements for Tunnel Deployment, considering this scenario covered by existing test cases;

- Conclusions

Observation 1: Propagation in the case of Tunnel Deployment for FR2 HST follows mainly a single path;

Proposal 1: RAN4 should not consider multipath channel to model propagation in the case of Tunnel Deployment for FR2 HST;

Proposal 2: To study the scope of FR2 HST Tunnel Deployments, RAN4 should consider the existing FR2 HST DPS channel model as baseline, Dmin = 1m to model reduced distance between Track and RRH;

Observation 2: A smaller Dmin results in a smaller ‘jump’ in Doppler shift when switching across RRH during the train path along the tracks;

Proposal 3: RAN4 should agree to not introduce dedicated Demodulation requirements for Tunnel Deployment, considering this scenario covered by existing test cases;

- References

[1] R4-2305892, 3GPP TSG-RAN WG4 Meeting #106bis-e, Online Meeting, 17th April- 26th April, 2023