3GPP TSG-RAN WG4 Meeting #107	R4-2309678

Incheon, Korea, May 22 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on RRM core requirements for NR Carrier Phase Positioning

Agenda item:	8.23.3.6

Document for:	Discussion

Introduction

The specification of NR carrier phase positioning (NR CPP) is one of the objectives in the Rel-18 WID on expanded and improved positioning [1].

This contribution summarizes RAN4 #106bis-e agreements and lists discussion on some of the open issues with regard to impacts to RRM core requirements for NR CPP. RAN1 #112bis-e agreements are listed in the Annex.

Discussion

RAN4 agreements and open issues

RAN4 agreements and list of open issues from RAN4 #106bis-e are captured in WF [2].

- Sub-topic 2-1 General and scenarios #### Issue 2-1-1: Measurement types to be defined for DL carrier phase measurement requirements: Agreements: - Wait for more progress and outcomes on the definition of measurement from RAN1/RAN2 to define the measurement period requirements for CPP. #### Issue 2-1-2: Measurement types to be defined for UL carrier phase measurement requirements: Agreements: - RAN4 not to define measurement period requirements for UL RSCP measurement and FFS for accuracy requirements. #### Issue 2-1-3: Applicable RRC state: Agreements: - Define core requirements for CPP in RRC_CONNECTED and RRC_INACTIVE states.

#### Issue 2-1-4: Applicable BW range of reference signals for DL and UL carrier phase measurement: Agreements: - The measurement period requirements (formula) for DL measurements are agnostic to the PRS BW. - FFS the PRS/SRS BW range for measurement accuracy requirements. < Way forward > - Option 1: - RAN4 to align the targeted PRS BW range for NR DL CPP in RRC_CONNECTED to that for positioning techniques specified in Rel-16 and in RRC_INACTIVE to that for positioning techniques specified in Rel-17 in regard to both BW related RRM performance requirements. - RAN4 to align the targeted BW range for both reference signals, SRS for positioning and MIMO SRS, for NR UL CPP to that for positioning techniques specified in Rel-16 in regard to RRM performance requirements. #### Issue 2-1-5: Applicable number of measurement instances: < Way forward > - Option 1: - RAN4 to investigate and specify the measurement period, reporting, and accuracy requirements for DL CPP measurements, for the cases of a single and multiple measurement instances aligned to requirements for legacy positioning techniques in Rel-16 and Rel-17. #### Issue 2-1-6: PRU support: < Way forward > - Option 1: - RAN4 to investigate measurement requirements for PRU, in particular if PRU is expected to support tighter measurement accuracy requirements than target UE. - Sub-topic 2-2 Core requirements #### Issue 2-2-1: Measurement period requirements for DL carrier phase measurement: < Way forward > - Option 1: - If RSCPD/RSCP being reported with other legacy measurements together, RAN4 can define the measurement reporting delay requirements for them with the existing requirements for the associated legacy measurement. - Option 2: - Current core requirements for PRS measurement are used as baseline for carrier phase measurement. Adaptations can be FFS e.g. sample number and applicable PRS configuration. - Option 3: - Other options are not precluded. #### Issue 2-2-2: Applicable RF frequencies for DL RSCP measurement period requirements: < Way forward > - Option 1: - The basic measurement requirement for DL RSCP is defined for each RF frequency. And RAN4 to discuss whether to define measurement requirements for multiple RF frequencies. #### Issue 2-2-3: Applicable RF frequencies for DL RSCPD measurement period requirements: < Way forward > - Option 1: - The RF frequencies for target TRP and reference TRP in RSCPD measurement can be different. #### Issue 2-2-4: Impact of UE mobility: Agreements: - The impact on measurement requirements for NR DL CPP due to mobility procedures should be investigated and specified by RAN4 similar as done for Rel-16/Rel-17 positioning techniques. #### Issue 2-2-5: Impact of collisions with other signals/channels in RRC_INACTIVE: Agreements: - RAN4 to investigate and specify RRM impacts from NR DL CPP in RRC_INACTIVE such as collisions with other signals/DL channels, assuming Rel-17 requirements can be re-used. - Sub-topic 2-3 Performance requirements #### Issue 2-3-1: Channel conditions for carrier phase measurement accuracy requirements: Agreements: - RAN4 to discuss and decide which fading channels to be used for evaluating and specifying the CPP accuracy requirements in addition to AWGN channel. #### Issue 2-3-2: PRS/SRS BW configurations for measurement accuracy requirements: < Way forward > - Option 1: - RAN4 to consider the PRS BW configurations used for accuracy requirements in Rel-16 and Rel-17 as baseline for NR CPP accuracy requirements - RAN4 to consider the SRS BW configurations used for accuracy requirements in Rel-16 as baseline for NR UL CPP accuracy requirements for both reference signals, SRS for positioning and MIMO SRS. #### Issue 2-3-3: Side condition: < Way forward > - Option 1: - SINR side condition for the NR carrier phase measurement can be same as these for Rel16/17 positioning measurement with PRS. #### Issue 2-3-4: Measurement reporting requirements: < Way forward > - Option 1: - RAN4 to investigate and specify measurement reporting for NR DL CPP based on DL measurement types specified in TS 38.215. - RAN4 to investigate and specify measurement reporting requirements for NR UL CPP based on UL measurement types specified in TS 38.215 - RAN4 to specify joint measurement reporting requirements for the use case scenario of coexistent NR CPP and legacy NR positioning techniques. - RAN4 to await further RAN1 progress on additional measurement reporting and assistance information to improve the measurement accuracy. #### Issue 2-3-5: Report mapping tables: < Way forward > - Option 1: - Introduce report mapping tables for absolute and differential carrier phase measurement, and discuss the following issues to define report mapping tables: - The unit for carrier phase report - The range for carrier phase report - The resolution for carrier phase report - Option 2: - The different report mapping tables shall be defined for RSCP and RSCPD #### Issue 2-3-6: Reporting range: < Way forward > - Option 1: - For DL RSCP and UL RSCP the reporting range shall be should be [0, 2pi], whereas for DL RSCPD, it should be [-pi, pi]. #### Issue 2-3-7: Reporting granularity: < Way forward > - Option 1: - For Rel18 CPP measurements, the reporting granularity for DL RSCP, DL RSCPD and UL RSCP should be configurable up to UE capability. - Option 2: - For carrier phase positioning, RAN4 should consider the higher positioning resolution compared to the one for the time-based measurement metric.

  - General and scenarios

Measurement types and reporting for DL CPP

RAN1 #112bis-e achieved following agreement on the measurement types for DL CPP.

Agreement Introduce DL reference carrier phase (DL RSCP) and NR DL reference carrier phase difference (DL RSCPD) as DL carrier phase measurements. - Note: It is up to RAN4 to decide whether and how to define the requirements for DL RSCP and/or DL RSCPD. No LS needed to RAN4 for this note. - DL RSCP can be reported together with UE Rx – Tx time difference measurement - DL RSCPD can be reported together with RSTD measurement - FFS: details on how to eliminate unknown initial Rx phase with RSCP/RSCPD reporting can be further discussed Note: Whether to support standalone DL RSCP and/or DL RSCPD reporting, or DL RSCP/DL RSCPD reporting with other new types of measurements (if agreed), can be further discussed.

Thus, RAN4 will need to define measurement requirements for DL RSCP and DL RSCPD. Measurement reporting requirements should be specified such, that DL RSCP is reported together with UE Rx-Tx time difference and DL RSCPD reported together with RSTD, whilst standalone DL RSCP or DL RSCPD measurements, respectively, are still subject to be agreed in RAN1.

RAN4 to define measurement period and measurement reporting requirements for DL RSCP, which can be reported together with UE Rx-Tx time difference.

RAN4 to define measurement period and measurement reporting requirements for DL RSCPD, which can be reported together with RSTD.

Measurement types and reporting for UL CPP

The measurement type for UL CPP was already agreed at RAN1 #112 to be UL RSCP based on SRS for positioning or MIMO SRS reference signals. RAN1 #112bis-e achieved following agreement on the reporting of the measurement type for UL CPP.

Agreement - Support enabling a TRP to report UL RSCP together with RTOA and/or gNB Rx-Tx time difference measurements to LMF - Note 1: The report of UL carrier phase measurement with gNB Rx – Tx time difference does not necessarily require the report of DL carrier phase measurement with UE Rx – Tx time difference. - Note 2: This doesn’t preclude standalone UL carrier phase measurements reporting.

Thus, RAN4 will need to define measurement requirements for UL RSCP. Measurement reporting requirements should be specified such, that UL RSCP is reported together with RTOA and/or gNB Rx-Tx time difference or even standalone. According to RAN4#106bis-e, RAN4 will not specify measurement period requirements for UL RSCP.

RAN4 to define measurement reporting requirements for UL RSCP, which can be reported together with RTOA, gNB Rx-Tx time difference or standalone.

Applicable RRC state

RAN1 has further detailed the support of DL CPP for the supported RRC states in the following agreement:

Agreement Support the reuse of existing physical layer procedures for DL positioning (e.g., DL-TDOA) with the necessary enhancements in measurement configuration, request and report (e.g., adding the configuration related to the NR DL CPP) for both UE-based and UE-assisted NR DL carrier phase positioning, including - UE in RRC_CONNECTED state with measurement gap. - FFS: UE in RRC_CONNECTED state without measurement gap - UE in RRC_ INACTIVE state

Likewise, RAN1 has reached following agreement for UL CPP:

Agreement For NR UL carrier phase positioning for UE in RRC_CONNECTED and RRC_INACTIVE states, support reuse of existing physical layer procedures for UL positioning (e.g., UL-TDOA), with necessary enhancements in the measurement configuration, measurement request and measurement report (e.g., the configuration related to the NR UL CPP). - FFS: the details of the enhancements.

The support of MG assisted DL measurement in RRC_CONNECTED state and support in RRC_INACTIVE as well as support of UL measurements in both RRC states, extending Rel-16 and Rel-17 measurement configurations, is confirmed, and RAN4 needs to wait on RAN1’s decision regarding support of gapless DL measurements in RRC_CONNECTED state. Thus, RAN4 should focus on defining measurement requirements for MG assisted DL CPP measurements in RRC_CONNECTED state and on DL CPP measurements in RRC_INACTIVE state and on UL CPP measurements for UE in RRC_CONNECTED and RRC_INACTIVE states.

RAN4 to focus on defining measurement requirements for MG assisted DL CPP measurements in RRC_CONNECTED state, for DL CPP measurements in RRC_INACTIVE state and for UL CPP measurements for UE in RRC_CONNECTED and RRC_INACTIVE states.

Applicable number of measurement instances

According to RAN1 TR 38.859 [3], baseline performance will use a single measurement instance. In addition, RAN4 should investigate the performance and latency impact from using multiple measurement instances for the investigated propagation channels and reuse the number of samples in Rel-16 (4) and for latency reduction in Rel-17 (1) as baseline assumption. This also takes into account aligned measurement reporting for legacy positioning and NR CPP measurements if normal or reduced latency positioning is configured.

RAN4 to investigate and specify measurement period, reporting, and accuracy requirements for DL RSCP and DL RSCPD measurements, for the cases of single and multiple measurement instances aligned to requirements for UE Rx-Tx and RSTD measurements, respectively, specified in Rel-16 and in Rel-17 for reduced latency.

PRU support

PRU is required for double differential carrier phase estimation both on DL and UL. In that respect it is crucial to perform simultaneous PRS carrier phase measurements at target UE and PRU in case of NR DL CPP, and simultaneous transmission of SRS for positioning or MIMO SRS, respectively, in case of NR UL CPP. RAN1 had following agreements for UL CPP related to enabling simultaneous transmissions and for DL CPP related to simultaneous UE measurements:

Agreement To enable simultaneous transmission of UL SRS for positioning by a target UE and a PRU, support the following enhancements: - Enabling LMF to request the serving gNB of a UE to configure the transmission of the [indicated] UL SRS resources from the UE within indicated time window(s). - FFS: the details of the time window, e.g., the start time, duration, periodicity for the time window(s), within the vicinity of a reference SRS configuration or use the existing message of Scheduled Location time - Enabling LMF to request the serving gNB and neighboring gNBs of the UE to measure the [indicated] UL SRS resources from the UE within indicated time window(s). - Note: this may be a different indicated time window Agreement To enable simultaneous measurements on same DL PRS by a target UE and a PRU, support the following enhancements: - Enabling LMF to request the UEs, including target UE and PRU(s), to perform measurements on [indicated] DL PRS resources occurring within indicated time window(s). - FFS: the details of the configuration of the indicated time window(s), e.g., the start time, duration, periodicity for the time window(s), as well as the relationship with the Scheduled Location time.

As location of PRU is known, high Tx timing accuracy for SRS for positioning / MIMO SRS for UL CPP and high DL-RSCP / DL-RSCPD measurement accuracy for DL CPP measurements will be beneficial considering the agreed operation in different time windows. Thus, RAN4 should investigate, if Tx timing accuracy requirements and DL RSCP/ DL-RSCPD measurement accuracy requirements can be tighter for PRU, e.g. for LOS path/phase detection, than for target UE, this being subject to PRU capability.

RAN4 to investigate, if Tx timing accuracy requirements and DL RSCP/ DL-RSCPD measurement accuracy requirements can be tighter for PRU than for target UE, this being subject to PRU capability.

Coexistence with legacy positioning techniques

RAN1 #112bis-e achieved following agreement on joint reporting of NR CPP with legacy NR positioning techniques.

Agreement Introduce DL reference carrier phase (DL RSCP) and NR DL reference carrier phase difference (DL RSCPD) as DL carrier phase measurements. - DL RSCP can be reported together with UE Rx – Tx time difference measurement - DL RSCPD can be reported together with RSTD measurement Agreement - Support enabling a TRP to report UL RSCP together with RTOA and/or gNB Rx-Tx time difference measurements to LMF

RAN1 considers concurrent operation of Rel-16 positioning techniques and NR CPP as important use case scenario. Hence RAN4 should specify joint measurement reporting requirements for DL CPP for this use case. Moreover, joint measurement reporting can be ensured if measurement periods for DL CP measurements are aligned to those for Rel-16 / Rel-17 measurement types. Hence RAN4 should investigate how measurement periods for DL CP measurements can be aligned to those for Rel-16 / Rel-17 measurement types.

RAN4 should investigate how measurement periods (MP) for DL CP measurements can be aligned to those for Rel-16 / Rel-17 measurement types (i.e. MP for DL RSCP aligned to that for UE Rx-Tx time difference and MP for DL RSCPD aligned to that for RSTD).

In this respect, it is important to perform NR CP measurements on the same PFL as used for Rel-16 positioning technique. This means, the PFLs configured for RSTD and UE Rx-Tx time difference, respectively, should also be configurable for DL RSCP and DL RSCPD.

Assume a network has configured two PFLs and some TRPs use PFL1 and others PFL2 for PRS transmission to reduce intercell interference. RSTD for a reference TRP on PFL1 and a target TRP on PFL2 is then determined across two PFL’s. The paired DL CP measurement is identified as DL RSCPD according to RAN1 #112bis-e agreement. If this DL CP measurement is restricted to single PFL for all cells, then either PRS transmission on PFL1 or PFL2 or another PFL3 is required in all cells, yielding the need to configure 2 PFLs for some or all cells configured for PRS transmission. In our view this increases complexity for network and UE and yields a lower network efficiency, as bandwidth is consumed by PRS transmission on the second PFL. Thus, if two or more PFL’s are configured in the network for RSTD (up to 4 PFLs are defined in Rel-16), the same PFL per each cell should be configurable for the paired DL CP measurement.

In case the network operates multiple PFL’s to support RSTD or UE Rx-Tx time difference measurements, the same PFL per each cell should be configurable for the paired DL CP measurement, i.e. DL RSCPD or DL RSCP, respectively.

Thus, we support option 1 in issue 2-2-2.

#### Issue 2-2-2: Applicable RF frequencies for DL RSCP measurement period requirements: - Option 1: - The basic measurement requirement for DL RSCP is defined for each RF frequency. And RAN4 to discuss whether to define measurement requirements for multiple RF frequencies.

As the above consideration on multiple PFLs applies similar to UE Rx-Tx time difference measurement, RAN4 should define measurement requirements for multiple PFL deployments, i.e. TRPs using different PFLs.

The RF frequencies for target TRP and serving TRP for the RSCP measurement can be different.

#### And we also support option 1 in issue 2-2-3.

#### Issue 2-2-3: Applicable RF frequencies for DL RSCPD measurement period requirements: - Option 1: - The RF frequencies for target TRP and reference TRP in RSCPD measurement can be different.

The RF frequencies for target TRP and reference TRP in RSCPD measurement can be different.

Channel conditions for carrier phase measurement accuracy requirements

RAN4#106bis-e found agreement to discuss and decide which fading channels to be used for evaluating and specifying the CPP accuracy requirements in addition to AWGN channel.

Related to propagation channels, TDL-A and TDL-C profiles were used for specifying measurement accuracy for fading channel in Rel-16/Rel-17. These are NLOS profiles. In the RAN1 feasibility study [3], the accuracy of NR carrier phase positioning has been evaluated under different scenarios (e.g., InF-SH, InF-DH) as defined in TR 38.901. In fact, for NR CPP, indoor office deployments as well as indoor factory (InF) deployments in the IIOT context are a major use case, where the LOS component is more expressed.

Thus, following proposal is made:

RAN4 to study typical fading channel profiles for NR CPP where LOS component is more expressed, in addition to AWGN channel.

Report mapping requirements

RAN4#106bis-e discussed different aspects on report mapping for DL and UL CPP measurements, such as appropriates report mapping tables, reporting range and reporting granularity.

#### Related to issue 2-3-6, the reporting range for DL RSCP and UL RSCP should be [0; 2pi] and for DL RSCPD [-pi, +pi].

RAN4 to define the reporting range [0; 2pi] for DL RSCP and UL RSCP and [-pi; +pi] for DL RSCPD.

Related to issue 2-3-7, we support option 2.

#### Issue 2-3-7: Reporting granularity: < Way forward > - Option 1: - For Rel18 CPP measurements, the reporting granularity for DL RSCP, DL RSCPD and UL RSCP should be configurable up to UE capability. - Option 2: - For carrier phase positioning, RAN4 should consider the higher positioning resolution compared to the one for the time-based measurement metric.

The reporting granularity should be smaller than for TOA measurements, which is 1 Tc (= 0.509 ns). As reported in our simulation results for NR DL CPP and AWGN channel [4], a resolution up to 5% of the carrier wavelength can be achieved. In 2 GHz, this represents a granularity of 0,05 Tc. Given that Tc represents the carrier wavelength in this case, there are only 20 codepoints, which can be covered by 5 bits. Hence we do not think the reporting granularity needs to be configurable.

RAN4 to consider lower reporting granularity for CP measurements than Tc used for TOA measurements.

Measurement accuracy requirements for UL RSCP

Regarding whether to define measurement accuracy requirements for UL RSCP, we agree to postpone the decision to performance part.

In our view, measurement accuracy requirements for UL RSCP could be specified following the approach for gNB Rx-Tx time difference in Rel-16, where BB performance was specified, and a manufacturer declared impairments margin added on top. In our view this approach can be reused for UL RSCP in Rel-18.

RAN4 to postpone the decision on whether to define UL RSCP measurement accuracy requirements to performance part.

RAN4 to consider whether the Rel-16 approach for gNB Rx-Tx time difference accuracy performance with specified BB performance and manufacturer declared impairments margin can be reused for defining UL RSCP accuracy performance in Rel-18.

- Conclusion

The scope of RRM impacts for NR carrier phase positioning (NR CPP) being one of the objectives in the Rel-18 WID on expanded and improved positioning [1], is investigated in this contribution.

The following proposals are made.

- RAN4 to define measurement period and measurement reporting requirements for DL RSCP, which can be reported together with UE Rx-Tx time difference.

RAN4 to define measurement period and measurement reporting requirements for DL RSCPD, which can be reported together with RSTD.

RAN4 to define measurement reporting requirements for UL RSCP, which can be reported together with RTOA, gNB Rx-Tx time difference or standalone.

RAN4 to focus on defining measurement requirements for MG assisted DL CPP measurements in RRC_CONNECTED state, for DL CPP measurements in RRC_INACTIVE state and for UL CPP measurements for UE in RRC_CONNECTED and RRC_INACTIVE states.

RAN4 to investigate and specify measurement period, reporting, and accuracy requirements for DL RSCP and DL RSCPD measurements, for the cases of single and multiple measurement instances aligned to requirements for UE Rx-Tx and RSTD measurements, respectively, specified in Rel-16 and in Rel-17 for reduced latency.

RAN4 to investigate, if Tx timing accuracy requirements and DL RSCP/ DL-RSCPD measurement accuracy requirements can be tighter for PRU than for target UE, this being subject to PRU capability.

RAN4 should investigate how measurement periods (MP) for DL CP measurements can be aligned to those for Rel-16 / Rel-17 measurement types (i.e. MP for DL RSCP aligned to that for UE Rx-Tx time difference and MP for DL RSCPD aligned to that for RSTD).

In case the network operates multiple PFL’s to support RSTD or UE Rx-Tx time difference measurements, the same PFL per each cell should be configurable for the paired DL CP measurement, i.e. DL RSCPD or DL RSCP, respectively.

The RF frequencies for target TRP and serving TRP for the RSCP measurement can be different.

The RF frequencies for target TRP and reference TRP in RSCPD measurement can be different.

RAN4 to study typical fading channel profiles for NR CPP where LOS component is more expressed, in addition to AWGN channel.

RAN4 to define the reporting range [0; 2pi] for DL RSCP and UL RSCP and [-pi; pi] for DL RSCPD.

RAN4 to consider lower reporting granularity for CP measurements than Tc used for TOA measurements.

RAN4 to postpone the decision on whether to define UL RSCP measurement accuracy requirements to performance part.

RAN4 to consider whether the Rel-16 approach for gNB Rx-Tx time difference accuracy performance with specified BB performance and manufacturer declared impairments margin can be reused for defining UL RSCP accuracy performance in Rel-18.

- References

- RP-230328, “Revised WID on Expanded and Improved NR Positioning”, Intel Corporation, RAN#99-e

- R4-2306351, “WF on RRM requirements for NR sidelink positioning and carrier phase positioning”, CATT

- 3GPP TR 38.859 V18.0.0, “Study on Expanded and Improved NR Positioning (Release 18)”

- R4-2301854, “Initial simulation results for DL NR carrier phase positioning”, Nokia, Nokia Shanghai Bell

- Annex: RAN1 Agreements on NR CPP

RAN1 #112

Conclusion A PRU can report its location and associated uncertainty as is the case for other UEs. It is not necessary to always include the PRU location information with the PRU measurements in the same report. The PRU location information and measurements should be decoupled, where decoupled means that the PRU location information is determined independently of the reported measurements, even if the PRU location information and the PRU measurements would be included in the same report. Agreement RAN1 will continue discussions on what enhancements to LPP, NRPPa, and/or RAN signaling are necessary to support simultaneous measurements of the same DL-PRS for multiple UEs, including a target UE and a PRU; and to support simultaneous transmission of SRS for multiple UEs, including a target UE and a PRU. RAN1 will include the outcome in an LS to SA2 and RAN2, and cc RAN3. Note: Enhancements might or might not have RAN1 specification impact. Agreement The draft LS reply in R1-2302145 is endorsed. Final LS in R1-2302146. Agreement To enable UE-based and UE-assisted NR carrier phase positioning (CPP), one or both of the following new measurements should be introduced: - DL carrier phase (CP), which is obtained by a UE measuring the DL PRS signal(s) from a TRP. - FFS: The detailed definition of the DL CP - DL carrier phase difference (CPD), which is the difference of two DL CPs from two TRPs - FFS: The detailed definition of the DL CPD To enable NG-RAN node-assisted NR carrier phase positioning (CPP), the following new measurement should be introduced: - UL carrier phase (CP), which is obtained by a TRP measuring the UL SRS for positioning or MIMO SRS from a UE. - FFS: The detailed definition of the UL CP Agreement For NR carrier phase positioning, at least support the following approach: enable a UE/TRP to report carrier phase measurements together with the legacy positioning measurements to LMF - FFS: which legacy positioning measurements among RSTD, RTOA, UE Rx-Tx time difference measurements, gNB Rx-Tx time difference measurements Agreement NR DL reference signal carrier phase (RSCP) (of i-th path) is defined as the phase of the channel response at the i-th path delay derived from the resource elements (REs) that carry the DL PRS signals configured for the measurement. A RSCP is associated with a specific RF frequency. - FFS: the reference point of the RSCP - FFS: whether/how the measurement timing is defined - Note: the i-th path is used for the sake of definition, whether only the first path or additional paths will be supported is subject to further discussion - Note: Whether to capture the above definition into TS 38.215 depends on whether RAN1 decides to introduce DL carrier phase measurement for NR CPP Agreement For NR DL reference signal carrier phase difference (RSCPD) measurement for NR CPP, the RSCPD is defined as the difference of RSCPs measured from the DL PRS signals from target TRP and reference TRP. - FFS: whether/how to define per path RSCPD - Note: Whether/how to capture the above definition into TS 38.215 depends on whether RAN1 decides to introduce DL carrier phase difference measurement for NR CPP Agreement NR UL reference signal carrier phase (RSCP) (of i-th path) is defined as the phase of the channel response at the i-th path delay derived from the resource elements (REs) that carry the UL SRS signal for positioning purpose configured for the measurement. A UL RSCP is associated with a specific RF frequency. - FFS: the reference point of the UL RSCP - FFS: whether/how the measurement timing is defined - Note: the i-th path is used for the sake of definition, whether only the first path or additional paths will be supported is subject to further discussion - Note: The support of MIMO SRS for positioning is transparent to UE Agreement To support NR carrier phase positioning, further consider the following options: - Option 1: Support a UE/TRP to report the carrier phase measurements of more than one frequency within a PFL/carrier to LMF - NOTE: the frequency can be the carrier frequency or the frequency of a subcarrier - FFS: the details of reporting, e.g., the maximum number of reported frequencies within a PFL/ carrier - Option 2: Introduce and report a new type of UE/TRP measurement based on carrier phase differentials across multiple subcarriers within a PFL/carrier - NOTE: carrier phase differentials across multiple subcarriers within a carrier can be related to time of arrival - Option 3: Support a UE/TRP to optionally report an estimated integer ambiguity and/or search range of the integer ambiguity to LMF - Option 4: Support LMF to provide the expected integer ambiguity range at least for UE-based NR CPP in the positioning assistance data. Agreement Rel-17 LOS/NLOS indication (when indicated) applies for the carrier phase measurement(s) in the same report.

RAN1 #112bis

Agreement Introduce DL reference carrier phase (DL RSCP) and NR DL reference carrier phase difference (DL RSCPD) as DL carrier phase measurements. - Note: It is up to RAN4 to decide whether and how to define the requirements for DL RSCP and/or DL RSCPD. No LS needed to RAN4 for this note. - DL RSCP can be reported together with UE Rx – Tx time difference measurement - DL RSCPD can be reported together with RSTD measurement - FFS: details on how to eliminate unknown initial Rx phase with RSCP/RSCPD reporting can be further discussed Note: Whether to support standalone DL RSCP and/or DL RSCPD reporting, or DL RSCP/DL RSCPD reporting with other new types of measurements (if agreed), can be further discussed. Agreement Support one of the following options for the definition of the reference point of the UE/TRP carrier phase measurements (down-selection in RAN1#113). - Option 1: - The reference point of the UE carrier phase measurements is defined the same as the reference point of RSTD for frequency range 1 and frequency range 2. - The reference point of the TRP carrier phase measurements is defined the same as the reference point of RTOA for frequency range 1 and frequency range 2. - Note: It is up to UE/TRP’s implementation on how to map the carrier phase to the reference point for reporting. - Option 2: - The reference point of the UE/TRP carrier phase measurements is defined as the antenna phase center of the UE/TRP Rx antenna for frequency range 1 and frequency range 2. - UE/TRP should provide the antenna phase center offset (PCO), i.e., the relative position between the antenna phase center and the antenna connector to LMF FFS: the more details of the PCO reporting, e.g., in LCS or GCS frame Agreement To enable simultaneous transmission of UL SRS for positioning by a target UE and a PRU, support the following enhancements: - Enabling LMF to request the serving gNB of a UE to configure the transmission of the [indicated] UL SRS resources from the UE within indicated time window(s). - FFS: the details of the time window, e.g., the start time, duration, periodicity for the time window(s), within the vicinity of a reference SRS configuration or use the existing message of Scheduled Location time - Enabling LMF to request the serving gNB and neighboring gNBs of the UE to measure the [indicated] UL SRS resources from the UE within indicated time window(s). - Note: this may be a different indicated time window Agreement To enable simultaneous measurements on same DL PRS by a target UE and a PRU, support the following enhancements: - Enabling LMF to request the UEs, including target UE and PRU(s), to perform measurements on [indicated] DL PRS resources occurring within indicated time window(s). - FFS: the details of the configuration of the indicated time window(s), e.g., the start time, duration, periodicity for the time window(s), as well as the relationship with the Scheduled Location time. Agreement Support the reuse of existing physical layer procedures for DL positioning (e.g., DL-TDOA) with the necessary enhancements in measurement configuration, request and report (e.g., adding the configuration related to the NR DL CPP) for both UE-based and UE-assisted NR DL carrier phase positioning, including - UE in RRC_CONNECTED state with measurement gap. - FFS: UE in RRC_CONNECTED state without measurement gap - UE in RRC_ INACTIVE state Agreement The specific RF frequency associated with a DL carrier phase measurement is defined as the center frequency of the DL PFL by default. - Note: It is open to further discussion whether a frequency other than the center frequency of the DL PFL can also be the specific RF frequency for non-default case(s), if RAN1 agrees to introduce them. Agreement The specific RF frequency associated with a UL carrier phase measurement is defined, by default, as the center frequency of the transmission bandwidth of the SRS for positioning purpose. - Note: It is open to further discussion whether a frequency other than the center frequency of the UL carrier can also be the specific RF frequency for a non-default case(s), if RAN1 agrees to introduce them. Agreement - Support enabling a TRP to report UL RSCP together with RTOA and/or gNB Rx-Tx time difference measurements to LMF - Note 1: The report of UL carrier phase measurement with gNB Rx – Tx time difference does not necessarily require the report of DL carrier phase measurement with UE Rx – Tx time difference. - Note 2: This doesn’t preclude standalone UL carrier phase measurements reporting. Agreement Further study whether and how to support a UE/TRP to report the carrier phase measurement quality indication for corresponding the phase measurements. Agreement For NR UL carrier phase positioning for UE in RRC_CONNECTED and RRC_INACTIVE states, support reuse of existing physical layer procedures for UL positioning (e.g., UL-TDOA), with necessary enhancements in the measurement configuration, measurement request and measurement report (e.g., the configuration related to the NR UL CPP). - FFS: the details of the enhancements. Agreement Adopt one of the following options for a timestamp associated with a reported RSCP/RSCPD measurement (make the decision in RAN1#113): - Option 1: - NR-TimeStamp, currently defined in TS 37.355, is reused as the timestamp with the granularity of a slot. - FFS: Whether to clarify in the specification the reported RSCP/RSCPD value presents the RSCP/RSCPD of a specific OFDM symbol within the slot identified by the NR-TimeStamp. - Option 2: - NR-TimeStamp, currently defined in TS 37.355, should be enhanced to include the OFDM symbol index in a slot, as the timestamp for RSCP/RSCPD measurements. Agreement To address the impact of the phase delays on Tx/Rx RF chains, support one or more of the following options (down-selection in RAN1#113): - Option 1a: introduce the definition of UE/TRP Tx/Rx phase error groups (PEGs) for the Tx/Rx of DL PRS/UL SRS signals - Rel-17 definitions of UE/TRP Tx/Rx TEGs can be used as the starting point for defining UE/TRP Tx/Rx PEGs. - FFS: the details of \the UE/TRP Tx/Rx PEGs - Option 1b: Introduce Tx/Rx RF antenna IDs or Tx/Rx RF chain IDs to identify the individual Tx/Rx RF chains for transmitting/receiving the DL PRS/UL SRS signals. - FFS: the details of the Tx/Rx RF antenna IDs or Tx/Rx RF chain IDs - Note: Device transmitting PRS or positioning SRS provides Tx antenna ID or Tx Chain ID. Device receiving PRS or positioning SRS provides Rx antenna ID or Rx Chain ID. - Option 1c: introduce the report of ARP ID for the Rx/Tx of DL PRS/UL SRS signals. - The transmission/reception associated with the same ARP ID is assumed from the same ARP. - FFS: the maximum number of ARP IDs. - Option 2: reuse or enhance the existing Rel-17 definitions of UE/TRP Tx/Rx TEGs with smaller margin value. - Option 3: RAN1 sends an LS to RAN4, requesting RAN4 to consider whether there is a need to define the new UE/TRP Tx/Rx phase error groups (PEGs), introduce new IDs (e.g., Tx/Rx RF antenna IDs ) to present the phase delays for the Tx/Rx of DL PRS/UL SRS signals, or reuse or enhance the existing Rel-17 definitions of UE/TRP Tx/Rx TEGs with smaller margin value, and provide the definitions if RAN4 decides it is needed.