3GPP TSG-RAN WG4 Meeting#107	R4-2309686

Icheon, Korea 22nd May – 26th May, 2023

Source: 	Qualcomm Incorporated

Title: 	Views on PDSCH Performance Requirements for FR2 Multi-Rx

Agenda item:	8.8.4.2

Document for:	Discussion

# 1. Introduction

After the conclusion of RAN4#106bis-e, there were a few open issues that need to be further discussed [1]. In this paper, we share our views on these open issues for PDSCH demodulation requirements.

2. Demodulation Requirements for PDSCH

- Proposals

  - Option 1: Consider including TDLA30-300 for 120kHz/200MHz requirements, if introduced

  - Option 2: Consider TDLA30-300 for 100 MHz/120 kHz.

  - Option 3: Consider TDLA30-75 for 100MHz/120KHz

Channel Model

We note that 75Hz Doppler captures a reasonable UE velocity and considering the use cases of FR2, we are of the opinion that TDLA30-75 a good channel model to define the PDSCH demodulation performance requirements for FR2 multi-Rx.

Proposal 1: Consider TDLA30-75 for 100MHz/120KHz (Option 3).

- Proposals

  - Option 1: One PTRS port per TRP for both sDCI and mDCI schemes

  - Option 2: One port PTRS for s-DCI SDM scheme

PTRS Pattern

We think that one port per TRP for PTRS is a feasible option for both sDCI and mDCI configurations. Due to correlated nature of channels in FR2 multi-Rx scenarios, having one PTRS port may prove to be difficult for common phase estimation.

Proposal 2: One PTRS port per TRP for both sDCI and mDCI schemes (Option 1).

  - Time offsets between TRPs

We think that time offsets between TRPs for FR2 can be chosen by scaling the FR1 time offsets, which is captured in option 1. We are not sure as to how certain time offsets were tied to different transmission scenarios for sDCI and mDCI schemes in Option 4.

- Proposals

  - Option 1: Use the time offset between the two TRPs transmission points as {0.25us, -0.0625us}.

  - Option 2: Use time offset of TRP1 with respect to TRP2 as {0.2us, -0.05us} for different test cases.

  - Option 3: For NR FR2-1 multi-Rx chain DL reception, consider sDCI and mDCI based SDM scheme with timing offset between TRxPs as follow

    - Timing offset of the second TRxP fro

    - m the first TRxP [us] as -0.0625 or 0.25.

  - Option 4: Select following value for timing offset of the second TRxP from the first TRxP for demodulation cases.

|  |  | Timing offset[us] |
| --- | --- | --- |
| Multi-DCI based multi-TRP | non-overlapping | -0.0625 |
|  | full-overlapping | -0.0625 |
| Single-DCI based multi-TRP | SDM | 0.25 |

  - Select 3000Hz frequency offset for all demodulation cases.

Proposal 3: Use the time offset between the two TRPs transmission points as {0.25us, -0.0625us} (Option 1).

  - Frequency offsets between TRPs

- Proposals

- Option 1: Select 3000Hz frequency offset for all demodulation cases as starting point.

- Option 2: 600Hz

- Option 3: 400Hz

Similar to the time offset, we are of the opinion that frequency offset between TRPs for FR2 can also be chosen by scaling the FR1 frequency offset.

Proposal 4: Consider 600Hz frequency offset between TRPs (Option 2).

3. Conclusions

In this paper, we share our views on PDSCH demodulation requirements for FR2 multi-Rx DL tests. The following have been proposed.

Proposal 1: Consider TDLA30-75 for 100MHz/120KHz (Option 3).

Proposal 2: One PTRS port per TRP for both sDCI and mDCI schemes (Option 1).

Proposal 3: Use the time offset between the two TRPs transmission points as {0.25us, -0.0625us} (Option 1).

Proposal 4: Consider 600Hz frequency offset between TRPs (Option 2).

4. References

[1] R4-2306000, “WF on UE demodulation and CSI requirements for FR2 multi-Rx DL reception,” 3GPP TSG RAN WG4 Meeting #106bis-e, Online, Apr 17 – 26, 2023.