3GPP TSG-RAN WG4 Meeting # 107						R4-2309716

Incheon, KR, May 22-26, 2023

Agenda Item:	8.2.3

Source: 		T-Mobile USA, Ericsson

Title: 	n71 capability signalling for symmetric bandwidths

Document for:	Approval

- Introduction

At RAN4#106e a Way Forward on adding new channel bandwidths to existing bands was approved in R4-2306545 [1]. The Way Forward included:

FFS if any signalling is needed to indicate the optional symmetrical UL/DL CBW support. Companies to confirm that support for n71 UL CHBWs are indicated via the channelBWs-UL IE in UE capabilities, and that support for UL 25 and UL 30 will indicate support for symmetric 25 and 30 MHz with no new signalling needed.

This contribution explains why we think no new signalling is needed.

- Discussion

The channelBWs-UL/ChannelBWs-UL-1590 and channelBWs-DL/ChannelBWs-DL-1590IEs indicate which channel BWs the UE supports on the UL and DL.

Observation 1: The channelBWs-UL/ChannelBWs-UL-1590 and channelBWs-DL/ChannelBWs-DL-1590  IEs indicate which channel BWs the UE supports on the UL and DL.

If asymmetric BCS0 is defined for an NR band, it is mandatory for a UE to support. Asymmetric BCS0 is defined for n71, so it is mandatory for the UE to support.

Observation 2: If asymmetric BCS0 is defined for an NR band, it is mandatory for a UE to support. Asymmetric BCS0 is defined for n71, so it is mandatory for the UE to support.

If a UE indicates support for uplink channel bandwidths greater than 20 MHz, it must support symmetric UL/DL with those channel bandwidths.

Observation 3: If a UE indicates support for uplink channel bandwidths greater than 20 MHz, it must support symmetric UL/DL with those channel bandwidths.

The conclusion is straightforward: There is no need for new signalling to indicate support for symmetric channel bandwidths greater than 20 MHz. The existing signalling is sufficient.

Proposal: There is no need for new signalling to indicate support for symmetric channel bandwidths greater than 20 MHz. The existing signalling is sufficient.

- Conclusion

Observation 1: The channelBWs-UL/ChannelBWs-UL-1590 and channelBWs-DL/ChannelBWs-DL-1590  IEs indicate which channel BWs the UE supports on the UL and DL.

Observation 2: If asymmetric BCS0 is defined for an NR band, it is mandatory for a UE to support. Asymmetric BCS0 is defined for n71, so it is mandatory for the UE to support.

Observation 3: If a UE indicates support for uplink channel bandwidths greater than 20 MHz, it must support symmetric UL/DL with those channel bandwidths.

Proposal: There is no need for new signalling to indicate support for symmetric channel bandwidths greater than 20 MHz. The existing signalling is sufficient.

# 4.	References

- R4-2306545 “WF on adding new channel bandwidths support to existing NR bands,” Ericsson