3GPP TSG-RAN4 Meeting #107	 R4-2309719

# Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on ATG Mobility Requirements

Agenda item:	8.14.4.2

Document for:	Discussion

Introduction

In [1], the work item for Air-to-Ground communication was approved to discuss requirements for the deployments of ATG UEs. The discussions are intended to address the characteristics of ATG networks, such as extremely large inter-site distance, and the list of objectives for RRM discussion include  “considering the different nature of ATG UEs and their view of the network, increased cell sizes and other relevant aspects”. The work has been progressing in RAN4 since then, and in RAN4 #106, a Way-Forward on the topic was discussed and approved in [3]. The WF presents still some relevant open issues to be addressed in this Work Item. In this document we’ll focus our contributions on the open item regarding:

- ATG Mobility Requirements

Discussion

In [2], several timing aspects for ATG operation were discussed. In this contribution we will focus on some of the key open issues on the cited WF.

RRC_IDLE

One of the issues regarding the mobility for ATG UEs in RRC IDLE was the discussion whether to introduce the cell reselection mechanism based on distance threshold and height. The issue was further captured in the WF [2]:

Issue 2-1-1-1: Cell re-selection mechanism - Option 1: Introduce distance-based triggering as an additional condition for intra- and inter-frequency cell measurement as NTN, whether and how to configure the feature is up to network implementation. - If Srxlev > S(non)IntraSearchP and Squal > S(non)IntraSearchQ, and the distance between UE and serving cell reference location is smaller than distanceThresh if distanceThresh is configured and UE has location information, then the UE shall search for inter/intra-frequency layers of higher priority. - If Srxlev ≤ S(non)IntraSearchP or Squal ≤ S(non)IntraSearchQ, or the distance between UE and serving cell reference location is larger than distanceThresh if distanceThresh is configured and UE has location information, then the UE shall search for and measure inter/intra-frequency layers of higher, equal or lower priority in preparation for possible reselection. - Option 4: - If ATG UE altitude higher than certain threshold, ATG UE perform measurement - If ATG UE altitude lower than certain threshold, ATG UE may not perform measurement - Option 5: No need to specify distance, altitude or speed-based enhancements to cell re-selection mechanism during Release 18. - If needed, such mechanisms may be discussed in future releases. Agreements: Candidate options for FFS: - Option 1 - Option 1+4 - Option 5 - For cell reselection, the A2G UE should resume the neighbor cell measurement in normal manner without any relaxation when the condition on distance between the aircraft and the reference location is no longer met. - Option 3: further consider the trade-off between the workload and performance gain brought by the new cell re-selection mechanisms.

In Option 1, the discussion is whether to introduce distance threshold for cell reselection in Idle Mode. The distance threshold parameter was introduced as part of the mobility framework for NTN in Rel-17. The main justification used by then was based on two main arguments:

- especially in NGSO scenarios, the UE was expected to face several reselections a day, caused by the movement of the satellite

- The “near-far” effect in NTN is much less pronounced than in previous deployments, as the distances between UE and satellite are so large that there was virtually no difference between the propagation conditions observed between the satellite and the closest or the farthest UE in a cell.

Because of those two aspects, the mobility in idle mode should be enhanced to minimize UE power consumption. This optimization could prevent a UE in Idle mode to be camped in a cell with degrading conditions, to not jeopardize the UE performance in case this UE makes the transition to RRC connected. Although the functionality might bring some gain for ATG UEs, the gain is not likely to be as high as it was in NTN. Another issue with Option 1 regards the availability of the distance threshold parameter.

RAN2 has sent a LS Reply to RAN4 (R2-2304565), indicating that they will provide signalling support for indicating the position of the base station (still under discussion whether the diluted or the real position is to be provided). As the adoption of SIB19 for NTN is not clearly defined, it cannot be expected that the distanceThresh parameter will be available for NTN.

The distance threshold parameter for cell reselection is broadcast in SIB19. It is still not clear whether RAN2 will utilize SIB19 to provide assistance information to the ATG UE or if this information will be provided in a different SIB.

As it remains unclear whether the distance threshold parameter will be available in ATG, not using it as an enhancement for cell reselection

For Option 4, it is not clear the gains that are harvested from such implementation. The measurement condition for cell reselection is based on the conditions of the serving cell. If the UE is below a “threshold height” but the radio signal conditions are degrading considerable, there is no reason to prevent this UE to perform measurements and move to a more suitable cell.

In IDLE Mode, the UE only perform measurements to evaluate the condition of the serving cell. The main advantage of reducing the number of measurements collected by the UE is to save UE power. But, this gain is rather limited and almost negligible:

- Aircrafts on the ground have very low mobility for most of the time. Unless they are landing or taking off, it is unlikely that the ATG UE will be moving in a very significant manner. Measurement relaxation for UEs with low mobility and/or not at cell edge can already be used here to minimize the impact of IDLE mode measurements.

- If the aircrafts are on the ground it is more likely that the CPE will be turned off. It is more likely that the CPE in the aircraft will be turned on when the aircraft is close to start moving. Therefore, creating extra exception for IDLE mode mobility does not seem justifiable here.

Do not introduce height-dependent requirements for measurement procedures for ATG UES.

There are no significant gains for adopting height threshold for measurements in idle mode, that justifies asking RAN2 to provide signalling support for this new threshold.

Do not adopt introduce height dependent threshold for ATG UEs.

Measurements using directional antennas

This WI has considered ATG implementations at frequencies around 2 and 4 GHz. As it is currently discussed in RF topics, the ATG UE operating at 4 GHz is assumed to be a CPE UE capable of directional transmissions towards the base station.

However, 4 GHz is still a frequency in the range of FR1 where the mobility requirements were designed for a UE using omni directional transmission. For cell reselection, UEs operating in the band above 10 GHz are expected to have beam directionality according to the WID.

A UE capable of directional transmission is performing “tracking” of the base station to a certain degree. This brings the advantage to enhance the antenna gain in the direction of the serving cell, but it potentially decreases the antenna gain in the direction of neighboring cells. Therefore, the UE shall perform measurements also in different directions when measuring neighbor cells.

A similar procedure was adopted in legacy (terrestrial) deployments in FR2, as depicted in the Example from Table 4.2.2.3-1, from TS 38.133 [4], for intra-frequency mobility in RRC_IDLE (highlights are our own):

Table 4.2.2.3-1: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra

| DRX cycle length [s] | Scaling Factor (N1) |  |  | Tdetect,NR_Intra [s] (number of DRX cycles) | Tmeasure,NR_Intra [s] (number of DRX cycles) | Tevaluate,NR_Intra [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2-1Note1 | FR2-2 Note2 |  |  |  |
| 0.32 | 1 | 8 | 12 | 11.52 x N1 x M2 (36 x N1 x M2) | 1.28 x N1 x M2 (4 x N1 x M2) | 5.12 x N1 x M2 (16 x N1 x M2) |
| 0.64 |  | 5 | 8 | 17.92 x N1 (28 x N1) | 1.28 x N1 (2 x N1) | 5.12 x N1 (8 x N1) |
| 1.28 |  | 4 | 6 | 32 x N1 (25 x N1) | 1.28 x N1 (1 x N1) | 6.4 x N1 (5 x N1) |
| 2.56 |  | 3 | 5 | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
| Note 1: Applies for UE supporting FR2-1 power class 2&3&4. For UE supporting FR2-1 power class 1 or 5, N1 = 8 for all DRX cycle length. Note 2: Applies for UE supporting FR2-2 power class 2&3. For UE supporting FR2-2 power class 1, N1 = 12 for all DRX cycle length. Note 3: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. If different SMTC periodicities are configured for different cells, the SMTC periodicity in this note is the one used by the cell being identified. During PSS/SSS detection, the periodicity of the SMTC configured for the intra-frequency carrier is assumed, and if the actual SSB transmission periodicity is greater than the SMTC configured for the intra-frequency carrier, longer Tdetect, NR_intra is expected. |  |  |  |  |  |  |

The table indicates that the time to detect may be enlarged to accommodate measurements across multiple beams at the UE side.

Therefore, for operation in 4 GHz, we support a scaling factor for measurements across different beams, but in a smaller degree than the one applied for FR2.

For UEs with directional antennas operating in 4 GHz, use a scaling factor for cell reselection requirements (Tdetect, Tevaluate, T measure) for measurements across different beams.

Conclusion

In the paper, we analysed some of the open issues for ATG timing requirements. The following Observations and Proposals were made:

Observation 1: The distance threshold parameter for cell reselection is broadcast in SIB19. It is still not clear whether RAN2 will utilize SIB19 to provide assistance information to the ATG UE or if this information will be provided in a different SIB.

Proposal 1: As it remains unclear whether the distance threshold parameter will be available in ATG, not using it as an enhancement for cell reselection

Proposal 2: Do not introduce height-dependent requirements for measurement procedures for ATG UES.

Observation 2: There are no significant gains for adopting height threshold for measurements in idle mode, that justifies asking RAN2 to provide signalling support for this new threshold.

Proposal 3: Do not adopt introduce height dependent threshold for ATG UEs.

Proposal 4: For UEs with directional antennas operating in 4 GHz, use a scaling factor for cell reselection requirements (Tdetect, Tevaluate, T measure) for measurements across different beams.

- References

- RP-220962, “New WID: Air-to-ground network for NR”, CMCC, RAN #95-e, March 17 – March 23, 2022.

- R4-2306344, “WF on NR ATG RRM requirements”, CMCC, RAN4 #106, Feb 27 – March 03, 2023, Athens, Greece