3GPP TSG-RAN WG4 Meeting #107	R4-2309723

Electronic Meeting, 22 May. – 26 May. 2023

Agenda Item:	7.1.1.2

Source:	Charter Communications, Inc.

Title:	Discussion of simulation results on UE RF NR-U UL CA A-MPR for PC5

Document for:	Discussion

# Introduction

In RAN4#106 meeting, a WF for introducing Carrier aggregation to Power class 5 (PC5) in NR-unlicensed spectrum for was approved [1].

“In Release 17 the CA_n96B/C UL CA and configurations are specified. Even so, it was clarified in RAN that although NR-U ULCA requirements were completed, NR-U ULCA could not be deployed since the A-MPR for n96B/C UL was not specified, especially for the U.S. NS_53 and NS_54.” [1]

Then in RAN4#106bis-e a Way Forward was provided with simulation requirements [2].

In this paper we will provide some initial simulation results for NR-U UL CA A-MPR for NS_54. The paper will start off recapping the general requirements as well for the NS_54. Then there will be the initial results.

Requirements for NS_53 will most likely be mostly based on the in-band emission requirement, especially for below aggregated bandwidths of 100MHz/n96B

# Discussion

## PC5 CA MPR

Simulations were done for unlicensed spectrum with PC5. The simulations were calibrated with the following PA calibration point and impairments with the list provided below:

## Simulation calibrations

### PC5 UL CA (A-)MPR:

- Power Class: PC5

- Calibration Point: 1dB MPR for QPSK DFT-s-OFDM 20MHz 100RB3 waveform at 27dB ACLR

- Post PA losses 4dB

- ACLR: 27 dB

- SEM: NR-U

- IQ image: -28dB

- Carrier leakage: -28dBc

General spectrum emission mask for intra-band contiguous CA operation with shared spectrum channel access can be seen below, with the modification of the spectrum emission mask highlighted with yellow.

Table 6.5F.2A.1.2-1: Spectrum emission mask for intra-band contiguous CA operation with shared spectrum channel access Spectrum emission limit (dBr) / Channel bandwidth. ΔfOOB (MHz); 10, 20, 40, 60, 80, 100 MHz; Measurement bandwidth (MBW). ± 0-1; -20 ∆fOOB; [100kHz]3. ± 1-(BWChannel_CA / 2); – 20 – (8 / A) ¦ΔfOOB – 1¦ where A = (BWChannel_CA / 2) – 1; 1 MHz. ± (BWChannel_CA / 2)-BWChannel_CA; – 16 – (624 / BWChannel_CA) ¦ΔfOOB¦. < - BWChannel_CA or > BWChannel_CA; -40. NOTE 1: Void NOTE 2: Void NOTE 3: The measured value shall be scaled by a factor equal to the ratio of the reference bandwidth (1 MHz) to the measurement bandwidth before the emission limit (dBr) is applied. NOTE 4: The carrier leakage exceptions from Table 6.4F.2.3-1 apply and carrier leakage contribution shall be removed prior to setting the 0dBr level of the mask, the reported carrier frequency location in txDirectCurrentLocation field of the UplinkTxDirectCurrentBWP can be used to cancel the carrier leakage contribution. If txDirectCurrentLocation is not available or is reported with value 3300 or 3301, a carrier frequency location at the center of the channel shall be assumed..

Table 3: Setup for MPR

| Test Number | Modulation | Allocation | Precoder | Sub-Band |
| --- | --- | --- | --- | --- |
| 1 | Pi/2-BPSK | Full | DFT-S-OFDM | All |
| 2 | Pi/2-BPSK | Wideband Full | DFT-S-OFDM | e.g. 0-1, 0-010, 0-100, 0-011, 1-100, 1-1100, 0-0111 |
| 3 | Pi/2-BPSK | Interlace | DFT-S-OFDM | All |
| 4 | Pi/2-BPSK | Wideband interlace | DFT-S-OFDM | e.g. 0-1, 0-010, 0-100, 0-011, 1-100, 1-1100, 0-0111 |
| 5 | QPSK | Full | DFT-S-OFDM | All |
| 6 | QPSK | Full | CP-OFDM | All |
| 7 | QPSK | Wideband Full | DFT-S-OFDM | e.g. 0-1, 0-010, 0-100, 0-011, 1-100, 1-1100, 0-0111 |
| 8 | QPSK | Wideband Full | CP-OFDM | e.g. 0-1, 0-010, 0-100, 0-011, 1-100, 1-1100, 0-0111 |
| 9 | QPSK | Interlace | DFT-S-OFDM | All |
| 10 | QPSK | Interlace | CP-OFDM | All |
| 11 | QPSK | Wideband interlace | DFT-S-OFDM | e.g. 0-1, 0-010, 0-100, 0-011, 1-100, 1-1100, 0-0111 |
| 12 | QPSK | Wideband interlace | CP-OFDM | e.g. 0-1, 0-010, 0-100, 0-011, 1-100, 1-1100, 0-0111 |
| 13 | 16QAM | Full | DFT-S-OFDM | All |
| 14 | 16QAM | Full | CP-OFDM | All |
| 15 | 16QAM | Wideband Full | DFT-S-OFDM | e.g. 0-1, 0-010, 0-100, 0-011, 1-100, 1-1100, 0-0111 |
| 16 | 16QAM | Wideband Full | CP-OFDM | e.g. 0-1, 0-010, 0-100, 0-011, 1-100, 1-1100, 0-0111 |
| 17 | 16QAM | Interlace | DFT-S-OFDM | All |
| 18 | 16QAM | Interlace | CP-OFDM | All |
| 19 | 16QAM | Wideband interlace | DFT-S-OFDM | e.g. 0-1, 0-010, 0-100, 0-011, 1-100, 1-1100, 0-0111 |
| 20 | 16QAM | Wideband interlace | CP-OFDM | e.g. 0-1, 0-010, 0-100, 0-011, 1-100, 1-1100, 0-0111 |
| 21 | 64QAM | Full | DFT-S-OFDM | All |
| 22 | 64QAM | Full | CP-OFDM | All |
| 23 | 64QAM | Wideband Full | DFT-S-OFDM | e.g. 0-1, 0-010, 0-100, 0-011, 1-100, 1-1100, 0-0111 |
| 24 | 64QAM | Wideband Full | CP-OFDM | e.g. 0-1, 0-010, 0-100, 0-011, 1-100, 1-1100, 0-0111 |
| 25 | 64QAM | Interlace | DFT-S-OFDM | All |
| 26 | 64QAM | Interlace | CP-OFDM | All |
| 27 | 64QAM | Wideband interlace | DFT-S-OFDM | e.g. 0-1, 0-010, 0-100, 0-011, 1-100, 1-1100, 0-0111 |
| 28 | 64QAM | Wideband interlace | CP-OFDM | e.g. 0-1, 0-010, 0-100, 0-011, 1-100, 1-1100, 0-0111 |
| 29 | 256QAM | Full | DFT-S-OFDM | All |
| 30 | 256QAM | Full | CP-OFDM | All |
| 31 | 256QAM | Wideband Full | DFT-S-OFDM | e.g. 0-1, 0-010, 0-100, 0-011, 1-100, 1-1100, 0-0111 |
| 32 | 256QAM | Wideband Full | CP-OFDM | e.g. 0-1, 0-010, 0-100, 0-011, 1-100, 1-1100, 0-0111 |
| 33 | 256QAM | Interlace | DFT-S-OFDM | All |
| 34 | 256QAM | Interlace | CP-OFDM | All |
| 35 | 256QAM | Wideband interlace | DFT-S-OFDM | e.g. 0-1, 0-010, 0-100, 0-011, 1-100, 1-1100, 0-0111 |
| 36 | 256QAM | Wideband interlace | CP-OFDM | e.g. 0-1, 0-010, 0-100, 0-011, 1-100, 1-1100, 0-0111 |

For sub-band, all means there is no LBT failure/occupancy in any of the 20MHz channels. Full allocation follows the Table 5.3.3-2 in TS 38.101-1 for how many resource blocks are used in each 20MHz channel [3] and all available resource blocks are utilized in the simulations.

## PC5 CA A-MPR

The networking signaling value NS_54 is used when operating in band n96. Agreement that to add Carrier aggregation (CA) with Power class 5 will be introduced.

## In-Band emissions

There are additional requirements for the transmit power density as seen in Table 6.2F.1-2 in TS 38.101-1 [3] for all unlicensed bands. A part of Table 6.2F.1-2 is shared below.

Approximately, for a full allocation in a 20MHz channel bandwidth, with maximum Power for PC5 (20dBm), the maximum mean power density (dBm/MHz) will be around 7, this means the in-band emission requirements for NS_53, will be leading the simulation results, while for NS_54 it will probably not be the restrictive/stringent requirement. The wider bandwidth is simulated, the more the power density is spread out in the channel bandwidth, the less restrictive the maximum mean power density is.

Table 6.2F.1-2: Additional requirements for transmit power density

| NR Band | NS value | Channel bandwidth (MHz) | Frequency range (MHz) | Maximum mean power density (dBm/MHz) |
| --- | --- | --- | --- | --- |
| n96 | NS_53 | 20, 40, 60, 80 | 5925 – 7125 | -1 |
|  | NS_54 | 20, 40, 60, 80 | 5925 – 6425 | 17 |
|  |  |  | 6525 – 6875 |  |

## Center Frequency at the lower edge of Band n96

Based on the defined NR-ARFCN number for band n96 (Table 5.4.2.3-3 in TS 38.101-1 [3]) , the lowest number for each bandwidth using the formula

FREF = FREF-Offs + ΔFGlobal (NREF – NREF-Offs)

Table 5.4.2.1-1: NR-ARFCN parameters for the global frequency raster

| Frequency range (MHz) | ΔFGlobal (kHz) | FREF-Offs (MHz) | NREF-Offs | Range of NREF |
| --- | --- | --- | --- | --- |
| 0 – 3000 | 5 | 0 | 0 | 0 – 599999 |
| 3000 – 24250 | 15 | 3000 | 600000 | 600000 – 2016666 |

This gives center frequencies for each edge channel depending of bandwidth of the lower edge of band n96 for bandwidth class B:

- 40MHz total aggregated Channel Bandwidth: 5965.02MHz

- 60MHz total aggregated Channel Bandwidth: 5974.98MHz

- 80MHz total aggregated Channel Bandwidth: 5985MHz

- 100MHz total aggregated Channel Bandwidth: 5995.02MHz

For band n96, the additional requirements are offset from the channel bandwidth edge by 20MHz. All other channels will follow the CA MPR backoff limits defined in Table 6.2F.2A.2-1 in TS 38.101-1 [3].

## A-MPR Simulation Results

Charter’s simulation results for NS_54 with Power Class 5, see Figure 1 and Table 1. This is simulated for shared spectrum access intra-band contiguous CA for bandwidth class B only and due to time constraint, we had only time to run some initial simulations with the hope to compare and calibrate with other companies during this May meeting.

Figure 1: Initial simulation results for UL CA A-MPR PC5 for Band n96B for NS_54.

Table 1: Initial simulation results for UL CA A-MPR PC5 for Band n96B with NS_54 for calibration purpose.

| Pre-coding | Modulation | RB Allocation |  |  |
| --- | --- | --- | --- | --- |
|  |  | Full2 (dB) | Partial3 (dB) | Exception for Full5 (dB) |
| DFT-s-ODFM | Pi/2 BPSK4 | ≤ [4.0] |  |  |
|  | QPSK | ≤ [4.0] |  |  |
|  | 16 QAM | ≤ [4.5] |  |  |
|  | 64 QAM | ≤ [5.0] |  |  |
|  | 256 QAM | ≤ [6.0] |  |  |
| CP-OFDM | QPSK | ≤ [5.0] |  |  |
|  | 16 QAM | ≤ [5.0] |  |  |
|  | 64 QAM | ≤ [6.0] |  |  |
|  | 256 QAM | ≤ [7.0] |  |  |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. The MPR applies to interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214 [10]. NOTE 2: Full RB allocation MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and sub-bands are transmitted according to configuration A in Table 6.2F.2A.2-2. NOTE 3: Partial RB allocation MPR applies when one or more RB’s in one or more sub-bands are not allocated and sub-bands are transmitted according to configuration A in Table 6.2F.2A.2-2. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. NOTE 5: Exception for Full RB allocation MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and sub-bands are transmitted according to configuration [B] in Table 6.2F.2A.2-2. |  |  |  |  |

# Conclusion

In this paper, Charter has provided some initial simulation results of MPR and A-MPR (for NS_54) for power class 5 shared spectrum access intra-band contiguous CA for bandwidth class B. We hope to get a calibration discussion with the end objective of adding A-MPR table for band n96 with NS_53 and NS_54 for intra-band contiguous CA for bandwidth class B and class C.

# Reference

[1] R4-2303484, WF on NR-U enhancement, Apple

[2] R4-2306481, WF on NR-U contiguous ULCA A-MPR, Charter Communications, Inc., Apple

[3] TS 38.101-1: User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone, V17.8.0 (2022-12)

[4] TR 37.890, Feasibility Study on 6 GHz for LTE and NR in Licensed and Unlicensed Operations V0.16.0 (2022-06)