3GPP TSG-RAN WG4 Meeting #107	R4-2309767

Incheon, KR, May 22nd – May 26th , 2023

Title: 	WF co-existence study of Above 10GHz NTN band

Agenda Item:	8.27.2

Source: 	Samsung

Document for:	Approval

Topic #1: Scenarios and layout

  - Agreement

Following agreements have been made:

- Take the table in Annex 1 as starting point for TN and NTN system deployment assumptions in co-existence studies.

- Consider the situation of NTN UE inside several TN clusters first

- No need to consider fixed NTN UE vertical angle value by the assumption that NTN UE can track the satellite by either mechanical or electronic method.

  - Further discuss the detailed simulation methodology.

- Further discuss the NTN UE elevation angle value: [10] & 90 degree

- Take the assumption that NTN UE antenna points to the satellite accurately as starting point.

- Mobile VSAT coexistence study for NGSO scenarios is out of the scope of co-existence study.

- FRF=2 with two polarizations (RHCP, LHCP)

  - Open issues

None

Topic #2: System parameters

2.1	Agreement

Following agreements have been made:

- For SAN NF, further evaluate based on following candidate options:

- Option 1: 3.5dB

- Option 2: 5.9 dB

- Take the normalised SAN antenna pattern for Ka-Band coexistence studies and related half-power beam-width (2θ-3dB).

| Equation | Half-power beam-width 2θ-3dB Value | GEO constellation type @ 17GHz | LEO constellation type @ 17GHz |
| --- | --- | --- | --- |
| Fθ=2 J1uu | 2θ-3dB=2×Arcsin1.616×λπD | 0.1763 degrees | 1.7337 degrees |

where:

- Jix  is the Bessel function of first type and ith order with argument x

- is the angle in a θ;φ spherical coordinates system,

- u=πDλsin⁡(θ)

- D : Antenna diameter

- : Wavelength

- For SAN antenna adjacent beam spacing (3dB BW),

ABS[rad] = sqrt(3) x sin(HPBW[degrees]/2) or ABS[rad] = sqrt(3) x sinr(HPBW[rad]/2)

- with ABS [degree]=180/pi x ABS[rad] and

- with HPBW the Half-Power BandWidth of the main lobe from the satellite antenna pattern.

- Align TN parameters as below

| Size of each nominal channel BW in MHz | 200 |
| --- | --- |
| Transmission bandwidth in MHz | 190.08 |
| UE number per cell | Same as the number of BS beam |

In the simulation assumptions, align Table 2.4.2-1 with the previous BS antenna table:

- The antenna element gain shall be 5.5 dBi.

- Antenna element vertical radiation pattern (dB), θ3dB=90°

- Antenna element horizontal radiation pattern (dB), φ3dB=90°

2.2	Open issues

- NTN UE NF for co-existence: await conclusions made by [143] in Main session. If no agreements can be made, then consider following options:

- Option 1: 5.9dB

- Option 2: 1.2dB or 2.1dB

- Option 3: 4dB

- For NTN UE parameters, await conclusions made by [143] in Main session.

Topic #3: System parameters

3.1	Agreements

- Use UMi as the propagation model for TN UE to L-ESIM.

- Use flat ACIR modelling for co-existence

3.2	Open issues

None

Annex 1	Deployment of TN and NTN systems for co-existence studies

| No. | Combination | Aggressor | Victim | Which NTN cell/UE to observe? | Which TN/UE to observe? | Which TN cells in a TN to observe? |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | NTN UL | TN UL | NTN cell: Nadir point. NTN UE: NTN UEs randomly dropped in TN clusters. | TN randomly placed in this NTN beam | Only the active TN clusters which contain NTN UE(s) . |
| 2 | TN with NTN | TN UL | NTN UL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: NTN UEs randomly dropped in TN clusters | Consider an active rate of 20% for TN. | Only the active TN cells in central NTN beam |
| 3 | TN with NTN | NTN UL | TN DL | NTN cell: Nadir point NTN UE: NTN UEs randomly dropped in TN clusters. | TN clusters randomly placed in this NTN beam | All active TN clusters which contain NTN UE(s) |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (to be further discussed for GEO and LEO，interested companies can bring analysis and results for other values). NTN UE: NTN UEs randomly dropped in TN clusters. | TN clusters randomly placed in this NTN beam | Only the active TN clusters which contain NTN UE(s). |
| 4 | TN with NTN | TN DL | NTN UL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE NTN UEs randomly dropped in TN clusters | Consider the active rate of 20% for TN. | All active TN cells in central NTN beam |
| 5 | TN with NTN | TN DL | NTN DL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: NTN UEs randomly dropped in TN clusters. | One cluster with 19 TN cells (57 sectors) randomly placed in the central NTN beam | Only the active TN clusters which contain the NTN UE(s). |
| 6 | TN with NTN | NTN DL | TN DL | NTN cell: Nadir point. NTN UE: NTN UEs randomly dropped in TN clusters. | TN clusters randomly placed in this NTN beam | All in central NTN beam |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (to be further discussed for GEO and LEO，interested companies can bring analysis and results for other values) NTN UE: NTN UEs randomly dropped in TN clusters | TN clusters randomly placed in this NTN beam |  |
| 7 | TN with NTN | NTN DL | TN UL | NTN cell: Nadir point NTN UE: NTN UEs randomly dropped in TN clusters. | TN clusters randomly placed in this NTN beam | Only the active TN clusters which contain NTN UE(s) |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (to be further discussed for GEO and LEO，interested companies can bring analysis and results for other values). NTN UE: NTN UEs randomly dropped in TN clusters. | TN clusters randomly placed in this NTN beam | Only the active TN clusters which contain NTN UE(s). |
| 8 | TN with NTN | TN UL | NTN DL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: NTN UEs randomly dropped in TN clusters | Consider the active rate of 20% for TN. | All active TN cells in central NTN beam |