3GPP TSG-RAN4 Meeting #107	R4-2309781

# Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0365 | rev | 1 | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for 38.101-4: Correction of wrongly placed section for FR2-2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz-Perf |  |  |  |  |  | Date: |  |  | 2023-05-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Section A.3.3.2.5 was added under the wrong parent section. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Move section A.3.3.2.5 to the correct parent section. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Section is placed under the wrong parent section. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.3.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R4-2307942 |  |  |  |  |  |  |  |  |

## << Start of change 1>>

#### A.3.2.2.8	Reference measurement channels for SCS 480 kHz FR2-2

Table A.3.2.2.8-1: PDSCH Reference Channel for TDD UL-DL pattern FR2.480-1 (QPSK)

| Parameter |  | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  |  | R.PDSCH.8-1.1 TDD |  |  |  |  |
| Channel bandwidth |  | MHz | 400 |  |  |  |  |
| Subcarrier spacing |  | kHz | 480 |  |  |  |  |
| Allocated resource blocks |  | PRBs | 66 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 20) = {15,16,17,18,19} for i from {0,…,639} |  |  | N/A |  |  |  |  |
| For Slots i = 320, 321 |  |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 20) = 14 for i from {0,…, 639} |  |  | 11 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1…13} for i from {1,…,639} |  |  | 13 |  |  |  |  |
| Allocated slots per 2 frames |  |  | 477 |  |  |  |  |
| MCS table |  |  | 64QAM |  |  |  |  |
| MCS index |  |  | 4 |  |  |  |  |
| Modulation |  |  | QPSK |  |  |  |  |
| Target Coding Rate |  |  | 0.30 |  |  |  |  |
| Number of MIMO layers |  |  | 1 |  |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 20) = {15,16,17,18,19} for i from {0,…,639} |  |  | N/A |  |  |  |  |
| For Slots i = 320, 321 |  |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 20) = 14 for i from {0,…, 639} |  |  | 12 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1…13} for i from {1,…,639} |  |  | 12 |  |  |  |  |
| Overhead for TBS determination |  |  | 6 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 20) = {15,16,17,18,19} for i from {0,…,639} |  | Bits | N/A |  |  |  |  |
| For Slots i = 320, 321 | Bits |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 20) = 14 for i from {0,…, 639} |  | Bits | 4480 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1…13} for i from {1,…,639} |  | Bits | 5504 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 20) = {15,16,17,18,19} for i from {0,…,639} |  | Bits | N/A |  |  |  |  |
| For Slots i = 320, 321 | Bits |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 20) = 14 for i from {0,…, 639} |  | Bits | 24 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1…13} for i from {1,…,639} |  | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 20) = {15,16,17,18,19} for i from {0,…,639} |  | CBs | N/A |  |  |  |  |
| For Slots i = 320, 321 | CBs |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 20) = 14 for i from {0,…, 639} |  | CBs | 1 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1…13} for i from {1,…,639} |  | CBs | 1 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 20) = {15,16,17,18,19} for i from {0,…,639} |  | Bits | N/A |  |  |  |  |
| For Slots i = 320, 321 | Bits |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 20) = 14 for i from {0,…, 639} |  | Bits | 15048 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1…13} for i from {1,…,639} |  | Bits | 18216 |  |  |  |  |
| Max. Throughput averaged over 2 frames |  | Mbps | 129.632 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |  |

Table A.3.2.2.8-2: PDSCH Reference Channel for TDD UL-DL pattern FR2.480-1 (16QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.8-2.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 400 |  |  |  |  |
| Subcarrier spacing | kHz | 480 |  |  |  |  |
| Allocated resource blocks | PRBs | 20 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 20) = {15,16,17,18,19} for i from {0,…,639} |  | N/A |  |  |  |  |
| For Slots i = 320, 321 |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 20) = 14 for i from {0,…, 639} |  | 11 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1…13} for i from {1,…,639} |  | 12 |  |  |  |  |
| Allocated slots per 2 frames |  | 477 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 13 |  |  |  |  |
| Modulation |  | 16QAM |  |  |  |  |
| Target Coding Rate |  | 0.48 |  |  |  |  |
| Number of MIMO layers |  | 1 |  |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 20) = {15,16,17,18,19} for i from {0,…,639} |  | N/A |  |  |  |  |
| For Slots i = 320, 321 |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 20) = 14 for i from {0,…, 639} |  | 12 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1…13} for i from {1,…,639} |  | 12 |  |  |  |  |
| Overhead for TBS determination |  | 6 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 20) = {15,16,17,18,19} for i from {0,…,639} | Bits | N/A |  |  |  |  |
| For Slots i = 320, 321 | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 20) = 14 for i from {0,…, 639} | Bits | 3904 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1…13} for i from {1,…,639} | Bits | 4736 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 20) = {15,16,17,18,19} for i from {0,…,639} | Bits | N/A |  |  |  |  |
| For Slots i = 320, 321 | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 20) = 14 for i from {0,…, 639} | Bits | 24 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1…13} for i from {1,…,639} | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 20) = {15,16,17,18,19} for i from {0,…,639} | CBs | N/A |  |  |  |  |
| For Slots i = 320, 321 | CBs | N/A |  |  |  |  |
| For Slot i, if mod(i, 20) = 14 for i from {0,…, 639} | CBs | 1 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1…13} for i from {1,…,639} | CBs | 1 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 20) = {15,16,17,18,19} for i from {0,…,639} | Bits | N/A |  |  |  |  |
| For Slots i = 320, 321 | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 20) = 14 for i from {0,…, 639} | Bits | 8160 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1…13} for i from {1,…,639} | Bits | 10080 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 111.6224 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

### A.3.2.3	HD-FDD

## << End of change 1 >>

## << Start of change 2 >>

#### A.3.3.2.5	Reference measurement channels for SCS 120 kHz FR2

Table A.3.3.2.5-1: PDCCH Reference Channels (Time domain allocation 1 symbol)

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDCCH.5-1.1 TDD | R.PDCCH.5-1.2 TDD | R.PDCCH.5-1.3 TDD | R.PDCCH. 5-1.4 TDD |  |  |
| Subcarrier spacing | kHz | 120 | 120 | 120 | 120 |  |  |
| CORESET frequency domain allocation |  | 60 | 60 | 60 | 60 |  |  |
| CORESET time domain allocation |  | 1 | 1 | 1 | 1 |  |  |
| Aggregation level |  | 2 | 4 | 8 | 8 |  |  |
| DCI Format |  | 1_0 | 1_1 | 1_1 | 2_6 |  |  |
| Payload (without CRC) | Bits | 40 | 56 | 56 | 12 |  |  |

Table A.3.3.2.5-2: PDCCH Reference Channel (Time domain allocation 2 symbols)

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDCCH.5-2.1 TDD |  |  |  |  |  |
| Subcarrier spacing | kHz | 120 |  |  |  |  |  |
| CORESET frequency domain allocation |  | 60 |  |  |  |  |  |
| CORESET time domain allocation |  | 2 |  |  |  |  |  |
| Aggregation level |  | 16 |  |  |  |  |  |
| DCI Format |  | 1_0 |  |  |  |  |  |
| Payload (without CRC) | Bits | 40 |  |  |  |  |  |

#### A.3.3.2.6	Reference measurement channels for SCS 480 kHz FR2-2

Table A.3.3.2.6-1: PDCCH Reference Channels (Time domain allocation 1 symbol)

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDCCH.6-1.1 TDD |  |  |  |  |  |
| Subcarrier spacing | kHz | 480 |  |  |  |  |  |
| CORESET frequency domain allocation |  | 60 |  |  |  |  |  |
| CORESET time domain allocation |  | 1 |  |  |  |  |  |
| Aggregation level |  | 8 |  |  |  |  |  |
| DCI Format |  | 1_1 |  |  |  |  |  |
| Payload (without CRC) | Bits | 56 |  |  |  |  |  |

Table A.3.3.2.6-2: PDCCH Reference Channel (Time domain allocation 2 symbols)

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDCCH.6-2.1 TDD |  |  |  |  |  |
| Subcarrier spacing | kHz | 480 |  |  |  |  |  |
| CORESET frequency domain allocation |  | 60 |  |  |  |  |  |
| CORESET time domain allocation |  | 2 |  |  |  |  |  |
| Aggregation level |  | 16 |  |  |  |  |  |
| DCI Format |  | 1_0 |  |  |  |  |  |
| Payload (without CRC) | Bits | 40 |  |  |  |  |  |

## A.3.4	Reference measurement channels for PBCH demodulation requirements

## << End of change 2 >>