3GPP TSG-RAN WG4 Meeting#107                               R4-2309783

Incheon, South Korea, 22 – 26 May 2023

Title: 	WF on system parameters and RF requirements

Agenda Item:	     8.29.7

Source: 	ZTE

Document for:	Approval

Agreement

## System parameter and declaration

Issue 1-1:  System parameter for NCR-MT

- Agreement:

  - Proposal 1 agreed

    - All the channel bandwidths per band should be optional for NCR_MT.

Issue 1-3-1:   Types declaration for NCR

- Agreement:

  - No restriction on the applicable types on NCR-MT and NCR-fwd link (BS side and UE sdide) from RF core requirements perspective

  - FFS whether needs to consider mix types for the introduction of conformance test cases and will be further discussed during performance stage

Issue 1-3-2: Class declarations for NCR

- Agreement:

  - NCR-fwd link DL side (input and output) : same class applied

  - NCR-fwd link UL side (input and output): same class applied

  - NCR-fwd link DL and UL: Separate classes can be applied

  - NCR-MT vs NCR-fwd UL: same class applied

## . RF requirement for NCR-MT

Issue 3-1-1   Output power dynamics and power control

- Agreement:

  - For local area NCR-MT: reuse legacy UE requirements (relative power tolerance and aggregated power tolerance requirements)

  - For wide area NCR-MT:[reuse the output dynamic range requirement, relative power tolerance and aggregated power tolerance requirements from wide area IAB-MT]

Issue 3-1-2   Transmitter ON-OFF power and transition period

- Agreement: Further discuss below two options:

  - Option 1: IAB-MT approach

  - Option 2: Legacy UE approach

Issue 3-1-3   OBW

- Agreement

  - Proposal 1 agreed

Issue 3-1-4   ACLR

- Agreement:

  - For wide area NCR-MT: BS requirements applied

  - For local area NCR-MT: UE requirements (PC2) applied

Issue 3-1-5   OBUE/SEM

- Agreement:

  - For wide area NCR-MT: BS requirements applied

  - For local area NCR-MT: FFS

  - Proposal 3 agreed

Issue 3-1-6   transmitter spurious emission

- Agreement:

  - Proposal 1 agreed

  - If the NCR supports simultaneous MT and FWD transmission, then the UL (i.e. BS side) TX emissions requirements should be defined on the total emissions from MT and FWD.

Issue 3-1-7  Transmitter transmitter intermodulation

- Agreement:

  - Further discuss below two options for NCR-MT:

    - Option 1: IAB-MT approach

    - Option 2: Legacy UE approach

Issue 3-2-1    REFSENS requirement

- Agreement:

- NF is assumed as 5dB for FR1 WA NCR-MT,

- FR1 LA NCR-MT:

- Option1: 13dB

- Option 2: legacy UE value

- NF is assumed as 10dB for 30GHz and 12dB for 45GHz for FR2 NCR-MT

Issue 3-2-2    Dynamic range requirement

- Agreement:

  - Proposal 1 agreed

Issue 3-2-3    ACS/IBB

- Agreement:

  - For wide area NCR-MT: reusing IAB-MT requirements

  - For local are NCR-MT: FFS

Issue 3-2-4    OOBB requirement

- Agreement:

  - For wide area NCR-MT: reusing IAB-MT requirements

  - For local are NCR-MT: FFS

Issue 3-2-6    receiver intermodulation requirement

- Agreement:

  - For wide area NCR-MT: reusing IAB-MT requirements

  - For local are NCR-MT: FFS

Issue 3-2-7   receiver in-channel selectivity requirement requirement

- Agreement:

  - Proposal 1 agreed

## . RF requirement for NCR-Fwd

### 1.3.1 NCR-Fwd type 1-H:

Repeat output power:

Agreement:

- For wide area NCR-fwd link, NTXU,counted = min(NTXU,active , 8*Ncells) for emission requirement only;

- For local area NCR-fwd link, NTXU,counted = min(NTXU,active ,4*Ncells) ;

Adjacent Channel Leakage Power Ratio:

Agreement:

For the absolute ACLR limit and relative ACLR limit for NCR-Fwd and NCR-MT simultaneous transmission, the measurement bandwidth for the absolute ACLR limit should be

For the contiguous case for NCR-MT and NCR-Fwd part:

- follow the nominal bandwidth of NCR-Fwd if NCR-MT bandwidth is placed within NCR-Fwd;

- follow the nominal bandwidth of NCR-Fwd+ NCR-MT bandwdith if NCR-MT bandwidth is placed outside of NCR-Fwd;

For the non-contiguous case for NCR-MT and NCR-Fwd part:

- to follow the CACLR principle.;

For scaling factor for NCR-Fwd absolute ACLR limit for contiguous case:

Agreement:

For wide area NCR-fwd link, NTXU,counted = min(NTXU,active , 8*Ncells) for emission requirement only;

For local area NCR-fwd link, NTXU,counted = min(NTXU,active ,4*Ncells) ;

For scaling factor for NCR-Fwd absolute ACLR limit for non-contiguous case:

Agreement:

To follow the CACLR principle to consider the accumulated emission;

For wide area NCR-fwd link, NTXU,counted = min(NTXU,active , 8*Ncells) for emission requirement only;

For local area NCR-fwd link, NTXU,counted = min(NTXU,active ,4*Ncells) ;

Operating band unwanted emissions

For the scaling factor for OBUE requirement of NCR-Fwd part

Agreement:

For wide area NCR-fwd link, NTXU,counted = min(NTXU,active , 8*Ncells) for emission requirement only;

For local area NCR-fwd link, NTXU,counted = min(NTXU,active ,4*Ncells) ;

For medium range  NCR-fwd link in DL, NTXU,counted = min(NTXU,active , 8*Ncells) for emission requirement only;

For simultaenous transmission for NCR-MT and NCR-Fwd link in the non-contiugous case:

To follow the BS multi-carrier approach without considering further relaxations;

For simultaneous transmission for NCR-MT and NCR-Fwd link in the contiguous case:

To follow the NCR-Fwd OBUE requirement for Fwd part in the uplink part;

Transmitter spurious emissions:

For scaling factors for NCR-Fwd part:

Agreement:

For wide area NCR-fwd link, NTXU,counted = min(NTXU,active , 8*Ncells) for emission requirement only;

For local area NCR-fwd link, NTXU,counted = min(NTXU,active ,4*Ncells) ;

For medium range -NCR-fwd link in DL, NTXU,counted = min(NTXU,active , 8*Ncells) for emission requirement only;

For simulatenous and non-simultaneous transmission of NCR-MT and NCR-Fwd part in the uplink direction:

- Only one limit should be defined;

- FFS to reuse UE spurious emission or repeater spurious emission requiremeent;