3GPP TSG RAN WG4 Meeting #107	R4-2309785

Incheon, KR, May 22 – May 26, 2023

Agenda item:			8.29.7

Source:	Moderator (ZTE)

Title:	Ad-hoc meeting minutes for [107][312] NR_netcon_repeater_RF

Document for:	Discussion

- Introduction

In this contribution, we mainly summarized the ad-hoc meeting minutes for NCR RF requirement which is focusing on the NCR-Fwd part.

- Discussion

## 2.1. NCR-Fwd type 1-H:

### 2.1.1 Repeat output power:

- Option 1: NTXU,counted = min(NTXU,active , 8*Ncells) [ZTE, CATT]

- Option 2: [CMCC]

- For LA, max scaling factor is 6dB, implicating max 30dBm output power, i.e. NTXU,counted = min(NTXU,active , 4*Ncells)

- For WA, max scaling factor is 9dB, i.e. NTXU,counted = min(NTXU,active , 8*Ncells)

Cmcc: the total of sum of output power could be larger than any power class.  The requirement might be met by such scaling factor considered.

Ericsson: agree to limit the power to the ue power class;

CMCC: for the emission requirement, we still needed the scaling factor .

Agreement:

- For wide area NCR-fwd link, NTXU,counted = min(NTXU,active , 8*Ncells) for emission requirement only;

- For local area NCR-fwd link, NTXU,counted = min(NTXU,active ,4*Ncells) ;

### 2.1.2. Adjacent Channel Leakage Power Ratio:

Ericsson: for the simultaneous transmission, propose the same bandwidth for NCR-MT instead of NCR-Fwd otherwise there are some conflict.

ZTE: how to consider the additional emission for nCR-Fwd part;

Agreement:

For the absolute ACLR limit and relative ACLR limit for NCR-Fwd and NCR-MT simultaneous transmission, the measurement bandwidth for the absolute ACLR limit should be

For the contiguous case for NCR-MT and NCR-Fwd part:

- follow the nominal bandwidth of NCR-Fwd if NCR-MT bandwidth is placed within NCR-Fwd;

- follow the nominal bandwidth of NCR-Fwd+ NCR-MT bandwdith if NCR-MT bandwidth is placed outside of NCR-Fwd;

For the non-contiguous case for NCR-MT and NCR-Fwd part:

- to follow the CACLR principle.;

For scaling factor for NCR-Fwd absolute ACLR limit for contiguous case:

Agreement:

For wide area NCR-fwd link, NTXU,counted = min(NTXU,active , 8*Ncells) for emission requirement only;

For local area NCR-fwd link, NTXU,counted = min(NTXU,active ,4*Ncells) ;

For scaling factor for NCR-Fwd absolute ACLR limit for non-contiguous case:

Agreement:

To follow the CACLR principle to consider the accumulated emission;

For wide area NCR-fwd link, NTXU,counted = min(NTXU,active , 8*Ncells) for emission requirement only;

For local area NCR-fwd link, NTXU,counted = min(NTXU,active ,4*Ncells) ;

### 2.1.3. Operating band unwanted emissions

For the scaling factor for OBUE requirement of NCR-Fwd part

Agreement:

For wide area NCR-fwd link, NTXU,counted = min(NTXU,active , 8*Ncells) for emission requirement only;

For local area NCR-fwd link, NTXU,counted = min(NTXU,active ,4*Ncells) ;

For medium range  NCR-fwd link in DL, NTXU,counted = min(NTXU,active , 8*Ncells) for emission requirement only;

For simultaenous transmission for NCR-MT and NCR-Fwd link in the non-contiugous case:

To follow the BS multi-carrier approach without considering further relaxations;

For simultaneous transmission for NCR-MT and NCR-Fwd link in the contiguous case:

To follow the NCR-Fwd OBUE requirement for Fwd part in the uplink part;

### 2.1.4. Transmitter spurious emissions:

For scaling factors for NCR-Fwd part:

Agreement:

For wide area NCR-fwd link, NTXU,counted = min(NTXU,active , 8*Ncells) for emission requirement only;

For local area NCR-fwd link, NTXU,counted = min(NTXU,active ,4*Ncells) ;

For medium range -NCR-fwd link in DL, NTXU,counted = min(NTXU,active , 8*Ncells) for emission requirement only;

For simulatenous and non-simultaneous transmission of NCR-MT and NCR-Fwd part in the uplink direction:

- Only one limit should be defined;

- FFS to reuse UE spurious emission or repeater spurious emission requiremeent;

Receiver spurious emissions:

FFS

## 2.2. NCR-Fwd type 1-O: