3GPP TSG-RAN WG4 Meeting #107	R4-2309829

# Incheon, 22nd - 26th May, 2023

Title: 	pCR on IOT NTN demodulation performance requirements (TS36.181, Rel-18)

Source: 	Huawei, HiSilicon

Agenda item:	9.7.7.2

Document for:	Approval

- Introduction

During RAN4#106-bis-e meeting, WF [1] for SAN demodulation requirements for IoT over NTN was approved. In this contribution, we provide the updated text proposal about FRC of NTN SAN performance requirements to TS 36.181 as per work split.

- Reference

- R4-2305965, WF on IoT NTN SAN demodulation requirements, RAN4#106-bis-e, Ericsson

# Text Proposal

# <START OF THE CHANGE 0>

#### 4.1.2.3	Measurement of performance requirement

Table 4.1.2.3-1: Maximum Test System Uncertainty for Performance Requirements

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 8.2.1 Performance requirements for NPUSCH format 1 | ± 0.6 dB | Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 8.2.2 ACK missed detection for NPUSCH format 2 | ± 0.6 dB | Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 8.2.3 Performance requirements for NPRACH | ± 0.6 dB | Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| In addition, the following Test System uncertainties and related constraints apply: AWGN Bandwidth; ≥ 1.08MHz; NRB x 180kHz according to BWConfig. AWGN absolute power uncertainty, averaged over BWConfig; ±1.5 dB. AWGN flatness and signal flatness, max deviation for any resource block, relative to average over BWConfig; ±2 dB. AWGN flatness over BWChannel, max deviation for any resource block, relative to average over BWConfig; +2 dB. AWGN flatness and signal flatness, max difference between adjacent resource blocks; ±0.5 dB. AWGN peak to average ratio; ≥10 dB @0.001%. Signal-to noise ratio uncertainty, averaged over uplink transmission Bandwidth; ±0.3 dB. Fading profile power uncertainty; Test-specific. Fading profile delay uncertainty, relative to frame timing; ±5 ns (excludes absolute errors related to baseband timing). |
| Note 1: Only the overall stimulus error is considered here. The effect of errors in the throughput measurements due to finite test duration is not considered. |  |  |

Table 4.1.2.3-2: Maximum Test System Uncertainty for Performance Requirements (FR1 OTA performance tests)

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 11.2.1 Performance requirements for NPUSCH format 1 | ± 0.6 dB | Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 11.2.2 ACK missed detection for NPUSCH format 2 | ± 0.6 dB | Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 11.2.3 Performance requirements for NPRACH | ± 0.6 dB | Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| In addition, the following Test System uncertainties and related constraints apply: AWGN Bandwidth; ≥ 1.08MHz; NRB x 180kHz according to BWConfig. AWGN absolute power uncertainty, averaged over BWConfig; ±1.5 dB. AWGN flatness and signal flatness, max deviation for any resource block, relative to average over BWConfig; ±2 dB. AWGN flatness over BWChannel, max deviation for any resource block, relative to average over BWConfig; +2 dB. AWGN flatness and signal flatness, max difference between adjacent resource blocks; ±0.5 dB. AWGN peak to average ratio; ≥10 dB @0.001%. Signal-to noise ratio uncertainty, averaged over uplink transmission Bandwidth; ±0.3 dB. Fading profile power uncertainty; Test-specific. Fading profile delay uncertainty, relative to frame timing; ±5 ns (excludes absolute errors related to baseband timing). |
| Note 1: Only the overall stimulus error is considered here. The effect of errors in the throughput measurements due to finite test duration is not considered. |  |  |

# <END OF THE CHANGE 0>

# <START OF THE CHANGE 1>

## 8.2	Performance requirements for Narrowband IoT

### 8.2.1	Performance requirements for NPUSCH format 1

#### 8.2.1.1	Definition and applicability

The performance requirement of NPUSCH format 1 is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in Annex A. The performance requirements assume HARQ re-transmissions.

The tests for 3.75KHz subcarrier spacing are applicable to the base stations supporting 3.75 kHz subcarrier spacing requirements. The tests for single-subcarrier/multi-subcarrier of 15KHz subcarrier spacing are applicable to the base stations supporting the number of subcarriers of 15 kHz subcarrier spacing requirements.

#### 8.2.1.2	Minimum Requirement

The minimum requirement is in TS 36.102 [2] subclause 8.2.1.

#### 8.2.1.3	Test Purpose

The test shall verify the receiver’s ability to achieve the throughput under multipath fading propagation conditions for a given SNR.

#### 8.2.1.4	Method of test

##### 8.2.1.4.1	Initial Conditions

Test environment:	Normal, see subclause D.2.

RF channels to be tested:	M; see subclause 4.7.

1)	Connect the SAN tester generating the wanted signal, multipath fading simulators and AWGN generators to all SAN antenna connectors for diversity reception via a combining network as shown in Annex I.3.2.

##### 8.2.1.4.2	Procedure

1)	Adjust the AWGN generator, according to the channel bandwidth, defined in Table 8.2.1.4.2-1.

Table 8.2.1.4.2-1: AWGN power level at the SAN input

| Channel bandwidth [KHz] | AWGN power level |
| --- | --- |
| 200 | -100.5dBm /180KHz |

2)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in annex A and the test parameters in Table 8.2.1.4.2-2.

Table 8.2.1.4.2-2: Test parameters for testing NPUSCH format 1

| Parameter | unit | Value |
| --- | --- | --- |
| Maximum number of HARQ transmissions |  | 4 |
| RV sequences |  | RV0, RV2 |
| Duration of NPUSCH format 1 segment transmission (Nsegmentprecompensation) | ms | 256 for 3.75kHz SCS, 16 for 15kHz SCS |
| Repetition |  | 4 for 3.75kHz SCS, 16 for 15kHz SCS |

3)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex B.

4)	Adjust the equipment so that required SNR specified in tables 8.2.1.5-1 to 8.2.1.5-3 is achieved at the SAN input.

5)	The signal generator sends a test pattern with the pattern outlined in Figure 8.2.1.4.2-1. The test signal generator sends frames, and the receiver tries to demodulate the content. The frames are sent with certain frequency and  gradual time offset per segment as described below in Table 8.2.1.4.2-3 and Table 8.2.1.4.2-4. The time offset is reset back to zero between segments.

Figure 8.2.1.4.2-1: Test signal pattern for NPUSCH format 1 demodulation tests

Table 8.2.1.4.2-3: Frequency Offsets for testing NPUSCH format 1

| Cases | Accumulated Frequency Offset |
| --- | --- |
| Table 8.2.1.5-1 | 128 Hz |
| Table 8.2.1.5-2 | 8 Hz |

Table 8.2.1.4.2-4: Timing Offsets for testing NPUSCH format 1

| Cases | Step size Δt |
| --- | --- |
| Table 8.2.1.5-1 | 0.32 us per RU |
| Table 8.2.1.5-2 | 0.01 us per RU |

6)	For each of the reference channels in Table 8.2.1.5-1 to 8.2.1.5-2 applicable for the base station, measure the throughput, according to annex E.

#### 8.2.1.5	Test Requirement

The throughput measured according to subclause 8.2.1.4.2 shall not be below the limits for the SNR levels specified in Table 8.2.1.5-1 for 3.75KHz subcarrier spacing tests and not be below the limits for the SNR levels specified in Table 8.2.1.5-2 for 15KHz subcarrier spacing with the supported number of subcarrier tests.

Table 8.2.1.5-1 Required SNR for NPUSCH format 1 test, 200KHz Channel Bandwidth, 3.75KHz subcarrier spacing, 1Tx

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Repetition number | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 3.75 KHz | 1 | NTN-TDLA100-1 | A16-1 | 4 | 70% | [-1.4] |
| 1 | 1 | 3.75 KHz | 1 | NTN-TDLC5-1 | A16-1 | 4 | 70% | [-2.3] |
| 1 | 2 | 3.75 KHz | 1 | NTN-TDLA100-1 | A16-1 | 4 | 70% | [-4.9] |
| 1 | 2 | 3.75 KHz | 1 | NTN-TDLC5-1 | A16-1 | 4 | 70% | [-5.4] |

Table 8.2.1.5-2 Required SNR for NPUSCH format 1 test, 200KHz Channel Bandwidth, 15KHz subcarrier spacing, multiple subcarriers, 1Tx

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Repetition number | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 15KHz | 12 | NTN-TDLA100-1 | A16-2 | 16 | 70% | [-3.2] |
| 1 | 1 | 15KHz | 12 | NTN-TDLC5-1 | A16-2 | 16 | 70% | [-4.1] |
| 1 | 2 | 15KHz | 12 | NTN-TDLA100-1 | A16-2 | 16 | 70% | [-7.4] |
| 1 | 2 | 15KHz | 12 | NTN-TDLC5-1 | A16-2 | 16 | 70% | [-8.0] |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in Annex G.

### 8.2.2	ACK missed detection for NPUSCH format 2

#### 8.2.2.1	Definition and applicability

The performance requirement of NPUSCH format 2 for ACK missed detection is determined by the two parameters: probability of false detection of the ACK and the probability of detection of ACK. The performance is measured by the required SNR at probability of detection equal to 0.99. The probability of false detection of the ACK shall be 0.01 or less.

The probability of false detection of the ACK is defined as a conditional probability of erroneous detection of the ACK when input is only noise.

The probability of detection of ACK is defined as conditional probability of detection of the ACK when the ACK was sent per NPUSCH format 2 transmission when the signal is present.

The tests for 3.75KHz subcarrier spacing are applicable to the base stations supporting 3.75 KHz subcarrier spacing requirements. The tests for 15KHz subcarrier spacing are applicable to the base stations supporting 15KHz subcarrier spacing requirements.

#### 8.2.2.2	Minimum Requirement

The minimum requirement is in TS 36.102 [2] subclause 8.2.2.1 and 8.2.2.2.

#### 8.2.2.3	Test purpose

The test shall verify the receiver’s ability to detect ACK under multipath fading propagation conditions for a given SNR.

#### 8.2.2.4	Method of test

##### 8.2.2.4.1	Initial Conditions

Test environment:	Normal, see subclause D.2.

RF channels to be tested:	M; see subclause 4.7

1)	Connect the SAN tester generating the wanted signal, multipath fading simulators and AWGN generators to all SAN antenna connectors for diversity reception via a combining network as shown in Annex I.3.2.

##### 8.2.2.4.2	Procedure

1)	Adjust the AWGN generator, according to the channel bandwidth defined in Table 8.2.2.4.2-1.

Table 8.2.2.4.2-1: AWGN power level at the SAN input

| Channel bandwidth [KHz] | AWGN power level |
| --- | --- |
| 200 | -100.5dBm /180KHz |

2)	The characteristics of the wanted signal shall be configured according to TS 36.211 [12] and the test parameters in Table 8.2.2.4.2-2.

Table 8.2.2.4.2-2: Test parameters for testing NPUSCH format 2

| Parameter | unit | Value |
| --- | --- | --- |
| Duration of NPUSCH format 2 segment transmission (Nsegmentprecompensation) | ms | 128 for 3.75kHz SCS, 32 for 15kHz SCS |
| Repetition |  | 16 |

3)	The multipath fading emulators shall be configured according to the corresponding channel model defined in Annex B.

4)	Adjust the equipment so that the SNR specified in tables 8.2.2.5-1 to 8.2.2.5-2 is achieved at the SAN input during the ACK transmissions.

5)	The signal generator sends a test pattern with the pattern outlined in figure 8.2.2.4.2-1. The following statistics are kept: the number of ACKs falsely detected in the idle periods and the number of missed ACKs. Each falsely detected ACK transmission in the idle periods is accounted as one error for the statistics of false ACK detection, and each missed ACK transmission per NPUSCH format 2 transmisson is accounted as one error for the statistics of missed ACK detection. The frames are sent with certain frequency and time offsets per segment as described below in Table 8.2.2.4.2-3 and Table 8.2.2.4.2-4. The time offset is reset back to zero between segments.

Figure 8.2.2.4.2-1: Test signal pattern for NPUSCH format 2 demodulation tests

Table 8.2.2.4.2-3: Frequency Offsets for testing NPUSCH format 2

| Cases | Accumulated Frequency Offset |
| --- | --- |
| Table 8.2.2.5-1 | 64 Hz |
| Table 8.2.2.5-2 | 16 Hz |

Table 8.2.2.4.2-4: Timing Offsets for testing NPUSCH format 2

| Cases | Step size Δt |
| --- | --- |
| Table 8.2.2.5-1 | 0.08 us per RU |
| Table 8.2.2.5-2 | 0.02 us per RU |

#### 8.2.2.5	Test Requirement

The fraction of falsely detected ACKs shall be less than 1% and the fraction of correctly detected ACKs shall be larger than 99% for the SNR listed in Table 8.2.2.5-1 and Table 8.2.2.5-2.

Table 8.2.2.5-1 Required SNR for NPUSCH format 2 test, 200KHz Channel Bandwidth, 3.75KHz subcarrier spacing, 1Tx

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Number of allocated subcarriers | Subcarrier spacing | Repetition number | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | NTN-TDLA100-1 | 1 | 3.75KHz | 16 | [1.9] |
| 1 | 2 | NTN-TDLA100-1 | 1 | 3.75 KHz | 16 | [-4.7] |

Table 8.2.2.5-2 Required SNR for NPUSCH format 2 test, 200KHz Channel Bandwidth, 15KHz subcarrier spacing, 1Tx

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Number of allocated subcarriers | Subcarrier spacing | Repetition number | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | NTN-TDLA100-1 | 1 | 15KHz | 16 | [2.0] |
| 1 | 2 | NTN-TDLA100-1 | 1 | 15KHz | 16 | [-4.4] |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in Annex G.

### 8.2.3	Performance requirements for NPRACH

#### 8.2.3.1	Definition and applicability

The performance requirement of NPRACH for preamble detection is determined by two parameters: the total probability of false detection of the preamble (Pfa) and the probability of detection of the preamble (Pd). The performance is measured for the required SNR

-	at probability of Pd which shall not be smaller than 99% and

-	at probability of Pfa which shall not be larger than 0.1%.

Pfa is defined as a conditional total probability of erroneous detection of the preamble (i.e. erroneous detection from any detector) when input is only noise.

Pd is defined as conditional probability of detection of the preamble when the signal is present. The erroneous detection consists of several error cases – detecting different preamble than the one that was sent, not detecting a preamble at all or correct preamble detection but with the wrong timing estimation. A timing estimation error occurs if the estimation error of the timing of the strongest path is larger than 3.646 us. The strongest path for the timing estimation error refers to the strongest path in the power delay profile.

The performance requirements for TDD are optional and only valid for base stations supporting TDD.

The parameters of NPRACH test preambles are listed in Table 8.2.3.1-1.

Table 8.2.3.1-1 NPRACH Test Parameters

| Parameter | Value | Value |
| --- | --- | --- |
| Narrowband physical layer cell identity | 0 | 0 |
| nprach-Periodicity (ms) | 80 | 320 |
| nprach-SubcarrierOffset | 0 | 0 |
| nprach-NumSubcarriers | 12 | 12 |
| numRepetitionsPerPreambleAttempt | 8 | 32 |

#### 8.2.3.2	Minimum Requirement

The minimum requirement is in TS 36.102 [2] subclause 8.2.3.1.1 and 8.2.3.2.1.

#### 8.2.3.3	Test purpose

The test shall verify the receiver’s ability to detect NPRACH preamble under multipath fading propagation conditions for a given SNR.

#### 8.2.3.4	Method of test

##### 8.2.3.4.1	Initial Conditions

Test environment:	Normal, see subclause D.2.

RF channels to be tested:	M; see subclause 4.7.

1)	Connect the SAN tester generating the wanted signal, multipath fading simulators and AWGN generators to all SAN antenna connectors for diversity reception via a combining network as shown in Annex I.3.2.

##### 8.2.3.4.2	Procedure

1)	Adjust the AWGN generator, according to the channel bandwidth.

Table 8.2.3.4.2-1: AWGN power level at the SAN input

| Channel bandwidth [KHz] | AWGN power level |
| --- | --- |
| 200 | -100.5dBm /180KHz |

2)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in Annex A.

3)	The multipath fading emulators shall be configured according to the corresponding channel model defined in Annex B.

4)	Adjust the frequency offset of the test signal according to Table 8.2.3.5-1.

5)	Adjust the equipment so that the SNR specified in Table 8.2.3.5-1 is achieved at the SAN input during the NPRACH preambles.

6) 	The test signal generator sends a preamble with repetitions and the receiver tries to detect the preamble. This pattern is repeated as illustrated in figure 8.2.3.4.2-1. The preambles are sent with a fixed timing offset of 0.5TCP during the test, where TCP is NPRACH cyclic prefix of length as defined in TS36.211 [12]. The following statistics are kept: the number of preambles detected in the idle period and the number of missed preambles.

Figure 8.2.3.4.2-1: NPRACH preamble test pattern

#### 8.2.3.5	Test Requirement

Pfa shall not exceed 0.1% and Pmd shall not exceed 1% for the SNRs in Table 8.2.3.5-1.

Table 8.2.3.5-1: NPRACH missed detection test requirements for FDD

| Number of TX antennas | Number of RX antennas | Repetition number | Propagation conditions and correlation matrix (Annex B) | Frequency offset [Hz] | SNR[dB] |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Preamble format 0 | Preamble format 1 |
| 1 | 1 | 8 | NTN-TDLA100-1 | 200 | [13.1] | [13.0] |
| 1 | 2 | 8 | NTN-TDLA100-1 | 200 | [5.3] | [5.4] |
| 1 | 1 | 16 | NTN-TDLA100-1 | 200 | [8.0] | [8.0] |
| 1 | 2 | 16 | NTN-TDLA100-1 | 200 | [1.4] | [1.4] |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in Annex G.

# <END OF THE CHANGE 1>

# <START OF THE CHANGE 2>

## 11.2	Performance requirements for Narrowband IoT

### 11.2.1	Performance requirements for NPUSCH format 1

#### 11.2.1.1	Definition and applicability

The performance requirement of NPUSCH format 1 is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in Annex A. The performance requirements assume HARQ re-transmissions.

The tests for 3.75KHz subcarrier spacing are applicable to the base stations supporting 3.75 kHz subcarrier spacing requirements. The tests for single-subcarrier/multi-subcarrier of 15KHz subcarrier spacing are applicable to the base stations supporting the number of subcarriers of 15 kHz subcarrier spacing requirements.

#### 11.2.1.2	Minimum Requirement

For SAN type 1-O, the minimum requirement is in TS 36.102 [2] subclause 11.1.1.

#### 11.2.1.3	Test Purpose

The test shall verify the receiver’s ability to achieve the throughput under multipath fading propagation conditions for a given SNR.

#### 11.2.1.4	Method of test

##### 11.2.1.4.1	Initial Conditions

Test environment:	Normal, see subclause D.2.

RF channels to be tested:	M; see subclause 4.7.

Direction to be tested: OTA REFSENS receiver target reference direction (see D.xx in table 4.6-1).

1)	Connect the SAN tester generating the wanted signal, multipath fading simulators and AWGN generators to all SAN antenna connectors for diversity reception via a combining network as shown in Annex I.3.2.

##### 11.2.1.4.2	Procedure

1)	Place the SAN with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex D.7.

2)	Align the manufacturer declared coordinate system orientation of the SAN with the test system.

3)	Set the SAN in the declared direction to be tested.

4)	Connect the SAN tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex D.7. Each of the demodulation branch signals should be transmitted on one polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in annex A, and according to additional test parameters listed in table 11.2.1.4.2-1.

Table 11.2.1.4.2-1: Test parameters for testing NPUSCH format 1

| Parameter | unit | Value |
| --- | --- | --- |
| Maximum number of HARQ transmissions |  | 4 |
| RV sequences |  | RV0, RV2 |
| Duration of NPUSCH format 1 segment transmission (Nsegmentprecompensation) | ms | 256 for 3.75kHz SCS, 16 for 15kHz SCS |
| Repetition |  | 4 for 3.75kHz SCS, 16 for 15kHz SCS |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex G.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the SAN receiver is as specified in clause 11.2.1.5 for SAN type 1-O, and that the SNR at the SAN receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 11.2.1.4.2-2.

Table 11.2.1.4.2-2: AWGN power level at the SAN input

| Channel bandwidth [KHz] | AWGN power level |
| --- | --- |
| 200 | -100.5dBm - ΔOTAREFSENS dBm /180KHz |
| NOTE 1: ΔOTAREFSENS as declared in D.43 in table 4.6-1 and clause 10.1 NOTE 2: The AWGN power level contains an AWGN offset of 16dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 16dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. |  |

8)	The signal generator sends a test pattern with the pattern outlined in Figure 11.2.1.4.2-1. The test signal generator sends frames, and the receiver tries to demodulate the content. The frames are sent with certain frequency and  gradual time offset per segment as described below in Table 11.2.1.4.2-2 and Table 11.2.1.4.2-3. The time offset is reset back to zero between segments.

Figure 11.2.1.4.2-1: Test signal pattern for NPUSCH format 1 demodulation tests

Table 11.2.1.4.2-2: Frequency Offsets for testing NPUSCH format 1

| Cases | Accumulated Frequency Offset |
| --- | --- |
| Table 11.2.1.5-1 | 128 Hz |
| Table 11.2.1.5-2 | 8 Hz |

Table 11.2.1.4.2-3: Timing Offsets for testing NPUSCH format 1

| Cases | Step size Δt |
| --- | --- |
| Table 11.2.1.5-1 | 0.32 us per RU |
| Table 11.2.1.5-2 | 0.01 us per RU |

8)	For each of the reference channels in Table 11.2.1.5-1 to 11.2.1.5-3 applicable for the SAN, measure the throughput, according to annex E.

#### 11.2.1.5	Test Requirement

The throughput measured according to subclause 11.2.1.4.2 shall not be below the limits for the SNR levels specified in Table 11.2.1.5-1 for 3.75KHz subcarrier spacing tests and not be below the limits for the SNR levels specified in Table 11.2.1.5-2 for 15KHz subcarrier spacing with the supported number of subcarrier tests.

Table 11.2.1.5-1 Required SNR for NPUSCH format 1 test, 200KHz Channel Bandwidth, 3.75KHz subcarrier spacing, 1Tx

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Repetition number | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 3.75 KHz | 1 | NTN-TDL100-1 | A16-1 | 4 | 70% | [-1.4] |
| 1 | 1 | 3.75 KHz | 1 | NTN-TDLC5-1 | A16-1 | 4 | 70% | [-2.3] |
| 1 | 2 | 3.75KHz | 1 | NTN-TDL100-1 | A16-1 | 4 | 70% | [-4.9] |
| 1 | 2 | 3.75KHz | 1 | NTN-TDLC5-1 | A16-1 | 4 | 70% | [-5.4] |

Table 11.2.1.5-2 Required SNR for NPUSCH format 1 test, 200KHz Channel Bandwidth, 15KHz subcarrier spacing, multiple subcarriers, 1Tx

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions and correlation matrix (Annex B) | FRC (Annex A) | Repetition number | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 15KHz | 12 | NTN-TDL100-1 | A16-2 | 16 | 70% | [-3.2] |
| 1 | 1 | 15KHz | 12 | NTN-TDLC5-1 | A16-2 | 16 | 70% | [-4.1] |
| 1 | 2 | 15KHz | 12 | NTN-TDL100-1 | A16-2 | 16 | 70% | [-7.4] |
| 1 | 2 | 15KHz | 12 | NTN-TDLC5-1 | A16-2 | 16 | 70% | [-8.0] |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in Annex G.

### 11.2.2	ACK missed detection for NPUSCH format 2

#### 11.2.2.1	Definition and applicability

The performance requirement of NPUSCH format 2 for ACK missed detection is determined by the two parameters: probability of false detection of the ACK and the probability of detection of ACK. The performance is measured by the required SNR at probability of detection equal to 0.99. The probability of false detection of the ACK shall be 0.01 or less.

The probability of false detection of the ACK is defined as a conditional probability of erroneous detection of the ACK when input is only noise.

The probability of detection of ACK is defined as conditional probability of detection of the ACK when the ACK was sent per NPUSCH format 2 transmission when the signal is present.

The tests for 3.75KHz subcarrier spacing are applicable to the base stations supporting 3.75 KHz subcarrier spacing requirements. The tests for 15KHz subcarrier spacing are applicable to the base stations supporting 15KHz subcarrier spacing requirements.

#### 11.2.2.2	Minimum Requirement

For SAN type 1-O, the minimum requirement is in TS 36.102 [2] subclause 11.2.2.1 and 11.2.2.2.

#### 11.2.2.3	Test purpose

The test shall verify the receiver’s ability to detect ACK under multipath fading propagation conditions for a given SNR.

#### 11.2.2.4	Method of test

##### 11.2.2.4.1	Initial Conditions

Test environment:	Normal, see subclause D.2.

RF channels to be tested:	M; see subclause 4.7

Direction to be tested:	OTA REFSENS receiver target reference direction (see D.44 in table 4.6-1).

##### 11.2.2.4.2	Procedure

1)	Place the SAN with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex D.7.

2)	Align the manufacturer declared coordinate system orientation of the SAN with the test system.

3)	Set the SAN in the declared direction to be tested.

4)	Connect the SAN tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex D.7. Each of the demodulation branch signals should be transmitted on one polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to TS 38.211 [8], and according to additional test parameters listed in table 11.2.1.4.2-1.

Table 11.2.1.4.2-1: Test parameters for testing NPUSCH format 2

| Parameter | unit | Value |
| --- | --- | --- |
| Duration of NPUSCH format 2 segment transmission (Nsegmentprecompensation) | ms | 128 for 3.75kHz SCS, 32 for 15kHz SCS |
| Repetition |  | 16 |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex G.2.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the SAN receiver is as specified in clause 11.2.1.5.1, and that the SNR at the SAN receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level quoted in table 11.2.1.4.2-2.

Table 11.2.2.4.2-2: AWGN power level at the SAN input

| Channel bandwidth [KHz] | AWGN power level |
| --- | --- |
| 200 | -100.5dBm /180KHz |
| NOTE 1: ΔOTAREFSENS as declared in D.43 in table 4.6-1 and clause 10.1 NOTE 2: The AWGN power level contains an AWGN offset of 16dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 16dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. |  |

8)	The signal generator sends a test pattern with the pattern outlined in figure 11.2.2.4.2-1. The following statistics are kept: the number of ACKs falsely detected in the idle periods and the number of missed ACKs. Each falsely detected ACK transmission in the idle periods is accounted as one error for the statistics of false ACK detection, and each missed ACK transmission per NPUSCH format 2 transmisson is accounted as one error for the statistics of missed ACK detection. The frames are sent with certain frequency and time offsets per segment as described below in Figure 11.2.2.4.2-3 and Table 11.2.2.4.2-4. The time offset is reset back to zero between segments.

Figure 11.2.2.4.2-1: Test signal pattern for NPUSCH format 2 demodulation tests

Table 11.2.2.4.2-3: Frequency Offsets for testing NPUSCH format 2

| Cases | Accumulated Frequency Offset |
| --- | --- |
| Table 11.2.2.5-1 | 64 Hz |
| Table 11.2.2.5-2 | 16 Hz |

Table 11.2.2.4.2-4: Timing Offsets for testing NPUSCH format 2

| Cases | Step size Δt |
| --- | --- |
| Table 11.2.2.5-1 | 0.08 us per RU |
| Table 11.2.2.5-2 | 0.02 us per RU |

#### 11.2.2.5	Test Requirement

The fraction of falsely detected ACKs shall be less than 1% and the fraction of correctly detected ACKs shall be larger than 99% for the SNR listed in Table 11.2.2.5-1 and Table 11.2.2.5-2.

Table 11.2.2.5-1 Required SNR for NPUSCH format 2 test, 200KHz Channel Bandwidth, 3.75KHz subcarrier spacing, 1Tx

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Number of allocated subcarriers | Subcarrier spacing | Repetition number | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | NTN-TDLA100-1 | 1 | 3.75KHz | 16 | [1.9] |
| 1 | 2 | NTN-TDLA100-1 | 1 | 3.75 KHz | 16 | [-4.7] |

Table 11.2.2.5-2 Required SNR for NPUSCH format 2 test, 200KHz Channel Bandwidth, 15KHz subcarrier spacing, 1Tx

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Number of allocated subcarriers | Subcarrier spacing | Repetition number | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | NTN-TDLA100-1 | 1 | 15KHz | 16 | [2.0] |
| 1 | 2 | NTN-TDLA100-1 | 1 | 15KHz | 16 | [-4.4] |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in Annex G.

### 11.2.3	Performance requirements for NPRACH

#### 11.2.3.1	Definition and applicability

The performance requirement of NPRACH for preamble detection is determined by two parameters: the total probability of false detection of the preamble (Pfa) and the probability of detection of the preamble (Pd). The performance is measured for the required SNR

-	at probability of Pd which shall not be smaller than 99% and

-	at probability of Pfa which shall not be larger than 0.1%.

Pfa is defined as a conditional total probability of erroneous detection of the preamble (i.e. erroneous detection from any detector) when input is only noise.

Pd is defined as conditional probability of detection of the preamble when the signal is present. The erroneous detection consists of several error cases – detecting different preamble than the one that was sent, not detecting a preamble at all or correct preamble detection but with the wrong timing estimation. A timing estimation error occurs if the estimation error of the timing of the strongest path is larger than 3.646 us. The strongest path for the timing estimation error refers to the strongest path in the power delay profile.

The performance requirements for TDD are optional and only valid for base stations supporting TDD.

The parameters of NPRACH test preambles are listed in Table 11.2.3.1-1.

Table 11.2.3.1-1 NPRACH Test Parameters

| Parameter | Value | Value |
| --- | --- | --- |
| Narrowband physical layer cell identity | 0 | 0 |
| nprach-Periodicity (ms) | 80 | 320 |
| nprach-SubcarrierOffset | 0 | 0 |
| nprach-NumSubcarriers | 12 | 12 |
| numRepetitionsPerPreambleAttempt | 8 | 32 |

#### 11.2.3.2	Minimum Requirement

For SAN type 1-O, the minimum requirement is in TS 36.102 [2] subclause 11.2.3.1.1 and 11.2.3.2.1.

#### 11.2.3.3	Test purpose

The test shall verify the receiver’s ability to detect NPRACH preamble under multipath fading propagation conditions for a given SNR.

#### 11.2.3.4	Method of test

##### 11.2.3.4.1	Initial Conditions

Test environment:	Normal, see subclause D.2.

RF channels to be tested:	M; see subclause 4.7.

Direction to be tested: OTA REFSENS receiver target reference direction (see D.44 in table 4.6-1).

##### 11.2.3.4.2	Procedure

1)	Place the SAN with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex D.7.

2)	Align the manufacturer declared coordinate system orientation of the SAN with the test system.

3)	Set the SAN in the declared direction to be tested.

4)	Connect the SAN tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex D.7. Each of the demodulation branch signals should be transmitted on one polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in annex A and the test parameter msg1-FrequencyStart is set to 0.

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex G.

7)	Adjust the AWGN generator, according to the SCS and channel bandwidth. The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 11.2.3.4.2-1.

Table 11.2.3.4.2-1: AWGN power level at the SAN input

| Channel bandwidth [KHz] | AWGN power level |
| --- | --- |
| 200 | -100.5dBm /180KHz |
| NOTE 1: ΔOTAREFSENS as declared in D.43 in table 4.6-1 and clause 10.1. NOTE 2: The AWGN power level contains an AWGN offset of 16dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 16dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. |  |

8) 	The test signal generator sends a preamble with repetitions and the receiver tries to detect the preamble. This pattern is repeated as illustrated in Figure 11.2.3.4.2-1. The preambles are sent with a fixed timing offset of 0.5TCP during the test, where TCP is NPRACH cyclic prefix of length as defined in TS36.211 [12]. The following statistics are kept: the number of preambles detected in the idle period and the number of missed preambles.

Figure 11.2.3.4.2-1: NPRACH preamble test pattern

#### 11.2.3.5	Test Requirement

Pfa shall not exceed 0.1% and Pmd shall not exceed 1% for the SNRs in Table 11.2.3.5-1.

Table 11.2.3.5-1: NPRACH missed detection test requirements for FDD

| Number of TX antennas | Number of RX antennas | Repetition number | Propagation conditions and correlation matrix (Annex B) | Frequency offset [Hz] | SNR[dB] |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Preamble format 0 | Preamble format 1 |
| 1 | 1 | 8 | NTN-TDLA100-1 | 200 | [13.1] | [13.0] |
| 1 | 2 | 8 | NTN-TDLA100-1 | 200 | [5.3] | [5.4] |
| 1 | 1 | 32 | NTN-TDLA100-1 | 200 | [8.0] | [8.0] |
| 1 | 2 | 32 | NTN-TDLA100-1 | 200 | [1.4] | [1.4] |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in Annex G.

# <END OF THE CHANGE 2>

# <START OF THE CHANGE 3>

# A.16	Fixed Reference Channels for NB-IoT NPUSCH format 1

Table A.16-1: FRC parameters for NB-IoT NPUSCH format 1

| Reference channel | A16-1 | A16-2 |
| --- | --- | --- |
| Subcarrier spacing (kHz) | 3.75 | 15 |
| Number of allocated subcarriers | 1 | 12 |
| Diversity | No | No |
| Modulation | BPSK | QPSK |
| ITBS / IRU | 0 / 1 | 9 / 0 |
| Payload size (bits) | 32 | 136 |
| Allocated resource unit | 2 | 1 |
| Code rate (target) | 1/3 | 2/3 |
| Code rate (effective) | 0.29 | 0.56 |
| Transport block CRC (bits) | 24 | 24 |
| Code block CRC size (bits) | 0 | 0 |
| Number of code blocks - C | 1 | 1 |
| Total number of bits per resource unit | 96 | 288 |
| Total symbols per resource unit | 96 | 144 |
| Channel estimation length (ms) Note 1 | 16 |  |
| Note 1: Channel estimation lengths are included in the table for information only. |  |  |

# <END OF THE CHANGE 3>

# <START OF THE CHANGE 4>

# G.3	Measurement of Performance Requirements

Table G.3-1: Derivation of Test Requirements (Performance tests)

| Test | Minimum Requirement in TS 36.102 | Test Tolerance (TT) | Test Requirement in TS 36.181 |
| --- | --- | --- | --- |
| 8.2.1 Performance requirements for NPUSCH format 1 | SINRs as specified | 0.6dB | Formula: SINR + TT T-put limit unchanged |
| 8.2.2 ACK missed detection for NPUSCH format 2 | SINRs as specified | 0.6dB | Formula: SNR + TT False ACK limit unchanged Correct ACK limit unchanged |
| 8.2.3 Performance requirements for NPRACH | SNRs as specified | 0.6dB | Formula: SNR + TT NPRACH False detection limit unchanged NPRACH detection limit unchanged |

Table G.3-2: Derivation of Test Requirements (FR1 OTA performance tests)

| Test | Minimum Requirement in TS 36.102 | Test Tolerance (TT) | Test Requirement in TS 36.181 |
| --- | --- | --- | --- |
| 11.2.1 Performance requirements for NPUSCH format 1 | SINRs as specified | 0.6dB | Formula: SINR + TTOTA T-put limit unchanged |
| 11.2.2 ACK missed detection for NPUSCH format 2 | SINRs as specified | 0.6dB | Formula: SNR + TTOTA False ACK limit unchanged Correct ACK limit unchanged |
| 11.2.3 Performance requirements for NPRACH | SNRs as specified | 0.6dB | Formula: SNR + TTOTA NPRACH False detection limit unchanged NPRACH detection limit unchanged |

# <END OF THE CHANGE 4>