# 3GPP TSG-RAN WG4 Meeting # 107	R4-2309832

Incheon, KR, May 22 – May 26, 2023

Title: 	WF on NCR_MT demodulation requirements

Agenda Item:	8.29.6

Source: 	ZTE Corporation

Document for:	Approval

- General

Issue 1-1-1: FR1 requirements

Agreement:

- 10MHz for 15kHz SCS and 40MHz for 30kHz SCS for NCR-MT demodulation requirements.

- FFS whether need to consider 5MHz/15kHz

Issue 1-1-2: FR2 requirements

Agreement:

- Option 1: New FR2 requirements shall be defined on PDCCH for signaling of the indication of changing the Access link beam.

- Option 2: Do not define new requirements on PDCCH for signaling of Access link beam change indication.

- PDSCH requirements

Issue 1-2-1: Test metric for FR1 and FR2

Agreement:

- Both 1% BLER(1st transmission) and 70% maximum throughput using the same simulation configurations with maximum number of HARQ transmission=4

Issue 1-2-2: MCS

Agreement:

- MCS 4 (QPSK) for FR1

- QPSK is the same as legacy UE for FR2

Issue 1-2-3: Channel model for FR2

Agreement:

- TDLA30-75

- PDCCH requirements

Issue 1-3-1:   PDCCH requirements

Agreement:

- Reuse IAB-MT and legacy UE requirements as much as possible as baseline.

- FFS for adapt PDCCH requirement for testing of DCI type 2_8 used for access link beam change indication.

  - Interesting companies can bring simulation results for DCI type 2_8.

Issue 1-3-2: CORESET for FDD

Agreement:

- 1TX: coreset duration 2

- 2TX: coreset duration 1

- CSI requirements

Issue 1-4-1: PMI requirements

Agreement:

- Option1: Define PMI requirements for MCR-MT.

    - Option 1A: Testing of performance requirements for PMI reporting is optional (Reuse from IAB-MT).

- Option 2: No