3GPP TSG-RAN WG4 Meeting # 107		R4-2309981

# Incheon, KR, May 22 – May 26, 2023

Agenda item:			9.7.8

Source:	Moderator (MediaTek inc.)

Title:	Topic summary for [107][236] LTE_NBIOT_eMTC_NTN_req

Document for:	Information

# Introduction

Briefly introduce background, the scope of this summary (e.g. list of treated agenda items).

This document is the topic summary for RRM requirements for NB-IoT/eMTC core & perf. requirements for NTN, including the following topics covered

- Topic#1: RRM core requirements (AI 9.7.5)

- Topic#2: RRM perf requirements (AI 9.7.6)

List of candidate target of discussions for this topic.

- 1st round: Proceed Topic 2 perf part first. Decide on the scope, priority, options and tentative agreement to be discussed in the 2nd round. Conclude issues with strict consensus, if any.

- 2nd round: Work on CR/WF.

# Topic #1: RRM core requirements (AI 9.7.5)

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2307894 | MediaTek inc. | Observation 1: The current specification already allows network parameters to differentiate the relaxation requirements. Proposal 1: the existing tables of relaxation factor N can be reused and no need to introduce new tables. |
| R4-2308311 | Huawei, HiSilicon | Observation 1: Legacy relaxation criteria will lead to very long delay for cell re-selection after t-service. Observation 2: In current requirements for neighbour cell measurement, it needs to be clarified that whether it requires UE to exit relaxed neighbour cell measurement Ttrigger before t-service. Observation 3: If UE is already performing neighbour cell measure normally before t-service, it does not make much sense for serving cell measurement relaxation for power saving purpose since UE is already wake up for measurement for each DRX. Proposal 1: In addition to the existing serving cell relaxation conditions, add following conditions for IoT NTN: - The time span to T-service is longer than k, where k = 4 DRX cycle if not configured with eDRX_IDLE cycle, and k = 1 eDRX cycle if configured with eDRX_IDLE cycle. Proposal 2: RAN4 not to define requirements for discontinuous coverage upon UE is in coverage, which is similar as Cell selection procedure. |
| R4-2308356 | Nokia, Nokia Shanghai Bell | Observation 1: The main reason to execute serving cell measurements is to trigger neighbor cell measurements. Observation 2: Measurement relaxation does not apply within T-trigger when t-service is broadcasted and applicable. Proposal 1: After Ttrigger UE is allowed to skip serving cell measurements and measurements of neighbor cells associated to the same serving satellite. Proposal 2: When the UE has reached measurement relaxation, and t-service is configured, the UE might perform cell selection procedure instead of cell reselection procedure, when the time span to T-service is smaller than k, where k = 2 DRX cycle if not configured with eDRX_IDLE cycle, and k = 1 eDRX cycle if configured with eDRX_IDLE cycle. Observation 3: The network is capable of controlling the interval between measurements by choosing the (e)-DRX cycle length and the parameter numDRX-CyclesRelaxed. Observation 4: Introducing a scaling factor for the relaxed cell measurements does not introduce any relaxation factor that can’t be already configured today. Moreover, it introduces additional complications by “unrelaxing” the measurements in certain scenarios. Proposal 3: Do not introduce a scaling factor for the relaxation factor for NGSO compared to GSO. The relaxation factor can already be modulated by the RRC parameter numDRX-cyclesRelaxed specified by RAN2. Proposal 4: RAN4 to agree that in discontinuous coverage scenarios, the UE shall resume measurements when the UE is in expected coverage as defined by the previously acquired SystemInformationBlockType32. Observation 5: Paging requirements only apply for UEs camped in a detected cell. Proposal 5: Do not create additional requirements after t-serviceStart regarding UE paging interruption. Proposal 6: Do not change DRX requirements based on t-service configuration. |
| R4-2309221 | Ericsson | - The values in tables used for deriving the serving cell measurement relaxation factors are reduced by factor 2 for NGSO for eMTC and NB-IoT in IDLE mode. - For serving cell: In addition to the existing serving cell relaxation conditions, add following conditions for IoT NTN (eMTC and NB-IoT): The time span to T-service is longer than k, where k = 4 DRX cycle if not configured with eDRX_IDLE cycle, and k = 1 eDRX cycle if configured with eDRX_IDLE cycle. - Whether to consider following option for relaxed neighbour cell measurement requirements depends on whether neighbour cell measurements are supported for GSO in first release of IoT NTN WI: - The eMTC and NB-IoT UE is allowed to meet the relaxed neighbour cell requirements provided that it has found more than [1] satellite including the serving satellite. - RAN4 to agree that in discontinuous coverage scenarios, the UE shall resume measurements when the UE is in expected in-coverage after UE acquired the SystemInformationBlockType32. - If the UE is configured with DRX cycle ≥ [1.28] sec, then the UE shall meet the serving cell evaluation requirements defined for DRX cycle of [640] ms starting from at least 2 configured DRX cycles before ‘t-Service-r17’. - If the UE is configured with ‘t-Service-r17’ [2] in the serving cell and eDRX_IDLE, then the UE shall meet the requirements defined for DRX cycle length of [2.56] s in Table 4.7A.2.1.1-1 starting from at least [2] DRX cycles before ‘t-Service-r17’. |

## Open issues summary

Before f2f meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### IDLE state mobility requirements - measurement relaxation

#### Issue 1-1: NGSO, serving cell measurement

Proposals

- Proposal 1: In addition to the existing serving cell relaxation conditions, add following conditions for IoT NTN (eMTC and NB-IoT): The time span to T-service is longer than k, where k = 4 DRX cycle if not configured with eDRX_IDLE cycle, and k = 1 eDRX cycle if configured with eDRX_IDLE cycle. (Huawei, Ericsson)

- Proposal 2: When the UE has reached measurement relaxation, and t-service is configured, the UE might perform cell selection procedure instead of cell reselection procedure, when the time span to T-service is smaller than k, where k = 2 DRX cycle if not configured with eDRX_IDLE cycle, and k = 1 eDRX cycle if configured with eDRX_IDLE cycle. (Nokia)

- Proposal 3: After Ttrigger UE is allowed to skip serving cell measurements and measurements of neighbor cells associated to the same serving satellite. (Nokia)

Recommended WF

  - Discuss proposals

#### Issue 1-2: NGSO, serving cell measurement - relaxation factor N

Proposals

- Option 1: Do not introduce a scaling factor for the relaxation factor for NGSO compared to GSO. The relaxation factor can already be modulated by the RRC parameter numDRX-cyclesRelaxed specified by RAN2. (Nokia, MTK)

- Option 2: The values in tables used for deriving the serving cell measurement relaxation factors are reduced by factor 2 for NGSO for eMTC and NB-IoT in IDLE mode. (Ericsson)

Recommended WF

  - Keep existing factor if no consensus.

#### Issue 1-3: GSO, neighbour cell measurement

Proposals

- Option 1: Whether to consider following option for relaxed neighbour cell measurement requirements depends on whether neighbour cell measurements are supported for GSO in first release of IoT NTN WI: The eMTC and NB-IoT UE is allowed to meet the relaxed neighbour cell requirements provided that it has found more than [1] satellite including the serving satellite. (Ericsson)

Recommended WF

  - Discuss proposals

### IDLE state mobility requirements – others

#### Issue 2-1: Discontinuous Coverage upon UE is in coverage

Background

Agreements (from RAN4#105)

When the UE is provided with t-serviceStart-r17 and has discontinuous coverage capabilities, then after t-service-r17 is reached and the UE is out of coverage, the UE may delay or resume cell measurements/search till when the UE is in coverage. Definition of in coverage is FFS

Excerpt from TS36.304 regarding SIB32

4	General description of Idle mode

If SystemInformationBlockType32 has been received and if the UE has determined that it is out of coverage using available satellite assistance information (e.g. ephemeris parameters and coverage parameters in current or previously received SystemInformationBlockType32, SystemInformationBlockType31, t-Service in SystemInformationBlockType3 or other parameters), the AS configuration (e.g. priorities provided by dedicated signalling and logged measurements) is kept, but the UE need not perform any idle mode tasks. It is up to UE implementation to handle running timers. The detection of out of coverage using satellite assistance information is up to UE implementation and once in coverage the UE shall perform all idle mode tasks.

Proposals

- Proposal 1: RAN4 to agree that in discontinuous coverage scenarios, the UE shall resume measurements when the UE is in expected coverage as defined by the previously acquired SystemInformationBlockType32. (Nokia)

- Proposal 1a: RAN4 to agree that in discontinuous coverage scenarios, the UE shall resume measurements when the UE is in expected in-coverage after UE acquired the SystemInformationBlockType32. (Ericsson)

- Proposal 2: RAN4 not to define requirements for discontinuous coverage upon UE is in coverage, which is similar as Cell selection procedure. (Huawei)

Recommended WF

  - Discuss proposals

#### Issue 2-3: Discontinuous Coverage - revision on paging requirement

Proposals

- Option 1: Do not create additional requirements after t-serviceStart regarding UE paging interruption. (Nokia)

Recommended WF: Option 1.

#### Issue 2-3: NGSO, serving cell measurement upon t-Service-r17

Proposals

- Proposal 1: Do not change DRX requirements based on t-service configuration. (Nokia)

- Proposal 2: (Ericsson)

  - If the UE is configured with DRX cycle ≥ [1.28] sec, then the UE shall meet the serving cell evaluation requirements defined for DRX cycle of [640] ms starting from at least 2 configured DRX cycles before ‘t-Service-r17’.

  - If the UE is configured with ‘t-Service-r17’ [2] in the serving cell and eDRX_IDLE, then the UE shall meet the requirements defined for DRX cycle length of [2.56] s in Table 4.7A.2.1.1-1 starting from at least [2] DRX cycles before ‘t-Service-r17’.

Recommended WF:

- Is the 2nd sub-bullet (eDRX part) in Proposal 2 agreeable?

### CR

| R4-2308358 | CR on timing advance requirements for NBIOT over NTN | Nokia, Nokia Shanghai Bell |
| --- | --- | --- |
| R4-2307619 | Correction on reference of NB-IoT for satellite access | China Telecom Corporation Ltd. |
| R4-2307895 | CR on RRM core requirements maintenance for LTE NB-IoT/eMTC over NTNRe-establishment | MediaTek inc. |
| R4-2308312 | CR on maintenance of NB-IoT for IoT NTN | Huawei, HiSilicon |
| R4-2309225 | Correction to IDLE mode cat-M IoT NTN requirements | Ericsson |

# Topic #2: RRM perf requirements (AI 9.7.6)

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2307601 | CMCC | Proposal 1: Define UE transmit timing accuracy test for UL segmented transmission in HD-FDD NB enhanced coverage test, only NGSO test configuration applies for this test. The test setup and test requirements in A.13.4.1.2.1 (HD-FDD NB1 standalone enhanced coverage test) Test 1 (DRX off) can be the starting point. Proposal 2: In test case, set Nsegmentprecompensation (npusch-TxDuration-r17) as 64ms. Proposal 3: Define the test case without UE supporting the capability of ntn-SegmentedPrecompensationGaps-r17. Proposal 4: For the test requirement of UL segmented transmission, following part with underline should be updated for new test - The test system shall verify that the UE shall not adjust the uplink transmission timing autonomously during an ongoing repetition period other than at initial transmission or at the start of a transmission segment boundary. - a) After a connection is set up with the cell, the test system sends NPDCCH including uplink grant for NPUSCH transmission and the test system shall measure the UE transmit timing offset (nTs) of the first transmission in each segment and verify that it is within Te (nTs ≤NTA_Ref+NTAoffset+NTA,common+NTA, UE-specific×TS ± 97×TS + Tmargin) with respect to the first detected path (in time) of the corresponding downlink frame of NB-IoT cell 1. |
| R4-2307896 | MediaTek inc. | Proposal 1: For Cat-M1, introduce test cases of measurement procedure and measurement performance for intra-frequency in GEO, including the following test cases: Measurement Procedure; Event triggered reporting; E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA; M1 6-1. E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX; M1 6-2. E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA; M1 6-3. E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX; M1 6-4. Measurement Performance Requirements; RSRP accuracy; FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA; M1 7-1. HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA; M1 7-2. |
| R4-2308313 | Huawei, HiSilicon | Observation 1: NW will mostly schedule maximum repetition level when PHR is lower than -23 dB. Observation 2: Though UE can report different PHR for negative values, it cannot help NW scheduling since NW can only configure largest repetition level as TN. Proposal 1: Reuse legacy PHR reporting values for NB-IoT GEO. Proposal 2: Define intra-frequency RRC Re-establishment TC for both NC and EC. |
| R4-2308355 | Nokia, Nokia Shanghai Bell | Proposal 1: Use the same PHR table for GSO and NGSO operations (Option 1). Proposal 2: The common delay parameters provided in SIB31 for the RRM test cases can be obtained assuming a reference point on Earth whose elevation angle to the satellite position given by the ephemeris does not fall below 30 degrees during the test. Proposal 3: RAN4 to introduce reference channels for Cat-M1 UEs in Annex A.3.1.3, for both MPDDCH and MPDSCH. |
| R4-2309222 | Ericsson | Observation 1: GEO satellites operate around 36 000 km above the earth while LEO satellites operates around 1 200 km above the earth. - For UE in normal coverage managed by GEO with only 4 reportable values, the PHR reporting values are defined as shown in Table 3. Table 3: Power headroom report mapping for UE category NB1 UEs for a cell (e.g. cell1) served by GEO satellite during random access procedure Reported value; Measured quantity value (dB). POWER_HEADROOM_0; -54 PH -10. POWER_HEADROOM_1; -10 PH -2. POWER_HEADROOM_2; -2 PH 6. POWER_HEADROOM_3; PH ≥ 6. - For UE in enhanced coverage managed by GEO with only 4 reportable values, the PHR reporting values are defined as shown in Table 4. Table 4 Power headroom report mapping for UE category NB1 UEs not supporting enhanced PHR when the enhanced coverage level other than 0 is selected during random access procedure and UE is served by GEO satellite Reported value; Measured quantity value (dB). POWER_HEADROOM_0; -54 PH -30. POWER_HEADROOM_1; -30 PH -20. POWER_HEADROOM_2; -20 PH 6. POWER_HEADROOM_3; PH ≥ 6. - Define UE initial transmission timing test for UL segmented transmission. |

## Open issues summary

Before f2f meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

#### Issue 1: PHR reporting for NB-IoT in GEO

Proposals

- Proposal 1: For NB-IoT, Reuse legacy PHR reporting values for NB-IoT GEO. (Huawei, Nokia)

- Proposal 2: For NB-IoT GEO, modify the values in exiting tables to include more negative PHR reporting values. (Ericsson)

- For UE in normal coverage managed by GEO with only 4 reportable values, the PHR reporting values are defined as shown in Table 3.

Table 3: Power headroom report mapping for UE category NB1 UEs for a cell (e.g. cell1) served by GEO satellite during random access procedure

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | -54 PH -10 |
| POWER_HEADROOM_1 | -10 PH -2 |
| POWER_HEADROOM_2 | -2 PH 6 |
| POWER_HEADROOM_3 | PH ≥ 6 |

- For UE in enhanced coverage managed by GEO with only 4 reportable values, the PHR reporting values are defined as shown in Table 4.

Table 4 Power headroom report mapping for UE category NB1 UEs not supporting enhanced PHR when the enhanced coverage level other than 0 is selected during random access procedure and UE is served by GEO satellite

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | -54 PH -30 |
| POWER_HEADROOM_1 | -30 PH -20 |
| POWER_HEADROOM_2 | -20 PH 6 |
| POWER_HEADROOM_3 | PH ≥ 6 |

Recommended WF

- Discuss proposals.

#### Issue 2-1: RMC for Cat-M1 with 1.4 MHz

Proposals

- Proposal 1: RAN4 to introduce reference channels for Cat-M1 UEs in Annex A.3.1.3, for both MPDDCH and MPDSCH. (Nokia)

Recommended WF

- Agree on Proposal 1.

#### Issue 2-2: Common delay in SIB31

Proposals

- Proposal 1: The common delay parameters provided in SIB31 for the RRM test cases can be obtained assuming a reference point on Earth whose elevation angle to the satellite position given by the ephemeris does not fall below 30 degrees during the test (Nokia)

Recommended WF

- Discuss proposals.

#### Issue 3: Test for Segmented UL Transmissions

Proposals

- Proposal 1: Define UE initial transmit timing accuracy test for UL segmented transmission. (Ericsson)

- Proposal 1a: Define UE transmit timing accuracy test for UL segmented transmission in HD-FDD NB enhanced coverage test, only NGSO test configuration applies for this test. The test setup and test requirements in A.13.4.1.2.1 (HD-FDD NB1 standalone enhanced coverage test) Test 1 (DRX off) can be the starting point. (CMCC)

  - set Nsegmentprecompensation  (npusch-TxDuration-r17) as 64ms.

  - Define the test case without UE supporting the capability of ntn-SegmentedPrecompensationGaps-r17.

  - For the test requirement of UL segmented transmission, following part with underline should be updated for new test

    - The test system shall verify that the UE shall not adjust the uplink transmission timing autonomously during an ongoing repetition period other than at initial transmission or at the start of a transmission segment boundary.

    - a) After a connection is set up with the cell, the test system sends NPDCCH including uplink grant for NPUSCH transmission and the test system shall measure the UE transmit timing offset (nTs) of the first transmission in each segment and verify that it is within Te (nTs ≤NTA_Ref+NTAoffset+NTA,common+NTA, UE-specific×TS   ± 97×TS + Tmargin) with respect to the first detected path (in time) of the corresponding downlink frame of NB-IoT cell 1.

Recommended WF

- Are the test configurations/procedure/requirement in Proposal 1a agreeable?

#### Issue 4: Test case list

Proposals

- Proposal 1: Define intra-frequency RRC Re-establishment TC for both NC and EC (Huawei).

- Proposal 2: For Cat-M1, introduce test cases of measurement procedure and measurement performance for intra-frequency in GEO, including the following test cases: (MTK)

| Measurement Procedure | Event triggered reporting | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA | M1 6-1 |
| --- | --- | --- | --- |
|  |  | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX | M1 6-2 |
|  |  | E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA | M1 6-3 |
|  |  | E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX | M1 6-4 |
| Measurement Performance Requirements | RSRP accuracy | FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA | M1 7-1 |
|  |  | HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA | M1 7-2 |

Recommended WF

- Are both Proposal 1 and Proposal 2 agreeable?

### CR

| R4-2308357 | DraftCR on Cell Reselection Test cases for M1 UEs in satellite access | Nokia, Nokia Shanghai Bell |
| --- | --- | --- |
| R4-2308359 | CR on reference configurations for Cat-M1 in Satellite Access | Nokia, Nokia Shanghai Bell |
| R4-2307602 | Draft CR on E-UTRAN RLM in DRX test and HO and CHO test for Cat-M1 UE in CEMode A | CMCC |
| R4-2307897 | Draft Big CR on NB-IoT/eMTC RRM performance requirements for NTN | MediaTek inc. |
| R4-2307898 | Introduction of test cases of Cell Re-Selection for NB-IoT | MediaTek inc. |
| R4-2307899 | Introduction of test cases of Intra-frequency Measurements and RSRP for M1 | MediaTek inc. |
| R4-2308314 | DraftCR on RRC reestablishment test cases of NB-IoT for GEO | Huawei, HiSilicon |
| R4-2309223 | RRC re-establishment test cases for eMTC over NTN | Ericsson |
| R4-2309224 | RMC for eMTC test cases over NTN | Ericsson |