3GPP TSG-RAN WG4 Meeting #107								               	              R4-2310064

Incheon, Korea, May 22 – 26, 2023

Agenda item:			8.25.6

Source:	Apple

Title:	WF on NR Mobility Enhancements RRM requirements (part 2)

Document for:	Approval

# Topic #1: NR-DC with selective activation of cell groups via L3 enhancements

Issue 1-1: TEvent_DU in subsequent CPC delay requirements

- Agreements:

  - Keep Tevent_DU in subsequent CPC delay requirements. Both TEvent_DU  0 and TEvent_DU = 0 can be covered. No need to explicitly define the cases where Tevent_DU=0.

Issue 1-2: whether to UE shall evaluate the execution condition of other candidate PSCells while executing CPC

- Agreements:

  - No. According to TS37.340 and RAN2 agreements, UE is not evaluating the execution condition of other candidate PSCells while executing CPC, and the evaluation is continued after finishing the PSCell addition or change. (CMCC, MTK, Apple, ZTE, Huawei, OPPO, QC)

Issue 1-3: CR splitting on core requirements:

- Agreement:

| Title | Section | Key aspects (based on agreements in RAN4#106-bis-e) | Responsible company |
| --- | --- | --- | --- |
| Subsequent conditional PSCell change | (new) 8.11E | - Starting point of subsequent CPC in RRM requirements is the time when UE transmits SN RRCReconfigurationcomplete message for the previous PSCell addition or change. - Ending point of subsequent CPC in RRM requirements is the transmission of PRACH preamble towards the target PSCell. - For subsequent CPC delay requirements: if starting point is the time when UE completes the previous CPC/CPA, Tconfig_PSCell_Subsequent_Change_Conditional = TEvent_DU + Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms and the definition of each component is the same as 8.11B.2 in TS38.133 except that TEvent_DU needs to be updated. - TEvent_DU is the delay uncertainty which is the time from [when UE transmits SN RRCReconfigurationcomplete message for the previous PSCell addition or change] until a condition exists at the measurement reference point which will trigger the subsequent conditional PSCell change. | Apple |

# Topic #2: Improvement on SCell/SCG setup delay

### Sub-topic 2-1 scope and overall solution

Issue 2-1-1: combination of solution 1 and 2

- Candidate solutions:

  - Option 1: (Nokia)

    - 1. UE having CA/DC measurements (Rel-16 EMR or other) available from IDLE/INACTIVE mode at RRC setup/resume.

    - 2. UE performing evaluation of the validity of available measurements starting from RRC setup/resume.

    - 3. UE reporting measurement status of the available measurements at RRC setup/resume complete.

    - 4. If needed, UE performing additional validation measurements during RRC CONNECTED mode.

    - 5. UE reporting measurement results as soon as validation is completed.

Issue 2-1-2: relationship between R16 EMR and R18 enahcement to SCell/SCG setup delay

- Candidate solutions:

  - Option 1: Rel-16 EMR and R18 enahcement to SCell/SCG setup delay are independent features. (CMCC, ZTE, Nokia)

  - Option 1a: in R18 enhancement to SCell/SCG setup delay, solutions based on existing cell reselection measurement and enhanced measurement are independent of UE support of R16 EMR. (Apple)

  - Option 2: For UE to utilize the cell reselection measurement as enhancement for Rel-18, to support Rel-16 Idle/Inactie measurement for CA/DC is a pre-requisite. (E///)

Issue 2-1-3: whether to consider measurement originated from CONNECTED mode before UE enters IDLE/INACTIVE mode

clarification on ‘Early measurement’

- Candidate solutions:

  - FFS: no. Legacy definition can be used as “Early measurements are performed during IDLE/INACTIVE mode”

### Sub-topic 2-2 solutions based on existing measurement

Issue 2-2-1: overall solution for UE which is configured with EMR measurement.

- Candidate solutions:

  - Option 1: introduce indication of availability and/or validity status of measurement results from UE to NW into the [existing] EMR reports. (CATT, vivo)

  - Option 2: introduce validity indication for solution based on existing measurement. (Apple, ZTE, Xiaomi)

  - Option 3: UE does not report the invalid measurement results to NW. All the results reported to NW should be considered valid without additional indication. (MTK, HW, OPPO, QC)

  - Option 4: Introduce a new indication for validity. But the final decision on validity should be made by the NW, and the UE can provide additional information to help NW decision through the new indication. Additional information could be (LG)

    - information about when the measurement result was obtained

    - Information about the serving cell to which the UE belongs when the measurement results were obtained.

  - Option 5: RAN4 shall agree on a validity solution based on VarMeasIdleReport by providing time stamps for the measurement results when 1 frequency carrier finishes the measurement and stored this time value within this UE variable. Details on how to introduce time stamps shall be upon to the RAN2 discussion. (E///)

  - Option 6: When returning from IDLE or INACTIVE mode, the UE indicates at RRC connection setup/resume complete the measurement status of IDLE/INACTIVE mode measurements out of the following options: 1. No measurements available, 2. Valid measurements available, 3. Measurements ongoing. (Nokia)

    - 1. No measurements available: The UE shall indicate “no measurements available” at RRC setup/resume complete and not report measurement results in the following scenarios:

      - UE does not have any measurement results available from IDLE/INACTIVE mode, or

      - The available measurements are invalid and the UE is not going to continue enhanced measurements in CONNECTED mode

    - 2. Valid measurements available (based on existing measurement):

      - When the UE has valid measurements available at RRC setup/resume complete, it reports the results immediately based on RAN2 agreed mechanism.

      - When returning from RRC INACTIVE mode, if the UE has valid measurements available at RRC resume complete, the UE may also directly report the results at this point, similarly to a Rel-16 EMR capable UE.

    - 3. Measurements ongoing

      - UE continues measurement (solution based on enhanced measurements)

      - When the UE continues enhanced measurements in CONNECTED mode, enhance the reporting mechanism so that the UE can report as soon as valid measurement results are available.

Issue 2-2-2: overall solution for UE which is not capable of EMR measurement.

- Candidate solutions:

  - Option 1: (CMCC)

    - the measurement results can be reported in following two ways:

      - in RRCResumeComplete (or RRCSetupComplete) message

      - after UE in connected mode (i.e. in UEInformationResponse message)

  - Option 1a: For UE which is not capable of or not configured with EMR measurement, UE can report the measurement results obtained in idle/inactive mode and follows the same procedure of EMR. The final decision of the reporting mechanism is up to RAN2. (MTK)

  - Option 2: RAN4 shall clarify the Rel-18 capability for using existing measurement by how cell reselection measurement shall be validated and reported. (E///)

Issue 2-2-3: definition of ‘valid’ in solution based on existing measurement

- Agreements in RAN4#106-bis-e:

  - Candidate criteria for measurements validity definition

    - A) the measurement are performed within the last [X] seconds before it is reported

    - B) the reported measurement results satisfy measurement accuracy

    - C) variation of serving cell RSRP/RSRQ does not exceed [Y] dB

  - FFS whether a single or several criteria should be used for measurements validity definition.

- Candidate solutions:

  - Option A) the measurement are performed within the last [X] seconds before it is reported

    - Support: MTK, Apple, vivo, LG (A+B), ZTE (A+B or A+C), Xiaomi (A+C), HW, E/// (A+B)

    - Object: QC, Nokia

    - Others:

      - Further discuss whether to consider multiple [X] per UE mobility speed (CATT)

  - Option B) the reported measurement results satisfy measurement accuracy

    - Support: CATT, LG (A+B), ZTE (A+B), E/// (A+B), QC, Nokia

    - Object: Apple

    - Others:

  - Option C) variation of serving cell RSRP/RSRQ does not exceed [Y] dB

    - Support: vivo, LG, ZTE (A+C), Xiaomi (A+C)

    - Object: Apple, QC

    - Others:

      - How does the UE evaluate the variation should be further discussed (CATT)

      - FFS (Nokia)

Issue 2-2-4: additional criteria to determine ‘invalid’

- Candidate solutions:

  - Option 1: If the cell reselection occurred after the measurement for CA/DC setup was stopped due to expiration of T331 OR if the cell which the UE initiates the RRCSetupRequest is inconsistent with the cell when the UE stops EMR measurement, the EMR measurement result in idle/inactive mode will be invalid. (vivo)

  - Option 2: FFS

Issue 2-2-5: indication of availability for UE configured with EMR

- Candidate solutions:

  - Option 1: it is necessary to introduce availability. (vivo, CATT?)

  - Option 2: reuse existing idleMeasAvailable-r16 for UE configured with EMR. (Apple)

  - Option 3: unnecessary (QC)

Issue 2-2-6: indication of validity for UE configured with EMR

- Candidate solutions:

  - Option 1: it is necessary to introduce validity. (vivo, CATT?)

  - Option 1a: the validity information can be added in the existing EMR reporting structure. (Apple, ZTE, Xiaomi, E///)

  - Option 2: unnecessary. (MTK, HW, OPPO, QC)

Issue 2-2-7: indication of availability for UE NOT capable/configured with EMR

- Candidate solutions:

  - Option 1: Regarding availability, whether a similar indication as ‘idleMeasAvailable-r16’ is necessary can be up to RAN2. (Apple)

  - Option 2: introduce availability information. (vivo)

  - Option 3: unnecessary (QC)

Issue 2-2-8: indication of validity for UE NOT capable/configured with EMR

- Candidate solutions:

  - Option 1: how/when to report the result and the validity information can be up to RAN2. (Apple)

  - Option 2: unnecessary. (MTK, HW, OPPO, QC)

  - Option 3: UE reports the indication of valid cell reselection measurement results in RRCResumeComplete message or UEInformationResponse message. (Xiaomi)

Issue 2-2-9: whether to report invalid results

- Agreements:

  - In new reporting in R18, UE does not report the invalid results to NW.

Issue 2-2-10: new timer for validity

- Candidate solutions:

  - Proposal 1: RAN4 shall agree on introduce a configurable validity timer Tvalidity. (E///)

    - The validity timer can be described as within x seconds until validity indication time (RRC setup/Resume complete) .

    - The value of this x seconds shall be indicated toward RAN2 as a value range for example from 5 to 60 seconds.

Issue 2-2-11: accuracy requirements

- Candidate solutions:

  - Proposal 1: for solutions based on existing measurement, if R18 enhancement to SCell/SCG setup delay are based on EMR, it is straightforward to reuse EMR accuracy requirements. (CMCC)

  - Proposal 2: for solutions based on existing measurement, if R18 enhancement to SCell/SCG setup delay are based on cell re-selection, the EMR accuracy requirements can be reused. (CMCC)

- Recommended WF

  - Discuss in performance part.

### Sub-topic 2-3 solutions based on enhanced measurement

Issue 2-3-1: overall solution

- Candidate solutions:

  - Option 1: (QC)

    - NW provide explicit measurement information for enhanced measurement.

    - NW trigger enhanced measurement based on NW demands.

    - UE start performing additional measurement from msg1 transmission

    - If UE has measurement results to report, UE can report in msg5. NW can initiate SCell/SCG setup.

    - If UE does not report in msg5, NW expect UE continue performing measurement for remaining measurement period. NW configure the MO at RRC CONNECTED for the same target frequency that UE is performing measurement during IDLE/INACTIVE state.

    - NW can initiate SCell/SCG setup after UE report enhanced measurement results.

  - Option 2: (Nokia)

    - If needed, UE performing additional validation measurements during RRC CONNECTED mode.

    - UE reporting measurement results as soon as validation is completed.

- Recommended WF

  - Continue discussion each step in corresponding issue.

Issue 2-3-2: whether it is necessary to indicate measurement status when entering connected mode.

- Candidate solutions:

  - Option 1: no. For safety, UE would indicate the worst case. In this way, UE can always meet the requirement but is still allowed to report the measurement results earlier if UE completes the measurement earlier. (MTK)

  - Option 2: If agreed to introduce solution based on additional measurement, the UE can indicate validation status to inform the network for validation of measurements in idle/inactive mode and/or connected mode. (OPPO)

Issue 2-3-3: feasibility of enhanced measurement which starts from RRC setup/resume procedure.

- Candidate solutions:

  - Option 1: It is questionable on the feasibility of introducing the enhanced measurement starting from RRC connection setup/resume procedure. (Xiaomi, HW, MTK)

  - Option 2: There is no feasibility issue to continue performing enhanced measurement after RRC CONNECTED state from configuring proper MO such as (QC)

    - If NW provide explicit measurement information for enhanced measurement, NW configure MO at RRC CONNECTED state for the same target frequency that UE is performing from RRC IDLE/INACTIVE.

Issue 2-3-4: definition of ‘valid’ for solutions based on enhanced measurement

- Candidate solutions:

  - FFS: Measurement result is valid when UE meet the measurement delay and accuracy requirements. No need to define additional criteria. (QC)

Issue 2-3-5: starting point of the enhanced measurement

- Candidate solutions:

  - Option 1: (Apple, vivo, ZTE)

    - For MT originating call, UE starts to perform additional measurement after paging reception.

    - for MO call, UE starts to perform additional measurement after first RACH preamble transmission, i.e. Msg1.

  - Option 2: RAN4 can consider defining unified delay requirement for both MT and MO call, starting from after Msg1 transmission. (QC)

  - Option 3: Validation of available IDLE/INACTIVE mode measurements may start at RRC connection setup/resume. (Nokia)

Issue 2-3-6: ending point of the enhanced measurement

- Candidate solutions:

  - Option 1: When UE sends RRCResumeComplete or SecurityModeComplete (CMCC, vivo, Apple)

  - Option 2: no later than MO configuration (Apple, ZTE, E///)

  - Option 3: RAN4 shall define measurement delay requirement instead of defining measurement ending point. (QC)

  - Option 3a: No need to define an explicit ending point for validation measurements. Enhanced measurements end when the UE has finished the measurements and reported the results. This may be before or after the CONNECTED mode measurement configuration. (Nokia)

Issue 2-3-7: UE measurement behavior after receiving MO configuration in connected mode

- Candidate solutions:

  - Option 1: requirements of existing measurement in connected mode shall not be impacted by enhanced measurement. UE is not expected to support parallel measurement of enhanced measurement and legacy connected mode measurement. (Apple)

  - Option 2: NW expect the UE continues performing measurement even after RRC CONNECTED, if NW provide explicit measurement information for enhanced measurement, NW configure MO at RRC CONNECTED state for the same target frequency that UE is performing from RRC IDLE/INACTIVE. (QC)

- Tentative agreement in Ad-hoc:

  - UE is [expected or allowed] to continue the measurements in CONNECTED mode if all newly configured MOs align with the frequency layers to be measured in the IDLE/INACTIVE mode

    - FFS the UE measurement requirements, e.g., the delay or starting/ending point

    - FFS if the configurations in IDLE/INACTIVE and CONNECTED mode are not 100% the same

- Recommended WF

  - Continue discussion on the tentative agreement in next meeting.

Issue 2-3-8: Reporting of the measurement results

- Candidate solutions:

  - Option 1: the measurement results can be reported in following two ways (i.e. same as EMR report): (CMCC)

    - in RRCResumeComplete (or RRCSetupComplete) message

    - after UE in connected mode (i.e. in UEInformationResponse message)

  - option 2: UE can report measurement results in msg5 during INACTIVE state or UE can indicate measurement report available in msg5 and report after RRC security complete if measurement result is available.  (QC)

Issue 2-3-9: network assistant information and measurement configuration

- Candidate solutions:

  - Option 1: NW shall provide explicit information for enhanced measurement. The measurement information can include target frequency, Cell ID, target SSB info. (QC)

  - Option 1a: For the UE to know how to measure during the validation period, NW can provide explicit information such as target frequency and/or Cell ID, and/or target SSB info for measurements in RRC configuration before IDLE mode or SIB. (Nokia)

  - Option 2: UE may receive the measurement configuration during previous connected mode, or alternatively UE may read SIB information. Signaling details are up to RAN2. (Nokia)

  - Option 3: UE can use previously stored and/or used FR2 SCell configuration. (Nokia)

Issue 2-3-10: number of samples, including whether Rx beam sweeping is needed

- Candidate solutions:

  - Option 1: To guarantee the measurement accuracy, the measurement samples are not supposed to be reduced. Not to reduce the scaling factor of Rx beam sweeping during the RRC connection setup/resume procedure. (Apple, HW)

  - Option 1a: As baseline, RAN4 shall not reduce the scaling factor of Rx beam sweeping when defining requirements for the new measurement during RRC connection setup/resume. (Xiaomi, QC)

  - Option 2: For the case that the carrier for mobility is in the same band with the carriers for improved measurement, the beam information from cell reselection measurement can be utilized to reduce the Rx beam sweeping factor in improved measurement. For other cases, to introduce UE capability for lower Rx beam sweeping factor like Rel-17 positioning. (vivo)

  - Option 3: During the additional measurement, for the further validity check, reduced samples and (or) reduced beam sweeping factors can be considered for the results obtained within the last [Y] seconds. Reduced samples or reduced beam sweeping factors may not be considered for results obtained more than [Y] seconds ago. Also, for the results obtained within the last [X] (Y>X) seconds, it can be treated as valid results without additional measurements. (LG)

  - Option 4: During re-evaluation/validation measurements, UE is not expected to perform full beam-sweeping and hence, the scaling factor associated with the beam sweeping can be reduced. (Nokia)

Issue 2-3-11: number of carriers

- Agreements in RAN4#106-bis-e:

  - UE is not expected to perform enhanced measurement on FR2 more than one carrier per band. FFS: on the selection of carriers if multiple carriers are configured per band in FR2.

- Candidate solutions:

  - FFS: Carrier(s) per band can be either be selected by UE or controlled by the network, or both. (Nokia)

Issue 2-3-12: measurement period

- Candidate solutions:

  - Option 1: RAN4 consider SSB period for the enhanced measurement. The measurement period requirements can be defined as (8 + [X]) TSSB ms. (QC)

  - Option 2: Enhanced measurement period can be based on SSB period instead of SMTC. (Nokia)

Issue 2-3-13: applicability rules to perform enhanced measurement

- Candidate solutions:

  - Option 1: (QC)

    - The enhanced measurement is applied when target cell SNR > [Y] dB (e.g Y = 6 dB).

    - UE is allowed to stop enhanced measurement upon RA procedure problem (e.g msg2 reception failure).

  - Option 2: FFS

Issue 2-3-14: whether to define measurement latency requirements

- Candidate solutions:

  - Option 1: Yes (CMCC)

  - Option 2: It is hard to define the measurement requirements if the overall measurement starts from IDLE/INACTIVE mode and ends in Connected mode. (MTK)

Issue 2-3-15: accuracy measurement

- Candidate solutions:

  - Option 1: it is preferred to specify accuracy requirements to guarantee the measurement performance. Rel-16 EMR accuracy requirements can be used as baseline. (CMCC)

  - Option 2: FFS

- Recommended WF

  - Discuss in performance part.

# Topic #3: Enhanced CHO configurations

### Sub-topic 3-1 CHO including target MCG and target SCG in NR-DC (obj. 3)

Issue 3-1-1: PCell handover delay in CHO including target MCG and target SCG in FR1+FR2 NR-DC (obj. 3)

- Agreements in RAN4#106-bis-e:

  - Pcell handover delay in CHO including target MCG and target SCG in FR1+FR2 NR-DC (obj. 3) is defined as:

    - DCHOwithPSCell_Pcell = TRRC + TEvent_DU + Tmeasure + Tprocessing + TIU + T∆ + Tmargin + TCHO_execution.

  - Definition of each component, except Tprocessing, is same as that defined in CHO requirement in TS38.133 clause 6.4.1.2.

  - Definition of Tprocessing:

    - Option 1: same as that defined in requirements of handover with PSCell. (CATT)

    - Option 2: same as that defined in requirements of CHO.

- Agreements in RAN4#107:

  - Tprocessing is same as that defined in requirements of handover with PSCell.

Issue 3-1-2: PSCell handover delay in CHO including target MCG and target SCG in FR1+FR2 NR-DC (obj. 3)

- Agreements:

  - DCHOwithPSCell_PSCell = TRRC + TEvent_DU + Tmeasure + TCHO_execution + Tprocessing + Tsearch_PCell_Conditional + Tsearch_PSCell + T∆_PSCell + TPSCell_ DU + 2 ms, where

    - The definitions of TRRC, TEvent_DU, Tmeasure, TCHO_execution, Tprocessing are the same as the definitions in the delay requirements for Pcell

    - Tsearch_PCell_Conditional is the time for obtaining the timing reference of target PCell. If SMTC of the target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync, Tsearch_PCell_Conditional = TΔ + Tmargin, where TΔ has the same definition in the delay requirements for PCell and Tmargin =2ms. Otherwise, Tsearch_PCell_Conditional = 0 ms.

    - TPSCell_ DU is the delay uncertainty in acquiring the first available PRACH occasion in the PSCell. TPSCell_ DU is up to the summation of SSB to PRACH occasion association period and 10 ms as UE can transmit RACH on different FR simultaneously in FR1+FR2 NR-DC.

### Sub-topic 3-2 CHO including target MCG and candidate SCG for CPC/CPA in NR-DC (obj. 4)

Issue 3-2-1: PCell handover delay in CHO including target MCG and candidate SCG for CPC/CPA in FR1+FR2 NR-DC (obj. 4)

- Candidate solutions:

  - Option 1: (CATT, ZTE)

    - consider DCHOwithCPAC = Max [DCHO,  DCPAC] as baseline. TEvent_DU needs to be updated.

      - DCHO = TRRC + TEvent_DU + Tmeasure + TCHO_execution +Tprocessing + TIU + T∆ + Tmargin

      - where all detail delay components shall follow TS38.133 clause 6.1.4.2 legacy definition

      - DCPAC = TRRC_delay + TEvent_DU + Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms

      - where all detail delay components shall follow TS38.133 clause 8.9A.2 legacy definition

      - TEvent_ DU should be until the condition at the measurement reference point which will trigger the conditional handover and the conditional PSCell addition/change

  - Option 1a: (Ericsson)

    - DCHOwithCPAC = Max [DCHO,  DCPAC].

      - DCHO = TRRC + TEvent_DU + Tcell_index_identify + TCHO_execution +Tprocessing + TIU + T∆ + Tmargin

      - DCPAC = TRRC_delay + TEvent_DU + Tcell_index_identify + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms

    - where all detail delay components shall follow legacy except from update of TEvent_DU and  Tcell_index_identify

  - Option 2: (CMCC)

    - DCHOwithPSCell_PCell = TRRC + TEvent_DU + Tmeasure + Tprocessing + TIU + T∆ + Tmargin + TCHO_execution if

      - CPC/CPA and CHO conditions are satisfied simultaneously

      - CPC/CPA condition is fulfilled firstly

  - Option 3: (MTK, Apple, HW, QC)

    - similar as obj.3, i.e., DCHOwithPCell_PCell = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution and Tinterrupt = Tprocessing + TIU + T∆ + Tmargin, except that

      - Tprocessing = 25 ms

      - TIU can be up to the summation of SSB to PRACH occasion association period and 10 ms as UE can transmit RACH on different FR simultaneously in FR1+FR2 NR-DC.

  - Option 4: (vivo)

    - if CHO and CPC conditions are met simultaneously or CPC conditions is met firstly and CHO condition is met later, DCHOwithCPC_PCell = TRRC_delay + max (TEvent_DU_PCell + Tmeasure_PCell, TEvent_DU_PSCell + Tmeasure_PSCell) + Tinterrupt + TCHO_execution

Issue 3-2-2: PSCell handover delay in CHO including target MCG and candidate SCG for CPC/CPA in FR1+FR2 NR-DC (obj. 4)

- Candidate solutions:

  - Option 1: (CATT, ZTE)

    - consider DCHOwithCPAC = Max [DCHO,  DCPAC] as baseline. TEvent_DU needs to be updated.

      - DCHO = TRRC + TEvent_DU + Tmeasure + TCHO_execution +Tprocessing + TIU + T∆ + Tmargin

      - where all detail delay components shall follow TS38.133 clause 6.1.4.2 legacy definition

      - DCPAC = TRRC_delay + TEvent_DU + Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms

      - where all detail delay components shall follow TS38.133 clause 8.9A.2 legacy definition

  - Option 1a: (E///)

    - DCHOwithCPAC = Max [DCHO,  DCPAC].

      - DCHO = TRRC + TEvent_DU + Tcell_index_identify + TCHO_execution +Tprocessing + TIU + T∆ + Tmargin

      - DCPAC = TRRC_delay + TEvent_DU + Tcell_index_identify + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms

    - where all detail delay components shall follow legacy except from update of TEvent_DU and  Tcell_index_identify

  - Option 2: (MTK)

    - DCHOwithPSCell_PCell = TRRC + TEvent_DU + Tmeasure + TCHO_execution + Tprocessing + Tsearch_PSCell + T∆_PSCell + TPSCell_ DU + 2 ms, where

      - The definitions of TRRC, TEvent_DU, Tmeasure, TCHO_execution, Tprocessing are the same as the definitions in the delay requirements for Pcell

      - TPSCell_ DU is the delay uncertainty in acquiring the first available PRACH occasion in the PSCell. TPSCell_ DU is up to the summation of SSB to PRACH occasion association period and 10 ms as UE can transmit RACH on different FR simultaneously in FR1+FR2 NR-DC.

  - Option 2a: (Apple)

    - DCHOwithPSCell_PCell = TRRC + TEvent_DU + Tmeasure + TCHO_execution + Tprocessing + Tsearch_PCell_Conditional + Tsearch_PSCell + T∆_PSCell + TPSCell_ DU + 2 ms, TEvent_DU needs to be updated

  - Option 3: (vivo)

    - DCHOwithCPC_PSCell = TRRC_delay + max (TEvent_DU_PCell + Tmeasure_PCell, TEvent_DU_PSCell + Tmeasure_PSCell) + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms

  - Option 4: (HW)

    - Additional Tmeasure_PCell is supposed to be added:

    - DCHOwithcandidatePSCell_PCell = = TRRC + TEvent_DU + Tmeasure_PCell+ Tmeasure_PSCell + Tprocessing + TPSCell_ DU + T∆ + Tmargin + TCHO_execution.

  - Option 5: (QC)

    - DCHOwithPSCell_PCell = TRRC + TEvent_DU + Tmeasure + TCHO_execution + Tprocessing + Tsearch_PSCell + T∆_PSCell + TPSCell_ DU + 2 ms, where

    - The definitions of TRRC, TEvent_DU, Tmeasure, TCHO_execution, Tprocessing are the same as the definitions in the delay requirements for Pcell

    - TPSCell_ DU is the delay uncertainty in acquiring the first available PRACH occasion in the PSCell. TPSCell_ DU is up to the summation of SSB to PRACH occasion association period and 10 ms as UE can transmit RACH on different FR simultaneously in FR1+FR2 NR-DC.

Issue 3-2-3: TEvent_DU

- Candidate solutions:

  - Option 1: (Apple)

    - TEvent_DU in PCell and PSCell handover delay requirement in FR1+FR2 NR-DC (obj. 4) is the delay uncertainty which is the time from when the UE successfully decodes the RRC command for CHO including target MCG and candidate SCG for CPC/CPA until conditions exist at the measurement reference point which will trigger the CHO and CPC/CPA.

  - Option 1a: (HW)

    - TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until conditions fulfilled for PCell and associated candidate PSCell at the measurement reference point which will trigger the conditional handover

  - Option 1c: (vivo)

    - TEvent_ DU should be until the condition at the measurement reference point which will trigger the conditional handover and the conditional PSCell addition/change

  - Option 2: (E///)

    - Resue the TS38.133 clause6.1.4

    - TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a CHO+CPAC command until a condition exists at the measurement reference point which will trigger the conditional handover (CHO).

Issue 3-2-4: Tmeasure

- Candidate solutions:

  - FFS: Tmeasure for CHO with CPA/C shall be updated to T cell_index_identify. Tcell_index_identify is the from the end of TEvent_DU until UE executes a handover to a target cell and interruption time starts. (E///)

Issue 3-2-5: if complementary CHO-only configuration is provided

- Agreements:

  - Legacy conditional handover can be reused for conditional PCell handover if complementary CHO-only configuration is provided

Issue 3-2-6: if complementary CHO-only configuration is NOT provided

- Candidate solutions:

  - Option 1: (CMCC)

    - For the case that CHO condition is met, but CPC condition is not met, if no complementary CHO-only configuration is provided, whether CHO execution is triggered is still under discussion in RAN2. RAN4 can wait for the conclusion from RAN2.

  - Option 2: (vivo)

    - PCell handover delay: DCHOwithCPC_PCell = TRRC_delay + max (TEvent_DU_PCell + Tmeasure_PCell, TEvent_DU_PSCell + Tmeasure_PSCell) + Tinterrupt + TCHO_execution and Tinterrupt = Tprocessing + TIU + T∆ + Tmargin where

      - The definition of TRRC_delay, TEvent_DU_PCell, Tmeasure_PCell, TCHO_execution, T∆, TIU, Tmargin are the same as the definitions in the delay requirement on conditional handover

      - Tprocessing = 25ms

    - PSCell change delay: DCHOwithCPC_PSCell = TRRC_delay + max (TEvent_DU_PCell + Tmeasure_PCell, TEvent_DU_PSCell + Tmeasure_PSCell) + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms

      - The definition of TEvent_DU_PSCell, Tmeasure_PSCell, TUE_preparation, T∆, TPSCell_ DU are the same as the definitions in the delay requirement on conditional PSCell change

      - Tprocessing = 25ms

Issue 3-2-7: others

- Candidate solutions:

  - Option 1: RAN4 to discuss the impact of parallel processing in FR1 + FR1 NR-DC on delay requirements (Nokia)

- Recommended WF

  - According to agreement in RAN4#106-bis-e, this can be postponed to RAN4#108.

Issue 3-2-8: CR spliting

- Agreements:

| Title | Section | Responsible company |
| --- | --- | --- |
| Conditional handover including target MCG in FR1 and target SCG in FR1 in NR-DC | (new) 6.1.6.1.1 | Qaulcomm |
| Conditional handover including target MCG in FR1 and target SCG in FR2 in NR-DC | (new) 6.1.6.1.2 | Apple |
| Conditional handover including target MCG in FR1 and candidate SCG for CPC/CPA in FR1 in NR-DC | (new) 6.1.7.1.1 | vivo |
| Conditional handover including target MCG in FR1 and candidate SCG for CPC/CPA in FR2 in NR-DC | (new) 6.1.7.1.2 | Ericsson |