3GPP TSG-RAN WG4 Meeting #107	R4-2310086

Incheon, KR, May 22nd – May 26th, 2023

Title: 	WF on RRM requirements for NR network energy saving

Agenda Item:	8.35.4

Source: 	Huawei, HiSilicon

Document for:	Approval

- Topic #1:	SSB-less SCell operation

- Sub-topic 1-1: Scenarios

Issue 1-1-1/2/3: Scenario 1 / 2 / 2a

- Agreements

  - Continue RAN4 work on the following SSB-less SCell scenarios

    - Scenario 1: SCell without SSB transmission and with TRS transmission

    - Scenario 2a: SCell without SSB transmission and without any other DL transmissions, but with UL reception at the NW side

      - Note: No RAN1 impacts are expected, and no RAN4 requirements will be defined if the scenario is not supported from RAN1 specification perspective.

  - Deprioritize RAN4 work on the following SSB-less SCell scenario

    - Scenario 2: SCell without SSB transmission and without TRS transmission

  - Send LS to RAN1/2 to check on support of Scenario 2a from RAN1/2 specifications perspective

Issue 1-1-4: Clarification on TRS

FFS:

  - Proposal 1: RAN4 to clarify what type of TRS is intended for SSB-less SCell operation in Scenario 1 (Nokia)

  - Proposal 2: Periodic TRS is available on the SSBless Scell. (QC)

  - Proposal 3: RAN4 to study the UEs dependency on TRS for data reception and whether no TRS would work or not. (Ericsson)

- Sub-topic 1-2: Feasibility conditions

Issue 1-2-1: RTD conditions for scenario 1

- Agreements

  - Further consider the following cases for requirements definition

    - Set 1: RTD ≤ 3us + X (X is FFS)

    - Set 2: 260ns < RTD < min(CP, 3us)

      - note: the CP corresponding to the largest SCS across CCs

    - Set 3: RTD ≤ 260ns

    - FFS whether all subsets are feasible from UE implementation perspective

Issue 1-2-2: RTD conditions for scenario 2

Scenario 2 is deprioritized based agreement in Issue 1-1-1/2/3.

Issue 1-2-3: RTD conditions for scenario 2a

FFS:

  - Proposal 1: RTD between SCell without SSB and the inter-band active serving cell is within 260ns (CTC, ZTE)

  - Proposal 2: RTD between SCell without SSB and the inter-band active serving cell is within RTD< CP/2-Te ns (CMCC)

Issue 1-2-4: Power difference conditions for scenario 1

FFS:

  - Proposal 1: The difference of the reception power with the FR1 inter-band active serving cell is within 6dB (Apple, CATT, Intel, MTK, CTC, CMCC, Vivo, Huawei, ZTE)

    - Proposal 1a: For a UE using single RF chain, the difference of the reception power with the FR1 inter-band active serving cell is within 6dB (Ericsson, CMCC)

    - Proposal 1b: Reception power difference between the reference cell with applying transmit power compensation and SSBless SCell is within 6dB. (QC)

  - Proposal 2: For a UE using dual RF chains, the maximum power difference UE can handle is FFS, and RAN4 to study whether UE can use TRS transmission in scenario 1 for computing AGC (Ericsson)

    - Proposal 2a: For a UE using dual RF chains, the maximum power difference UE can larger than 6dB, the SCell activation delay should be further studied. (CMCC)

  - Proposal 3: The difference of reception power is expected to vary between 6dB and 25dB. (Nokia)

Issue 1-2-5: Power difference for scenario 2

Scenario 2 is deprioritized based agreement in Issue 1-1-1/2/3.

Issue 1-2-6: Power difference conditions for scenario 2a

FFS:

  - Proposal 1: The difference of the reception power with the FR1 inter-band active serving cell is within 6dB (CTC)

    - Proposal 1a: For a UE using single RF chain, the difference of the reception power with the FR1 inter-band active serving cell is within 6dB (Ericsson)

Issue 1-2-7: Others related to power difference

FFS:

  - Proposal 1: NW can provide transmit power offset between reference Cell and SSBless SCell, where the transmit power offset = Transmit power of RS on reference Cell – Transmit power of RS on SSBless SCell. (QC)

  - Proposal 2: Difference of the reception power be discussed in RF session (LG)

Issue 1-2-8: QCL/TCI indication

FFS:

  - Proposal 1: RS of SCell without SSB is QCL-A with TRS of the SCell without SSB, and the TRS(s) of the SCell is (are) further QCL-TypeC with SSB(s) of an inter-band active serving cell.  (Vivo, Apple, CATT, MTK, Huawei)

    - Proposal 1a: NW shall be able to provide TCI state information for the SSBless SCell based on measurement and report conducted from the reference Cell. (QC)

  - Proposal 2: TCI state indication may not be needed (CTC, Ericsson)

  - Proposal 3: Consider both with and without QCL configuration between the RSs from inter-band carriers. (Nokia)

Issue 1-2-9: Frequency range

FFS:

  - Proposal 1: RAN4 should study the feasible range of frequency separation between inter-band carriers to enable the SSB-less SCell operation. (Nokia)

  - Proposal 2: Frequency domain separation can be in RF session (LG, CTC, ZTE)

  - Proposal 3: RRM requirement discussion of SSB-less SCell operation for FR1 inter-band collocated CA can assume adjacent inter-band scenario as starting point. (Apple)

  - Proposal 4: (Vivo)

    - For scenario 1, instead of specifying frequency domain separation condition between 2 bands, RAN4 discuss the UE capability implying the same antenna set is used in the inter-band CA combination:

    - If UE does not report the corresponding capability, CSI-RS based L3 measurement should be supported for the SSB-less SCell. Send LS to RAN2 to inform this decision and ask them to clarify in RAN2 spec about the freqBandIndicatorNR configured under MeasObjectNR for this case.

Issue 1-2-10: Performant evaluation for feasibility study

FFS

  - Proposal 1: RAN4 to discuss and agree that the performance evaluation metrics to establish the feasibility of SSB-less SCell operation include at least metrics related to the demodulation performances of the PDSCH transmitted by the SSB-less SCell (e.g. BLER). (Nokia)

  - Proposal 2: For all Scenario 1, 2 and 2a, no need to perform PDCCH/PDSCH performance evaluation. (ZTE)

    - Proposal 2a: For band combination with small frequency domain separation, no need to carry out the performance evaluation. (CTC)

- Sub-topic 1-3: RRM requirements for SSB-less SCell activation

Issue 1-3-1: Whether TRS is used in SCell activation

FFS:

  - Proposal 1: RAN4 to study the SSB-less SCell operation for scenario 1 considering the following cases: (Nokia)

    - Case 1: TRS monitoring is not needed for SSB-less SCell activation (i.e. 3ms SCell activation delay is achieved).

    - Case 2: TRS monitoring is needed for SSB-less SCell activation (wherein >3ms SCell activation delay is expected).

  - Proposal 2: For band combinations with small frequency domain separation, there is no need to consider requirements based on TRS transmission during SCell activation. (CTC)

Issue 1-3-2: SCell activation requirements

FFS:

  - Proposal 1: For FR1 co-located SSB-less inter-band CA, when the conditions defined above are met, the SCell activation delay can be further reduced to 3ms (CATT, CTC, Huawei)

  - Proposal 2: Use fast SCell Activation Delay Requirement for FR1(section 8.3.16 of TS38.133) as the baseline to specify the requirement for FR1 SSB-less SCell activation, e.g., A-TRS is used for fine timing tracking and fine AGC settling (Apple)

  - Proposal 2a: (Vivo)

For an SSB-less SCell that configured with CSI-RS based L3 measurement, the known/unknown condition of the SCell is defined based on the L3 MR of the CSI-RS based L3 measurement

If an SSB-less SCell is configured with CSI-RS based L3 measurement, and UE has reported one MR within a predefined period, the SSB-less SCell is unknown

    - The activation delay of a known SSB-less SCell is TfirstTRS + 5ms or TfirstA-TRS + 5ms.

Otherwise, the SSB-less SCell is unknown

    - The activation delay of an unknow SSB-less SCell is TfirstTRS + TTRS + 5ms or TFirstATRS + Tgap + TATRS + 5ms

  - Proposal 3: There is at least one active serving cell as reference cell which can provide SSB for T/F sync, AGC for the SSB-less Scell. The reference cell shall keep being activated. (QC)

    - SSBless SCell for interband CA is considered as known SCell from Scell activation latency perspective. (e.g, UE can skip coarse AGC, measurement and report for TCI activation can be skipped)

  - Proposal 4: RAN4 to discuss the SCell activation delay for activating an inter-band co-located SSB-less SCell after the feasibility study is concluded. (Nokia)

  - Proposal 5: (CMCC)

    - In scenario 1, for the case of RTD> CP/2-Te, and/or reception power difference larger than 6dB, the SCell activation delay should be further studied, to at least capture fine sync delay and/or AGC adjustment delay, and considering the impaction due to lack of QCL-ed relationship.

Issue 1-3-3: Other RRM requirements

FFS:

  - Proposal 1: If SSB less SCell supports UL, RAN4 to study UL timing requirements of SSB less SCell. (Ericsson)

- Sub-topic 1-4: L1 measurement for SSB-less Scell

Issue 1-4-1: Whether to have L1 measurement on SSB-less SCell

FFS:

  - Proposal 1: L1 measurement on less SCell is not needed. (MTK, ZTE, CATT, Huawei)

    - Proposal 1a: For SSB-less SCell activation, when the conditions about RTD, power imbalance and TRS are met, the L1/L3 measurement can be skipped (CATT)

    - Proposal 1b: When CSI-RS resources for L1 measurement are not configured, as long as the RTD, power difference conditions are ensured between two carriers, the SSB-less SCell can fully leverage the information from an already activated serving cell, the corresponding L1 measurements can be skipped (Huawei)

  - Proposal 2: CSI-RS based L1 measurement is needed on SSB-less SCell

    - Proposal 2a: RAN4 to assume no SSB but with CSI-RS resource for L1 measurement on the FR1 inter-band SSB-less SCell (Apple)

    - Proposal 2b: If the conditions are not met but CSI-RS based measurement is supported and configured, L1 measurement needs to be specified for SSB-less SCell operation. (CATT)

    - Proposal 2c: When CSI-RS resources for L1 measurement are configured, the legacy requirements for CSI-RS based L1 measurement can be reused for SSB-less SCell operation. (Huawei)

  - Proposal 3a: RAN4 needs to discuss the impact on the CSI-RS based L1 measurement requirements due to SSB-less SCell operation.  (Nokia)

  - Proposal 3b: RAN4 can study whether and how to perform RLM/BFD/CBD on the SSBless SCell based on reference Cell measurement. (Vivo, QC)

  - Proposal 3c: For L1/L3 measurement, following two options can be discussed in parallel in RRM session. （LGE）

    - Option 1: No SSB but with CSI-RS resource for L1/L3 measurement on the inter-band SSB-less SCell

    - Option 2: No SSB and No CSI-RS resource for L1/L3 measurement on the inter-band SSB-less SCell

  - Proposal 4 (from moderator):

    - When RTD, power difference conditions are ensured and CSI-RS based L1 measurement is not configured, L1 measurement on SSB-less SCell is not needed

    - When CSI-RS based L1 measurement is configured, legacy requirements can apply.

- Sub-topic 1-5: L3 measurement for SSB-less Scell

Issue 1-5-1: Whether to have L3 measurement on SSB-less SCell

FFS:

  - Proposal 1: L3 measurement is no needed on SSB-less SCell (Appe, CATT, MTK, Huawei, ZTE, QC)

    - Proposal 1a: For SSB-less SCell activation, when the conditions about RTD, power imbalance and TRS are met, the L3 measurement can be skipped (CATT)

    - Proposal 1b: As long as the RTD, power difference conditions are ensured between two carriers, the SSB based L3 measurements on PCC can be leveraged for SSB-less SCC. (Huawei)

  - Proposal 2: If the conditions are not met but CSI-RS based measurement is supported and configured, L3 measurement needs to be specified for SSB-less SCell operation. (CATT)

  - Proposal 3a: RAN4 needs to discuss the impact on the CSI-RS based L3 measurement requirements due to SSB-less SCell operation.  (Nokia)

  - Proposal 3b: For L3 measurement, following two options can be discussed in parallel in RRM session. （LGE）

    - Option 1: No SSB but with CSI-RS resource for L3 measurement on the inter-band SSB-less SCell

    - Option 2: No SSB and No CSI-RS resource for L3 measurement on the inter-band SSB-less SCell

- Sub-topic 1-6: Others

Issue 1-6-1: T/F information indication

FFS:

  - Proposal 1: regarding T/F information acquisition for the SSB-less SCell operation for inter-band CA for FR1 and co-located cells, RAN4 to consider one of following alternatives: (Apple)

    - Alt 1: expand the definition of QCLed-typeC to indicate the RTD between the SSB-less SCell and the inter-band active serving cell is within a small range, e.g., ±260ns. RAN4 needs to check with RAN1 for this solution.

    - Alt 2: introduce an indication from network to UE to indicate which inter-band active serving cell or which SSB on inter-band active serving cell can be used as timing source for the SSB-less SCell. RAN4 needs to check with RAN2 for this solution.

Issue 1-6-2: UE capability

FFS:

  - Proposal 1: If FR1 inter-band SSB-less SCell operation is introduced in R18 NES, a new capability like existing scellWithoutSSB shall be introduced to cover the R18 FR1 inter-band CA case: (Apple)

  - Proposal 2: It is left to RAN2 to decide whether to reuse existing SCellwithoutSSB or introduce new signalling (CATT)

  - Proposal 3: UE capability for SSBless SCell should be defined per band combo where the band combo indicate the bands of reference cell and the SSBless SCell (QC)

  - Proposal 4: RAN4 to study how to allow NW to understand which bands UE can support for SSB less SCell operation (Ericsson)

Issue 1-6-3: Intra-band CA

FFS:

  - Proposal 1: Activation requirements specified for inter-band SSB-less SCell also apply to intra-band non-contiguous SSB-less SCell activation (Intel)

  - Proposal 2: As the baseline requirements for R18 scenario 2a discussion, RAN4 discuss and agree on the activation delay for intra-band contiguous SCell without DL, which is also 3ms (Vivo)

Issue 1-6-4: TAG

FFS:

  - Proposal 1: No RRM requirements are specified if the SSB-less SCell is not configured in the same TAG as the reference serving cell (Vivo)

Issue 1-6-5: Reference Cell

FFS:

  - Proposal 1: There is at least one active serving cell as reference cell which can provide SSB for T/F sync, AGC for the SSB-less Scell. The reference cell shall keep being activated. (QC)

    - RAN4 consider PCell first as the reference cell transmitting SSB. FFS: another Scell can be used as the reference cell (QC)

  - Proposal 2: NOT to limit the reference CC as PCC (Vivo)

  - Proposal 3: Reference CC may be indicated by NW based on the UE capability exchange. The content of the UE exchange is FFS (Ericsson)

Issue 1-6-6: Margin for multiple Rx chains

FFS:

  - Proposal 1: To address UE vendors’ concern on different Rx chain assumptions between intra-band CA and inter-band CA, the time margin between multiple Rx chains can be clarified (ZTE)

Issue 1-6-7: LS to other WG

FFS:

  - Proposal 1: For SSB-less operation of an inter-band SCell in FR1, send the LS to RAN2 to trigger discussion regarding the following (Vivo)

    - Clarify in TS 38.331 that the SSB frequency configuration can be absent if the timing information can be provided from an inter-band active serving cell.

    - Clarify/introduce the RRC signalling for indicating which inter-band active serving cell is the timing reference of the cell without SSB.

  - Proposal 2: (Vivo)

    - For scenario 1, instead of specifying frequency domain separation condition between 2 bands, RAN4 discuss the UE capability implying the same antenna set is used in the inter-band CA combination:

    - If UE does not report the corresponding capability, CSI-RS based L3 measurement should be supported for the SSB-less SCell. Send LS to RAN2 to inform this decision and ask them to clarify in RAN2 spec about the freqBandIndicatorNR configured under MeasObjectNR for this case.

Issue 1-6-8: AGC information indication

FFS:

  - Proposal 1: regarding AGC settling for the SSB-less SCell operation for inter-band CA for FR1 and co-located cells, RAN4 to consider one of following alternatives: (Apple)

    - Alt 1: if T/F information is reused from an inter-band FR1 serving cell, the AGC info of same inter-band FR1 serving cell can also be used for the target SSB-less SCell.

    - Alt 2: introduce an indication from network to UE to indicate which inter-band active serving cell or which SSB on inter-band active serving cell can be used as AGC source for the SSB-less SCell.

  - Proposal 2: RAN4 to study whether UE can use TRS transmission for computing AGC for SSB less SCell (Ericsson)

- Topic #2:	RRM impact due to other objectives

Issue 2-1: RRM impacts of Cell DTX/DRX (obj#2)

FFS:

  - Proposal 1: (Apple)

To indicate RAN1 that:

Based on network configuration, Rel-18 UE supporting cell DTX does not expect to receive and/or process the following signals/channels from the gNB, during non-active periods of cell DTX:

    - PRS

    - CSI-RS configured by measObjectNR (for RRM)

    - CSI-RS associated with RadioLinkMonitoringConfig and BeamFailureDectection (for RLM and BFD)

    - Periodic/Semi-persistent CSI-RS (for BM)

    - FFS: PDCCH in USS to be decided by RAN1/2

    - FFS: PDCCH in Type-3 CSS to be decided by RAN1/2

    - To not impact legacy UEs that do not support NES feature, TRS is still maintained during non-active duration of cell DTX.

  - Proposal 2: RAN4 is to discuss the interruption requirements due to changes in Cell DTX/DRX configurations (Intel)

  - Proposal 3: RAN4 shall wait for more progress in RAN1/2 to investigate whether there is RRM impacts of DTX/DRX (LGE, Huawei)

Issue 2-2: RRM impacts of Spatial and power domain techniques (obj#3)

FFS:

  - Proposal 1: RAN4 shall wait for more progress of RAN1 on spatial and power adaptation to investigate whether there is RRM impact

Issue 2-3: RRM impacts of Cell selection/reselection (obj#4)

FFS:

  - Obj#4 (mechanism to prevent legacy UEs camping on cells adopting the Rel-18 NES techniques) has no RRM impact.

Issue 2-4: RRM impacts of CHO (obj#5)

FFS:

  - Proposal 1: RAN4 shall wait for more progress of RAN2 on CHO to investigate whether there is RRM impacts. (Huawei)