3GPP TSG-RAN WG4 Meeting # 107												R4-2310149

Incheon, KR, May 22nd – May 26th , 2023

Agenda item:			5.2.5.2, 5.2.5.3

Source:	Moderator (Ericsson)

Title:	WF on R17 NR RedCap RRM requirements maintenance

Document for:	Approval

# Core part

Handover to a BWP with unmeasured SSB of a known cell

Agreement: It’s a common understanding that UE may not be configured to measure SSBs having same PCI but transmitted from two different cells. This is a corner case and no specific UE behaviour is defined to handle this scenario.

Revision of the intra/inter-frequency definition in handover

Agreement: Handover for a RedCap UE is defined as intra-frequency handover if the center frequency and subcarrier spacing (SCS) of the reference SSB of the serving cell is same as the center frequency and SCS of the reference SSB of the target cell, where:

- The reference SSB of the serving cell is the SSB configured in the active DL BWP of the serving cell

- The reference SSB of the target cell is the SSB configured in the DL firstActiveBWP of the target cell

RAN4 to further discuss whether and how to update the intra-frequency handover definition when the active DL BWP of serving cell/target cell doesn’t have SSB.

- Option 1: The intra-frequency handover is defined if the center frequency of the active DL BWP of serving cell is the same as the center frequency of the first active DL BWP of target cell.

- Other option is not precluded

Intra-frequency cell definition for RRC re-establishment

Agreement: A neighbour cell for RRC re-establishment procedures for a RedCap UE is defined as an intra-frequency cell if the centre frequency and subcarrier spacing (SCS) of the reference SSB of the serving cell is same as the centre frequency and SCS of the reference SSB of the neighbor cell; else it is considered as inter-frequency cell, where:

- The reference SSB of the target cell is the CD-SSB of the target cell.

- FFS: The reference SSB of the serving cell

  - Option 1: SSB configured in the BWP-specific servingCellMO, if configured, else the SSB configured in the servingCellMO

  - Option 2: CD-SSB of the serving cell

  - Option 3: SSB within the active BWP

# Topic #2: Performance part

Offset for cell selection and not-at-cell-edge criteria thresholds

- Remove the offset from all not-at-cell-edge thresholds (i.e. s-SearchThresholdP-r16, s-SearchThresholdQ-r16, s-SearchThresholdP2-r17 and s-SearchThresholdQ2-r17).

- Keep the existing offset (-1 dB) to Qrxlevmin and Qqualmin

Offset for missing thresholds: rsrp-ThresholdMsg3

Add -1 dB of offset to rsrp-ThresholdMsg3.

Offset for missing thresholds: rsrp-ThresholdSSB-SUL

- Option 1: Add +1 dB of offset to rsrp-ThresholdSSB-SUL.

- Option 2:  Not to consider an offset for rsrp-ThresholdSSB-SUL as this threshold is not applicable to RedCap UE’s.

  - Option 2a: RAN4 to inform RAN2 that RAN4 will not to consider an offset for rsrp-ThresholdSSB-SUL as this threshold is not applicable to RedCap UE’s