3GPP TSG-RAN WG4 Meeting # 107				                                   R4-2310163

Incheon, KR, May 22 – May 26, 2023

Title:	WF on NR Positioning – SL positioning and CPP

Agenda item:			8.23.4

Source:	CATT

Document for:	Approval

- Introduction

This WF includes the agreements and open issues discussed in topic summary for [107][220] NR_pos_enh2_part2.

- Topic #1: Sidelink Positioning (agenda 8.23.3.2)

  - Sub-topic 1-1 Measurement period requirements

Issue 1-1-1: Measurement types to be defined for SL positioning measurement requirements:

Agreements:

- RAN4 to start with defining measurement period requirements for SL-PRS based RSTD measurement.

Issue 1-1-2: The principle for defining the measurement period requirements:

Agreements:

- As a general principle, RAN4 aims to define a common measurement period requirement for different SL positioning measurement types. Adaptations to specific measurements will be discussed on a case-by-case basis.

Issue 1-1-3: Parameters in the measurement period requirements:

GTW Agreements

- The measurements period requirements equation is FFS

- Number of samples for positioning measurements is FFS

  - Option 1: 1

  - Option 2: 2

  - Option 3: 3

  - Option 4: 4

- The number of Rx beams is equal to 1 in FR1.

Issue 1-1-4: Impact of SL-DRX

Agreements:

- RAN4 to start with defining the measurement period requirements based on non-DRX.

- FFS whether the defined requirements apply for non-DRX as well as SL DRX case.

Issue 1-1-5: Impact of other channels/signals

Agreements:

- RAN4 to discuss possible impacts of sharing/collision between SL PRS measurement and other SL measurements/signals on measurement period requirements based on RAN1 conclusions on this topic.

Issue 1-1-6: Impact of network coverage change

Agreements:

- RAN4 to discuss the impact of a change in coverage status (e.g., going between in-coverage, out-of-coverage, partial coverage) on the measurement period requirements and UE behaviour for SL positioning.

Issue 1-1-7: Impact of Uu link connection

Agreements:

- RAN4 to discuss the impact of Uu link mobility on SL positioning measurements, e.g., due to handover, RLF or RRC re-establishment in the Uu connection to gNB.

Issue 1-1-8: initiation/cease of SL PRS tx

Agreements:

- RAN4 to discuss whether and how to define the requirements for initiation/cease of SL PRS transmissions for positioning.

  - Sub-topic 1-2 Timing related requirements

Issue 1-2-1: Timing error limit requirements of SL UE for positioning (both anchor UE and target UE)

Agreements:

- Discuss the SL positioning requirements based on existing sidelink Te requirements.

- If any technical issues identified, the agreements can be revisited.

Issue 1-2-2: Impact due to the Tx/Rx timing difference of multiple anchor Ues

Agreements:

- SL timing related positioning measurement requirements are defined under the RAN4 assumption that there is no timing misalignment among multiple anchor UEs due to different synchronization reference sources.

Issue 1-2-3: The scenarios on synchronization reference resource change

Agreements:

- FFS whether to differentiate the possible scenarios with the synchronization reference source change.

  - Scenario A: Impact on an SL measurement due to the synch change in the same SL UE performing the measurement:

    - Receiving UE for unidirectional measurements

    - Receiving and transmitting UE for bidirectional measurements

  - Scenario B: Impact on an SL measurement due to the synch change in another SL UE (tx or ) involved in the same measurement:

    - Transmitting UE for unidirectional or bidirectional measurements

  - Scenario C: Impact on SL measurements at multiple SL UEs which are sharing the same common synchronization reference source, due to the change of the common synchronization source,

  - Scenario D: Impact on an SL positioning result due to the change in another SL UE involved in the same positioning session but not necessarily the same SL measurement (multi-RTT scenario).

Issue 1-2-4: UE behavior and the impact on requirements when synchronization reference source change occurs

Way forward:

- Proposal 1:

  - The measurement period requirements are not impacted due to synchronization reference source change.

- Proposal 2:

  - At least SL PRS timing measurements are impacted by the synchronization reference source.

- Proposal 3:

  - When the synchronization reference source changes during the on-going SL measurement or right before the SL measurement:

    - Alt 1: Delaying/postponing the start of the synch change until the SL measurement is complete

    - Alt 2: Dropping or restarting the impacted SL measurement

- Proposal 4:

  - RAN4 to discuss the impact of transition (sync source change) after requirements for non-transition case are stable.

  - Sub-topic 1-3 Others

Issue 1-3-2: Report mapping

Way forward:

- Option 1:

  - For SL-PRS RSRP, SL-PRS RSRPP, SL-PRS AOA/ZOA, SL-PRS RSTD, SL-PRS RTOA, the report mapping requirements including reporting range, resolution step can be same as Rel-16/Rel-17 report mapping requirement.

- Option 2:

  - The measurement ranges for SL positioning measurements (except for SL-PRS AOA/ZOA) can be different compared to those specified for Uu positioning measurements. RAN4 to discuss the relevant range for each SL positioning measurement.

- Option 3:

  - Reuse the UL AoA measurement report mapping in 38.133 for SL AoA measurements.

Issue 1-3-3: Simulation assumption

Agreements:

- SL PRS pattern:

  - Case 1: Fully staggered SL PRS patterns

  - Case 2: Partially staggered SL PRS patterns

- Channel model for RSTD, Rx-Tx and RSRP:

  - AWGN

  - TDL-A (30 ns delay spread, 5Hz Doppler spread)

  - TDL-B (100ns delay spread, 200Hz Doppler spread)

- Antenna:

  - 1T/2R

- SL PRS BW:

  - 10 MHz, 20 MHz, 40 MHz

- SCS:

  - 15kHz, 30kHz, 60kHz

- CBW:

  - equal to the SL PRS BW

- Sampling rate:

  - Reuse the assumption for NR PRS based measurements

- Other details can be found in R4-2310077 and its revision.

Topic #2: Carrier Phase Positioning (agenda 8.23.3.6)

  - Sub-topic 2-1 Measurement period requirements

Issue 2-1-1: Measurement period requirements for DL RSCP/DL RSCPD:

Agreements:

- FFS: When DL RSCP/RSCPD is reported together with legacy positioning measurement, the same requirements should apply for DL RSCP/RSCPD and the associated legacy measurements.

- FFS whether to reuse the existing Rel-17 requirements based on RAN1 discussion on time window for CPP measurement.

Issue 2-1-2: Gap conditions for DL carrier phase measurement requirements:

Agreements:

- For DL carrier phase measurement in RRC_CONNECTED state, MG-based measurement requirements are defined.

Issue 2-1-3: Applicable RF frequencies for DL carrier phase measurement requirements:

Agreements:

- For RSCP measurement:

  - The measurement is associated with the specific RF frequency.

  - This specific RF frequency is the center frequency of the DL PFL by default.

- For RSCPD measurement:

  - RF frequencies for target TRP and reference TRP should belong to same PFL.

  - RAN4 will define the requirements starting from the same RF frequency for target and reference TRP.

Issue 2-1-4: Impact of UE mobility in RRC_CONNECTED:

Agreements:

- FFS: The mobility impact in RRC_CONNECTED in Rel-17 can also be reused.

Issue 2-1-5: Impact of collisions with other signals/channels in RRC_INACTIVE:

Agreements:

- FFS: The collision handling in RRC_INACTIVE state in Rel-17 can also be reused.

Issue 2-1-6: Applicable requirements for PRU:

Agreements:

- The measurement period requirements defined in RAN4 apply to both target UE and PRU(s).

  - Sub-topic 2-2 Measurement reporting requirements

Issue 2-2-1: Measurement reporting requirements for DL carrier phase measurement:

Agreements:

- FFS: When RSCP or RSCPD is reported together with legacy measurements, measurement reporting delay requirements for legacy measurements apply for CP measurements.

  - Sub-topic 2-3 Others

Issue 2-3-1: Report mapping for DL RSCP/RSCPD:

Agreements:

- Reporting range:

  - [0, 360) degrees for DL RSCP.

  - FFS for DL RSCPD.

- Granularity is FFS

  - Option 1: Fixed value

  - Option 2: Up to NW configuration and UE capability

Issue 2-3-2: Report mapping for UL RSCP:

Agreements:

- Same as DL RSCP.

Issue 2-3-3: Simulation assumption for RSCPD:

Agreements

- PRS configuration

  - Reuse Rel-17 configuration of RSTD for RSCPD

- Side condition

  - Reuse Rel-17 RSTD side condition for RSCPD

- Channel model:

  - AWGN

  - TDL-A (30 ns delay spread, 5Hz Doppler spread)

  - 2-tap model (same as for PRS-RSRPP)

- Number of samples:

  - Nsample = 1, 4

- Other details can be found in R4-2310078 and its revision.

Reference

- R4-2309965 Topic summary for [107][220] NR_pos_enh2_part2, CATT, RAN4#107