3GPP TSG-RAN WG4 Meeting #107	R4-2310257

Incheon, KR, May 22 – May 26, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.872 | CR | Draft | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TR 38.872 to introduce some potential RF impacts for for CA_n26-n28 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Skyworks Solutions Inc., Qualcomm, Telstra |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_sub1GHz_combo_enh |  |  |  |  |  | Date: |  |  | 2023-05-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To introduce some potential RF impacts for for CA_n26-n28 |  |  |  |  |  |  |  |  |
| Summary of change: |  | To introduce some potential RF impacts for for CA_n26-n28. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The outcomes for CA_n26-n28 for this SI can’t be captured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.3, 5.6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of Change for TR 38.872>>

5.6.3	Common for 1 band UL and 2 bands UL of CA_n26-n28

5.6.3.1	Operating bands for CA

Table 5.6.3.1-1:  CA band combination of band n26+n28

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz | FDD |
| n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

A note may be needed if case 1 is considered for CA_n26-n28, e.g. band n28 is restricted at lower 30MHz (UL: 703~733MHz, DL: 758~788MHz).

In addition, referring to the contribution R4-2309358, the following options about different frequency range restrictions are listed to trade-off the pros and cons. From a high level perspective, based on the proponent operator’s feedback, one possibility is that RAN4 move forward with option 4 (i.e. both options 1 and 3 being captured in the specification) which would allow maximum implementation flexibility.

Option 1 Keep current CA_n26A-n28A with entire n28 frequency range and specify n26 UL and UL CA support for entire frequency range

Option 2 Modify current CA_n26A-n28A to include only lower 30MHz of n28 and specify n26 UL and UL CA support for lower 30MHz frequency range

Option 3 Keep current CA_n26A-n28A with entire n28 frequency range and specify n26 UL and UL CA support for lower 30MHz frequency range

Option 4 Combination of options 1 and 3

5.6.3.2	Channel bandwidths per operating band for CA

Referring to table 5.5A.3.1-1h of TS 38.101-1, DL_n26A-n28A_BCS0 and BCS1 has been specified

5.6.3.3	Identify the potential MSD issues due to harmonic/harmonic mixing/cross band isolation

There is no need to specify harmonics and harmonics mixing exception for CA_n26-n28 as we didn’t specify them in current spec.

For cross band isolation issue:

Case 1: The 2nd adjacent channel leakage from band n26 may fall into band DL frequency range (774~788) when 20MHz UL CBW is assumed for band n26.

Case 2: the 1st adjacent channel leakage from band n26 may fall into n28 DL frequency range (796~803) when 20MHz UL CBW is assumed for band n26.

5.6.3.4	∆TIB,c and ∆RIB,c values

Option 1 is proposed based on the agreed Delta Tib and Rib values for CA_n5-n28 in WF R4-2306468 which can be reused for CA_n26-n28.

Option 2 is proposed in the contribution R4-2309358.

Table 5.6.3.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] option 1 | ΔTIB,c [dB] option 2 |
| --- | --- | --- | --- |
| CA_n26A-n28A | n26 | 0.7 | 0.5 |
|  | n28 | 0.7 | 0.5 |

Table 5.6.3.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] option 1 | ΔRIB,c [dB] option 2 |
| --- | --- | --- | --- |
| CA_n26A-n28A | n26 | 0.2 | 0.3 |
|  | n28 | 0.2 | 0.3 |

5.6.3.5	REFSENS exception evaluation for one UL band

1) Referring to the contribution R4-2308583, the following band n28A MSD evaluations, assuming 35dB and 45dB Tx filter rejection of band n26.

Table 5.6.3.5-1: band n28A MSD evaluations due to cross band isolation for CA_n26-n28

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n26 | n28A | 824 | 20 | 15 | 25 (RBstart=0) | 785.5 | 5 | 25.6 | ACLR2 |
| n26 | n28A | 824 | 20 | 15 | 25 (RBstart=0) | 785.5 | 5 | 15.9 | ACLR2 |

Referring to the contribution R4-2308583, we have the following band n28B MSD evaluations, assuming filter performance with set 1 and set 2 in Table 5.6.3.5-3.

Table 5.6.3.5-2: band n28B MSD evaluations due to cross band isolation for CA_n26-n28

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n26 | n28B | 824 | 20 | 15 | 25 (RBstart=0) | 801.5 | 5 | 45.9 (Set 1) | ACLR1 |
| n26 | n28B | 824 | 20 | 15 | 25 (RBstart=0) | 801.5 | 5 | 28.5 (Set 2) | ACLR1 |

Table 5.6.3.5-3: Filter performance assumption

|  | Set 1 | Set 2 |
| --- | --- | --- |
| n28B Rx filter rejection at 814~849MHz | 12 dB | 33 dB |
| n26 Tx filter rejection at 773~803MHz | 35 dB | 45 dB |

2) Referring to the contribution R4-2307477, the following analysis were provided.

To evaluate the potential 1UL and 2UL for CA_n26-n28 with band n28 full (n28F) or the lower 30MHz (n28A), it is useful to compare existing cases for CA_n5-n28F and CA_n18-n28F and CA_n18-n28A. This can be done by observing the IMD landscape provided in Figure 5.6.3.5-1, and the related wanted UL RB allocation and its image IMDs overlaps for 5MHz and 30MHz n28DL for n28F or n28A scenarios that are collected in Table 5.6.3.3-4.

In addition, 1UL cross band MSD for n28F and n28A were proposed in 1UL cross band MSD for n28F and n28A.

Figure 5.6.3.5-1: IMD landscape of n5, n18, n26 and n28 for n28F and n28A cases.

Table 5.6.3.5-4: Distance to n28 victim DL and IMD overlap for different UL and DL scenarios

| Band | Max UL BW | #RB | Distance to (MHz) |  | IMD overlap |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | n28A | n28F | n28A 5MHz | n28F 5MHz | n28A 30MHz | n28F 30MHz |
| n26 | 20MHz | 25 | 26 | 11 | IMD5/7 | IMD3 | IMD5/7 | IMD3/5/7 |
| n5 | 20MHz | 20 | 21 | 36 | IMD5/7 | IMD5 | IMD5/7 | IMD5/7 |
| n18 | 15MHz | 25 | 17 | 12 | IMD5/7 | IMD3/5 | IMD5/7 | IMD3/5/7 |
| n28A | 30MHz | 25 | 25 | na | na | na | IMD3/5 | na |
| n28F | 30MHz | 25 | na | 10 | na | na | na | IMD3/5 |

Table 5.6.3.5-5: 1UL cross band MSD for n28F and n28A

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n26 | n28 | 822.5 | 20 | 15 | 25 (RBstart=0) | 800.5 | 5 | [>30] | ACLR1 |
| n26 | n28 | 822.5 | 20 | 15 | 25 (RBstart=0) | 785.5 | 5 | [18] | ACLR2 |

3) Referring to the contribution R4-2309358, the technical analysis for two cases (case 1: n28 Cross-band MSD for lower 30MHz of n28 / case 2: n28 Cross-band MSD for entire n28) are shown below.

Table 5.6.3.5-6: Cross-band MSD for lower 30MHz of n28

Table 5.6.3.5-7: n28 Cross-band MSD for entire n28

It’s observed that MSD is about 15-20dB higher if the entire n28 must be supported, compared to case when only lower 30MHz of n28 is supported.

Table 5.6.3.5-8: n28 Cross-band MSD test point for lower 30MHz of n28 and entire n28

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n26 | n28 | 824 | 20 | 15 | 25 (RBstart=0) | 785.5 | 5 | 17.0 | ACLR2 |
| n26 | n28 | 824 | 20 | 15 | 25 (RBstart=0) | 800.5 | 5 | 33.7 | ACLR |

5.6.4	Specific for 2 bands UL of CA_n26-n28

5.6.4.1	Maximum output power for inter-band CA

Power class 3 is assumed for UL CA_n26-n28

5.6.4.2	UE co-existence studies

Table 5.6.4.2-1 lists Band n26 + Band n28 2UL bands CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.6.4.2-1: Band n26 and Band n28 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 814 | 849 | 703 | 748 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 146 | 66 | 1517 | 1597 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 880 | 995 | 557 | 682 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2331 | 2446 | 2220 | 2345 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1694 | 1844 | 1260 | 1430 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3145 | 3295 | 2923 | 3093 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 132 | 292 | 3034 | 3194 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2178 | 1963 | 2693 | 2508 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 616 | 411 | 1141 | 946 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3626 | 3841 | 3959 | 4144 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3737 | 3942 | 3848 | 4043 |

3rd order IMD may also fall into Rx frequencies of bands n26 when both Band n26 and Band n28 transmit the UL signals.

Table 5.6.4.2-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n26-n28 | E-UTRA Band 2, 3, 5, 18, 19, 25, 26, 31, 34, 40, 73 NR Band n79 | FDL_low | - | FDL_high | -50 |  |  |
|  | E-UTRA Band 4, 41, 42, 43, 50, 51, 52, 65, 66, 74 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 2, 11, 15 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 2, 11, 12 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 11 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 4, 14 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 13 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 4 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 4 |
|  | Frequency range | 773 | - | 799 | -50 | 1 |  |
|  |  | Frequency range |  | 799 |  | - |  | 803 |  | -40 |  | 1 |  | 4 |  |
|  |  | Frequency range |  | 945 |  | - |  | 960 |  | -50 |  | 1 |  |  |  |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7 MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 11: Applicable when the assigned NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 12: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned NR carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.3.1-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 13: This requirement is applicable for 5 and 10 MHz NR channel bandwidth allocated within 718 - 728 MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and Rbstart < 48. NOTE 14: This requirement is applicable in the case of a 10 MHz NR carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 15: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). |  |  |  |  |  |  |  |

5.6.4.3	REFSENS exception evaluation for two UL band

1) MSD due to IMD

If the following test frequency range was assumed, we can calculate the frequency range which is hit by IMD3 from UL CA_n26-n28 as 880~895 MHz. It can be observed the IMD3 missed the DL channel of band n26 by 5MHz.

n26: UL 814~819	DL 859~864

n28: UL 743~748	DL 798~803

IMD3 range is 880~895 MHz

That means there is no direct test frequency point which can be found for IMD3 MSD definition.

2) MSD due to two UL cross band isolation

Even if the performance of duplexer/filter’s isolation can be guaranteed by one UL cross band isolation MSD, two PA linearity performance should be guaranteed simultaneously by two UL cross band isolation MSD requirements especially for the case that two 1st / 2nd adjacent channel interferences from two different UL aggressor bands hit the DL part of victim band. The following test points were proposed in contribution R4-2307477 for further discussion.

Table 5.6.4.3-1: 2UL cross band MSD for n28F and n28A

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n26 | n28 | 824 | 20 | 15 | 25 (RBstart=0) | 788 | 30 | [>6] | ACLR1 from n26 UL band and ACLR1+ACLR2 from n28 UL band |
| n28 |  | 733 | 30 | 15 | 25 (RBstart=135) |  |  |  |  |
| n26 | n28 | 824 | 20 | 15 | 25 (RBstart=0) | 773 | 30 | [3] | ACLR2 from n26 UL band and ACLR1+ACLR2 from n28 UL band |
| n28 |  | 718 | 30 | 15 | 25 (RBstart=135) |  |  |  |  |

<<End of Change>>