3GPP TSG-RAN WG4 Meeting # 107				                    		R4-2310309

Incheon,KR, May 22nd  – May 26th , 2023

Agenda Item:	8.31.2.1.3

Source: 	OPPO

Title: 	TP to TR on the SL-e RX requirement

Document for:	Approval

- Introduction

The sidelink unlicensed RX requirement has been discussed in the previous RAN4 meeting and couple of WF [1][2][3][4] has been agreed. For the SL-e un-licensed RX requirements, there are still some of them left un-fininshed and hence in this paper we give further discussion and try to finalize them.,

This is the revision of R4-2308985.

- Discussion

TP to TR 38.786

7	Receiver characteristics for NR SL evolution

7.1	Rx requirements for NR SL single carrier operation in unlicensed bands

7.1.1	Reference sensitivity power level for NR SL-U

Background information for NR V2X reference sensitivity requirement has been captured in the TR 38.886 as captured below and can be reused for NR SL-U

The reference sensitivity power level REFSENS is the minimum mean power applied to the UE antenna connector at which the throughput shall meet or exceed 95% of the maximum throughput of the reference measurement channels.

The V2X UE REFSENS is defined by the following equation:

REFSENSV2X=kTB + SNRV2X +10log10(LCRB*SCS*12/RX_BW) +( NFV2X+ IM) – Diversity gain

Where

-	kTB: Thermal noise level is [-174dBm(kT) + 10*log10(RX BW)]dBm.

-	NF: Noise figure. 13 dB is used for LAA and can be reused for NR V2X requirements. Assumed NF is 9dB < 3GHz, NF is 10dB>= 3GHz (e.g B42, n77, n78, n79…) at licensed bands at FR1.

-	IM: 2.5 dB is assumed. When the number of RB size equal to or less than 24RBs, 0.5dB additional relaxation is allowed.

-	Target SNR: -0.5 dB

-	Diversity gain: 3dB

For NR-U, the following equations in table below are used to calculate the REFSENS requirement and it can be further re-used in NR SL-U in bands n46, n96 and n102, while taking account that for NR-U target SNR of -1dB was used and therefore the REFSENS values in table below are 0.5dB higher than with NR SL-U target SNR of 0.5dB.

Table 7.1.1-1 REFSENS for NR-U

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm) | Duplex Mode |
| n46 | 15 | 20, 40 | -89.7 + 10log10(NRB/106) | HD |
|  | 30 | 20, 40, 60, 80, 100 | -89.9 + 10log10(NRB/51) |  |
|  | 60 | 20, 40, 60, 80, 100 | -90.1 + 10log10(NRB/24) |  |
| n96, n102 | 15 | 20, 40 | -89.2 + 10log10(NRB/106) | HD |
|  | 30 | 20, 40, 60, 80, 100 | -89.4 + 10log10(NRB/51) |  |
|  | 60 | 20, 40, 60, 80, 100 | -89.6 + 10log10(NRB/24) |  |
| NOTE 1: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the RB configuration for SL-U REFSENS as shown in table 7.1.1-1. |  |  |  |  |

For the NR-U REFESENS requirement, the RB configuration for larger than 20MHz is considered as wideband operation and hence all intra-cell guard bands between sub-bands should be filled. In this case, the full RB allocation of NR is used to define the reference sensitivity requirement. However, for V2X, specific RB allocation using the sub-channels based on the RAN2 specification TS 38.331 is used as captured below:

Following the TS 38.331 definitions Table 7.1.1-2 shows the RB configuration for NR SL-U reference sensitivity requirement.

Table 7.1.1-2 NR SL-U RB configurations for REFSENS

|  |  | NR band / SCS / Channel bandwidth / Duplex mode |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NR SL-U Band | SCS kHz | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz | Duplex Mode |
| n46 | 15 | 105 | 216 |  |  |  | HD |
|  | 30 | 50 | 105 | 160 | 216 | 270 |  |
|  | 60 | 24 | 50 | 75 | 105 | [135] |  |
| n96 | 15 | 105 | 216 |  |  |  | HD |
|  | 30 | 50 | 105 | 160 | 216 | 270 |  |
|  | 60 | 24 | 50 | 75 | 105 | [135] |  |
| n102 | 15 | 105 | 216 |  |  |  | HD |
|  | 30 | 50 | 105 | 160 | 216 | 270 |  |
|  | 60 | 24 | 50 | 75 | 105 | [135] |  |

With the agreed SNR and RB configuration as above, the REFNSENS requirement for NR SL-U is  shown in Table 7.1.1-3.

Table 7.1.1-3 REFSENS for SL-U

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm) | Duplex Mode |
| n46 | 15 | 20, 40 | [-89.2 + 10log10(NRB/105)] | HD |
|  | 30 | 20, 40, 60, 80, 100 | [-89.4 + 10log10(NRB/50)] |  |
|  | 60 | 20, 40, 60, 80, 100 | [-89.6 + 10log10(NRB/24)] |  |
| n96, n102 | 15 | 20, 40 | [-88.7 + 10log10(NRB/105)] | HD |
|  | 30 | 20, 40, 60, 80, 100 | [-88.9 + 10log10(NRB/50)] |  |
|  | 60 | 20, 40, 60, 80, 100 | [-89.1 + 10log10(NRB/24)] |  |
| NOTE 1: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the RB configuration for NR SL-U REFSENS as shown in table 7.1.1-2 |  |  |  |  |

7.1.3	Adjacent Channel Selectivity for NR SL-U

For the ACS requirement, the ACS for NR-U was defined for bands above 3.3GHz considering the unlicensed bands of n46, n96, n102 are all high bands. However, the requirement format is different from NR single CC which can be found below, i.e. the ACS is adjusted according to the CBW while the NR single CC is fixed as 33dB for bands above 3.3GHz. The ACS requirement has been extensively discussed in NR-U WI with various views considering the difference of ACLR, ACIR impacts, and comparison with WIFI system, and coexistence with LAA, and the final value is a compromised outcome.

ACS Requirement for NR-U

Furthermore, for NR normal requirement, there are two test cases for ACS as case 1 and case 2. During the NR-U discussion, the case 2 was considered not necessary for operation in unlicensed bands.

It has been agreed to reuse NR-U ACS requirement for NR SL-U using the NR SL-U RMC, as discussed in general part.

7.1.4	Blocking characteristics for NR SL-U

The in band blocking requirement for NR V2X is captured in clause 7.6E.2.1 and for NR-U it is captured in clause 7.F.2.1. The transmission power is the same for 20 and 40MHz and the difference is the bandwidth of interferer. The 10MHz as smallest CBW for NR V2X is used as BWinterferer and corresponding offset frequency is decided. In NR SL-U, 10MHz is not supported and 20MHz is supported as the smallest CBW for single CC.

In this case, the NR-U IBB can be reused for NR SL-U.

The NR sidelink OBB requirement is reused from LTE sidelink and hence different power level for the transmission power based on different bandwidth. For NR-U OBB requirement, it is based on the NR normal requirement since there is no Wi-Fi corresponding requirement. Further modification has applied for range 3 considering the shallower frequency response of the band filter at high frequencies. In this case, the NR-U requirement can be reused for NR SL-U.

7.1.5	Spurious response for NR SL-U

The NR sidelink spurious response requirement is reused from LTE sidelink and hence different power level for the transmission power based on different bandwidth is used.For NR SL-U operation in unlicensed bands, the  NR-U requirement can be reused.

7.1.6	Intermodulation characteristics for NR SL-U

The NR sidelink Intermodulation requirement is reused from LTE sidelink and hence different power level for the transmission power based on different bandwidth is used.For NR SL-U operation in unlicensed bands,  the  NR-U requirement can be reused.