3GPP TSG-RAN WG4 Meeting # 107																	R4-2310322

Incheon, KR, May 22nd – May 26th, 2023

Agenda item:			7.1

Source:	Moderator (Skyworks Solutions, Inc.)

Title:	Ad hoc minutes for NR_Baskets_Part_1

Document for:	Information

# Introduction

Briefly introduce background, the scope of this email discussion (e.g. list of treated agenda items) and provide some guidelines for email discussion if necessary.

Topic 1: Combinations with Triple beat case

Topic 2: LBLB 1UL combinations

Topic 3: NR-U contiguous ULCA A-MPR

Topic 4: BCS4/5 additions

Topic 5: R18 Corrections to band combinations configurations and MSDs

# Topic #1: Combinations with Triple beat case

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2309088 Triple-beat for DC_3A-8B_n78A | Apple | Observation 1: If the TB product is resulted from the frequency sum of the 2 discrete RBs in the contiguous UL CA mixing with the other band UL carrier, the TB product would be no different from the IMD3 mixed by only 2 UL carriers for the same UL band combination. Proposal 1: If TB is resulted from the frequency sum of the 2 discrete RBs in the contiguous UL CA, there is no need to specify the TB test configuration as the requirement can already be verified by the fallback 2UL IMD3. Observation 2: If the TB test configuration with 2 discrete RBs in intra-band contiguous UL CA is within an FDD band, the IMDs generated by the 2 RBs could result in more severe REFSENS impact to its own DL band than the TB interference to the 3rd DL band. Proposal 2: The TB test configuration should ensure the FDD band self-interference would not interrupt the 3rd DL band MSD verification, if REFSENS or MSD for all DL bands would not be tested simultaneously. |
| R4-2309741 Triple Beat MSD for DC_3A-8B_n78A | Skyworks Solutions Inc. | Proposal: Interested companies are invited to study the band 3 MSD for the test point captured in Table 1. Table 1: Band 3 TB MSD test point proposal for DC_3A-8B_n78A ENDC band combination; NR/LTE band; UL Fc (MHz); UL/DL BW (MHz); UL LCRB; DL Fc (MHz); MSD (dB); Duplex mode; IMD order. 3; N/A; 5; N/A; 1845; [FFS]; FDD; IMD3. DC_3A-8B_n78A; 8; 900; 10; 1 (RBSTART=20); 945; N/A; FDD; N/A. 910; 10; 1 (RBSTART=45); 955; N/A; FDD; N/A. n78; 3660; 10; 25 (RBSTART=0); 3660; N/A; TDD; N/A. |
| R4-2309450 TP for TR 37.718-21-11: updated MSD for DC_3A_8B_n78A with UL DC_8B_n78A | CHTTL | TP to capture input on Triple Beat MSD in band 3 for DC_3A_8B_n78A with UL DC_8B_n78A |
| R4-2309097 MSD analysis for CA_n5-n77(2A) due to CA_n77(2A) 2UL IMD4 | Apple | Moderator: moved from block approval after initial summary Proposal: Table 3-1 shows our proposed value for the n5 MSD for CA_n5-n77 (2A) combination. Band / Channel bandwidth / NRB / Duplex mode; Source of IMD. NR CA band combination; NR band; UL Fc ( MHz ); UL/DL BW ( MHz ); UL CLBR; UL Fc ( MHz ); MSD ( dB ); Duplex mode. CA_n5-77 (2A); n5; N/A; 5; NA; 880; 6.81; FDD; IMD4. n77; 3410; 10; 1 RBSTART =25; 3410; N/A; TDD; N/A. 3850; 10; 1 RBSTART =25; 3850. Note 1: Based on the n77 UL dual-PA architecture |

## Open issues summary

### Sub-topic 1-1

Sub-topic description: Discussion on triple beat cases that are equivalent to an IMD case and Triple beat to third band verification

Issue 1-1: Triple Beat vs IMD3 and verification

- Proposals

  - Proposal 1: If TB is resulted from the frequency sum of the 2 discrete RBs in the contiguous UL CA, there is no need to specify the TB test configuration as the requirement can already be verified by the fallback 2UL IMD3.

  - Proposal 2: The TB test configuration should ensure the FDD band self-interference would not interrupt the 3rd DL band MSD verification, if REFSENS or MSD for all DL bands would not be tested simultaneously.

- Recommended WF

  - Proposals 1 can simplify triple beat MSD assessment and specification

  - Proposal 2 may need input to RAN5

  - Discuss proposals on-line and a WF may be needed to capture agreements

- AdHoc discussion

  - Apple: TB case is the same interference issue as 2UL IMD3, simplify this TB case with testing only IMD3.

  - CHTTL: this is more general issue. WF TB3. TB test point is in agreement with WF.

  - Huawei: in the 1RB+1RB case is not very efficient. Other allocation may be more useful.

  - Skyworks: WF in R4#105 said that TB should be anlyzed in a TP or discussion paper. Question is if two test are needed.

  - Apple: TB makes more sense is FDD band + large difference is different

  - Moderator: discussion is only for the TB case where the resulting interference is the same that 2UL IMD3.

  - CHTLL: is proposal 2 assuming that TB is tested?

  - Skyworks: other desense happen with band 8.

  - CHTTL: revised TP needed

- Recommendation:

  - revised TP for R4-2309450 to capture proposed MSD value by Skyworks of 4.1dB in Rev_R4-230941

    - Revision is R4-2310298

  - Recommend a WF on TB test cases versus 2UL IMD3 to Apple, R4-2309088 is noted

    - R4-2310316	WF on TB test cases versus 2UL IMD3	Source: Apple

  - R4-2309741 is revised to capture the MSD value proposed, the revision can then be noted

    - Revision number missing

### Sub-topic 1-2

Sub-topic description: Triple Beat MSD for DC_3A_8B_n78A with UL DC_8B_n78A

Open issues and candidate options before meeting:

Issue 1-2: Triple Beat MSD for DC_3A_8B_n78A with UL DC_8B_n78A

- Proposals

  - R4-2309088 there is no need to specify the TB test configuration as the requirement can already be verified by the fallback 2UL IMD3

  - R4-2309741Further study Triple beat MSD

    - Moderator: an MSD value was provided in a late contribution at: 4.1dB see Rev_ R4-2309741. If companies agree this MSD value may be considered.

- Recommended WF

  - Based on outcome in Issue 1-1, capture it in revision of R4-2309450

- AdHoc discussion

  - See Recommendation in Issue 1-1

### Sub-topic 1-3

Sub-topic description: CA_n5-n77(2A) due to CA_n77(2A) 2CC IMD4

Open issues and candidate options before meeting:

Issue 1-3: CA_n5-n77(2A) due to CA_n77(2A) 2CC IMD4

- AdHoc discussion

  - Apple: proposal for dual PA architecture. Current specification considers

  - Skywork: current value is 8.6

  - Huawei: one specification to cover all implementations.

  - Apple: according to our evaluation single PA would be >20dB

  - Qualcomm: captured test point in the spec has issue

  - Skyworks: same issue in -3

- Recommendation: current value is kept, given the frequency separation, 2PA is assumed. R4-2309097 is noted

  - Observation: A number of test point capture will need a fix next meeting due to row missing or row shifted

# Topic #2: LBLB 1UL combinations

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2309290 DraftCR for TS 38.101-1 to introduce CA_n12-n71 Requirements | Skyworks Solutions Inc. | To specify the following band combination specific requirements. 1) Cross band MSD of n12 UL in n71 DL 2) Cross band MSD of n71 UL in n12 DL These changes are based on agreements from way forward R4-2306479 WF on CA_n71-n85 and related band 12 and 71 combinations, T Mobile USA, Apple, Murata, Nokia, Skyworks |
| R4-2309298 Input on CA_n5-n71 cross-band MSD and DeltaT/R | Skyworks Solutions Inc. | Proposal on architecture: CA_n5-n71 requirement is based on a two-antenna implementation with: - Band n71 and n105 co-banding - n5DL+n105UL+n105DL triplexer on antenna 1 - n105DL+n5UL+n5DL triplexer on antenna 2 - Support of multiple LBLB combinations. Proposal on Delta T and Delta R: the values in Table 1 and 2 are used for CA_n5-n71. Table 1: ΔTIB,c due to NR CA (two bands) Inter-band CA combination; ΔTIB,c for NR bands (dB)9. Component band in order of bands in configuration10. CA_n5-n71; 0.5; 0.5. NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3.. Table 2: ΔRIB,c due to NR CA (two bands) Inter-band CA combination; ΔRIB,c for NR bands (dB)8. Component band in order of bands in configuration9. CA_n5-n71; 0.5; 0.3. NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77.. Proposal for CA_n5-n71 1UL cross-band MSD: see Table 4 below. Table 4: CA_n5-n71 1UL cross-band MSD UL band; DL band; UL Fc; UL BW; SCS of UL band; UL RB Allocation; DL Fc; DL BW; MSD; Cross-band Interference source. (MHz); (MHz); (kHz); LCRB; (MHz); (MHz); (dB). n71; n5; 688; 20; 15; 20 (RBstart=86); 871.5; 5; 1.3; >ACLR2. n5; n71; 834; 20; 15; 20 (RBstart=0); 649.5; 5; 1.1; >ACLR2. |
| R4-2309533 TP for 38.718-02-01 on MSD value due to cross band isolation for CA_n5A-n71A combination | Ericsson, Rogers | Table 5.39.1.4-1: ΔTIB,c Inter-band CA combination; ΔTIB,c for NR bands (dB)9. Component band in order of bands in configuration10. CA_n5-n71; 0.5; 0.5. NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3.. Table 5.39.1.4-2: ΔRIB,c Inter-band CA combination; ΔRIB,c for NR bands (dB)8. Component band in order of bands in configuration9. CA_n5-n71; -; -. NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77.. based on values for CA_n3-n7. Table 5.39.1.5-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC3 aggressor NR UL band for NR CA FR1 UL band; DL band; UL Fc; UL BW; SCS of UL band; UL RB Allocation; DL Fc; DL BW; MSD; Cross-band Interference source. (MHz); (MHz); (kHz); LCRB; (MHz); (MHz); (dB). n71; n5; 688; 20; 15; 20 (RBstart=86); 871.5; 5; 0.6; >ACLR2. n5; n71; 834; 20; 15; 20 (RBstart=0); 649.5; 5; 0.6; >ACLR2. |
| R4-2309363 New test point for CA_n71-n85 cross band MSD | Skyworks Solutions Inc. | Proposal on baseline CA_n71-n85 MSD test points in Table 1: - This applies to UEs not supporting the R18 newly introduced symmetric UL/DL CBW >20MHz. Table 1: MSD test point for R18 mandatory support UL band; DL band; UL Fc; UL BW; SCS of UL band; UL RB Allocation; DL Fc; DL BW; MSD; Cross-band Interference source. (MHz); (MHz); (kHz); LCRB; (MHz); (MHz); (dB). n71; n85; 688; 20; 15; 20 (RBstart=86); 730.5; 5; 8.2X; ACLR2. n85; n71; 705.5; 15; 15; 20 (Rbstart=59); 649.5; 5; 3.8; >ACLR2. NOTE X: applicable to UE not supporting n71 symmetrical UL/DL channel bandwidths above 20MHz. Proposal on additional largest band n71 UL CBW MSD test point in Table 2 and Table 3 alternatives: - This applies to UEs supporting any of the R18 newly introduced symmetric UL/DL CBW >20MHz - Assuming that a 35MHz symmetrical UL/DL for n71 is approved in RAN#100, the following test point in Table 2 is evaluated for the RAN4#108 meeting. Table 2: MSD test point for R18 optional support UL band; DL band; UL Fc; UL BW; SCS of UL band; UL RB Allocation; DL Fc; DL BW; MSD; Cross-band Interference source. (MHz); (MHz); (kHz); LCRB; (MHz); (MHz); (dB). n71; n85; 680.5; 35; 15; 20 (Rbstart=168); 730.5; 5; TBDX; ACLR1. NOTE X: applicable to UE supporting n71 symmetrical UL/DL channel bandwidths above 20MHz. - If 35MHz symmetrical UL/DL for n71 is not approved in RAN#100, then the following test point in Table 3 is evaluated for theRAN4#108 meeting. Table 3: MSD test point for R18 optional support UL band; DL band; UL Fc; UL BW; SCS of UL band; UL RB Allocation; DL Fc; DL BW; MSD; Cross-band Interference source. (MHz); (MHz); (kHz); LCRB; (MHz); (MHz); (dB). n71; n85; 683; 30; 15; 20 (Rbstart=140); 730.5; 5; TBDX; ACLR2. NOTE X: applicable to UE supporting n71 symmetrical UL/DL channel bandwidths above 20MHz. - The proper alternative will be captured in a CR after RAN#100 (earliest is RAN4#108 assuming that the MSD value is agreed). |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 2-1

Sub-topic description: CA_n12-n71 requirement

Issue 2-1: CA_n12-n71 requirement

- Proposals

  - Cross band MSD of n12 UL in n71 DL

  - Cross band MSD of n71 UL in n12 DL

- Recommended WF

  - Based on agreed WF, Endorse R4-2309290

- AdHoc discussion

  - Recommendation: No comment,  R4-2309290 CR should be agreeable and captured in BigCR on 2 band NRCA

### Sub-topic 2-2

Sub-topic description CA_n5-n71 Delta R

Issue 2-2: CA_n5-n71 Delta R

- Proposals

  - Value already endorsed in R4-2309533

  - Corrected Delta R values from R4-2309298

- Recommended WF

  - Discuss on-line and capture agreements in revision R4-2309533 if needed

- AdHoc discussion

  - T-Mobile: is this BCS5?

  - Moderator: no this is a new band combination wo BCS4/5

- Recommendation: revision of R4-2309533 to capture new delta R from R4-2309298 which can be noted

  - Revision is R4-2310317

### Sub-topic 2-3

Sub-topic description: CA_n5-n71 cross-band MSD

Issue 2-3: CA_n5-n71 cross-band MSD

- Proposals

  - 1.3/1.1dB MSD for n5/n71 in R4-2309533 based on evaluation

  - 0.6/0.6dB MSD for n5/n71 in R4-2309298 based on CA_n3-n7

- Recommended WF

  - Discuss on-line and capture agreements in revision R4-2309533 if needed

- AdHoc discussion

  - Qualcomm: we have values to propose: MSD 2.6/5.6dB for n5/n71.

  - Huawei: 0.6/0.6dB is based on two mid bands, how can it be reused for LBLB

  - Ericsson: OK with option 1

- Recommendation: use option 1 numbers (1.3/1.1dB MSD for n5/n71) in brackets in revision of R4-2309533  as starting point to discussion to accomodate Qualcomm’s numbers. After offline discussion, averaging is agreed and draft is circulated

### Sub-topic 2-4

Sub-topic description CA_n71-n85 MSD test points

Issue 2-4: CA_n71-n85 MSD test points

- Proposals

  - Cross band MSD for baseline (UL CBW up to 20MHz). values already agreed in WF. Only note added

  - Cross band MSD for new maximum UL CBW in n71. 30MHz in RAN99 but 35MHz requested in RAN#100

- Recommended WF

  - Discuss on-line and agree on handling of CA_n71-n85 handling of requirement for mandatory vs optional UL CBW support in R18.

- AdHoc discussion

  - Recommendation: WF on MSD test point for n71-n85 depending on max BW and legacy/new BW support to T-Mobile USA, Skyworks based on Skyworks input. R4-2309363 can be noted

    - R4-2310318	WF on MSD test point for n71-n85 depending on max BW and legacy/new BW support	Source: T-Mobile USA (add Skyworks as co-source)

# Topic #3: NR-U contiguous ULCA A-MPR

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2309723 Discussion of simulation results on UE RF NR-U UL CA A-MPR for PC5 | Charter Communications, Inc | Discussion paper for simulation calibration purpose Table 1: Initial simulation results for UL CA A-MPR PC5 for Band n96B with NS_54 for calibration purpose. Pre-coding; Modulation; RB Allocation. Full2 (dB); Partial3 (dB); Exception for Full5 (dB). DFT-s-ODFM; Pi/2 BPSK4; ≤ [4.0]. QPSK; ≤ [4.0]. 16 QAM; ≤ [4.5]. 64 QAM; ≤ [5.0]. 256 QAM; ≤ [6.0]. CP-OFDM; QPSK; ≤ [5.0]. 16 QAM; ≤ [5.0]. 64 QAM; ≤ [6.0]. 256 QAM; ≤ [7.0]. NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. The MPR applies to interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214 [10]. NOTE 2: Full RB allocation MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and sub-bands are transmitted according to configuration A in Table 6.2F.2A.2-2. NOTE 3: Partial RB allocation MPR applies when one or more RB’s in one or more sub-bands are not allocated and sub-bands are transmitted according to configuration A in Table 6.2F.2A.2-2. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. NOTE 5: Exception for Full RB allocation MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and sub-bands are transmitted according to configuration [B] in Table 6.2F.2A.2-2.. |

## Open issues summary

### Sub-topic 3-1

Sub-topic description: Discussion paper for simulation calibration purpose providing backoff per waveforms for NS_54 A-MPR

Issue 3-1: NS_54 A-MPR

- Proposals

  - Simulation results for calibration purpose

- Recommended WF

  - Data to be considered by other contributing companies

  - Note R4-2309723

- AdHoc discussion

  - Charter: OK with recommended WF.

  - Recommendation: Note R4-2309723 and ask chairman to add following text in report:

    - “Back-off values provided for the set of waveforms in this document can be used as reference for future contributions for calibration purpose.

    - Companies are encouraged to provide back off values for NS54 contiguous ULCA in next meeting”

# Topic #4: BCS4/5 additions

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2308562 Addition of FR1 intra-band BCS 4 and 5 | Huawei, HiSilicon, CATT | Added new FR1 CA_n48(2A) intra-band BCS 4 and 5 as per the approved WID RP-230067. Some existing FR1 intra-band BCS other than BCS 4 and 5 have no corresponding fallback BCS, significant signalling overhead will be introduced. |
| R4-2308563 Addition of FR1 inter-band BCS 4 and 5 | Huawei, HiSilicon, CATT | Added new FR1 inter-band BCS 4 and 5 as per the approved WID RP-230133. |
| R4-2308560 Updated TP for TR 38.718-02-01 to introduce BCS4 and BCS5 for CA_n5-n77 | Huawei, HiSilicon, CATT | Moderator: moved from block approval after initial summary There’s nothing new to adding n77(2A) to these requirements for CA_n5A-n77(3A) with UL n77(2A). The MSD test points due to harmonic mixing for band n5 25MHz are introduced as below. Table 5.14.1.5-1: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1 UL band; DL band; UL BW; SCS of UL band; UL RB Allocation; DL BW; MSD; UL/DL fc condition; UL/DL harmonic order. (MHz); (kHz); LCRB; (MHz); (dB). n77; n5; 25; 15; 130 (RBstart=0); 25; 1.7; NOTE 8; UL1/DL4. NOTE 8: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band.. Table 5.14.1.5-2: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC2 aggressor NR UL band for DL NR CA FR1 UL band; DL band; UL BW; SCS of UL band; UL RB Allocation; DL BW; MSD; UL/DL fc condition; UL/DL harmonic order. (MHz); (kHz); LCRB; (MHz); (dB). n77; n5; 25; 15; 130 (RBstart=0); 25; 3.3; NOTE 8; UL1/DL4. NOTE 5: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band.. Table 5.14.1.5-3: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC1.5 aggressor NR UL band for DL NR CA FR1 UL band; DL band; UL BW; SCS of UL band; UL RB Allocation; DL BW; MSD; UL/DL fc condition; UL/DL harmonic order. (MHz); (kHz); LCRB; (MHz); (dB). n77; n5; 25; 15; 130 (RBstart=0); 25; 4.1; NOTE 8; UL1/DL4. NOTE 8: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band.. |

## Open issues summary

### Sub-topic 4-1

Sub-topic description: FR1 intra-band BCS 4 and 5

Issue 4-1: CA_n48(2A) intra-band BCS 4 and 5

- Proposals

  - Adding CA_n48(2A) 1UL BCS4/5

- Recommended WF

  - Approve R4-2308562

  - (moderator note: missing “-“ in UL configuration, was missing before the CR)

- AdHoc discussion

  - ATT: may need revision for missing fallbacks of inter-band case.

- Recommendation: revision of R4-2308562 needed to fix the typo

  - Revised to R4-2310319

### Sub-topic 4-2

Sub-topic description FR1 inter-band BCS 4 and 5

Issue 4-2: BCS4/5 inter-band additions

- Proposals

  - Option 1: TBA

  - Option 2: TBA

- Recommended WF

  - Approve R4-2308563

- Moderator note: when introducing BCS4/5 there may be new MSD cases to be checked due to new smallest or largest CBW or aggregated BW, it would be useful that some text in cover page provides evidence of the checks

  - CA_n2-n77/78 introduces a new largest UL CBW for n2, cross band issue needs to be checked: check OK

  - CA_n8-n78 introduces a new largest UL CBW for n8, cross band issue needs to be checked: check OK

  - CA_n71-n78 introduces a new largest UL CBW for n71, cross band issue needs to be checked: check OK

- AdHoc discussion

  - ATT: number of cases n77C/n78C/n78(2A) configurations not in intra band. n2-n77 has new max DL CBW for harmonic mixing.

  - Nokia: CA_n77C BCS4/5 agreed in 106b.

  - Huawei: CA_n78C and n78(2A) BCS4/5 endorsed in bis meeting

  - moderator: Revison needed to point at 106b agreement for intra CA cases.

  - ATT: CA_n2-n77 BCS4/5 harmonic mixing for max DL CBW is needed and introduction postponed. WF needed. ATT: CA_n2-n77 not complete before we agree new harmonic mixing MSD.

    - HW: second test point not mandatory.

    - Verizon: further study if MSD is worse.

    - Skyworks: second test point was related to specific deployment

    - Verizon: FFS if second test point is needed if worse.

- Nokia: WF needed on handling BCS4/5 introducing new max/min CBW.

- Skyworks: already captured in an old WF.

- Recommendation:

- Revision of R4-2308563 TP to point at the intra-band fall back configurations introduced in R4#106b to clarify that al fallbacks are specified

  - Revised to R4-2310320

- WF to Skyworks  covering:

  - Guidelines on checks/specification updates required when BCS4/5 introduces a new smallest or largest CBW

  - CA_n2-n77 and CA_n5-n77 harmonic mixing update for new largest DL CBW needed or not

    - R4-2308560 is return to depending on WF agreement on updating CA_n5-n77 harmonic mixing update for new largest DL CBW

  - R4-2310321	WF on guidelines on checks/specification updates required when BCS4/5 introduces a new smallest or largest CBW	Source: Skyworks

### Sub-topic 4-3

Sub-topic description BCS4/5 for CA_n5-n77

Issue 4-3: BCS4/5 for CA_n5-n77

- AdHoc discussion

  - See recommendation for Issue 4-2

# Topic #5: R18 Corrections to band combinations configurations and MSDs

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2308564 Draft CR for TS 38.101-3 to align the MSD test configurations due to harmonic and harmonic mixing for ENDC | Huawei, HiSilicon | Based on the agreements in WF R4-2306583, the MSD table format should be further evolved. 1) MSD table format for harmonic interference exceptions in clause 7.3B.2.3.1 is updated. 2) MSD table format for harmonic mixing interference exceptions in clause 7.3B.2.3.2 is updated. |
| R4-2309081 CR for 38.101-1 18.1.0 Bug fixes for CA/DC tables | Apple | Correcting some notation bugs. For four UL combinations the marking “n” is missing and has been added |
| R4-2309082 CR for 38.101-3 18.1.0 Bug fixes and fallback additions for CA/DC tables | Apple | 1. Correcting some notation bugs 2. Adding missing Fallback combinations Many combinations have been added in the Release 18.1.0 spec with R4-2301500, however, there have many bugs introduced by using a wrong notation for the added combinations and also many fallbacks are missing. Some other minor bugs are corrected as well. |

## Open issues summary

### Sub-topic 5-1

Sub-topic description: MSD test configurations due to harmonic and harmonic mixing for ENDC

Issue 5-1: MSD table format for harmonic cases

- Proposals

  - Update UL harmonic and harmonic mixing MSD tables to the agreed new format

- Recommended WF

  - Endorse R4-2308564

- AdHoc discussion

  - Xxxx

  - Similar: proposal for cross band in draft document from Skyworks may be discussed if time allows

  - Qualcomm: coordination is needed for implementation

  - No comment in the room,

- Recommendation: unless there is on-line comment in Main session, R4-2308564 can be endorsed

### Sub-topic 5-2

Sub-topic description: 38.101-1 Bug fixes for CA/DC tables

Issue 5-2: Issue with band combinations notation

- Proposals

  - Simple fix of “n” missing for some bands

- Recommended WF

  - Approve R4-2309081

- AdHoc discussion

  - No comment in the room

- Recommendation: unless there is on-line comment in Main session, R4-2309081 can be endorsed

### Sub-topic 5-3

Sub-topic description 38.101-3 Bug fixes and fallback additions for CA/DC tables

Issue 5-3: Issue with band combinations notation and fallbacks

- Proposals

  - Fixing bugs and fallbacks

- Recommended WF

  - Approve R4-2309082

- AdHoc discussion

  - No comment in the room

- Recommendation: unless there is on-line comment in Main session, R4-2309082 can be endorsed