3GPP TSG-RAN WG4 Meeting # 107	R4-2310326

Incheon Meeting, May 22nd – 27th, 2023

Title:	TP to TR 37.718-21-11: Addition of DC_2-4_n78

Source:	Nokia, AMX

Agenda item:	7.4.2

Document for:	Approval

# 1	Introduction

This is a TP to TR 37.718-21-11 to add DC_2-4_n78.

************************************* Start of TP*****************************************

## 5.x	DC_2-4_n78

### 5.x.1	Operating bands for EN-DC

Table 5.x.1-1: EN-DC Band combinations (three bands)

| EN-DC band | E-UTRA CA band | NR band |
| --- | --- | --- |
| DC_2-4_n78 | CA_2-4 | n78 |

### 5.x.2	Configuration for DC

Table 5.x.2-1:  Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-4A_n78A | DC_2A_n78A DC_4A_n78A | CA_2A-4A | n78A |

### 5.x.3			Co-existence studies

### This is a 3-band combination, so uplink harmonic and harmonic mixing analysis is already done in the fallbacks. Only IMD for two uplink configurations is analysed.

### A study of the uplink combination affecting the downlink of the third band is shown in Table 5.x.3-1 for DC_2A_n78A into band 4 downlink.

Table 5.x.3-1: IMD analysis for DC_2A_n78A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 1850 | 1910 | 3300 | 3800 |
| DL Frequency [MHz] | 1930 | 1990 | 3300 | 3800 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1390 | 1950 | 5150 | 5710 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 100 | 520 | 4690 | 5750 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7000 | 7620 | 8450 | 9510 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1750 | 2430 | 7990 | 9550 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3900 | 2780 | 10300 | 11420 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8850 | 9530 | 11750 | 13310 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13350 | 11290 | 4340 | 3600 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 7700 | 6080 | 870 | 2050 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15050 | 17110 | 10700 | 11440 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13600 | 15220 | 12150 | 13330 |

The analysis shows the 4th order IMD product falls inside band 4 downlink

### A study of the uplink combination affecting the downlink of the third band is shown in Table 5.x.3-2 for DC_4A_n78A into band 2 downlink.

Table 5.x.3-2: IMD analysis for DC_4A_n78A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 1710 | 1755 | 3300 | 3800 |
| DL Frequency [MHz] | 2110 | 2155 | 3300 | 3800 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1545 | 2090 | 5010 | 5555 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 380 | 210 | 4845 | 5890 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6720 | 7310 | 8310 | 9355 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1330 | 1965 | 8145 | 9690 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 4180 | 3090 | 10020 | 11110 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8430 | 9065 | 11610 | 13155 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13490 | 11445 | 3720 | 3040 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 7980 | 6390 | 1335 | 2470 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14910 | 16955 | 10140 | 10820 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13320 | 14910 | 11730 | 12865 |

The analysis shows the 2nd, 4th and 5th order IMD product falls inside band 2 downlink.

Based on Table 5.x.3-1 and Table 5.x.3-2:

- The 4th order IMD product of DC_2_n78 falls inside band 4 downlink

- The 2nd, 4th and 5th order IMD product of DC_4_n78 falls inside band 2 downlink

### 5.x.4		∆TIB and ∆RIB values

For DC_2-4_n78, the TIB,c and RIB,c values are reused from DC_2-66_n78 and are given in the tables below.

Table 5.x.4-1:ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_2-4_n78 | 0.6 | 0.6 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.x.4-2:ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_2-4_n78 | 0.3 | 0.3 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

### 5.x.5	REFSENS requirements

MSD values are reused from DC_2A-66A_n78A, but band 4 frequencies requires new test points.

Table 5.x.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-4A_n78A | 2 | 1875 | 5 | 25 | 1955 | N/A | N/A |
|  | 4 | 1745 | 5 | 25 | 2145 | 10.3 | IMD4 |
|  | n78 | 3480 | 10 | 50 | 3480 | N/A | N/A |
|  | 2 | 1880 | 5 | 25 | 1960 | 32.1 | IMD2 |
|  | 4 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | n78 | 3700 | 10 | 50 | 3700 | N/A | N/A |
|  | 2 | 1860 | 5 | 25 | 1940 | 9.1 | IMD4 |
|  | 4 | 1750 | 5 | 25 | 2150 | N/A | N/A |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | N/A |
|  | 2 | 1870 | 5 | 25 | 1950 | 2.1 | IMD5 |
|  | 4 | 1750 | 5 | 25 | 2150 | N/A | N/A |
|  | n78 | 3600 | 10 | 50 | 3600 | N/A | N/A |

************************************* End of TP******************************************